3GPP TSG-RAN WG4 Meeting # 99-e	R4-2108357

# Electronic Meeting, May 19-27, 2021

Title:	LS Reply on Timing Assumption for Inter-Cell DL Measurement

Work Item:	NR_feMIMO-Core

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

Name:	Xutao Zhou

E-mail Address:	xutao.zhou@samsung.com

Attachments:

- Overall Description:

RAN4 thanks RAN1 for their enquiry on the topic of Timing Assumption for Inter-Cell DL Measurement and would like to provide the below responses to the questions asked.

Question 1: In the context of L1/L2-centric inter-cell mobility, what are the implication(s)/benefit(s), if any, (from RAN4 perspective) of limiting the measurement for L1-RSRP on measurement RS of a non-serving cell within the SMTC window(s) (only applicable to SSB) for the above agreed L1-RSRP reporting? Conversely, what are the implication(s)/benefit(s), if any, of not limiting the measurement for L1-RSRP to be carried out within the SMTC window(s)?

[RAN4] RAN4 has discussed the implication(s)/benefit(s) on limiting L1-RSRP measurement within and outside SMTC. RAN4 cannot conclude the suggestion to RAN1 on whether to limit L1-RSRP measurement within SMTC or not. The below implication(s)/benefit(s) have been identified

- If measurement for L1-RSRP is limited within SMTC

- In FR1, no impact on limiting L1-RSRP measurement within SMTC window. In FR2, resource sharing factor (e.g. Psharing factor=3 as defined for the L1-RSRP measurements of serving cell in section 9.5.4 in TS38.133) shall be considered

- An SMTC periodicity larger than SSB periodicity would lead to longer L1-RSRP measurement period.

- If the non-serving cell resources would require a measurement gap to be measured, the measured resources would need to be confined within the measurement gap.

- If measurement for L1-RSRP is not limited within SMTC

- Impact on UE measurement burden is expected. For FR2, scheduling restriction may be expected due to non-collocation cell measurement.

- Network may have full flexibility in configuration of the RS periodicity and offset

Question 2: In the context of L1/L2-centric inter-cell mobility, for measurement on measurement RS of a non-serving cell, with the above agreed L1-RSRP reporting, if the receive timing of the measurement RS from the non-serving cell can be different from the receive timing of the signals from the serving cell, what are the implication(s)/benefit(s), if any, from RAN4 perspective?

[RAN4] Based on the following understandings, RAN4 concluded different receive timing of the measurement RS from the non-serving cell and the receive timing of the signals from the serving cell can be supported.

- In FR1 TDD and FR2 the intra-frequency neighbor cell is assumed to be synchronous and the timing difference between serving cell and neighbor cell can be limited within a certain value which depends on cell phase synchronization error and propagation delay difference. For FR1 FDD the intra-frequency cell could be asynchronous and have different receive timing compared to serving cell.

- Performance degradation or increased UE complexity can be expected if timing difference exceeds the minimum requirements of UE (e.g., 1 CP)

2. To RAN WG1 group.

ACTION: RAN4 requests RAN1 to take the above responses into consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #100-e                  August 16 – 27, 2021         	 	 Online