3GPP TSG-RAN WG4 Meeting # 99-e 													R4-2108438

Electronic Meeting, 19th – 27th May 2021

Agenda item:			9.12.2

Source:	Moderator (Samsung)

Title:	Email discussion summary for [99-e][313] NTN_Solutions_Part2

Document for:	Information

Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

This lead summary document captures issues related to NR NTN coexistence aspects. It contains a summary of the contributions under sections 9.12.2 at TSG-RAN WG4 #99-e, together with identified key open issues and recommends topics/questions to be handled via email discussions. The goal of this document is also to provide recommendation on prioritization of discussion and whether any issues should be postponed.

A total of 14 TDOCs have been received for this agenda (See Appendix 1) and 5 topics are listed as below to cover proposals and contents in these documents as appropriate.

- Topic #1: Coexistence scenarios

- Topic #2: Network layout model & methodology

- Topic #3: Other simulation assumptions

- Topic #4: HAPS

- Topic #5: Calibration and alignment

To progress the discussion, it is proposed that the meeting could:

- 1st round: Focus on Topic #1 and 3, targeting on narrowing down co-existence scenarios and agreeing on parameters of NTN & TN systems.

- 2nd round: Focus on Topic #2, 4 and 5. Simulation assumptions will be captured in one or two separate document(s) as appropriate. Target to agree on WFs for simulation assumptions to provide results in RAN4 #99e.

Topic #1: Coexistence scenarios

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109544 | Samsung | Proposal 1: De-prioritize the NR/NB IoT indoor scenario until the O2I propagation model and its input variables are discussed and agreed, considering Observations 1, 2 and 3. Observation 1: To consider indoor NR/NB IoT BS and UE, the O2I propagation model provided in TR 38.811 requires many inputs to be agreed to make simulation results from different companies comparable. Observation 2: The O2I loss models in TR 38.803 and TR 36.942 are not designed for space-to-earth path which the propagation path usually penetrates from rooftop to indoor. Observation 3: For indoor case, the NTN ACI would be further reduced due to O2I penetration. |
| R4-2111462 | Thales | Proposal 1: RAN4 shall update “Simulation assumptions for NTN co-existence study” document with the Figure 1, the Figure 2 and the Figure 3. Figure 1: S-band NTN-TN adjacent band coexistence scenarios with TN in FDD mode Figure 2: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n34) Figure 3: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n41) |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1

Open issues and candidate options before e-meeting:

Issue 1-1: Indoor scenario of NR/NB-IoT

- Proposals

  - Option 1: De-prioritize the NR/NB IoT indoor scenario until the O2I propagation model and its input variables are discussed and agreed

  - Option 2: TBA

- Recommended WF:

The moderator would like to give a try to reduce the work load as much as possible. So following approaches are recommended.

  - Approach 1: satellite operators are invited to clarify whether NTN UE will be served with indoor scenario. If not, the indoor scenario of NR/NB-IoT can be removed.

  - Approach 2: If Approach 1 cannot be agreed, the NR/NB-IoT indoor scenario can be considered in Phase 2 with a clear agreement on O2I propagation model.

Sub-topic 1-2

Open issues and candidate options before e-meeting:

Issue 1-2: Interference chart

- Proposals

  - Option 1: RAN4 shall update “Simulation assumptions for NTN co-existence study” document with the Figure 1, the Figure 2 and the Figure 3.

Figure 1: S-band NTN-TN adjacent band coexistence scenarios with TN in FDD mode

Figure 2: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n34)

Figure 3: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n41)

- Recommended WF

  - Incorporate the Aggressor and victim table agreed in last RAN4 meeting show as below and capture Figure 1, 2 & 3 into the “Simulation assumptions for NTN co-existence study” document.

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN - NTN | TN DL | NTN DL |  | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL |  | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL |  | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL |  | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with n34 TDD. | Phase 1 |
| 7 | TN with NTN | TN UL | NTN DL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 2 |
| 8 | TN with NTN | NTN DL | TN UL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 1 |
| 9 | NTN with NTN | NTN DL | NTN DL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |

Companies views’ collection for 1st round

Open issues

Sub topic 1-1

| Company | Comments |
| --- | --- |
| Ericsson | Approach 1 is ok, considering all NTN UEs will always be outdoor. |
| Huawei | Option 1 |
| Xiaomi | Option 1. Indoor case can be De-prioritize |
| Hughes/EchoStar | Option 1 |
| THALES | Option 1, Approach 1. Indoor case can be de-prioritized. |
| CATT | Option 1 |
| Samsung | We support our option 1, and prefer approach 1. |
| ZTE | Option 1 |
| Inmarsat | Option 1 |
| Fraunhofer | Indoor NTN UEs should be de-prioritized |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX |  |
| Ericsson | It’s ok to add the table. For the figure, we are wondering what is the added value? It only shows possible DL-DL, UL-UL, DL-UL and UL-DL types of interferences, which are anyway captured in the table… |
| Huawei | No strong view on this. But I think current table reflect the agreement correctly. |
| Xiaomi | No strong view |
| Hughes/EchoStar | Table is OK but consider de-prioritize NTN-NTN |
| THALES | Option 1 is helpful to better visualize the interference types. We can also use the description to further discuss on the interference source types, the number of sources, etc. NTN-NTN scenario can be deprioritized if no interest from NTN operators. |
| CATT | No strong view for the figure as it just visualized the scenario. But seems not so necessary from co-existence work point of view. |
| Qualcomm | It is OK to add the tale. For the figure, we don’t have strong view. |
| ZTE | Table is sufficient we think. |
| Inmarsat | Ok, but we should deprioritize NTN-NTN |
| Fraunhofer | Table could be captured in informative annex if needed. |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1 Indoor scenario of NR/NB-IoT | All support Option 1 and several companies support Approach 1 to remove NR/NB-IoT Indoor scenario. Tentative agreements: Remove NR/NB-IoT Indoor scenario since NTN UE s will always be outdoor. Candidate options:N/A Recommendations for 2nd round:N/A |

|  | Status summary |
| --- | --- |
| Issue 1-2 Interference chart | All support updating the “Simulation assumptions for NTN co-existence study” with the agreed Aggressor and victim table. There are doubts on the necessity to capture Figure 1, 2 & 3. There are additional proposals to deprioritize NTN-NTN scenarios. However, NTN-NTN scenarios have already been moved to Phase 2 as the agreement in last RAN4 meeting Tentative agreements: Update the “Simulation assumptions for NTN co-existence study” with the agreed Aggressor and victim table. Candidate options: - Capture the Figure 1, 2 &3 as informative Annex. Recommendations for 2nd round: 1. Try to agree on the Candidate option since from moderator’s point of view, it is not harmful to capture these figures as they are helpful to visualize the interference type. 2. Further clarification on deprioritizing NTN-NTN scenarios is needed – Does it mean DO NOT consider NTN-NTN scenarios? |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Topic #2: Network layout model & methodology

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109118 | CATT | Proposed updates based on R4-2106105 Coordination System Option 2 There is no need to consider the curvature of earth for layout, assuming one satellite beam for the simulation. The distances for LEO-600, LEO-1200 and GEO can be assumed as 600km, 1200km and 35786km separately for any point under the 3dB satellite beam. Deployment of NTN UE Option 4 NTN UE should be randomly generated within the NTN area. How does it co-locate with TN network depends on how we place the 2 networks. Coexistence between NTN and NTN The following 2 cases should be considered. - One satellite carries two neighbour carriers, where the footprints of the 2 carriers are the same and coordinated see figure 2.2-1. - Two satellites (GEO and LEO) operate on two neighbour carriers but at different height, see figure 2.2-2. The number of LEO satellite and footprints are FFS. Figure2.1-1 Layout for coexistence between NTN and TN (TBD) Figure 2.2-2 Layout for coexistence between NTN systems Figure 2.2-3 Layout for coexistence between NTN systems (different height satellites) |
| R4-2109119 | CATT | Observation: The impact of earth curvature on co-existence performance under TN DL-NTN DL scenario seems acceptable. Proposal: It is proposed not to consider earth curvature for TN-NTN co-existence. |
| R4-2109645 | CATT | This paper presented a simplified method for TN UL/DL --> NTN UL simulation. The proposed method is as the following steps. - Randomly generate TN system (19 sites with wraparound) within the coverage of satellite beams. - Calculate the interference Pi from TN system to satellite. If the maximum number of loops I reached, go to the next step. Otherwise, go back to 1). - Calculate the total interference: Where, is the coverage rate of TN systems, S0 is the coverage area of a TN system and S is the coverage area of satellite beams. I is a parameter that balances the complexity and accuracy. Lager loop number I could promises better accuracy but costs more calculating resources and time. - Calculate the performance of NTN UE. In this step, the interference from TN systems is expressed by Ptotal and no more TN sites are generated. |
| R4-2110119 | Ericsson | Proposal 2: When NTN is aggressor in DL, a TN located at satellite’s cell border shall also be observed for FRF=1. Proposal 3: When TN is victim in DL, a scenario where the satellite is at low elevation shall also be analysed, considering a NTN cell at satellite coverage edge. Proposal 4: When TN is victim in UL, NTN impact shall only be analysed on TN cells hosting a NTN UE. Summary of Proposal 1~4 (to be shown as Issue 2-2 - Proposal - Option 2) |
| R4-2110412 | Huawei, HiSilicon | Coordination System Proposal 1: If the beam size refers to the Nadir pointing of the satellite, we can take option 2. There is no need to consider the curvature of earth for layout, assuming one satellite beam for the simulation. Simulation Methodology For following two cases, more TN sites might be needed due to large coverage per beam of NTN node. The heterogeneous network layout is shown in figure 3-1. No.; Combination; Aggressor; Victim. 1; TN - NTN; TN DL (TN BS); NTN UL (NTN satellite). 2; TN - NTN; TN UL (TN UE); NTN UL (NTN satellite). 3; To be added. Figure 3-1 The heterogeneous network layout The number of TN networks has been analyzed in the contribution [3], the number of layers can be calculated by equation as below: Layer = D-beam/ISD/2 + 1 (1) Where D-beam is the footprint. The total number of sites can be calculate as below Site_total = 3* Layer *( Layer -1) + 1 (2) Observation 1: There are some options to address the heterogeneous network layout with so many sites. Option 1: The heterogeneous network layout with all the sites can be simulated. Option 2: The heterogeneous network layout with parts of the sites can be simulated. After we get the interference levels of parts of the sites, we can calculate the interference levels of all the sites by scaling factor. Option 3: Other solutions are not precluded. |
| R4-2110799 | Qualcomm | Coordination System Proposal 1: RAN4 to consider the used assumptions in TR 38.811 for 3D global coordinate system as well the curvature of earth formula to calculate the slant distance between the UE and satellite. Simulation Methodology Proposal 2: RAN4 to consider one TN network (i.e., 19 cells * 3 sectors) dropped randomly every snapshot with different location inside the NTN cell. One option is the TN network centres could be defined by creating a location grid in order to cover all the locations in the NTN cell as shown in Figure 1. Figure 1: possible TN networks locations across one NTN cell In addition to that, consider the random NTN UE dropping within the TN network. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1

Open issues and candidate options before e-meeting:

Issue 2-1: Coordinate system

- Proposals

  - Option 1: RAN4 to consider the used assumptions in TR 38.811 for 3D global coordinate system as well the curvature of earth formula to calculate the slant distance between the UE and satellite. (Option 1 of R4-2106105)

  - Option 2: There is no need to consider the curvature of earth for layout, assuming one satellite beam for the simulation. The distances for LEO-600, LEO-1200 and GEO can be assumed as 600km, 1200km and 35786km separately for any point under the 3dB satellite beam. (Option 2 of R4-2106105)

- Recommended WF

  - Option 2 (as illustrated in several input documents differences are very small.)

Sub-topic 2-2

Open issues and candidate options before e-meeting:

Issue 2-2: Method to generate TN sites for NTN-TN co-existence studies

- Proposals

  - Option 1 (CATT & Huawei):

For following two cases, more TN sites might be needed due to large coverage per beam of NTN node. The heterogeneous network layout is shown in figure 3-1.

| No. | Combination | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | TN - NTN | TN DL (TN BS) | NTN UL (NTN satellite) |
| 2 | TN - NTN | TN UL (TN UE) | NTN UL (NTN satellite) |
| 3 | To be added |  |  |

Figure 3-1 The heterogeneous network layout

The number of layers can be calculated by equation as below:

Layer = D-beam/ISD/2 + 1                                (1)

Where D-beam is the footprint. The total number of sites can be calculate as below

Site_total = 3* Layer *(Layer -1) + 1                           (2)

There are some options to address the heterogeneous network layout with so many sites.

- Option 1-1: The heterogeneous network layout with all the sites can be simulated.

- Option 1-2: The heterogeneous network layout with parts of the sites can be simulated. After we get the interference levels of parts of the sites, we can calculate the interference levels of all the sites by scaling factor.

- Option 1-3: Following steps can be used to simplify the simulation

- Randomly generate TN system (19 sites with wraparound) within the coverage of satellite beams.

- Calculate the interference Pi from TN system to satellite. If the maximum number of loops I reached, go to the next step. Otherwise, go back to 1).

- Calculate the total interference:

Where,  is the coverage rate of TN systems, S0 is the coverage area of a TN system and S is the coverage area of satellite beams.

I is a parameter that balances the complexity and accuracy. Lager loop number I could promises better accuracy but costs more calculating resources and time.

- Calculate the performance of NTN UE. In this step, the interference from TN systems is expressed by Ptotal and no more TN sites are generated.

- Option 1-4: Other solutions are not precluded.

  - Option 2 (Ericsson): consider following table

| No. | Combination | Aggressor | Victim | Which NTN cell/TN to consider? | Which TN cells to consider? | Additional remark |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | All TNs in NTN cell | All | How many active TNs? |
| 2 | TN with NTN | TN UL | NTN UL | All TNs in NTN cell? | All | How many active TNs? |
| 3 | TN with NTN | NTN DL | TN DL | - NTN cell at Nadir point: - TN randomly placed in the NTN cell. - TN at NTN cell edge if FRF=1. - NTN cell with satellite at low elevation: - TN randomly placed in the NTN cell - TN at NTN cell edge if FRF=1. | All |  |
| 4 | TN with NTN | NTN UL | TN UL | NTN cell at Nadir point: TN randomly placed in this cell | Only the TN cells hosting NTN UE(s) |  |
| 5 | TN with NTN | NTN UL | TN DL | - NTN cell at Nadir point: - TN randomly placed in the NTN cell - TN at NTN cell edge if FRF=1. - NTN cell with satellite at low elevation: - TN randomly placed in the NTN cell - TN at NTN cell edge if FRF=1. | All |  |
| 6 | TN with NTN | TN DL | NTN UL | All TNs in NTN cell | All | How many active TNs? |
| 7 | TN with NTN | TN UL | NTN DL | TBD |  |  |
| 8 | TN with NTN | NTN DL | TN UL | NTN cell at Nadir point: TN randomly placed in this cell | Only the TN cells hosting NTN UE(s) |  |
| 9 | NTN with NTN | NTN DL | NTN DL | TBD | NA |  |
|  |  | NTN UL | NTN UL | TBD | NA |  |
| Further clarification from Ericsson: So far, RAN4 has only considered NTN central beam at Nadir position. But for TN DL as victim, we propose to look also at satellite at low elevation angle, where the NTN central beam won’t be at Nadir position (see R4-21100119 for justification). Note that for the simulations alignment, only NTN central beam at Nadir point could be considered. |  |  |  |  |  |  |

  - Option 3(Qualcomm): For NTN-TN coexistence studies, one TN network (i.e., 19 cells * 3 sectors) is dropped randomly every snapshot with different location inside the NTN cell.

- Option 3-1: Follow the traditional method to randomly drop TN network centres every snapshot within in NTN cell

- Option 3-2: TN network centres could be randomly drooped among a location grid in order to cover all the locations in the NTN cell. Such location grid could be created by equal sampling points per side of the hexagon. As shown in Figure 1, 11 points per side is used and the distance between successive points is about 12 km

Figure 1: possible TN networks locations across one NTN cell

- Recommended WF

  - Furth discuss options and sub-options above.

Sub-topic 2-3

Issue 2-3: Co-existence between NTN and NTN

- Proposals

  - Option 1: (remove [] in Section 2.2.2 of R4-2106105)

The following 2 cases should be considered.

- One satellite carries two neighbour carriers, where the footprints of the 2 carriers are the same and coordinated see figure 2.2-2.

- Two satellites (GEO and LEO) operate on two neighbour carriers but at different height, see figure 2.2-3. The number of LEO satellite and footprints are FFS.

Figure 2.2-2 Layout for coexistence between NTN systems

Figure 2.2-3 Layout for coexistence between NTN systems (different height satellites)

  - Option 2: TBA

- Recommended WF

  - Agree on Option 1. Other layouts are not precluded and contribuitons are invited.

Companies views’ collection for 1st round

Open issues

Sub topic 2-1 Coordinate system

| Company | Comments |
| --- | --- |
| Ericsson | As we are proposing to also look at satellite with low elevation angle (DL-DL cases), option 1 is still preferred. The Option 2 would only be ok when NTN central beam is at Nadir point. |
| Huawei | When NTN central beam is at Nadir point, we prefer option 2. However, if RAN4 has to check the low elevation angle, more discussion is needed. Maybe footprint also should be discussed. |
| Xiaomi | For the simulation, we think option 2 is simple and enough. However, option 1 is also OK for us |
| THALES | As previously explained during RAN4#98bis-e, we prefer option 1. |
| CATT | Option 2 is feasible for the scenarios in our paper R4-2109119. Other scenarios can be discussed further if there is a preference. |
| Qualcomm | Option 1. As explained by Ericsson, if low elevation angle is considered, option 1 shall be considered. Clarification questions to CATT: In figure 5 of R4-2109119 with the TP loss, there is indeed a gap, why the observation is saying no big difference? |
| Samsung | We prefer option 1 for all scenarios. But we are also OK with option 2 for scenarios only has nadir beam cases. This may needs further discussion on detailed beam assumption in each scenario. |
| ZTE | Maybe option 1 is better if considering difference case like low evaluation or nadiar point |
| Fraunhofer | Since the 3dB beamwidths are not captured in option 2 – it is not acceptable. Anyways, even if the Earth’s curvature could be neglected for some special cases, we think we should stick with option 1. |

Sub topic 2-2 Method to generate TN sites for NTN-TN co-existence studies

| Company | Comments |
| --- | --- |
| Moderator | For Option 3, a further clarification on how the TN location grid is created, e.g. number of spots, distribution, etc., has been provided by Qualcomm. |
| xxx |  |
| Ericsson | Option 1 approach is ok for the mentioned cases. But when we calculate the total interference, we can’t assume all TNs will trasnmit at full capacity, this is not realistic, it never happens. Our option 2 is our proposal based on the fight NTN central beam at Nadir point is not always the worst case scenario: when satellite is a low elevation angle, the higher path loss is largely compensated by the higher TN BS antenna gain (AAS and non AAS). Option 3: option 3-2 would give a good overview but might be resource consuming… |
| Huawei | Option 1, we can consider some scaling factor if it’s impossible that all TNs will transmit at full capacity. For Option 2, if we go to a low elevation angle, there are some open issues and parameter to be addressed and assumed. |
| THALES | Agree with proposed WF. However, we should consider worst cases, and then we can compute a mean interference based on the number of interfering sources and/or their densities. |
| CATT | Option 1. |
| Qualcomm | For option 1, clarifications on the figure for the layout: does it mean TN network centre should be located per the grids calculated by the equations? If yes, how to randomly distribute the TN network as mentioned in the option 1-3? How many TN network are needed? We think option 1-3 is a good start. The clarifications on how to define , S0, and S are needed. For option 2, we can add further consider the case for low elevation angle. For option 3, seems it is similar as option 1 in general. But need to confirm. |
| Samsung | We are open to all options, as they are not really conflicting each other. But for option 1, we would like to echo Ericsson’s comment that we should not assume all TN networks are transmitting simultaneously in simulation. For TN to NTN scenarios, we provided reference from ITU-R WP 5D below for lower loading factor as a possible approach, and not preclude other approaches: - IMT network loading/activate factor Urban/suburban macro; Rural macro; Urban small cell (outdoor)/Micro cell. Network loading factor (average base station activity) (Note 1); 20%, 50%; 20%, 50%; 20%, 50%. Note 1: For areas greater than 50 km2 a network loading factor of 20% should be used.. |
| ZTE | Fine with option 1 which is more clear, in addition loading factor or active factor should also been considered |
| Fraunhofer | Agree with Thales. |

Sub topic 2-3 Co-existence between NTN and NTN

| Company | Comments |
| --- | --- |
| xxx |  |
| Ericsson | Fine with Moderator’s recommendation, we could start with this option. |
| Xiaomi | Option 1 |
| Hughes/EchoStar | This need to be de-prioritized. There is no NTN band adjacent to S-band (1980-2010) and (2170-2200) MHz |
| THALES | NTN – NTN coexistence should be de-prioritized if no interest from operators. Moreover, the use case does not seem applicable for S-Band. |
| CATT | Support moderator’s recommendation. |
| Qualcomm | OK with option 1. |
| Samsung | Support moderator’s recommended way forward |
| ZTE | Fine with moderator’s recommendation. |
| Inmarsat | We should deprioritize NTN-NTN coexistence for now. |
| Fraunhofer | There are 2 cases but three figure captions and it seems that the cases do not refer to the correct figures. Please find our changes above. We are not sure what the benefit is to remove the brackets if other layouts are not precluded. In addition, there seems to be no consensus on whether NTN-NTN coexistence is really needed. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1: Coordinate system | 7 companies are OK with Option 1. 4 companies are OK with Option 2 when NTN central beam is at Nadir point. Tentative agreements: N/A Candidate options: - Option 1: RAN4 to consider the used assumptions in TR 38.811 for 3D global coordinate system as well the curvature of earth formula to calculate the slant distance between the UE and satellite. (Option 1 of R4-2106105) - Option 2: There is no need to consider the curvature of earth for layout, assuming one satellite beam for the simulation. The distances for LEO-600, LEO-1200 and GEO can be assumed as 600km, 1200km and 35786km separately for any point under the 3dB satellite beam. (Option 2 of R4-2106105) Recommendations for 2nd round: Try to agree on Option 1 as it convers more cases. Otherwise following approach is suggested: - Step 1: Go on with Option 2 for calibration purpose assuming NTN central beam is at Nadir point. - Step 2: Further study and discuss the difference between Option 1 and 2 in low elevation angle cases. If there are no big differences, then we adopt Option 1. |
| Issue 2-2: Method to generate TN sites for NTN-TN co-existence studies | Various views and comments have been provided. And it indeed needs further discussion. However, some common views can be achieved. Tentative agreements: - It should not be assumed that all TN networks are transmitting simultaneously in simulation. - Low elevation angle cases should be further considered based on the table in Option 2. Candidate options: - Option 1 - Option 3 - Option 4 Recommendations for 2nd round: Further clarifications on sub Options in Option 1 are invited and then discuss candidate options to see any agreements can be made. |
| Issue 2-3: Co-existence between NTN and NTN | 6 companies agree with Option 1. But there are proposals to deprioritize NTN-NTN scenarios. Tentative agreements: N/A Candidate options: - Option 1 Recommendations for 2nd round: Whether to deprioritize NTN-NTN scenarios shall be determined in Issue 1-2. For NTN-NTN layout itself, try to agree with Option 1 as a starting point. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #3: Other simulation assumptions

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109118 | CATT | Satellite Channel BW 5MHz FRF 3 Number of active NTN UE UL: 10, DL: 10 NTN system Noise figure in dB 5dB TN parameters Updates based on Table 2.3-5 of R4-2106105 (shown as Issue 3-x ) TN BS antenna For AAS antennas and non-AAS antennas, refer to the ITU LS reply in R4-20xxxxx. For UE antenna, an Omni-directional radiation pattern with antenna gain 0dBi is assumed NTN Performance metric The average throughput loss and 5%-ile throughput loss should be less than 5%. |
| R4-2109544 | Samsung | Proposal 2: Use FRF = 1 and BW = 30 MHz for NTN in co-ex assumption. Proposal 3: Adopt option 2 in Table 2.3-5 together with Table 2.3-6 as the NR parameters for NR assumptions for co-ex study. Observation 6: Option 1 cannot reflect the differences of inter-site distance (ISD), BS height, shadow fading, etc. among different deployment scenarios as urban macro, suburban macro and rural macro. Option 2 with an extra table as Table 2.3-6 in R4-2106105 clearly shows the different sets of parameters for different scenarios. |
| R4-2110119 | Ericsson | Proposal 1: To clarify FRF assumption, and as captured in RAN4#98bis-e Chair’s report, 2 phases will be considered for FRF: FRF=1 in phase 1 for simplification; FRF=3 in phase 2 or it is found FRF=1 is too stringent. Proposal 5: Clarify TR 38.811 propagation model. In the meantime, to not delay the calibration phase, this TR 38.811 propagation model could still be used as much as possible, further calibration might then be needed later if this model would need some update. Proposal 6: Adopt NTN BS noise figure values as proposed in Table 2. Satellite; GEO; LEO 600; LEO 1200. G/T (dB K-1); 19; 1.1; 1.1. G_Rx (dBi); 51; 30; 30. NF (dB); 7.4; 4.3; 4.3. Proposal 7: Assume 80% of TN UEs are outdoor and 20% indoor for TN macro urban and suburban. Proposal 9: Adopt the extended BS AAS antenna model (Table 4 Extended BS AAS antenna model) for TN in the scope of NTN simulations. Observation: The CLx-ile value should be adapted for rural, dense urban and indoor scenarios. |
| R4-2110412 | Huawei, HiSilicon | Proposal 2: Whether to use NTN UL power control or set 23 dBm may depend on the relation between CLx-ile, Path loss and Satellite Rx max Gain. Proposal 3: the noise figure F for NTN system (dB) should be further evaluated based on the couple loss assumption between satellite and gateway. Proposal 4: the parameters in tale 5-1 can be reused for the following parameters , SNIRMIN, and SNIRMAX, but other options are not precluded. Table 5-1: Parameters describing baseline Link Level performance for 5G NR Parameter; DL; UL; Notes. α, attenuation; 0.6; 0.4; Represents implementation losses. SNIRMIN, dB; -10; -10; Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL). SNIRMAX, dB; 30; 22; Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL). It’s infeasible to achieve 30dB/22dB DL/UL maximum SNIR for NR NTN, so the following parameters need to be further studied and RAN4 need to check them with RAN1. Attenuation factor, representing implementation losses SNIRMIN Minimum SNIR of the code set, dB SNIRMAX Maximum SNIR of the code set, dB |
| R4-2110799 | Qualcomm | Proposal 3: RAN4 to consider 20 MHz channel bandwidth with considering the number of active NTN UEs to be 3 in UL and 1 in DL that is in-line with the TN assumptions. Proposal 4: RAN4 to refer to TR 36.942 for the BS non-AAS antenna pattern. Proposal 5: RAN4 to reuse the same TN TPC for NTN with SNR target 15dB. |
| R4-2111423 | Thales | Proposal 4: For the purpose of coexistence analysis between adjacent bands, 20 MHz bandwidth size can be considered for simulation purpose. Observation 1: However, in order to simplify the simulations, coexistence analysis could take into account the 30 MHz configuration as an example for NTN deployment in S-band. |
| R4-2111462 | Thales | Proposal 2: Update with following information The satellite max Tx power can be calculated by the equation as below: maxTx power[dBm]=EIRP density[dBWMHz]+30+10 log10NRB*SCSMHz*12-Max Gain[dBi] NRB configuration per BandWidth size and SCS: Configuration FR1 S-band; NRB (5MHz BW); NRB (10MHz BW); NRB (15MHz BW); NRB (30MHz BW). SCS 15 kHz; 25; 52; 79; 160. SCS 30 kHz; 11; 24; 38; 78. Set-1 satellite parameters for co-existence study (TR 38.821) Satellite orbit; GEO; LEO-1200; LEO-600. Satellite altitude; 35786 km; 1200 km; 600 km. Payload characteristics for DL transmissions. Satellite EIRP density; 2GHz; 59 dBW/MHz; 40 dBW/MHz; 34 dBW/MHz. Satellite Tx max Gain; 51 dBi; 30 dBi; 30 dBi. Satellite max TX power in dBm; SCS15: 44.53; 47.71; 49.53; 52.59dBm SCS30: 43.98; 47.37; 49.36; 52.48dBm; SCS15: 46.53; 49.71; 51.53; 54.59dBm SCS30: 45.98; 49.37; 51.36; 54.48dBm; SCS15: 40.53; 43.71; 45.53; 48.59dBm SCS30: 39.98; 43.37; 45.36; 48.48dBm. Channel bandwidth; 5; 10; 15; 30 MHz; 5; 10; 15; 30 MHz; 5; 10; 15; 30 MHz. 3dB beamwidth; 0.4011 deg; 4.4127 deg; 4.4127 deg. Satellite beam diameter; 250 km; 90 km; 50 km. Proposal 3: Update with following information Satellite orbit; GEO; LEO-1200; LEO-600. Satellite altitude; 35786 km; 1200 km; 600 km. NTN UE Assumptions (Distribution, Density/km2) for 15 MHz and 30 MHz configurations (FRF1). Satellite Beam Diameter; 2GHz; 250 km; 90 km; 50 km. Mean Throughput Rate per (Active) NTN UE; 250 kbps; 250 kbps; 250 kbps. Satellite Spectrum Efficiency Assumption; 1.35bit/sec/Hz; 1.35bit/sec/Hz; 1.35bit/sec/Hz. Channel Bandwidth Assumption per Beam; 5; 10; 15; 30 MHz; 5; 10; 15; 30 MHz; 5; 10; 15; 30 MHz. Maximum Throughput per Beam; 6.75; 13.5; 20.25; 40.5 Mbps; 6.75; 13.5; 20.25; 40.5 Mbps; 6.75; 13.5; 20.25; 40.5 Mbps. Number of NTN (Active) UEs per Beam; 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam; 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam; 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam. NTN UE Density per Given Service Type (for Simulation Purpose); 30 MHz assumption: 0.0033 NTN UEs/km2 15 MHz assumption: 0.0016 NTN UEs/km2; 30 MHz assumption: 0.0255 NTN UEs/km2 15 MHz assumption: 0.0127 NTN UEs/km2; 30 MHz assumption: 0.0825 NTN UEs/km2 15 MHz assumption: 0.0412 NTN UEs/km2. Proposal 4: The maximum BW size used for NTN coexistence simulation purpose in MSS S-Band should consider the same maximum configurable BW size as for TN n65. Note1: It should be considered that the TN UE operating in n65 has the capability to operate also in the considered NTN MSS S-Band, since the UE uses the same implementation for both TN and NTN connectivity. For this reason, for the coexistence analysis, same BW configuration should be probably used for both TN and NTN. Note2: The TN maximum BW size in n65 is currently 20 MHz, and therefore NTN maximum supported BW size in MSS S-Band should also be 20 MHz. Note3: Simulation assumptions may be further updated accordingly. Proposal 5: RAN4 should consider the number of TN UEs or their respective densities as a function of interference type. Proposal 6: RAN4 should consider the number of NTN UEs or their respective densities as a function of interference type. Proposal 7: Depending on who is the aggressor, and who is the victim, the simulation approach may be different. For generating interference, RAN4 simulations may consider multiple sources, but for measuring interference, RAN4 simulation may use one single destination in different locations. Proposal 8: NTN simulation assumptions should follow similar Monte-Carlo simulation approach as for the TN simulation assumptions. Proposal 9: If TN simulation assumptions are using one single TN UE scheduled approach, NTN simulation assumptions should also use one single NTN UE scheduled approach. Proposal 10: If TN simulation assumptions are using a density of TN UEs, NTN simulation assumptions should also use a density of NTN UEs. Proposal 11: The simulation assumptions should consider UEs connecting with higher priority to the network providing higher QoS. Proposal 12: The simulation assumptions could be further simplified with NTN UEs at the edge of TN network or outside TN network. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1

This sub-topic focus on NTN parameters

Open issues and candidate options before e-meeting:

Issue 3-1: NTN FRF

- Proposals

  - Option 1: 2 phases will be considered for FRF: FRF=1 in phase 1 for simplification; FRF=3 in phase 2 or it is found FRF=1 is too stringent. (Agreement in RAN4#98-bis-e meeting)

  - Option 2: FRF =3

- Recommended WF

  - Follow the agreement in RAN4#98-bis-e meeting.  2 phases will be considered for FRF: FRF=1 in phase 1 for simplification; FRF=3 in phase 2 or it is found FRF=1 is too stringent. (Agreement in RAN4#98-bis-e meeting)

  - Update “Simulation assumptions for NTN co-existence” document with this agreement as appropriate.

Issue 3-2: Satellite channel bandwidth (BW)

- Proposals

  - Option 1: BW=5MHz (in associated with FRF=3)

  - Option 2: BW=30MHz (in associated with FRF=1)

  - Option 3: BW=20MHz

The maximum BW size used for NTN coexistence simulation purpose in MSS S-Band should consider the same maximum configurable BW size as for TN n65.

- Recommended WF

  - Take into account the agreement on FRF, consider BW=30MHz (FRF=1) in phase 1 and 5MHz (FRF=3) in phase 2.

Issue 3-3: Satellite max TX power

- Proposals

  - Option 1: The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

NRB configuration per BandWidth size and SCS:

| Configuration FR1 S-band | NRB (5MHz BW) | NRB (10MHz BW) | NRB (15MHz BW) | NRB (30MHz BW) |
| --- | --- | --- | --- | --- |
| SCS 15 kHz | 25 | 52 | 79 | 160 |
| SCS 30 kHz | 11 | 24 | 38 | 78 |

Following information is to be updated in Set-1 satellite parameters table.

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite max TX power in dBm |  | SCS15: 44.53; 47.71; 49.53; 52.59dBm SCS30: 43.98; 47.37; 49.36; 52.48dBm | SCS15: 46.53; 49.71; 51.53; 54.59dBm SCS30: 45.98; 49.37; 51.36; 54.48dBm | SCS15: 40.53; 43.71; 45.53; 48.59dBm SCS30: 39.98; 43.37; 45.36; 48.48dBm |
| Channel bandwidth |  | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz |

- Recommended WF

  - Agree on Option 1.

Issue 3-4: NTN system Noise figure in dB

- Proposals

  - Option 1: 5dB

  - Option 2: Adopt NTN BS noise figure values in the table below.

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 19 | 1.1 | 1.1 |
| G_Rx (dBi) | 51 | 30 | 30 |
| NF (dB) | 7.4 | 4.3 | 4.3 |

  - Option 3: the noise figure F for NTN system (dB) should be further evaluated based on the couple loss assumption between satellite and gateway.

- Recommended WF

  - TBA.

Sub-topic 3-2

This sub topic focus on TN parameters

Open issues and candidate options before e-meeting:

Issue 3-5: TN Parameters

- Proposals

  - Option 1: See Table 2.3-5 and Table 2.3-6 below.

  - Option 2: See Table 2.3-5 and Table 2.3-6 below.

Table 2.3-5 Simulation assumptions of TN respectively based on NB-IoT and NR

|  | NB-IoT standalone | NR |  |
| --- | --- | --- | --- |
|  |  | Option 1 | Option 2 |
| Carrier frequency in GHz | 2 | 2 | 2 |
| Size of each nominal channel BW in MHz | 0.2 | 20 | 20 |
| Transmission bandwidth in MHz | 0.18 | 9 |  |
| Environment | Urban macro Sub-urban Rural | Urban macro Sub-urban Rural | Deployment scenario related, check Table 2.3-6. |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around | 19-sites 57 sectors with wrap-around |
| Inter-site distance in meter | 500 for 2GHz band for UMA | 500 for 2GHz band for UMA FFS for other scenarios | Deployment scenario related, check Table 2.3-6 |
| System loading and activity | Full buffer 100% | Full buffer 100% | Full buffer 100% |
| Network location | FFS | The centre of TN network is aligned with the centre of centre beam of NTN | TN as victim: Randomly generated in NTN central beam |
| DL subcarrier spacing | 15kHz | 15kHz | 15kHz |
| UL | See RP-152284 | OFDMA | OFDMA |
| DL power control | No | No | No |
| UL power control | 36.942 section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB | 36.942 | 36.942 Section 9.1 |
| Frequency reuse | 1 | 1 | 1 |
| Number of scheduled UE per cell (DL) | 1 | 1 | 1 |
| Number of scheduled UE per cell (UL) | 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone | 3 | 3 |
| UE antenna height in meter | 1.5 | 1.5 | Deployment scenario related, check Table 2.3.6 |
| UE TX power in dBm | -40 to 23 | -40 to 23 | -40 to 23 |
| UE antenna gain in dBi | 0 | 0 | 0 |
| Building penetration loss | 45.820 Annex D.1 | In Pathloss model, TR 38.901 | In Pathloss model, TR 38.901 |
| Cell selection margin in dB | 3 | 3 | 3 |
| BS-MS min couple loss in dB | 70 | 70 for outdoor scenario in Table 2.1-1. 45 for indoor scenario in Table 2.1-1. | Proposed ‘Minimum BS-UE distance in meter’ instead of MCL. Deployment scenario related, check Table 2.3-6. |
| BS noise figure in dB | 5 | 5 | 5 |
| UE noise figure in dB | 9 | 9 | 9 |
| BS-UE path-loss model | TR36.942 macro urban | TR38.803 | TR 38.901 |
| Standard deviation of BS-UE log-normal shadow fading in dB | 10 | Deployment scenario related, referring to TR 38.901 | Deployment scenario related, referring to TR 38.901. |
| Shadowing correlation | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 |  |
| Link-level performance model |  | section 2.9 |  |
| UE distribution |  |  | Uniform |
| Evaluation metrics | SINR vs ACS (as victim) | Throughput loss (refer to section 2.8) | Throughput loss, referring to TR 38.803 section 5.2.7 |

Table 2.3-6 Deployment-related parameters of TN (2 GHz)

|  | Urban Macro | Suburban Macro | Rural Macro | Remarks |
| --- | --- | --- | --- | --- |
| Cell radius in meters | 500 | 1000 | 5000 | ITU-R Report M.2292 |
| BS Antenna height in meters | 25 | 30 | 30 |  |
| Base Station Antenna Characteristics |  |  |  |  |
| Antenna Pattern | For AAS, see TR 37.842 Section 5.3.3 |  |  | TR 37.842 |
| Element Gain in dBi | 6.4 | 7.1 | 7.1 | 3GPP LS to ITU-R WP5D RP-200559 and ITU-R WP5D [IMT_Parameters] |
| H and V 3dB beamwidth of single element in degree | 90º for H 65º for V | 90º for H 54º for V | 90º for H 54º for V |  |
| H and V front-to-back ratio in dB | 30 for both H/V | 30 for both H/V | 30 for both H/V |  |
| Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º |  |
| Antenna array configuration (Row × Column) | 8 x 8 elements | 8 x 8 elements | 8 x 8 elements |  |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 0.9 of wavelength for V | 0.5 of wavelength for H, 0.9 of wavelength for V |  |
| Conducted power per antenna element in dBm | 25 | 25 | 25 |  |
| Mechanical downtilt in degree | 10 | 6 | 3 |  |
| UE Parameters |  |  |  |  |
| UE Outdoor/indoor | 100% Outdoor |  |  |  |
| UE height in meter | 1.5 | 1.5 | 1.5 | 3GPP LS to ITU-R WP5D RP-200559 and ITU-R WP5D [IMT_Parameters] |
| Minimum BS-UE distance in meter | 35 | 35 | 35 |  |

- Recommended WF

  - TBA

Issue 3-6: TN BS AAS Antenna

- Proposals

  - Option 1: For AAS antennas and non-AAS antennas, refer to the ITU LS reply in R4-2008924.

  - Option 2: Adopt the extended BS AAS antenna model for TN in the scope of NTN simulations.

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ |

- Recommended WF

  - TBA

Issue 3-7: TN BS non-AAS Antenna

- Proposals

  - Option 1: For AAS antennas and non-AAS antennas, refer to the ITU LS reply in R4-2008924.

  - Option 2: RAN4 to refer to TR 36.942 for the BS non-AAS antenna pattern.

Note that Table 2.4.3-1 of R4-2106105 defined the antenna element pattern for non-AAS antennas.RAN4 needs to define the antenna pattern for non-AAS BS for simulation.

- Recommended WF

  - TBA

Sub-topic 3-3

This sub topic focus on TN and NTN UE deployment

Open issues and candidate options before e-meeting:

Issue 3-8: Deployment of NTN UE

- Proposals

  - Option 1: NTN UE should be randomly generated within the NTN area. How does it co-locate with TN network depends on how we place the 2 networks. (Option 4 of R4-2106105)

  - Option 2: Consider the random NTN UE dropping within the TN network.

  - Option 3: NTN simulation assumptions should follow similar Monte-Carlo simulation approach as for the TN simulation assumptions.

  - Option 4: Take into account following factors.

- Depending on who is the aggressor, and who is the victim, the simulation approach may be different. For generating interference, RAN4 simulations may consider multiple sources, but for measuring interference, RAN4 simulation may use one single destination in different locations.

- The simulation assumptions should consider UEs connecting with higher priority to the network providing higher QoS.

- The simulation assumptions could be further simplified with NTN UEs at the edge of TN network or outside TN network.

- Recommended WF

  - Agree on Option 1 as it seems Option 1, 2, 3 are the same.

  - Further discuss Option 4.

Issue 3-9: Principles to consider UE numbers

- Proposals

  - Option 1: Following principles should be applied

- RAN4 should consider the number of TN UEs or their respective densities as a function of interference type.

- RAN4 should consider the number of NTN UEs or their respective densities as a function of interference type.

- If TN simulation assumptions are using one single TN UE scheduled approach, NTN simulation assumptions should also use one single NTN UE scheduled approach.

- If TN simulation assumptions are using a density of TN UEs, NTN simulation assumptions should also use a density of NTN UEs.

- Recommended WF

  - TBA.

Issue 3-10: Number of active NTN UE

- Proposals

  - Option 1: 10 in UL and 10 in DL (align with TR 38.821)

  - Option 2: 3 in UL and 1 in DL (same with TN)

  - Option 3: Set-1 satellite parameters should be update with following NTN UE density assumptions.

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| NTN UE Assumptions (Distribution, Density/km2) for 15 MHz and 30 MHz configurations (FRF1) |  |  |  |  |
| Satellite Beam Diameter | 2GHz | 250 km | 90 km | 50 km |
| Mean Throughput Rate per (Active) NTN UE |  | 250 kbps | 250 kbps | 250 kbps |
| Satellite Spectrum Efficiency Assumption |  | 1.35bit/sec/Hz | 1.35bit/sec/Hz | 1.35bit/sec/Hz |
| Channel Bandwidth Assumption per Beam |  | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz | 5; 10; 15; 30 MHz |
| Maximum Throughput per Beam |  | 6.75; 13.5; 20.25; 40.5 Mbps | 6.75; 13.5; 20.25; 40.5 Mbps | 6.75; 13.5; 20.25; 40.5 Mbps |
| Number of NTN (Active) UEs per Beam |  | 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam | 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam | 30 MHz assumption: 162 NTN UEs/beam 15 MHz assumption: 81 NTN UEs/beam |
| NTN UE Density per Given Service Type (for Simulation Purpose) |  | 30 MHz assumption: 0.0033 NTN UEs/km2 15 MHz assumption: 0.0016 NTN UEs/km2 | 30 MHz assumption: 0.0255 NTN UEs/km2 15 MHz assumption: 0.0127 NTN UEs/km2 | 30 MHz assumption: 0.0825 NTN UEs/km2 15 MHz assumption: 0.0412 NTN UEs/km2 |

- Recommended WF

  - TBA

Issue 3-11: Indoor/outdoor TN UE

- Proposals

  - Option 1: Assume 80% of TN UEs are outdoor and 20% indoor for TN macro urban and suburban.

  - Option 2: 100% Outdoor

- Recommended WF

  - TBA

Sub-topic 3-4

Open issues and candidate options before e-meeting:

Issue 3-12: NTN UL TPC

- Proposals

  - Option 1: Whether to use NTN UL power control or set 23 dBm may depend on the relation between CLx-ile, Path loss and Satellite Rx max Gain.

  - Option 2: The CLx-ile value should be adapted for rural, dense urban and indoor scenarios.

  - Option 3: RAN4 to reuse the same TN TPC for NTN with SNR target 15dB.

- Recommended WF

  - TBA.

Issue 3-13: NTN Performance metric

- Proposals

  - Option 1: The average throughput loss and 5%-ile throughput loss should be less than 5%.

- Recommended WF

  - TBA.

Issue 3-14: Throughput ~ SNR mapping

- Proposals

  - Option 1: the parameters in tale 5-1 can be reused for the following parameters , SNIRMIN, and SNIRMAX, but other options are not precluded.

Table 5-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

It’s infeasible to achieve 30dB/22dB DL/UL maximum SNIR for NR NTN, so the following parameters need to be further studied and RAN4 need to check them with RAN1.

Attenuation factor, representing implementation losses
SNIRMIN  Minimum SNIR of the code set, dB
SNIRMAX  Maximum SNIR of the code set, dB

- Recommended WF

  - TBA.

Issue 3-15: Interference impact analysis when TN UL is victim

- Proposals: When analyzing the interference (throughput) impact of NTN on TN UEs:

  - Option 1: Only consider the TN cells hosting NTN UEs (i.e. impact should be checked only for TN UEs inside those TN cells).

  - Option 2: Consider all TN cells.

  - Option 3: other

- Recommended WF

  - TBA.

Sub-topic 3-5

Issue 3-16: Propagation model in TR 38.811

- Proposals: Clarify propagation in TR 38.811 (see R4-2110119).

- Recommended WF

  - Companies who contributed to TR 38.811 section 6.6.2 are invited and strongly encouraged to provide information w.r.t. this matter.

Companies views’ collection for 1st round

Open issues

Sub topic 3-1 NTN parameters

| Company | Comments |
| --- | --- |
| Ericsson | Issue 3-1: NTN FRF Ok with the recommended WF but if we have to switch to FRF=3 in phase 2, FRF=1 shall be forbidden then. Issue 3-2: Satellite channel bandwidth (BW) To simplify the simulations and avoid ACIR adaptation, we prefer option 3 so that we have same BW for TN and NTN. Issue 3-3: Satellite max TX power Ok, but power values in dBm should be rounded Issue 3-4: NTN system Noise figure in dB Option 2, 5dB is only for TN… |
| Huawei | Issue 3-1: NTN FRF We can follow the agreement in RAN4#98bis Issue 3-2: Satellite channel bandwidth (BW) We can go option 3 as compromise. Issue 3-3: Satellite max TX power If we take 20MHz, the corresponding Tx power should be added. Issue 3-4: NTN system Noise figure in dB Option 2 is OK for satellite Noise figure, but if we need to assume the NTN system Noise figure, the couple loss assumption between satellite and gateway should be considered. |
| Hughes/EchoStar | Issue 3-1: NTN FRF We can follow the agreement in RAN4#98bis Issue 3-2: Satellite channel bandwidth (BW) For simulation, option 3 as compromise. Issue 3-3: Satellite max TX power If we take 20MHz, the corresponding Tx power should be added. |
| THALES | Issue 3-1: NTN FRF We can follow the agreement in RAN4#98bis, in any case the FRF should be an operator choice. Also agree with WF. Issue 3-2: Satellite channel bandwidth (BW) Option 3 is the most reasonable, but Option 2 could also be an option. However, we should consider NTN operator inputs. N65 has been defined to use MSS band for terrestrial, so it makes sense to consider n65 as input for NTN maximum bandwidth configuration. Issue 3-3: Satellite max TX power We agree that the corresponding Tx power should be also added for 20 MHz. For 30 and 15 MHz is already defined. Issue 3-4: NTN system Noise figure in dB We are fine with the values for G/T and G_Rx (similar as in TR 38.821), but the NTN NF value should be further discussed. |
| CATT | Issue 3-1: NTN FRF We can follow the agreement in RAN4#98bis. Issue 3-2: Satellite channel bandwidth (BW) Ok with option 3 as compromise. Issue 3-3: Satellite max TX power Ok, but power should be aligned with channel bandwidth. Issue 3-4: NTN system Noise figure in dB OK with option 2. |
| Qualcomm | Issue 3-1: NTN FRF OK with WF. Issue 3-2: Satellite channel bandwidth (BW) Option 3. Per the input from satellite companies in tread 312, 20MHz is the reference deployment. So it is reasonable to use 20Mhz in co-ex simulation. And 20Mhz for both NTN and TN can simplify the simulation. Issue 3-3: Satellite max TX power OK with option 1 and need to revise to consider the channel BW of 20Mhz per the conclusion on Issue 3-1 |
| Samsung | Issue 3-1: NTN FRF Support recommended WF. Issue 3-2: Satellite channel bandwidth (BW) Support recommended WF. |
| ZTE | Issue 3-1: NTN FRF Support recommended WF. Issue 3-2: Satellite channel bandwidth (BW) Option 3 to have 20MHz |
| Panasonic | Issue 3-1: NTN FRF We can follow the agreement in RAN4#98bis. |
| Inmarsat | Issue 3-1: NTN FRF Agreement in RAN4#98bis is fine as starting point. We are ok with WF, and should keep in mind that FRF is an operator choice. Issue 3-2: Satellite channel bandwidth (BW) How does Option 3 work with FRF=3 ? We need something realistic that works for FRF=1 and FRF>=3 at least. Issue 3-3: Satellite max TX power We agree Tx power should be also added for 20 MHz. Issue 3-4: NTN system Noise figure in dB NTN NF value may need further discussion |
| Fraunhofer | Issue 3-1: NTN FRF Agree with WF |

Sub topic 3-2 TN parameters

| Company | Comments |
| --- | --- |
| Moderator | Issue 3-7 & 3-8 To CATT Further clarification on “reply LS to ITU R4-xxxxxx” is appreciated. Is it RP-200559 or any other document? CATT: It is LS in R4-2008924. |
| XXX | Issue 3-5: TN Parameters Issue 3-6: TN BS AAS Antenna Issue 3-7: TN BS non-AAS Antenna |
| Ericsson | Issue 3-5: TN Parameters We prefer option 2 with following comments: Network location: we have different proposal, see our table. BS-UE path loss: NB-Iot and NR should be aligned. Antenna pattern: better to use the latest model, the enhanced one or at least the one from TR 38.921. TN #UEs: should we have 100% outdoor for all scenarios? We would still be fine with this. Issue 3-6: TN BS AAS Antenna Option 2 or at least, used the latest agreed model in TR 38.921 Issue 3-7: TN BS non-AAS Antenna Option 2 is common model used in RAN4. Option 1 model is coming from TR 36.873, we are ok to use this one but the parameters are not correct for 2 GHz: vertical coverage should be ~10deg. (and not 30) and the gain should be around 17-18 dBi (not 12). |
| Huawei | Issue 3-5: TN Parameters We can highlight the difference between these two options firstly. Then we can further merge them to reach an agreement. Issue 3-6: TN BS AAS Antenna Option 1 For option 2, we should wait the conclusion on thread [337]. Issue 3-7: TN BS non-AAS Antenna I will check with Ericsson’s proposal. |
| THALES | Issue 3-5: TN Parameters We are fine with the parameters. One question: should we consider different transmission power for the BS (between urban/suburban/macro scenarios for example)? For the time being, it seems that only the propagation factors and the antenna tilt are having a predominant impact. Different antenna gain can be also useful. Issue 3-6: TN BS AAS Antenna To further discuss. Issue 3-7: TN BS non-AAS Antenna To further discuss. Not a strong view. In any case, some beamforming/directional beams on TN BS side would be useful in order to alleviate interference (to or from TN BS). |
| CATT | Issue 3-5: TN Parameters Propose to merge the same ones and highlight the differences to ease the discussion. Issue 3-6: TN BS AAS Antenna Option 1 Issue 3-7: TN BS non-AAS Antenna Option 1. |
| Qualcomm | Issue 3-5: TN Parameters We prefer option 2. Issue 3-6: TN BS AAS Antenna We don’t see the BS AAS antenna model in R4-2008924. Maybe the proponent can help to point it out. Issue 3-7: TN BS non-AAS Antenna Option 2 which was widely used in co-ex on 2GHz. We don’t see the BS non-AAS antenna model in R4-2008924. Maybe the proponent can help to point it out |
| Samsung | Issue 3-5: TN Parameters Support our option 2. Issue 3-6: TN BS AAS Antenna We proposed to use the table 2.3-6 for AAS. And the pattern is indicated as TR 37.842, and other input parameters are in the table. But we also can agree on R4-2008924, where pattern indicated as ITU-R M.2101. And the input parameters are also covered in that LS document. |
| ZTE | Issue 3-5: TN Parameters Option 2 Issue 3-6: TN BS AAS Antenna Antenna is still under the discussion in thread ITU-R rely LS, Issue 3-7: TN BS non-AAS Antenna It might be too time consuming with both AAS and non-AAS |
| Fraunhofer | Issue 3-5: TN Parameters Is there any difference in the options? - Option 1: See Table 2.3-5 and Table 2.3-6 below. - Option 2: See Table 2.3-5 and Table 2.3-6 below. There seems to be two options in Table 2.3-5 for NR TN. With regard to the network location, it should probably read “centre of central beam”. What is the difference between TN NR UL power control in options 1 and 2? Since we are not considering indoor UE, please remove assumptions related to that. Which section 2.9 is referenced in TN NR Link-level performance? Which section 2.8 is referenced in TN NR Evaluation metrics? |

Sub topic 3-3 TN and NTN UE deployment

| Company | Comments |
| --- | --- |
| XXX | Issue 3-8: Deployment of NTN UE Issue 3-9: Principles to consider UE numbers Issue 3-10: Number of active NTN UE Issue 3-11: Indoor/outdoor TN UE |
| Ericsson | Issue 3-8: Deployment of NTN UE Option 4 has some interesting proposal, like considering NTN UE at the edge of TN which is probably worst case for UL-UL. 1) seems also aligned with our proposal. Option 1 is the usual approach. Issue 3-9: Principles to consider UE numbers 1) and 2) NTN UEs density and TN UE density is not really depending on the interference type. 3)it doesn’t have to be a 1 to 1 relation. 4) is ok Issue 3-10: Number of active NTN UE Option 2 but we should make sure NTN UEs are distributed in TN. Issue 3-11: Indoor/outdoor TN UE We are fine with option 2 but wanted to highlight this is not usual assumptions for urban scenarios. |
| Huawei | Issue 3-8: Deployment of NTN UE Option 2 Issue 3-9: Principles to consider UE numbers Should we consider the general principle or we come up with a feasible assumption? Issue 3-10: Number of active NTN UE Option 2 or Option 3 Issue 3-11: Indoor/outdoor TN UE Option 1. |
| Xiaomi | Issue 3-8: Deployment of NTN UE To simplify the simulation, we prefer the option 2 in R4-2106105 that is distributing the NTN UEs within the TN network boundaries or centers randomly corresponding to simulation cases. Issue 3-9: Principles to consider UE numbers No strong view Issue 3-10: Number of active NTN UE Option 2 or Option 3 Issue 3-11: Indoor/outdoor TN UE Option 2 |
| THALES | Issue 3-8: Deployment of NTN UE It can be Option 1 combined with Option 4. We agree with the WF Issue 3-9: Principles to consider UE numbers Option 1 can use the UE density or directly the number of the UEs. With respect to the interference type, the idea was: depending on who is the aggressor, and who is the victim, the simulation approach may be different. For instance, for generating interference, we can consider multiple sources, but for measuring interference, we can use one destination at a time/in different locations (e.g. we randomly change position of the destination over the simulation time). This approach can be also generalized for both NTN and TN, as we could apply the same considerations also for TN. Also, depending on the density or number of the interference sources, we can also compute a mean interference value, and then a total interference value based on the mean interference value. This could further simplify the simulation complexity. Therefore, we can also simplify the simulation (depending on scenario) e.g. with less sources just around the destination, or we can also consider a mean value of interference in order to generate the total interference value (from an average value per source we can obtain total interference from all sources, if we do not want to generate all sources). Issue 3-10: Number of active NTN UE Option 3 or Option 2. Issue 3-11: Indoor/outdoor TN UE Option 2. We can consider only outdoor TN UEs for simplification purposes. |
| CATT | Issue 3-8: Deployment of NTN UE Option 2 Issue 3-9: Principles to consider UE numbers We don’t know what the typical case is. We should consider the impact on co-existence effect. Issue 3-10: Number of active NTN UE Ok to accept Option 2 or Option 3 Issue 3-11: Indoor/outdoor TN UE Option 1. |
| Qualcomm | Issue 3-8: Deployment of NTN UE Option 2. For Option 1, if we randomized NTN UE in NTN cell, in that case the interference will be very low from TN to NTN. Issue 3-9: Principles to consider UE numbers No need to have the principles for UE number if we can have conclusion on Issue 3-8 and 3-10. Issue 3-10: Number of active NTN UE Option 2 but we should make sure NTN UEs are distributed in TN. Issue 3-11: Indoor/outdoor TN UE Option 2 |
| Samsung | Issue 3-8: Deployment of NTN UE Support recommended WF. Issue 3-9: Principles to consider UE numbers Option 1-4) seems a fair approach. Issue 3-10: Number of active NTN UE For simplification, we prefer option 2. But option 3 also provided quite useful information, it may be related to Issue 3-9. Issue 3-11: Indoor/outdoor TN UE We support our option 2, because of O2I model issue. |
| ZTE | Issue 3-8: Deployment of NTN UE Support recommended WF. Issue 3-11: Indoor/outdoor TN UE support Option 2 |
| Fraunhofer | Issue 3-8: Deployment of NTN UE In case the whole NTN cell is overlayed with TN cells, randomly dropping UEs within the NTN area is fine. In case there the aggregated TN area is smaller than the NTN cell, we should probably drop the UEs in the smaller area. Issue 3-10: Number of active NTN UE Option 3 Issue 3-11: Indoor/outdoor TN UE Option 2 |

Sub topic 3-4 Others

| Company | Comments |
| --- | --- |
| XXX | Issue 3-12: NTN UL TPC Issue 3-13: NTN Performance metric Issue 3-14: Throughput ~ SNR mapping Issue 3-15: Interference impact analysis when TN UL is victim |
| Ericsson | Issue 3-12: NTN UL TPC All options are ok. Issue 3-13: NTN Performance metric Option 1 is ok Issue 3-14: Throughput ~ SNR mapping Keep FFS for the time being, not needed anyway for the calibration Issue 3-15: Interference impact analysis when TN UL is victim Option 1. |
| Huawei | Issue 3-12: NTN UL TPC We should define the CLx-ile firstly and then we can know whether 23dBm should be always set. Issue 3-13: NTN Performance metric Option 1. Issue 3-14: Throughput ~ SNR mapping Option 1 Issue 3-15: Interference impact analysis when TN UL is victim Option 1 |
| Xiaomi | Issue 3-12: NTN UL TPC Issue 3-13: NTN Performance metric Issue 3-14: Throughput ~ SNR mapping Option 1 Issue 3-15: Interference impact analysis when TN UL is victim Option 1 |
| THALES | Issue 3-12: NTN UL TPC Option 1 Issue 3-13: NTN Performance metric We can start with Option 1, but for NTN can be higher than 5% (we don’t know yet since simulations are not finished). However, initial (calibration results) show that such value can be possible. Issue 3-14: Throughput ~ SNR mapping Option 1 seems fine. Please note that we should also probably need to define new reference measurement channels for testing purposes (see 312 discussion from RAN4#99e). Issue 3-15: Interference impact analysis when TN UL is victim Option 1 or Option 2, we can also average the interference sources, as previously explained. |
| CATT | Issue 3-12: NTN UL TPC Option 1 Issue 3-13: NTN Performance metric Option 1. Issue 3-14: Throughput ~ SNR mapping Option 1 Issue 3-15: Interference impact analysis when TN UL is victim Option 1 |
| Qualcomm | Issue 3-12: NTN UL TPC Option 3 if satellite operators are OK Issue 3-13: NTN Performance metric Option 1 Issue 3-14: Throughput ~ SNR mapping Option 1 Issue 3-15: Interference impact analysis when TN UL is victim Option 1 |
| Samsung | Issue 3-15: No strong view, but try Option 3: Consider the impact to the whole TN area (19-Cell, 57 Sectors), if NTN UE is dropped within this TN area (19-cell, 57 sectors), we consider the impact. |
| ZTE | Issue 3-12: NTN UL TPC All options should be considered. Issue 3-13: NTN Performance metric Option 1 Issue 3-14: Throughput ~ SNR mapping The original one could be reused, just with no SINR mapping for that range. Issue 3-15: Interference impact analysis when TN UL is victim Option 1. |

Sub topic 3-5 TR 38.811 propagation model clarification

| Company | Comments |
| --- | --- |
| Moderator | Issue 3-16: Propagation model in TR 38.811 Companies who contributed to TR 38.811 section 6.6.2 are invited and strongly encouraged to provide information w.r.t. this matter. |
| Ericsson | Issue 3-16: Propagation model in TR 38.811 As explained in R4-2110119, we noticed some strange behavior with the propagation model described in TR 38.811, the given shadow fading values and resulting SINR cdf. It’s so difficult to trust any simulation outcomes with those observations. Any clarification on this model would be highly appreciated. |
| Huawei | Issue 3-16: Propagation model in TR 38.811 Need further check this issue proponent raised. |
| THALES | Issue 3-16: Propagation model in TR 38.811 Fine with the proposed WF. |
| ZTE | Issue 3-16: Propagation model in TR 38.811 More study is needed. |
| Inmarsat | Issue 3-16: Propagation model in TR 38.811 We are ok with the WF. |
| Fraunhofer | Issue 3-16: Propagation model in TR 38.811 Fine with the proposed WF. With regard to the simulation results presented in R4-2110119, it seems that LOS with no SF std was considered for the “freespace” and “ITU-P619” curves. Not sure why ITU-P.619 is assumed to not consider clutter loss. Please have a look at clause 2.7. The SF std describes the variance around the mean path loss. A value of zero seems to be highly unlikely. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 3-1: NTN FRF | Tentative agreements: Follow RAN4#98bis-e agreement on NTN FRF and update “Simulation assumptions for NTN co-existence” document accordingly. Candidate options:N/A Recommendations for 2nd round:N/A |
| Issue 3-2: Satellite channel bandwidth (BW) | Most companies can live with Option 3: 20MHz BW and there is a question on corresponding FRF. Tentative agreements: Candidate options: - 20MHz (in associated with FRF=1) Recommendations for 2nd round: Try to agree on the candidate option. |
| Issue 3-3: Satellite max TX power | All agree with Option 1 by adding Tx Power for 20MHz BW. Tentative agreements: Update Option 1 by adding Tx Power for 20MHz BW. Candidate options:N/A Recommendations for 2nd round:N/A |
| Issue 3-4: NTN system Noise figure in dB | No agreements can be made Tentative agreements:N/A Candidate options: - Option 2 Recommendations for 2nd round: Further discuss NTN NF value based on Option 2. |
| Issue 3-5: TN Parameters | Tentative agreements:N/A Candidate options:N/A Recommendations for 2nd round: Merge same items of Option 1 & 2 and highlight different ones for further discussion. |
| Issue 3-6: TN BS AAS Antenna | 3 companies support Option 1 and 1 company support Option 2. It should be noted relative for Option 2 discussion on going in thread 337 ITU-R reply LS Tentative agreements:N/A Candidate options: - Option 1 - Option 2 - FFS Recommendations for 2nd round: Try to agree on Option 1, which means AAS antenna pattern refers to that in ITU-R M.2101 and input parameters follow Table 2.3-6 (Antenna row will be updated to ITU-R M.2101). Note 1: TR 38.921 pattern is the same with ITU-R M.2101 pattern. |
| Issue 3-7: TN BS non-AAS Antenna | 1 company supports Option 1, 1 company supports Option with modifications and 1 company support Option 2. There’s a concern raised w.r.t consumption to consider both AAS and non-AAS BS. Tentative agreements: Candidate options: - Option 1 with modifications that vertical coverage should be ~10deg. (and not 30) and the gain should be around 17-18 dBi (not 12). - Option 2 - FFS Recommendations for 2nd round: - Proponents of Option1 are invited to introduce where non-AAS BS has been defined in R4-2008924. - Try to agree on Option 1 with proposed modification as Option 2 does not have vertical values for BS antenna which would be important for NTN studies. |
| Issue 3-8: Deployment of NTN UE | Tentative agreements: N/A Candidate options: - Merged Option 1 to 4: NTN UE should be randomly generated within the NTN area. And when co-locate with TN network, co-ex study only consider those NTN UE dropped within the TN network. The simulation should follow similar Monte-Carlo approach as for TN simulation assumptions, and FFS Option 4 - New Option: distribute the NTN UEs within the TN network boundaries or centers randomly corresponding to simulation cases. Recommendations for 2nd round: Further discuss candidate options in 2nd round |
| Issue 3-9: Principles to consider UE numbers | Difference views have been expressed on this issue and no consensus can be made so far. Tentative agreements:N/A Candidate options: - Option 1 Recommendations for 2nd round: Further discuss Option 1 and it may be resolved based on results of Issue 3-8 and 3-10. |
| Issue 3-10: Number of active NTN UE | 5 companies are OK to both Option 2 and Option 3 2 support Option 2 with the condition that NTN UE should be distributed in TN networks. 1 company supports Option 3 only. Tentative agreements: N/A Candidate options: - Option 2 - Option3 Recommendations for 2nd round: Try to agree on Option 2. |
| Issue 3-11: Indoor/outdoor TN UE | 7 companies are OK with Option 2 and 2 companies support Option 1. Tentative agreements: N/A Candidate options: - Option 1 - Option 2 Recommendations for 2nd round: Try to agree on Option 2 for simplification purpose. [Observation from moderator: Option 2 seems the worst case for both NTN to TN and TN to NTN scenarios.] |
| Issue 3-12: NTN UL TPC | Diverse views have been expressed and no consensus can be made so far. Tentative agreements: N/A Candidate options: - Option 1 - Option 2 - Option 3 Recommendations for 2nd round: - Use Option 3 as a starting point for calibration. - FFS these options in 2nd round. |
| Issue 3-13: NTN Performance metric | 5 companies support Option 1 Tentative agreements: The average throughput loss and 5%-ile throughput loss should be less than 5%. Candidate options:N/A Recommendations for 2nd round:N/A |
| Issue 3-14: Throughput ~ SNR mapping | 5 companies support Option 1. 1 company proposes FFS. 1 company proposes to reuse the original one. Tentative agreements:N/A Candidate options: - Option 1 Recommendations for 2nd round: Try to agree with Option 1 as the starting point. |
| Issue 3-15: Interference impact analysis when TN UL is victim | 6 companies support Option 1. 1 company is fine with both Option 1 and 2. A new proposal was made to consider the impact to the whole TN area. Tentative agreements:N/A Candidate options: - Option 1 - Option 2 - Option 3 (new): Consider the impact to the whole TN area (19-Cell, 57 Sectors), if NTN UE is dropped within this TN area (19-cell, 57 sectors), we consider the impact. Recommendations for 2nd round: Further discuss candidate options in 2nd round. |
| Issue 3-16: Propagation model in TR 38.811 | All agree to further study this issue. Tentative agreements: Companies who contributed to TR 38.811 section 6.6.2 are invited and strongly encouraged to provide information w.r.t. this matter. Candidate options:N/A Recommendations for 2nd round:N/A |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #4: HAPS

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2110119 | Ericsson | Proposal 10: Adopt the antenna parameters proposed in Table 5 for HAPS simulations. Table 5: HAPS antenna parameters Number of cells; 7. Antenna array configuration (row x column); 2 x 2 for 1st layer cell 4 x 2 for 2nd layer cell. Antenna polarization; Linear ±45°. Element gain; 5.5 dBi. Loss; 2.0 dB. Element HPBW horizontal/vertical; 90° for both H/V. Element front-to-back ratio horizontal/vertical; 30 dB for both H/V. Element spacing horizontal/vertical; 0.5 wavelength for both H/V. Antenna panel tilt (from the horizon); 90° for 1st layer cell 23° for 2nd layer cell. Tx power per antenna panel; 43 dBm. Noise figure; 5 dB. Indoor UE percentage; 0%. Coverage area (7 cells combined); A 100 Km radius circular area centered by the serving HAPS. UE distribution; Uniformly distributed in the coverage area. |
| R4-2110689 | Nokia, Nokia Shanghai Bell | Proposal 1: For the HAPS network, UL scheduled bandwidth is 2 RBs per UE and 10 UEs are scheduled per cell. Scheduled UE resources are randomly distributed across the bandwidth. Proposal 2: For the TN network, UL scheduled bandwidth is 16 RBs per UE and 6 UEs are scheduled per cell. Scheduled UE resources are randomly distributed across the bandwidth. Proposal 3: Use the following parameters to set the UE’s UL transmit power in the agreed UL power control model: UL power control parameter; TN; HAPS. Pmax (dBm); 23; 23. Rmin (dB); -54; -54. γ; 1; 1. X, transmission bandwidth (MHz); 2.88; 0.36. Y, BS noise figure (dB); 5; 5. |
| R4-2110800 | Qualcomm | Proposal 1: the coexistence simulation layout between HAPS and TN shown in Figure 3 is proposed to be considered. Figure 3. The simulation layout between HAPS and TN Proposal 2: The HAPS EIRP is proposed to use 55dBm for the 1st layer cell and 58dBm for the 2nd layer cell. Proposal 3: The UE noise figure is proposed to be 9dB. Proposal 4: For uplink simulations, 3UEs are usually assumed for simultaneous transmission in TN network. For the number of scheduled UEs per HAPS cell, two options, 3 or 10, can be considered as candidates. Proposal 5: The CLx-ile value can be determined by the targeted SNR value. The targeted SNR value is related with operator’s deployment requirement, and it would be much helpful if operator could comment on the target SNR value in HAPS system. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 4-1

This sub-topic focus on network layouts of HAPS co-existence study.

Open issues and candidate options before e-meeting:

Issue 4-1: Layout between HAPS and TN

- Proposals

  - Option 1: Consider the coexistence simulation layout between HAPS and TN  shown below

In every snapshot of the Monte Carlo simulation, one TN cluster is dropped randomly into HAPS coverage. Each cluster consists of 19 cells, 57 sectors. Through large number of snapshots, the TN clusters can locate at different places of HAPS coverage. The HAPS UE and TN UE are uniformly distributed in the TN cluster’s coverage.  By wrap-around, each HAPS UE is surrounded by TN UEs in three layers cells. This can be considered as the worst case. In realistic deployment, the HAPS UEs are very sparse.

Random dropping TN cluster is the easiest way to implement to cover different places of HAPS area. A candidate location set can be considered as the other option to implement, which has higher complexity but also higher accuracy to model the interference. A candidate location set contains the possible centers of TN cluster in the simulation, and the center of TN cluster will select one from the candidate location set for each snapshot.  For example, the center-to-center inter-system distance 0, 10, 20, 30, 40, 50km in Table 1 can be considered as a candidate location set, but the size is not enough to cover different places in HAPS area.  The set size can be flexible, which depends on the granularities.

  - Option 2: TBA

- Recommended WF

  - TBA

Sub-topic 4-2

This sub-topic focus on HAPS parameters.

Open issues and candidate options before e-meeting:

Issue 4-2: Uplink scheduled bandwidth

- Proposals

  - Option 1:

- For the HAPS network, UL scheduled bandwidth is 2 RBs per UE and 10 UEs are scheduled per cell. Scheduled UE resources are randomly distributed across the bandwidth.

- For the TN network, UL scheduled bandwidth is 16 RBs per UE and 6 UEs are scheduled per cell. Scheduled UE resources are randomly distributed across the bandwidth.

  - Option 2:

- For the HAPS network, 3 or 10

- For the TN network, 3UEs are usually assumed for simultaneous transmission

- Recommended WF

  - TBA

Issue 4-3: Antenna parameters

- Proposals

  - Option 1: See changes marked in yellow below

  - Option 2: See changes marked in yellow below

|  | Option 1 | Option 2 |
| --- | --- | --- |
| Element gain | 5.5 dBi | 8dBi |
| Loss | 2.0 dB |  |
| Element HPBW horizontal/vertical | 90° for both H/V | 65° for both H/V |
| Tx power per antenna panel | 43 dBm |  |
| HAPS Platform e.i.r.p./cell |  | 55 dBm (1st layer cell), 58 dBm (2nd layer cell) |
| HAPS Platform e.i.r.p. Spectral Density/cell |  | 42 dBm/MHz (1st layer cell), 45 dBm/MHz (2nd layer cell) |

- Recommended WF

  - TBA

Issue 4-4: HAPS UE Noise Figure

- Proposals

  - Option 1: 9dB

- Recommended WF

  - TBA

Issue 4-5: HAPS UE UL TPC

- Proposals

  - Option 1: Use the following parameters to set the UE’s UL transmit power in the agreed UL power control model:

| UL power control parameter | TN | HAPS |
| --- | --- | --- |
| Pmax (dBm) | 23 | 23 |
| Rmin (dB) | -54 | -54 |
| γ | 1 | 1 |
| X, transmission bandwidth (MHz) | 2.88 | 0.36 |
| Y, BS noise figure (dB) | 5 | 5 |

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Issue 4-1

| Company | Comments |
| --- | --- |
| Nokia | Issue 4-1: Option 1 requires randomly drops of TNs in the HAPS coverage area. How many TNs to drop is also another question. How many simulation drops are needed to get statistically stable results is yet another question. In our view this is overly complicated – not only make simulations more difficult to run, but also make the results more difficult to understand. We prefer the simpler model in RAN4#98bis-e HAPS simulation assumption (R4-2106106), where HAPS and TN are separated by a distance: Simulations can be run with various HAPS-TN inter-system distances. Not only simulations would be easier to execute and calibrate, but the impact of ACI at different scenarios can be directly compared from the results. |
| Qualcomm | Issue 4-1: Option 1. Option 1 is line with the approach used for NTN-coexistence. For most of the cases, only one TN network (19cell with 57 sectors) randomly distributes in the NTN area. For the following two cases, more TN networks might be needed since the interference is related with BS sites number. But it can be further discussed if we need to consider the following two cases in HAPS co-ex simulation. No.; Combination; Aggressor; Victim. 1; TN - NTN; TN DL (TN BS); NTN UL (NTN satellite). 2; TN - NTN; TN UL (TN UE); NTN UL (NTN satellite). 3; To be added. If snapshot number is enough, there is no issues to cover all the NTN area (The similar approach is used in NTN co-ex even the footprint for GEO/LEO is much larger than HAPS). For the proposal in R4-2106106, the issue is it only considers several center distances that could not cover all the possible area of NTN. And how to assume the distance will have big impact on the final results. For example, if the points marked in the figure are selected, the SINR for HAPS would be small. |

Issue 4-2

| Company | Comments |
| --- | --- |
| XXX |  |
| Ericsson | Option 2 |
| Huawei | Option 2 |
| Nokia | Issue 4-2: Support Option 1. In particular, we need to consider its power limited nature of HAPS UL. For most UEs to meet the required minimum SINR, the scheduled bandwidth for HAPS UE needs to be small. |
| CATT | Option 2 |
| Qualcomm | Option 2. And we can assume the same uplink scheduled bandwidth for TN and NTN for simplify the ACIR modeling in the simulation. |

Issue 4-3

| Company | Comments |
| --- | --- |
| Ericsson | Option 1. Option 2 is not correct and has been fixed. Option 2 would result on errors. |
| Huawei | OK with option 1 |
| Nokia | Issue 4-3: We can agree to add a 2 dB cable loss to be consistent with the ITU proposal. With that, the EIRP of Option 2 is consistent with the parameters in RAN4#98bis-e HAPS simulation assumption (R4-2106106). And the HAPS antenna parameter table can be expanded as follows. Antenna array configuration (row x column); 2 x 2 for 1st layer cell 4 x 2 for 2nd layer cell. Antenna polarization; Linear ±45°. Element gain; 8 dBi. Element HPBW horizontal/vertical; 65° for both H/V. Element front-to-back ratio horizontal/vertical; 30 dB for both H/V. Element spacing horizontal/vertical; 0.5 wavelength for both H/V. Antenna panel tilt (from the horizon); 90° for 1st layer cell 23° for 2nd layer cell. Tx power per antenna panel; 43 dBm. Cable loss; 2 dB. HAPS EIRP/cell; 55 dBm (1st layer cell), 58 dBm (2nd layer cell). HAPS EIRP spectral density/cell; 42 dBm/MHz (1st layer cell), 45 dBm/MHz (2nd layer cell). |
| CATT | OK with Option 1 |
| Qualcomm | OK with option 1 but need clarification for the definition of loss. What kind of loss here in the antenna model? Cable loss? |
| SoftBank | Agree with new table Nokia updated above. |

Issue 4-4

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Huawei | Option 1 |
| Nokia | Issue 4-4: We agree to change UE noise figure to 9 dB. Note: HAPS UE has the same assumption as TN UE. |
| CATT | Option 1 |
| Qualcomm | Option 1 |

Issue 4-5

| Company | Comments |
| --- | --- |
| Ericsson | Transmission BW should be aligned with number of scheduled UEs. |
| Nokia | Issue 4-5: Support Option 1. |
| Qualcomm | Transmission BW should be aligned with number of scheduled UEs. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 4-1: Layout between HAPS and TN | No agreements can be made so far Tentative agreements:N/A Candidate options: - Option 1 - Layouts in R4-2106106 Recommendations for 2nd round: Further discuss candidate options in 2nd round |
| Issue 4-2: Uplink scheduled bandwidth | 4 companies support Option 2 and 1 supports Option 1. Tentative agreements:N/A Candidate options: - Option 1 - Option 2 Recommendations for 2nd round: Try to agree on Option 2. |
| Issue 4-3: Antenna parameters | 4 companies support Option 1 and 2 supports Option 2 with updates. Tentative agreements:N/A Candidate options: - Option 1 - Option 2 with updates Recommendations for 2nd round: Further discuss candidate options in 2nd round |
| Issue 4-4: HAPS UE Noise Figure | All are OK with Option 1: 9dB. Tentative agreements: HPAS UE NF=9dB Candidate options:N/A Recommendations for 2nd round:N/A |
| Issue 4-5: HAPS UE UL TPC | 1 company supports Option 1. 2 companies is of the view that the parameter “Transmission BW” in Option 1 should be aligned with number of scheduled UEs. Tentative agreements:N/A Candidate options: - Option 1 Recommendations for 2nd round: Further discuss Option 1 esp. the Transmission BW in 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #5: Calibration and alignment

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2110121 | Ericsson | TN: DL SINR cdf , the UE Tx cdf and the UL SINR cdf of Rural & Urban macro NTN: DL SINR cdf, UE Tx power cdf, and UL SINR cdf of LEO600, LEO1200 & GEO observed on a TN layout area LEO600 - DL SINR cdf with 5 MHZ (left) and 30MHz (right) channel BW LEO600 - UL SINR cdf with 5 MHZ (left) and 30MHz (right) channel BW LEO1200 - DL SINR cdf with 5 MHZ (left) and 30MHz (right) channel BW LEO1200 - UL SINR cdf with 5 MHZ (left) and 30MHz (right) channel BW GEO - DL SINR cdf with 5 MHZ (left) and 30MHz (right) channel BW GEO - UL SINR cdf with 5 MHZ (left) and 30MHz (right) channel BW HAPS: DL SINR cdf, UE Tx power cdf, and UL SINR cdf observed on a TN layout area LEO600: It could be noticed that UEs are always transmitting at maximum power, whatever its location is in the TN area. Also, as expected with such small area, DL SINR and UL SINR have small variations (~0.04dB delta) over the all area. LEO1200: It could be noticed that UEs are always transmitting at maximum power, whatever its location is in the TN area. Also, as expected with such small area, DL SINR and UL SINR have small variations (~0.01dB delta) over the all area. Looking at UL SINR values, it seems 30MHz channel BW would be too high value for LEO1200 satellite. GEO: It could be noticed that UEs are always transmitting at maximum power, whatever its location is in the TN area. Also, as expected with such small area, DL SINR and UL SINR have small variations (0.001dB delta) over the all area. Looking at UL SINR values, it seems 30MHz channel BW would be much too high value for GEO satellite. HAPS: It could be noticed that UEs make use of the power control in this case, when located straight under the HAPS. Also, as the zone which was observed is relatively small (a TN) comparing to the HAPS coverage area, small variation is observed for the UL SINR values and the DL SINR values. |
| R4-2110193 | Xiaomi | Simulation results for NTN DL to TN DL （CBW=30MHz for NTN，CBW=20MHz for TN） |
| R4-2110690 | Nokia, Nokia Shanghai Bell | Alignment Figure 1. DL SINR of TN in UMa and RMa environments Figure 2. DL SINR of HAPS in rural environment (a) (b) Figure 3. DL SINR of TN with the presence of HAPS ACI Figure 4. UL SINR of HAPS Figure 5. UL SINR of HAPS with the presence of TN ACI Results Table 5. Worst case ACIR for 5% throughput loss in HAPS UL Simulation scenario; ACIR for 5% average throughput loss (dB); ACIR for 5% cell-edge throughput loss (dB). Rural macro TN + HAPS; 13.9; 24.2. HAPS + HAPS; 9.7; 26.2. Observation 1: For HAPS UL, ACI causes a higher degradation in cell-edge throughput than in average throughput. Observation 2: Initial simulation results indicate that the required ACS for HAPS is 29 dB. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 5-1

Open issues and candidate options before e-meeting:

Issue 5-1: Calibration

- Proposals

  - Option 1: Develop tables to calibrate following values for TN, NTN and HAPS respectively.

- TN: DL SINR cdf , the UE Tx cdf and the UL SINR cdf

- NTN: DL SINR cdf , the UE Tx cdf and the UL SINR cdf

- HAPS: DL SINR cdf , the UE Tx cdf and the UL SINR cdf

- Recommended WF

  - Agree on Option 1

  - Conduct the calibration during and after this meeting.

  - Finalize the calibration by RAN4#100e.

Companies views’ collection for 1st round

Open issues

Sub topic 1-1

| Company | Comments |
| --- | --- |
| Ericsson | We support the recommended WF. |
| Huawei | OK with Option 1. |
| Xiaomi | We support the recommended WF. |
| Nokia | We are fine with the recommended WF. |
| THALES | Fine with recommended WF. |
| CATT | We support the recommended WF. |
| Qualcomm | OK with recommended WF. Some clarification questions to better understand the results: For R4-2110121, what’s the UE bandwidth and UE number used in the simulation. Seems the all the UEs are not reaching 23dBm. For R4-2110689, in TR36.942, UE bandwidth is 16RB, but this assumption is based on 3 UEs and 10MHz bandwidth in total. When the bandwidth increasing from 10MHz to 20MHz, does the paper use (16RB per UE, 6 UE in total ) or (3 UE, 32 RB per UE)? For R4-2110690, why UL SINR of HAPS is up to 30dB, which is different with the 15dB assumption? |
| Samsung | Support recommended WF. |
| ZTE | Support recommended WF. |
| Inmarsat | Ok with recommended WF |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 5-1 Calibration | Most agree with the recommended WF. Questions have been made to proponents w.r.t simulation results in R4-2110121, R4-2110689 and R4-2110690 Tentative agreements: 1) Develop tables to calibrate following values for TN, NTN and HAPS respectively. - TN: DL SINR cdf , the UE Tx cdf and the UL SINR cdf - NTN: DL SINR cdf , the UE Tx cdf and the UL SINR cdf - HAPS: DL SINR cdf , the UE Tx cdf and the UL SINR cdf 2) Conduct the calibration during and after this meeting. 3) Finalize the calibration by RAN4#100e. Candidate options:N/A Recommendations for 2nd round:N/A |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on [313] NTN_Solutions_Part2 | Samsung |  |
| Simulation assumptions for NTN co-existence | Samsung, CATT |  |
| Simulation assumptions for HAPS co-existence | TBA |  |
| Calibration table for [313] NTN_Solutions_Part2 | TBA |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Appendix 1. TDOC list for this agenda

A total of 14 TDOCs have been provided for this agenda listed as below.

| TDoc Number | TDoc Type | Title | Company | Status | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- | --- |
| R4-2109118 | discussion | Updated simulation assumptions for NTN co-existence | CATT | available | Approval | 9.12.2.1 |
| R4-2109119 | discussion | Comparison of co-existence performance w/w.o. consideration on earthe curvature | CATT | available | Discussion | 9.12.2.2 |
| R4-2109544 | discussion | Proposed simulation assumptions for NTN co-existence study | Samsung | available | Approval | 9.12.2.1 |
| R4-2109645 | discussion | On simplification of TN UL --> NTN UL simulation | CATT | available | Approval | 9.12.2.1 |
| R4-2110119 | other | NTN Simulations assumptions | Ericsson | available | Approval | 9.12.2.1 |
| R4-2110121 | discussion | NTN - simulation results for alignment | Ericsson | available | Discussion | 9.12.2.2 |
| R4-2110193 | discussion | Preminary simulation result for coexistence study on NR to support non-terrestrial networks | Xiaomi | available | Discussion | 9.12.2.2 |
| R4-2110412 | other | Further discussion on NTN simulation assumptions | Huawei, HiSilicon | available | Approval | 9.12.2.1 |
| R4-2110689 | discussion | HAPS simulation assumptions for coexistence study | Nokia, Nokia Shanghai Bell | available | Approval | 9.12.2.1 |
| R4-2110690 | discussion | HAPS adjacent channel coexistence simulation results | Nokia, Nokia Shanghai Bell | available | Approval | 9.12.2.2 |
| R4-2110799 | discussion | Simulation scenarios and assumptions for NTN co-existence | Qualcomm Incorporated | available |  | 9.12.2.1 |
| R4-2110800 | discussion | HAPS coexistence simulation assumptions | Qualcomm Incorporated | available |  | 9.12.2.1 |
| R4-2111423 | discussion | Reference Deployment Scenario for NTN MSS S-Band | THALES | available | Discussion | 9.12.1.1 |
| R4-2111462 | discussion | On the S-band NTN Scenarios and Parameters for Coexistence Study | THALES | available | Discussion | 9.12.2.1 |