3GPP TSG-RAN WG4 Meeting # 99-e	R4-2108639

# Electronic Meeting, May 19 - 27, 2021

Source: 	Huawei

Title: 	WF on BS RX RF requirements for 52.6 – 71 GHz

Agenda Item:	9.15.5

Document for:	Approval

# Introduction

Contributions treating on BS RF Rx requirements were submitted to the RAN4#99-e meeting in [2-5]. Those were discussed during the first round on for [99-e][315] NR_exto71GHz_BSRF topic in [1].

Based on the discussion in [1], number of proposals were formulated in section 2.

# Agreements

In the following clauses, WF agreements are listed, unless otherwise stated:

# Way forward on general and sensitivity related requirements

- Sensitivity levels

  - Use the same principle for derivation of sensitivity level requirements as for current FR2 (i.e. declaration of sensitivity level),

  - Further check if sensitivity declarations based on 100 MHz reference measurement channel can be the basis instead of 50MHz (as in existing FR2),

  - Further check whether the allowed range of sensitivity values to be declared needs an update.

- FRCs

  - FRCs for 52.6 GHz – 71 GHz include 100 MHz/120 kHz, 400 MHz/480 kHz, 400 MHz/960 MHz are to be defined

  - Further check on the possibility to reuse parameters from FR2.

# Way forward on ACS and in-band blocking

- ACS

The following options were discussed:

- The middle ACS value between the values at 50GHz and 70GHz in TR 38.803 (i.e. 22 dB) can be adopted for NR operation in 52.6 – 71 GHz range.

- Agree ACS of [22-23] dB with 100 MHz interfering signal BW

- Should wait for the conclusion of co-existence simulation discussion.

The following is agreed:

- Postpone the decision till the co-existence simulation discussion is concluded, once this is done collect more analyses, including:

  - For the ACS values: provide further analyses and motivation for ACS other than options a and b above.

  - Re-evaluate the interferer signal type, including its SCS, channel bandwidth and frequency offset.

- In-band blocking

The following options were discussed:

- Option 1: In-band blocking level of BS type 2-O can be used as baseline for NR operation in 52.6 – 71 GHz range, but consideration should be placed to ensure alignment between in-band selectivity and ACS.

- Option 2: 33dB offset from the reference sensitivity used for interfering signal is applied for 60GHz

The following is agreed:

  - Option 1 as baseline, i.e. In-band blocking level of BS type 2-O can be used as baseline for NR operation in 52.6 – 71 GHz range, but consideration should be placed to ensure alignment between in-band selectivity and ACS.

(option 2 was the FR2 assumption, i.e. the FR2 interferer power level was 33dB higher than the OTA REFSENS level)

  - Re-evaluate the interferer signal type, including its SCS, channel bandwidth.

# Way forward on OOB blocking

- Blocker level

Option 1: reuse current existing FR2 OOB blocker level for 52.6-71GHz

- Frequency definitions

  - For out-of-band blocking RF core requirement define the interferer signal upper frequency limit to 2nd harmonic similar as for FR2.

  - For receiver blocking further consider ΔfOBUE and decide if ΔfOOB needs to be aligned

  - Postpone measurement step size discussion to performance part.

# For further considerations

- Receiver spurious emissions

Option 1: Align the general receiver spurious emissions limits with Tx general spurious emission limits.

Option 2: use ETSI EN 303 722 for unlicensed operation in Europe (regional requirement).

- Rx IMD

The interferer levels for general receiver intermodulation for NR operation in 52.6 – 71 GHz range can be further discussed and decided once the in-band blocking and ACS levels are agreed.

- ICS

ICS can be further discussed and decided based on IoT level obtained from SLS results.

# References

- R4-2108440		Email discussion summary for [99-e][315] NR_exto71GHz_BSRF, Moderator (Nokia)

- R4-2109115		Discussion on BS RX RF requirements for 52 6-71GHz, CATT

- R4-2109385		Proposals on BS receiver requirements for extending current NR operation to 71 GHz, Nokia, Nokia Shanghai Bell

- R4-2109871		On BS RF receiver requirements for the frequency range 52 to 71 GHz, Ericsson

- R4-2110602		Discussion on BS Rx requirements for 60GHz, ZTE corporation