3GPP TSG-RAN WG4 Meeting #99-e	R4-2108750

# Electronic meeting, 19 – 27 May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.105 | CR | 0230 | rev | 1 | Current version: | 15.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.105: Regional requirements for band 41 in Japan, Rel-15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, SoftBank Corp., KDDI Corporation, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Additional requirements for Band 41 are not aligned with the regional requirements in Japan |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the regional requirements list. Deleted addional requirements related to Band 41 in Japan. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Additional requirements are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5, 6.2.2.2.2, 6.2.2.4.2, 6.7.2.4, 6.6.5.4.7, 6.6.6.4, 6.7.4.2, 9.3.2.2.2, 9.3.2.4.2, 9.7.5.2.4.6, 9.7.5.4.6.4, 9.7.6.4.3.2, 9.8.2.4, 9.8.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS37.145-1, TS37.145-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

4.5	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or as mandatory requirements set by local and regional regulation. It is normally not stated in the 3GPP specifications under what exact circumstances the regional requirements apply, since this is defined by local or regional regulation.

Table 4.5-1 lists all requirements in the present specification that may be applied differently in different regions. Non-AAS BS requirements are applicable as defined in the present document. In many cases, such requirements include regional requirements that are implicitly referenced from the present specification, and listed in the specification for the specifications concerned 3GPP TS 25.104 [2], 3GPP TS 25.105 [3], 3GPP TS 36.104 [4] and 3GPP TS 37.104 [5].

Table 4.5-1: List of regional requirements

| Clause number | Requirement | Comments |
| --- | --- | --- |
| 4.6 | Operating bands and Band Categories | Some operating bands may be applied regionally. |
| 6.6.2, 9.7.2 | Occupied bandwidth and OTA Occupied bandwidth | The requirement may be applied regionally. There may also be regional requirements to declare the Occupied bandwidth according to the definition. |
| 6.6.4, 9.7.4 | Spectrum emission mask and OTA Spectrum emission mask | The mask specified may be mandatory in certain regions. In other regions this mask may not be applied. Additional spectrum protection requirements may apply regionally. |
| 6.6.5, 9.7.5 | Operating band unwanted emissions and OTA Operating band unwanted emissions | Category A or Category B operating band unwanted emissions limits may be applied regionally. |
| 6.6.5, 9.7.5 | Operating band unwanted emissions and OTA Operating band unwanted emissions | The BS may have to comply with the applicable emission limits established by FCC Title 47 [15], when deployed in regions where those limits are applied and under the conditions declared by the manufacturer. |
| 6.6.5, 9.7.5 | Operating band unwanted emissions and OTA Operating band unwanted emissions | The requirements for protection of DTT may apply regionally. |
| 6.6.5, 9.7.5 | Operating band unwanted emissions and OTA Operating band unwanted emissions | Regional requirement as defined in 3GPP TS 37.104, subclause 6.6.2.4.4 [9] may be applied for the protection of systems operating in frequency bands adjacent to band 1 as defined in 3GPP TS 37.104, subclause 4.5, [9] in geographic areas in which both an adjacent band service and UTRA and/or E-UTRA are deployed. |
| 6.6.5, 9.7.5 | Operating band unwanted emissions and OTA Operating band unwanted emissions | Additional requirements defined for Band 24 in 3GPP TS 37.104, subclause 6.6.2.4.5 may apply in regions where FCC regulation applies. |
| 6.6.5, 9.7.5 | Operating band unwanted emissions and OTA Operating band unwanted emissions | Additional band 32 unwanted emissions requirements may apply in certain regions |
| 6.6.6, 9.7.6 | Spurious emissions and OTA Spurious emissions | Category A limits are mandatory for regions where Category A limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [14] apply. Category B limits are mandatory for regions where Category B limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [14] apply. |
| 6.6.6, 9.7.6 | Spurious emissions and OTA Spurious emissions | Additional spurious emissions requirements may be applied for the protection of system operating in frequency ranges other than the AAS BS operating band as described in 3GPP TS 37.104 [9] subclause 6.6.1.3 (NOTE). |
| 6.6.6, 9.7.6 | Spurious emissions and OTA Spurious emissions | In addition to 3GPP requirements, the BS may have to comply with the applicable emission limits established by FCC Title 47 [15], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |
| 6.6.6, 9.7.6 | Spurious emissions and OTA Spurious emissions | The emission limits specified as the basic limit + X (dB) are applicable, unless stated differently in regional regulation. |
| 6.7, 9.8 | Transmitter intermodulation and OTA Transmitter intermodulation | Additional requirements may apply in certain regions. |
| 7.5, 10.6 | Blocking and OTA Blocking | For the Public Safety LTE BS in Korea from 718 to 728 MHz in Band 28, regional blocking requirement is specified in TS 36.104 [8], subclause 7.6.3. |
| 7.6, 10.7.4 | Rx spurious emissions and OTA Rx Spurious emissions | The emission limits specified as the basic limit + X (dB) are applicable, unless stated differently in regional regulation. |

[Unaffected Portions Skipped]

6.2	Base station output power

6.2.1	General

The configured carrier power is the target maximum power for a specific carrier for the operating mode set in the BS within the limits given by the manufacturer's declaration.

6.2.2	Maximum output power

6.2.2.1	General

The rated carrier output power of the AAS BS shall be as specified in table 6.2.2.1-1.

Table 6.2.2.1-1: AAS BS rated output power limits for BS classes

| AAS BS class | PRated,c,sys | PRated,c,TABC |
| --- | --- | --- |
| Wide Area BS | (NOTE) | (NOTE) |
| Medium Range BS | ≤ 38 dBm +10log(NTXU,counted) | ≤ 38dBm |
| Local Area BS | ≤ 24 dBm +10log(NTXU,counted) | ≤ 24dBm |
| NOTE: There is no upper limit for the PRated,c,sys or PRated,c,TABC of the Wide Area Base Station. |  |  |

6.2.2.2	Minimum requirement for MSR operation

6.2.2.2.1	General

In normal conditions, Pmax,c,TABC shall remain within +2 dB and -2 dB of the configured carrier power for each TAB connector as declared by the manufacturer.

In extreme conditions, Pmax,c,TABC shall remain within +2,5 dB and -2,5 dB of the configured carrier power for each TAB connector as declared by the manufacturer.

In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

6.2.2.2.2	Additional requirements (regional)

6.2.2.3	Minimum requirement for single RAT UTRA operation

The minimum requirement for single RAT UTRA BS is the same as that defined in subclause 6.2.2.2.

6.2.2.4	Minimum requirement for single RAT E-UTRA operation

6.2.2.4.1	General

The minimum requirement for single RAT E-UTRA BS is the same as that defined in subclause 6.2.2.2.

6.2.2.4.2	Additional requirements (regional)

[Unaffected Portions Skipped]

6.6.5.4.7	Additional requirements

The E-UTRA operating band unwanted emission basic limits for additional requirements are the same as the basic limit is specified in 3GPP TS 36.104 [8], subclause 6.6.3.3.

Additional requirements specified in TS 36.104 [8], subclause 6.6.3.3 for Band 41 and Band 49 are not applicable for AAS BS.

[Unaffected Portions Skipped]

6.6.6.4	Minimum requirement for single RAT E-UTRA operation

Single RAT E-UTRA operation spurious emission basic limits are the same as those specified in 3GPP TS 36.104 [8], subclauses 6.6.4.1-6.6.4.4. Spurious emission limits specified in TS 36.104 [8], subclauses 6.6.4.1-6.6.4.4 for Band 41, Band 46 and Band 49 are not applicable for AAS BS.

[Unaffected Portions Skipped]

6.7.2.4	Additional co-location minimum requirements

[Unaffected Portions Skipped]

6.7.4.2	Void

-

Table 6.7.4.2-1: 
Void

[Unaffected Portions Skipped]

9.3.2.2	Minimum requirement for MSR operation

9.3.2.2.1	General

In normal conditions, Pmax,c,TRP shall remain within +2,0 dB and -2,0 dB of the configured carrier TRP as declared by the manufacturer.

In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

9.3.2.2.2	Additional requirements (regional)

9.3.2.3	Minimum requirement for single RAT UTRA operation

The minimum requirement for single RAT UTRA BS is the same as that defined in subclause 9.3.2.2.

9.3.2.4	Minimum requirement for single RAT E-UTRA operation

9.3.2.4.1	General

The minimum requirement for single RAT E-UTRA BS is the same as that defined in subclause 9.3.2.2.

9.3.2.4.2	Additional requirements (regional)

[Unaffected Portions Skipped]

9.7.5.2.4.6	Void

Table 9.7.5.2.4.6-1: Void

|  | - |  |  |  |
| --- | --- | --- | --- | --- |

[Unaffected Portions Skipped]

9.7.5.4.6.4	Void

Table 9.7.5.4.6.4-1: Void

|  | - |  |  |  |
| --- | --- | --- | --- | --- |

[Unaffected Portions Skipped]

Table 9.7.6.4.3.2-5: Void

[Unaffected Portions Skipped]

9.8.2.4	Additional minimum requirements

[Unaffected Portions Skipped]

9.8.4.2	Void

-

Table 9.8.4.2-1: Void

[End of change]