3GPP TSG-RAN4 Meeting #99-e	R4-2108805

Online, , 19th May 2021 - 27th May 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 					On UL Duty Cycle method for NR uplink inter band CA

Agenda Item: 			8.35.2

Document for:	Approval

# 1	Introduction

RAN4#98-bis-e further discussed how to handle SAR candidate solutions and approved an WF of [1], where the following formulas are captured.

This contribution focuses on discussing Duty Cycle based solutions, specifically on how to handle SARratioNR.

# 2	Discussion

If we substitute the formula (2) into the formula (1), we get the following.

DutyNR, x *( PNR,x/ P26)* 50%/DutycycleNR, x + DutyNR, y *(PNR, y/ P26)* 50%/DutycycleNR, y ≤ Duty threshold (3)

If we multiply both sides of (3) by DutycycleNR, x and divide them by 50%, (3) becomes as follows.

DutyNR, x *( PNR,x/ P26) + DutyNR, y *(PNR, y/ P26)*(DutycycleNR, x /DutycycleNR, y) ≤ DutycycleNR, x *Duty threshold/50%  (4)

For simplicity, let’s define DutycycleNR, x /DutycycleNR, y as SARratio.

DutyNR, x *( PNR,x/ P26) + DutyNR, y *(PNR, y/ P26)* SARratio ≤ DutycycleNR, x * Duty threshold/50% (5)

Supposed that PNR,x = PNR, y = 200 mW(23dBm), the above formula (5) becomes as follows.

DutyNR, x + DutyNR, y * SARratio ≤ DutycycleNR, x * Duty threshold/100%  (6)

Assume that DutycycleNR, x = DutycycleNR, y = 50%, (5) becomes as follows.

DutyNR, x + DutyNR, y ≤ 0.5 * Duty threshold  (7)

To make the total duty cycle for uplink inter band CA is 50%, the UE has to report Duty threshold = 100%. The network, however, has to consider the total duty cycle is 50% even the UE reports 100%.

Observation 1:

- The following formula will generate inconsistency between the reported total duty cycle for uplink inter band CA by UE and the total(actual) duty cycle to be scheduled.

  - DutyNR, x *( PNR,x/ P26)*SARratioNR, x + DutyNR, y *(PNR, y/ P26)* [SARratioNR, y ] ≤ Duty threshold (1)

  - SARratioNR, x = 50%/DutycycleNR, x   ; SARratioNR, y = 50%/DutycycleNR, y

Moreover, in the end, UE vendors have authority to put the number of Duty threshold. That means whatever DutycycleNR, x and DutycycleNR, y are, the total duty cycle can be determined by UE vendor. For instance, assume that DutycycleNR, x = 60 and DutycycleNR, y = 50%, SARratio becomes 5/6.

DutyNR, x + DutyNR, y * 5/6 ≤ 0.6 * Duty threshold  (8)

If a UE vendor would like to limit the total(actual) duty cycle down to 50%, the UE just reports Duty threshold as 50/0.6=83%.

Observation 2:

- With respect to the formulas (1) and (2), whatever DutycycleNR, x and DutycycleNR, y are, UE vendors can set the total(actual) duty cycle considering DutycycleNR, x and DutycycleNR, y by adjusting Duty threshold.

Hence, we propose the following formula.

Proposal: In case duty cycle method is adopted, the formula should be as follows.

- DutyNR, x *( PNR,x/ P26) + DutyNR, y *(PNR, y/ P26)* SARratioNR ≤ Duty threshold

  - The definition of SARratioNR is FFS. Possible candidates are DutycycleNR, x /DutycycleNR, y or reported by UE together with Duty threshold.

# 3	Conclusion

In this contribution, we analysed the solution proposed in [1] and it turns out that the formula captured in [1] is redundant and no need to include two SARratioNR terms.

Observation 1:

- The following formula will generate inconsistency between the reported total duty cycle for uplink inter band CA by UE and the total(actual) duty cycle to be scheduled.

  - DutyNR, x *( PNR,x/ P26)*SARratioNR, x + DutyNR, y *(PNR, y/ P26)* [SARratioNR, y ] ≤ Duty threshold (1)

  - SARratioNR, x = 50%/DutycycleNR, x   ; SARratioNR, y = 50%/DutycycleNR, y

Observation 2:

- With respect to the formulas (1) and (2), whatever DutycycleNR, x and DutycycleNR, y are, UE vendors can set the total(actual) duty cycle considering DutycycleNR, x and DutycycleNR, y by adjusting Duty threshold.

Proposal: In case duty cycle method is adopted, the formula should be as follows.

- DutyNR, x *( PNR,x/ P26) + DutyNR, y *(PNR, y/ P26)* SARratioNR ≤ Duty threshold

  - The definition of SARratioNR is FFS. Possible candidates are DutycycleNR, x /DutycycleNR, y or reported by UE together with Duty threshold.

# 6	References

[1] R4-2105341, WF on SAR solutions for PC2 NR inter-band CA and SUL configurations, China Telecom Incorporated