3GPP TSG-RAN WG4 Meeting#98-bis             	R4-2108913

E-meeting, 12th – 20th  April, 2021

Title:	UE RF IBM requirements for CA configurations within same frequency group

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	9.4.2.1.3

Document for:	Approval

# 1	Introduction

In previous meeting [1] TP was approved to capture agreements CA_n258A-n260A and CA_n257A-n259A based on IBM into TR 38.851. Once the corresponding CRs are agreed then specification have three band combinations for IBM and all of those are combinations between the frequency groups. Agreement of the CRs would complete the work in this WI for UE RF IBM requirements for CA configurations between the frequency groups.

In this contribution we continue the discussion for UE RF IBM requirements for CA configurations within same frequency group

# 2	Discussion

In previous RAN4 meeting [2] email discussion summary for this topic had two questions

Issue 5-1-1: whether IBM inter-CA requirement framework established for n260+n261 shall be applied to any requested CA band pair from the same frequency group (parameter values discussed separately)

- Proposals

  - Option 1: Yes

  - Option 2: No

Companies felt that option 1 is reasonable approach and do not see a need to develop new framework for IBM requirements for band combinations within same (single) frequency group.

Tentative agreements:

Option 1: IBM inter-CA requirement framework established for n260+n261 shall be applied to any requested CA band pair from the same frequency group (parameter values discussed separately) with IBM”

Recommendations for 2nd round:

Discuss if tentative agreement is acceptable first in GTW session.

However in the GTW session no agreement was reached partially due to limited time and importance of focusing the agreements for combinations between the frequency groups. But as all companies who commented during email discussion supported that IBM inter-CA requirement framework established for n260+n261 shall be applied to any requested CA band pair from the same frequency group (parameter values discussed separately) we will make the proposal again.

Proposal 1: IBM inter-CA requirement framework established for n260+n261 shall be applied to any requested CA band pair from the same frequency group (parameter values discussed separately) with IBM

Issue 5-2-1: how to define the relaxation values of FR2 inter-band CA within same frequency group for IBM

- Proposals

  - Option 1: reuse 3.5dB for all band combinations

  - Option 2: reuse 3.5dB for CA_n257+n258. FFS for CA_n259+n260.

  - Option 3: other

During the email discussion proposal 2 got most support but also both option 1 and especially option 3 were supported. Option 2 is actually a subset of option 1 hence not against option 1. Supporters for option 3 were calling out operator request for CA_n257+n258. In this contribution we propose that RAN4 will use CA_n257+n258 as an  example band combination and captures requirements into specification without direct operator request. This is not unusual infact during REL10 RAN4 chose such CA configuration as an example that was not usable to anybody CA_1A-5A altoung later it was found out that is was usable. Anyways CA_n257+n258 is not unusable it could be used in several countries. Therefore we propose following

Proposal 2: use 3.5dB for CA_n257+n258 for reference sensitivity relaxation and  EIS spherical coverage requirement relaxation value

If proposals 1 and 2 are acceptable then CR can be provided to August RAN4 meeting and objective is completed.

# 3	Conclusion

In this contribution we have made following proposals.

Proposal 1: IBM inter-CA requirement framework established for n260+n261 shall be applied to any requested CA band pair from the same frequency group (parameter values discussed separately) with IBM

Proposal 2: use 3.5dB for CA_n257+n258 for reference sensitivity relaxation and  EIS spherical coverage requirement relaxation value

If proposals 1 and 2 are acceptable then CR can be provided to August RAN4 meeting and objective is completed.

# 4 	References

[1] R4-2105495, FR2 inter-band CA for different frequency band groups with IBM (TP to TS 38.851), Nokia, Nokia Shanghai Bell, NTT DOCOMO, INC.

[2] R4-2105467, 	Email discussion summary for [98-bis-e][130] NR_RF_FR2_req_enh2_Part_1