3GPP TSG-RAN WG4 Meeting #99-e                                                         R4-2108997

Electronic Meeting, 17-28 May, 2021

Source:	Ligado Networks

Title:	TP for TR 38.717-03-01: CA_n24-n48-n77 combinations

Agenda item:			8.11.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-01 to include CA_n24-n48-n77 as per [1].

# 2. Reference

- RP-210593, Revised WID: Rel-17 NR inter-band CA for 3 bands DL with 1 band UL

# 3. Text Proposal

<Start of Text Proposal>

## 6.x	CA_n24-n48-n77

#### 6.x.1		Operating bands for CA

Table 6.x.1-1: 3DL Inter-band CA operating bands

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n24 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 6.x.2		Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported bandwidths per CA band combination for 3DL inter-band CA

| NR CA Configuration | UL Configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n24A-n48A-n77A |  | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n24A-n48(2A)-n77A |  | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(2A) BCS0 in Table 5.5A.2-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n24A-n48A-n77(2A) |  | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n77 | See CA_n77(2A) BCS0 in Table 5.5A.2-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n24A-n48(2A)-n77(2A) |  | n24 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n48 | See CA_n48(2A) BCS0 in Table 5.5A.2-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) BCS0 in Table 5.5A.2-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.x.3		Co-existence studies

Table 6.x.3-1 summarizes frequency ranges where harmonics occur for CA_n24-n48-n77.

Table 6.x.1.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |  |
| n24 | 1626.5 | 1660.5 | 1525 | 1559 | 3253 | 3321 | 4879.5 | 4981.5 | 6506 | 6642 | 8132.5 | 8302.5 |  |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |  |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |  |

Based on above table, the 2nd harmonic of n24 UL may fall into the lower DL frequency range of n77 which need to be addressed in lower order combination.

Table 6.x.1.3-2 summarizes frequency ranges where harmonics mixing occur for CA_n24-n48-n77.

Table 6.x.1.3-2: Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n24 | 1626.5 | 1660.5 | 1525 | 1559 | 3050 | 3118 | 4575 | 4677 | 6100 | 6236 | 7625 | 7795 |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there are no harmonic mixing issues for the CA_n24-n48-n77 combinations.

#### 6.x.4		∆TIB and ∆RIB values

For CA_n24-n48-n77, the ∆TIB,c and ∆RIB,c values are given according to CA_n41-n66-n77 relaxation values.

Table 6.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n24-n48-n77 | n24 | 0.6 |
|  | n48 | 0.8 |
|  | n77 | 0.8 |

Table 6.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n24-n48-n77 | n24 | 0.2 |
|  | n48 | 0.5 |
|  | n77 | 0.5 |

#### 6.x.5		REFSENS requirements

The 2nd harmonic issues from n24 UL into n77 DL is addressed in lower order combination.

<End of Text Proposal>