3GPP TSG-RAN WG4 Meeting # 99-e	R4-2109015

Electronic Meeting, May. 19-27, 2021

Source: 	CATT

Title: 	Co-existence simulation assumptions and some simulation results

Agenda item:	9.15.4.1

Document for:	Approval

- Introduction

This contribution provides our proposals for co-existence simulation assumptions and some preliminary simulation results are also provided.

- Discussion

2.1 Simulation assumptions

There was some co-existence simulation assumption discussion in last RAN4 meeting and the WF[5] was not approved due to some different understandings. According to the last meeting’s discussion, 100 MHz CBW is assumed to be the worst case. For the BS output power, we assume the BS antenna array, output power should be aligned with the assumption in TR 38.808. For the UE output power, we reused the assumption in [5]. So we have the following simulation assumption proposals for the indoor and outdoor.

Table 1: Indoor co-existence simulation assumption

| Deployment |  | Indoor Office A and Office C as specified in [1] |
| --- | --- | --- |
| System Parameters | Carrier Frequency | 60 GHz, 70 GHz |
|  | Channel BW | 100 and 2000 MHz |
|  | Number of active UEs | 1 |
|  | Channel model | InH open office specified in [2] |
|  | LBT | No LBT |
|  | SCS | 100 MHz 120kHz SCS 2000 MHz 960kHz SCS (166 RB) |
| BS | (Mg, Ng, M, N, P) | (1,1,4,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-7 in [3] for ceiling mount |
|  | Max Tx Power | 52.8 dBm EIRP (or max TX conducted power 32.8dBm) |
|  | Noise Figure | 13 dB |
| UE | (Mg, Ng, M, N, P) | (1,2,2,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-8 in [3] |
|  | Max Tx Power | 20 dBm EIRP(or max TX conducted power 3dBm) |
|  | Noise figure | 13 dB |
|  | LoS/ NLoS | LoS probability for indoor office defined in [4] |

Table 2: Outdoor co-existence simulation assumption

| Deployment |  | Dense urban scenario outdoor A as specified in [1], coordinated and uncoordinated |
| --- | --- | --- |
| System Parameters | Carrier Frequency | 60 GHz or 70 GHz |
|  | Channel BW | 100 and 2000 MHz |
|  | Number of active UEs | 1 |
|  | Channel model | Umi model as specified in [2] |
|  | LBT | No LBT |
|  | SCS | 100 MHz 120kHz SCS 2000 MHz 960kHz SCS (166 RB) |
| BS | (Mg, Ng, M, N, P) | (1,1,16,16,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table 7.3-1 of [2] |
|  | Max Tx Power | 61.8 dBm EIRP |
|  | Noise Figure | 13 dB |
| UE | (Mg, Ng, M, N, P) | (1,2,2,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-8 in [3] |
|  | Max Tx Power | 20 dBm EIRP(or max TX conducted power 3dBm) |
|  | Noise figure | 13 dB |
|  | LoS/ NLoS | LoS probability for indoor office defined in [4] |

2.2 Preliminary simulation results for outdoor scenario

According to the simulation assumption in 2.1, we have some preliminary simulation results for outdoor scenario.

In order to calibrate the simulation platform, we first used the same simulation parameters as [8]. Figure 1 and Figure 2 are our simulation results to compare with [8].

Figure 1: DL SINR CDF for 60 GHz using the simulation assumption in [8](left: our result, right: from[8])

Figure 2: Average DL throughput loss for 60 GHz using the simulation assumption in [8](left: our result, right: from [8])

The CDF curve in Figure 1 is closely aligned, but there’s  ~5 dB difference for the ACIR requirement in the two simulations. Our ACIR result is ~12 dB for the 5% TP loss.

Figure 3 to Figure 6 are our simulation results using the simulation assumption in Table 2.

Figure 3: 60 GHz SINR CDF using the simulation assumption in Table 2.

Figure 4: 70 GHz SINR CDF using the simulation assumption in Table 2.

Figure 5: DL throughput loss for 60 GHz using the simulation assumption in Table 2

Figure 6: DL throughput loss for 70 GHz using the simulation assumption in Table 2

From the simulation results, we observed ~6 dB ACIR requirement for 60 GHz and ~4 dB ACIR requirement for 70 GHz. It’s very different with the observation in [8]. The reason for the difference may come from the different BS power assumption. We propose to align simulation assumptions to further simulate and discuss the ACIR requirement.

Proposal: Discuss and agree the simulation assumption for the ACIR requirement.

- Conclusion

We provide indoor and outdoor simulation results based on the assumption in Table 1 and Table 2. In order to align the simulation assumption, we propose,

Proposal: Discuss and agree the simulation assumption for the ACIR requirement.

- References

- 3GPP TR 38.808: "Study on supporting NR from 52.6 GHz to 71 GHz".

- 3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

- 3GPP TR 38.802: "Study on New Radio Access Technology Physical Layer Aspects".

- 3GPP TR 38.803: "Study on new radio access technology: Radio Frequency (RF) and co-existence aspects ".

- R4-2105415, Way forward on system simulations for 60 GHz, Qualcomm

- R4-2104806, Discussion on the co-exit simulation assumption, CATT

- R4-2105172, Simulation results for NR coexistence study indoor deployment at 60GHz, Qualcomm CDMA Technologies

- R4-2105173, Simulation results for NR coexistence study: dense urban micro deployment at 60GHz, Qualcomm CDMA Technologies