3GPP TSG-RAN WG4 Meeting #99-e                                                             R4-2109104

Electronic Meeting, May. 19 - 27, 2021

Source: 	CATT

Title: 	Simulation results for PUSCH 256QAM performance requirement

Agenda item:	9.11.3.1

Document for:	Approval

- Introduction

In RAN4 #98-bis-e, a way forward on PUSCH 256QAM performance requirements was approved [1]. The simulation need to be done for alignment based on agreement of test parameter, and the remaining issues such as MCS, additional DM-RS, 2TX etc. in WF also need to be decided based on simulation results.

In this contribution, we provide our simulation results for alignment.

- Discussion

The simulation assumptions are listed as in Table A5-1. And the following cases are evaluated.

1)  Case1: MCS 24, 1TX, DMRS 1+1, PDSCH mapping type A.

2)  Case2: MCS24, 1TX, DMRS 1+1, PDSCH mapping type B.

3)  Case3: MCS22, 1TX, DMRS1+1, PDSCH mapping type A.

4)  Case4: MCS24, 1TX, DMRS1+1+1, PDSCH mapping type A.

5)  Case5: MCS24, 2TX, DMRS1+1, PDSCH mapping type A.

The summary of simulation results for case1, case2, case 3, case 4 and case 5 are shown in Table 2-1, Table 2-2, Table 2-3, Table 2-4 and Table 2-5 respectively.

Table 2-1: Summary of simulation results for case 1: MCS 24, 1TX, DMRS 1+1, PDSCH mapping type A

| SCS | CBW | TX/RX | MCS | DMRS | PUSCH mapping type | SNR @70% of maximum throughput(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| 15kHz | 5MHz | 1T2R | 24 | 1+1 | type A | 31.26 | 33.76 |
| 15kHz | 5MHz | 1T4R | 24 | 1+1 | type A | 29.69 | 32.19 |
| 15kHz | 5MHz | 1T8R | 24 | 1+1 | type A | 26.05 | 28.55 |
| 15kHz | 10MHz | 1T2R | 24 | 1+1 | type A | 24.43 | 26.93 |
| 15kHz | 10MHz | 1T4R | 24 | 1+1 | type A | 20.54 | 23.04 |
| 15kHz | 10MHz | 1T8R | 24 | 1+1 | type A | 16.65 | 19.15 |
| 15kHz | 20MHz | 1T2R | 24 | 1+1 | type A | 23.56 | 26.06 |
| 15kHz | 20MHz | 1T4R | 24 | 1+1 | type A | 19.41 | 21.91 |
| 15kHz | 20MHz | 1T8R | 24 | 1+1 | type A | 15.29 | 17.79 |
| 30kHz | 10MHz | 1T2R | 24 | 1+1 | type A | 27.84 | 30.34 |
| 30kHz | 10MHz | 1T4R | 24 | 1+1 | type A | 25.07 | 27.57 |
| 30kHz | 10MHz | 1T8R | 24 | 1+1 | type A | 21.3 | 23.8 |
| 30kHz | 20MHz | 1T2R | 24 | 1+1 | type A | 24.04 | 26.54 |
| 30kHz | 20MHz | 1T4R | 24 | 1+1 | type A | 19.94 | 22.44 |
| 30kHz | 20MHz | 1T8R | 24 | 1+1 | type A | 16.05 | 18.55 |
| 30kHz | 40MHz | 1T2R | 24 | 1+1 | type A | 23.49 | 25.99 |
| 30kHz | 40MHz | 1T4R | 24 | 1+1 | type A | 19.23 | 21.73 |
| 30kHz | 40MHz | 1T8R | 24 | 1+1 | type A | 14.46 | 16.96 |
| 30kHz | 100MHz | 1T2R | 24 | 1+1 | type A | 25.25 | 27.75 |
| 30kHz | 100MHz | 1T4R | 24 | 1+1 | type A | 20.51 | 23.01 |
| 30kHz | 100MHz | 1T8R | 24 | 1+1 | type A | 16.44 | 18.94 |

Table 2-2: Summary of simulation results for case2: MCS24, 1TX, DMRS 1+1, PDSCH mapping type B

| SCS | CBW | TX/RX | MCS | DMRS | PUSCH mapping type | SNR @70% of maximum throughput(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| 15kHz | 5MHz | 1T2R | 24 | 1+1 | type B | 31.17 | 33.67 |
| 15kHz | 5MHz | 1T4R | 24 | 1+1 | type B | 29.64 | 32.14 |
| 15kHz | 5MHz | 1T8R | 24 | 1+1 | type B | 26.11 | 28.61 |
| 15kHz | 10MHz | 1T2R | 24 | 1+1 | type B | 24.35 | 26.85 |
| 15kHz | 10MHz | 1T4R | 24 | 1+1 | type B | 20.53 | 23.03 |
| 15kHz | 10MHz | 1T8R | 24 | 1+1 | type B | 16.59 | 19.09 |
| 15kHz | 20MHz | 1T2R | 24 | 1+1 | type B | 23.47 | 25.97 |
| 15kHz | 20MHz | 1T4R | 24 | 1+1 | type B | 19.44 | 21.94 |
| 15kHz | 20MHz | 1T8R | 24 | 1+1 | type B | 15.21 | 17.71 |
| 30kHz | 10MHz | 1T2R | 24 | 1+1 | type B | 27.76 | 30.26 |
| 30kHz | 10MHz | 1T4R | 24 | 1+1 | type B | 24.95 | 27.45 |
| 30kHz | 10MHz | 1T8R | 24 | 1+1 | type B | 21.28 | 23.78 |
| 30kHz | 20MHz | 1T2R | 24 | 1+1 | type B | 23.97 | 26.47 |
| 30kHz | 20MHz | 1T4R | 24 | 1+1 | type B | 19.94 | 22.44 |
| 30kHz | 20MHz | 1T8R | 24 | 1+1 | type B | 16.08 | 18.58 |
| 30kHz | 40MHz | 1T2R | 24 | 1+1 | type B | 23.51 | 26.01 |
| 30kHz | 40MHz | 1T4R | 24 | 1+1 | type B | 19.19 | 21.69 |
| 30kHz | 40MHz | 1T8R | 24 | 1+1 | type B | 14.19 | 16.69 |
| 30kHz | 100MHz | 1T2R | 24 | 1+1 | type B | 25.2 | 27.7 |
| 30kHz | 100MHz | 1T4R | 24 | 1+1 | type B | 20.49 | 22.99 |
| 30kHz | 100MHz | 1T8R | 24 | 1+1 | type B | 16.41 | 18.91 |

Table 2-3: Summary of simulation results for case3: MCS22, 1TX, DMRS1+1, PDSCH mapping type A

| SCS | CBW | TX/RX | MCS | DMRS | PUSCH mapping type | SNR @70% of maximum throughput(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| 15kHz | 5MHz | 1T2R | 22 | 1+1 | type A | 23.04 | 25.54 |
| 15kHz | 5MHz | 1T4R | 22 | 1+1 | type A | 20.09 | 22.59 |
| 15kHz | 5MHz | 1T8R | 22 | 1+1 | type A | 15.88 | 18.38 |
| 15kHz | 10MHz | 1T2R | 22 | 1+1 | type A | 20 | 22.5 |
| 15kHz | 10MHz | 1T4R | 22 | 1+1 | type A | 14.06 | 16.56 |
| 15kHz | 10MHz | 1T8R | 22 | 1+1 | type A | [TBD] | [TBD] |
| 15kHz | 20MHz | 1T2R | 22 | 1+1 | type A | 19.31 | 21.81 |
| 15kHz | 20MHz | 1T4R | 22 | 1+1 | type A | 10.62 | 13.12 |
| 15kHz | 20MHz | 1T8R | 22 | 1+1 | type A | [TBD] | [TBD] |
| 30kHz | 10MHz | 1T2R | 22 | 1+1 | type A | 22.86 | 25.36 |
| 30kHz | 10MHz | 1T4R | 22 | 1+1 | type A | 19.56 | 22.06 |
| 30kHz | 10MHz | 1T8R | 22 | 1+1 | type A | 15.31 | 17.81 |
| 30kHz | 20MHz | 1T2R | 22 | 1+1 | type A | 20.14 | 22.64 |
| 30kHz | 20MHz | 1T4R | 22 | 1+1 | type A | 10.09 | 12.59 |
| 30kHz | 20MHz | 1T8R | 22 | 1+1 | type A | [TBD] | [TBD] |
| 30kHz | 40MHz | 1T2R | 22 | 1+1 | type A | 20.02 | 22.52 |
| 30kHz | 40MHz | 1T4R | 22 | 1+1 | type A | 8.88 | 11.38 |
| 30kHz | 40MHz | 1T8R | 22 | 1+1 | type A | [TBD] | [TBD] |
| 30kHz | 100MHz | 1T2R | 22 | 1+1 | type A | 22.07 | 24.57 |
| 30kHz | 100MHz | 1T4R | 22 | 1+1 | type A | 13.63 | 16.13 |
| 30kHz | 100MHz | 1T8R | 22 | 1+1 | type A | [TBD] | [TBD] |

Table 2-4: Summary of simulation results for case4: MCS24, 1TX, DMRS1+1+1, PDSCH mapping type A

| SCS | CBW | TX/RX | MCS | DMRS | PUSCH mapping type | SNR @70% of maximum throughput(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| 15kHz | 5MHz | 1T2R | 24 | 1+1+1 | type A | 30.66 | 33.16 |
| 15kHz | 5MHz | 1T4R | 24 | 1+1+1 | type A | 29.07 | 31.57 |
| 15kHz | 5MHz | 1T8R | 24 | 1+1+1 | type A | 25.34 | 27.84 |
| 15kHz | 10MHz | 1T2R | 24 | 1+1+1 | type A | 24.66 | 27.16 |
| 15kHz | 10MHz | 1T4R | 24 | 1+1+1 | type A | 20.69 | 23.19 |
| 15kHz | 10MHz | 1T8R | 24 | 1+1+1 | type A | 16.69 | 19.19 |
| 15kHz | 20MHz | 1T2R | 24 | 1+1+1 | type A | 23.73 | 26.23 |
| 15kHz | 20MHz | 1T4R | 24 | 1+1+1 | type A | 19.64 | 22.14 |
| 15kHz | 20MHz | 1T8R | 24 | 1+1+1 | type A | 15.4 | 17.9 |
| 30kHz | 10MHz | 1T2R | 24 | 1+1+1 | type A | 29.32 | 31.82 |
| 30kHz | 10MHz | 1T4R | 24 | 1+1+1 | type A | 26.68 | 29.18 |
| 30kHz | 10MHz | 1T8R | 24 | 1+1+1 | type A | 22.86 | 25.36 |
| 30kHz | 20MHz | 1T2R | 24 | 1+1+1 | type A | 24.14 | 26.64 |
| 30kHz | 20MHz | 1T4R | 24 | 1+1+1 | type A | 19.95 | 22.45 |
| 30kHz | 20MHz | 1T8R | 24 | 1+1+1 | type A | 15.86 | 18.36 |
| 30kHz | 40MHz | 1T2R | 24 | 1+1+1 | type A | 23.72 | 26.22 |
| 30kHz | 40MHz | 1T4R | 24 | 1+1+1 | type A | 19.45 | 21.95 |
| 30kHz | 40MHz | 1T8R | 24 | 1+1+1 | type A | 14.54 | 17.04 |
| 30kHz | 100MHz | 1T2R | 24 | 1+1+1 | type A | 25.56 | 28.06 |
| 30kHz | 100MHz | 1T4R | 24 | 1+1+1 | type A | 20.77 | 23.27 |
| 30kHz | 100MHz | 1T8R | 24 | 1+1+1 | type A | 16.66 | 19.16 |

Table 2-5: Summary of simulation results for case5: MCS24, 2TX, DMRS1+1, PDSCH mapping type A

TBA

- Conclusions

This contribution provides our link level simulation results for 256QAM for alignment.

- References

- R4-2106120, Way forward on PUSCH 256QAM performance requirements, Huawei, HiSilicon, RAN4#98bis-e

- Annex

Table A5-1 simulation assumptions for PUSCH 256QAM performance requirement

| Parameter | Value |
| --- | --- |
| Carrier frequency (GHz) | 4 |
| Transform precoding | Disabled |
| Number of TX antenna | 1, [2] |
| Number of RX antenna | 2, [4], 8 |
| Number of layers | 1 TX: 1 [2 TX: 2] |
| Transmission scheme | Identity matrix (TPMI index 0) |
| DMRS type | type 1 with single-symbol DM-RS |
| Number of DMRS symbols | 1+1, [1+1+1] |
| Additional DM-RS | 1+1: dmrs-AdditionalPosition = ‘pos1’, type A: , 11; type B: , 10 [1+1+1: dmrs-AdditionalPosition = ‘pos2’, type A: , 7, 11; type B: , 5,10] |
| Start symbol index | 0 |
| Symbols length | 14 |
| Time domain resource allocation type | type A and type B |
| Frequency domain resource | Full applicable test bandwidth |
| MCS index | 24, [22] |
| Propagation condition | TDLA30-10 Low |
| SCS and BW | 15kHz: 5MHz, 10MHz, [20MHz]; 30kHz: 10MHz, [20MHz], 40MHz, [100MHz]; |
| PTRS | With and without PTRS PTRS configuration: KPT-RS=2, LPT-RS=1 |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions | 4 |
| RV sequence | {0,2,3,1} |
| CP | Normal |
| SRS | Not configured |
| Testing metric | SNR @70% of maximum throughput |