3GPP TSG-RAN WG4 Meeting #99-e                                                              R4-2109105

Electronic Meeting, May. 19 - 27, 2021

Source: 	CATT

Title: 	Discussion on PUSCH demodulation requirements for FR1 256QAM

Agenda item:	9.11.3.1

Document for:	Approval

- Introduction

In RAN4#98-bis-e, a way forward on PUSCH 256QAM performance requirements was approved [1]. The remaining issues include the MCS, DMRS, RX antenna, TX antenna and TX layer, etc.

In this contribution, we provide our views on these issues for FR1 256QAM.

- Discussion

MCS

As per the WF [1], the agreements and the remaining issues concerning MCS are shown as below:

- MCS: Evaluate {MCS24, MCS22} as starting point for next meeting, based on the simulation results to decide if MCS24 is feasible, otherwise to check lower MCS22 is feasible or not.

Table 2-1: SNR gain of MCS 22 relative to MCS 24

| SCS | CBW | TX/RX | DMRS | PUSCH mapping type | SNR @70% of maximum throughput(dB) |  | SNR gain of MCS 22 relative to MCS 24 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | MCS 24 | MCS 22 |  |
| 15kHz | 5MHz | 1T2R | 1+1 | type A | 31.26 | 23.04 | 8.22 |
| 15kHz | 5MHz | 1T4R | 1+1 | type A | 29.69 | 20.09 | 9.6 |
| 15kHz | 5MHz | 1T8R | 1+1 | type A | 26.05 | 15.88 | 10.17 |
| 15kHz | 10MHz | 1T2R | 1+1 | type A | 24.43 | 20 | 4.43 |
| 15kHz | 10MHz | 1T4R | 1+1 | type A | 20.54 | 14.06 | 6.48 |
| 15kHz | 20MHz | 1T2R | 1+1 | type A | 23.56 | 19.31 | 4.25 |
| 15kHz | 20MHz | 1T4R | 1+1 | type A | 19.41 | 10.62 | 8.79 |
| 30kHz | 10MHz | 1T2R | 1+1 | type A | 27.84 | 22.86 | 4.98 |
| 30kHz | 10MHz | 1T4R | 1+1 | type A | 25.07 | 19.56 | 5.51 |
| 30kHz | 10MHz | 1T8R | 1+1 | type A | 21.3 | 15.31 | 5.99 |
| 30kHz | 20MHz | 1T2R | 1+1 | type A | 24.04 | 20.14 | 3.9 |
| 30kHz | 20MHz | 1T4R | 1+1 | type A | 19.94 | 10.09 | 9.85 |
| 30kHz | 40MHz | 1T2R | 1+1 | type A | 23.49 | 20.02 | 3.47 |
| 30kHz | 40MHz | 1T4R | 1+1 | type A | 19.23 | 8.88 | 10.35 |
| 30kHz | 100MHz | 1T2R | 1+1 | type A | 25.25 | 22.07 | 3.18 |
| 30kHz | 100MHz | 1T4R | 1+1 | type A | 20.51 | 13.63 | 6.88 |

For the 256QAM in E-UTRA, the SNR @ 70% of maximum throughput for 1T2R is 21.4dB. Referring to our simulation results [2], we summarize the SNR gain of MCS 22 relative to MCS 24 as shown in Table 2-1. From the Table 2-1, it is observed that SNR gain of MCS 22 relative to MCS 24 is in range of 3.18-10.35dB. The SNR @ 70% of maximum throughput for MCS 24 for 1T2R is 31.26dB which is too high and far larger than 21.4dB. The maximum SNR @ 70% of maximum throughput for1T2R for MCS 22 is 23.04dB, which is close to 21.4dB.  So MCS 22 is more feasible for 256QAM for NR.

Proposal 1: To adopt MCS index =22 for 256QAM demodulation.

DMRS

As per the WF [1], the agreements and the remaining issues concerning DMRS are shown as below:

- DM-RS type: Type 1 with single-symbol DM-RS. - Additional DM-RS (dmrsAdditionalPosition): pos1, FFS pos2 - Decide based on interesting companies’ feedback for next meeting

Table 2-2: SNR gain of DMRS 1+1+1 relative to DMRS 1+1

| SCS | CBW | TX/RX | MCS | PUSCH mapping type | SNR @70% of maximum throughput(dB) |  | SNR gain of DMRS 1+1+1 relative to DMRS1+1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | DMRS1+1 | DMRS1+1+1 |  |
| 15kHz | 5MHz | 1T2R | 24 | type A | 31.26 | 30.66 | 0.6 |
| 15kHz | 5MHz | 1T4R | 24 | type A | 29.69 | 29.07 | 0.62 |
| 15kHz | 5MHz | 1T8R | 24 | type A | 26.05 | 25.34 | 0.71 |
| 15kHz | 10MHz | 1T2R | 24 | type A | 24.43 | 24.66 | -0.23 |
| 15kHz | 10MHz | 1T4R | 24 | type A | 20.54 | 20.69 | -0.15 |
| 15kHz | 10MHz | 1T8R | 24 | type A | 16.65 | 16.69 | -0.04 |
| 15kHz | 20MHz | 1T2R | 24 | type A | 23.56 | 23.73 | -0.17 |
| 15kHz | 20MHz | 1T4R | 24 | type A | 19.41 | 19.64 | -0.23 |
| 15kHz | 20MHz | 1T8R | 24 | type A | 15.29 | 15.4 | -0.11 |
| 30kHz | 10MHz | 1T2R | 24 | type A | 27.84 | 29.32 | -1.48 |
| 30kHz | 10MHz | 1T4R | 24 | type A | 25.07 | 26.68 | -1.61 |
| 30kHz | 10MHz | 1T8R | 24 | type A | 21.3 | 22.86 | -1.56 |
| 30kHz | 20MHz | 1T2R | 24 | type A | 24.04 | 24.14 | -0.1 |
| 30kHz | 20MHz | 1T4R | 24 | type A | 19.94 | 19.95 | -0.01 |
| 30kHz | 20MHz | 1T8R | 24 | type A | 16.05 | 15.86 | 0.19 |
| 30kHz | 40MHz | 1T2R | 24 | type A | 23.49 | 23.72 | -0.23 |
| 30kHz | 40MHz | 1T4R | 24 | type A | 19.23 | 19.45 | -0.22 |
| 30kHz | 40MHz | 1T8R | 24 | type A | 14.46 | 14.54 | -0.08 |
| 30kHz | 100MHz | 1T2R | 24 | type A | 25.25 | 25.56 | -0.31 |
| 30kHz | 100MHz | 1T4R | 24 | type A | 20.51 | 20.77 | -0.26 |
| 30kHz | 100MHz | 1T8R | 24 | type A | 16.44 | 16.66 | -0.22 |

Referring to our simulation results [2], we summarize the SNR gain of DMRS1+1+1 relative to DMRS1+1 as shown in Table 2-2. From the Table 2-2, it is observed that SNR gain of DMRS1+1+1 relative to DMRS1+1 is in range of -1.61-0.71dB. The SNR gain of DMRS1+1+1 relative to DMRS1+1 is not obvious. So DMRS 1+1 (pos1) is sufficient for 25QAM for NR FR1.

Proposal 2: To adopt pos1 additional DM-RS position for 256QAM demodulation.

PTRS and Phase noise modelling

As per the WF [1], the agreements and the remaining issues concerning PTRS are shown as below:

- PT-RS configuration: FFS configure PT-RS. - Further discuss and decide whether to configure PT-RS or not based on feedback from interesting companies in next meeting. - Phase Noise modelling: - Realistic phase noise modelling is left up to the contributing entities. - FFS how to consider phase noise impact based on further discussion and evaluations. - Interesting company is welcome to do investigation on the PN impact on 256QAM performance for next meeting.

The PTRS is mainly used to compensate common phase noise error (CPE), and we think the phase noise impact in FR1 for 256QAM is minor. So we prefer to not configure PTRS for NR FR1 256QAM. The phase noise impact can be considered in implementation margin for impairment result.

Proposal 3: Not to configure PTRS for 256QAM demodulation.

Antenna configuration

As per the WF [1], the agreements and the remaining issues concerning antenna configuration are shown as below:

- Number of Tx: - Option 1: Only 1Tx - Option 2: Both 1Tx and 2Tx - Number of Rx: - Option 1: 2/8 Rx - Option 2: 2/4/8 Rx - Number of layers: - Option 1: Only 1 layer - Option 2: Both of 1 and 2 layers - Applicability rule for different antenna configurations - Option 1: Reuse the existing test applicability rule defined in clause 8.1.2.0 of TS38.141-1 - Other options

Only 1TX antenna for 256QAM demodulation in TS 36.104 is required. So 1TX antenna is sufficient for 256QAM for NR FR1. And only 1 layer for 1TX antenna is sufficient.

Proposal 4: To adopt Option 1: Only 1Tx for 256QAM demodulation.

Proposal 5: To adopt Option 1: Only 1 layer for 256QAM demodulation.

Multiple RX antennas enable higher SNR, which is beneficial to reflecting the high throughput advantage of 256QAM. To allow different antenna configuration, the number of RX antennas 2, 4, 8 defined for 64QAM demodulation in TS 38.104 can be reused for 256QAM.

Proposal 6: To adopt Option 2: 2/4/8 Rx for 256QAM demodulation.

We think existing test applicability rule defined in clause 8.1.2.0 of TS38.141-1 can be reused for 256QAM demodulation.

Proposal 7: To adopt Option 1: Reuse the existing test applicability rule defined in clause 8.1.2.0 of TS38.141-1 for 256QAM demodulation

SCS/CBW combinations

As per the WF [1], the agreements and the remaining issues concerning SCS/CBW combinations are shown as below:

- SCS and bandwidth - 15kHz SCS: - Option 1:5MHz and 10MHz - Option 2: 5MHz, 10MHz and 20MHz. - 30kHz SCS - Option 1:10MHz and 40MHz - Option 2:10MHz, 20MHz, 40MHz and 100MHz. - Applicability rules for different SCS and CBW - Option 1: Reuse the existing applicability rules defined in sections 8.1.2.1.1 and 8.1.2.1.2 of TS 38.141-1 - Other options.

For SCS/CBW combinations, 256QAM demodulation is not different from other modulation. The other modulations (QPSK, 16QAM, and 64QAM) testing covers the SCS/CBW combinations: 15 kHz SCS: 5MHz, 10MHz, 20MHz and 30 kHz SCS: 10MHz, 20MHz, 40MHz, 100MHz. So the same SCS/CBW combinations can be adopted for 256QAM demodulation. But if the SCS/CBW combinations for 256QAM are different from the SCS/CBW combinations for other modulations, the closest channel bandwidth lower than widest supported bandwidth for 256QAM testing will be different from that for other modulations testing for some widest supported bandwidth.

Proposal 8: To adopt 15 kHz SCS: Option 2: 5MHz, 10MHz, 20MHz and 30 kHz SCS: Option 2:10MHz, 20MHz, 40MHz, 100MHz for 256QAM demodulation.

We think existing applicability rules defined in sections 8.1.2.1.1 and 8.1.2.1.2 of TS 38.141-1 can be reused for 256QAM demodulation.

Proposal 9: To adopt Option 1:  Reuse the existing applicability rules defined in sections 8.1.2.1.1 and 8.1.2.1.2 of TS 38.141-1 for 256QAM demodulation.

TX EVM

As per the WF [1], the agreements and the remaining issues concerning TX EVM are shown as below:

- Tx EVM: - Interesting companies are welcome to check the performance difference with and without Tx EVM (3.5% as baseline) impact considered. - RAN4 will discuss and decide whether additional margin should be considered in alignment results if no Tx EVM modelling in next meeting as per the evaluations results.

Table 2-3: SNR drop for SNR=20 -30dB

| SNR(dB) | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNRe (dB) | 19.5 | 20.3 | 21.2 | 22.1 | 22.8 | 23.6 | 24.3 | 24.9 | 25.5 | 26.0 | 26.5 |
| SNR drop(dB) | 0.5 | 0.7 | 0.8 | 0.9 | 1.2 | 1.4 | 1.7 | 2.1 | 2.5 | 3 | 3.5 |

For 3.5% TX EVM, the EVM power , and for noise, the noise power . Final effective SNR can be expressed as .  The SNR drop due to TX EVM equals to , and SNR drop for SNR=20 -30dB are shown in Table 2-3.  From Table 2-3, the SNR drop is 0.5dB at SNR =20dB and the SNR drop is 3.5dB at SNR =30dB. As the SNR increases, the SNR drop increases.  Referring to table 2-1, if the MCS 22 is adopted for 256QAM demodulation, the maximum SNR @70% of maximum throughput is 23.04 dB, and the SNR drop for SNR=23dB is 0.9dB. But if MCS 24 is adopted 256QAM demodulation, the maximum SNR @70% of maximum throughput is 31.26 dB and the SNR drop for SNR=31.26dB is larger than 3.5dB. So SNR drop due to TX EVM is dependent with MCS. The SNR drop can be considered as additional margin, so additional margin due to TX EVM is dependent with MCS

Observation 1: Additional margin due to TX EVM is dependent with MCS.

- Conclusion

This contribution provides analysis on PUSCH demodulation requirements for FR1 256QAM. The following proposals are concluded as follows:

Proposal 1: To adopt MCS index =22 for 256QAM demodulation.

Proposal 2: To adopt pos1 additional DM-RS position for 256QAM demodulation.

Proposal 3: Not to configure PTRS for 256QAM demodulation.

Proposal 4: To adopt Option 1: Only 1Tx for 256QAM demodulation.

Proposal 5: To adopt Option 1: Only 1 layer for 256QAM demodulation.

Proposal 6: To adopt Option 2: 2/4/8 Rx for 256QAM demodulation.

Proposal 7: To adopt Option 1: Reuse the existing test applicability rule defined in clause 8.1.2.0 of TS38.141-1 for 256QAM demodulation.

Proposal 8: To adopt 15 kHz SCS: Option 2: 5MHz, 10MHz, 20MHz and 30 kHz SCS: Option 2:10MHz, 20MHz, 40MHz, 100MHz for 256QAM demodulation.

Proposal 9: To adopt Option 1:  Reuse the existing applicability rules defined in sections 8.1.2.1.1 and 8.1.2.1.2 of TS 38.141-1 for 256QAM demodulation.

Observation 1: Additional margin due to TX EVM is dependent with MCS.

- References

- R4-2106120, Way forward on PUSCH 256QAM performance requirements, Huawei, HiSilicon,RAN4#98bis-e

- 3GPP TS 36.104 V17.0.0 (2020-12)

- 3GPP TS 38.104 V17.0.0 (2020-12)