3GPP TSG-RAN WG4 Meeting # 99-e 												              R4-2109119

Electronic Meeting, 19th – 27th May, 2021

Agenda item:			9.12.2.2

Source:	CATT

Title:	Comparison of co-existence performance w/w.o. considering earth curvature

Document for:	Approval

- Introduction

In RAN4#99e meeting, simulation assumption has been approved with some open issues to be further studied [1]. One of the open issues is whether to consider earth curvature in the simulation. This document presents simulation results on comparison of co-existence performance with/without considering earth curvature.

- Discussion

The simulation is carried out under the scenario of TN DL to NTN DL interference direction. Because the satellite beam of GEO has a larger beam diameter, it is more sensitive to curvature. So GEO is selected for the simulation.

Two topological assumptions are used, one is to consider the curvature of the earth, as shown in Figure 1 and Figure 2. Under this assumption, the location of the TN system (19 sites with wraparound) is randomly selected within the coverage of the satellite's central beam. Then, the coordinate system of TN system is regarded as local coordinate system (LCS), the coordinate system of satellite and earth is regarded as global coordinate system (GCS), and the angles such as satellite beam pointing direction is transformed from GCS to LCS. Because the scale of each TN system is small enough relative to the earth, curvature can be neglected within each TN system.

Figure 1 The topology considering curvature

Figure 2 TN surface

Another topological assumption is that the curvature is not considered, as shown in Figure 3. The TN system is also randomly selected in the central beam.

Figure 3 The topology not considering curvature

Both topologies select enough number of TN BS and then statistical results are collected. Other parameters and path loss models for NTN and TN are based on [1], including the LOS probability, cluster loss and shadow fading related to elevation. Seven NTN beams using the same frequency band are assumed, and one UE is generated in each satellite beam. Without losing generality, it is assumed that the NTN UE is located in a TN system, since the TN system is generated randomly in the central satellite beam. The set 2 assumption for satellite beam with lager beamwidth is used.

Figure 4 shows the CDF of SINR for two topologies. Figure 5 shows the throughput loss. It is observed that the difference between the two topologies seems acceptable, so the influence of the earth curvature might be neglected.

Figure 4 The CDF of NTN DL SINR							Figure 5. Throughput Loss o NTN DL

Observation: The impact of earth curvature on co-existence performance under TN DLNTN DL scenario seems acceptable.

Proposal: It is proposed not to consider earth curvature for TN-NTN co-existence.

- Conclusion

This paper compared the co-existence performance w/w.o. considering earth curvature. According to our simulation results the following observation and proposal are concluded.

Observation: The impact of earth curvature on co-existence performance under TN DLNTN DL scenario seems acceptable.

Proposal: It is proposed not to consider earth curvature for TN-NTN co-existence.

- Reference

[1] R4-2106105, Simulation assumptions for NTN co-existence study, Samsung, CATT