3GPP TSG-RAN WG4 Meeting #99-e		R4-2109192

E-meeting, 19 May – 27 May, 2021

Agenda item:	6.2.4.2.1

Source:	Intel Corporation

Title:	Simulation results for NR V2X single link PSSCH requirements

Document for:	Information

# Introduction

In the RAN4 #98-e meeting, WF on single link tests for NR V2X demodulation performance was agreed [1].

In this paper we provide our alignment and impairment results for V2X single link PSSCH requirements based on agreed WF [1] and simulation assumptions [2].

# Simulation results

Based on agreements from RAN4 #97-e and #98-e meetings the following scenarios were agreed for requirements definition:

  - Scenario 1: QPSK and 500 km/h relative velocity

  - Scenario 2: 16QAM and 260km/h relative velocity

  - Scenario 3: 64QAM and 30km/h relative velocity

In Figure 1 we provide the detailed simulation results for all considered scenarios. In Table 1 we provide alignment and impairments results.

| QPSK and 500 km/h relative velocity | 16QAM and 260km/h relative velocity |
| --- | --- |
| 64QAM and 30km/h relative velocity |  |
| Figure 1. PSSCH simulation results |  |

Table 1. Summary of PSSCH results

|  | Scenario 1 | Scenario 2 | Scenario 3 |
| --- | --- | --- | --- |
| Alignment results | 1.4 | 5.9 | 12.2 |
| Impairment results | 2.9 | 7.9 | 14.7 |

# Conclusion

In this paper we provided alignment and impairment results for V2X demodulation PSSCH single link requirements.

# References

- R4-2103990 “WF on single link tests for NR V2X demodulation performance”, LGE, RAN4 #98-e, February 2021.

- R4-2103991 “Updated simulation assumption for NR V2X single link test cases”, Huawei, RAN4 #98-e, February 2021.