3GPP TSG-RAN WG4 Meeting #99-e	R4-2109423

Electronic Meeting, 19th - 27th May, 2021

Source:	ZTE Corporation

Title:	Further discussion on UL Tx switching between 2 uplink bands in Rel-17

Agenda Item:	9.3.2.3

Document for:	Discussion

# Introduction

In RAN4#98-e, new Rel-17 UL Tx switching, i.e., 2Tx to 2Tx switching for 2 carriers, and both 1Tx to 2Tx and 2Tx to 2Tx switching between one carrier at Band A and two consecutive carriers at Band B, has been discussed [1], where switching between two uplink bands consists of two schemes: 1Tx-2Tx switching and 2Tx-2Tx switching as shown in below figure:

Fig. 1, UL Tx switching between two uplink bands in Rel-17

In this contribution, we observe that the new switching schemes require to combine inter-band and intra-band contiguous CA together with UL-MIMO, and further discuss some potential issues related to this combining.

# Discussion

Fig. 2, Combining inter-band and intra-band contiguous CA with UL-MIMO

Fig. 2 illustrates how Tx switching between two uplink bands involves combining inter-band and intra-band contiguous CA with UL-MIMO. The whole framework is the inter-band CA between band A (Carrier #1) and band B (Carrier#2 and #3), and there is also intra-band contiguous CA operation within band B (Carrier#2 and #3), in addition to 2Tx for band B.

Observation 1: Tx switching between two uplink bands in Rel-17 involves combining inter-band and intra-band contiguous CA with UL-MIMO.

Subsequently, some RF requirements need to be studied and clarified on the operation of combining inter-band, intra-band contiguous and UL-MIMO, e.g., MPR/A-MRP, configured transmitted power etc.

Proposal 1: RAN4 to study and clarify some RF requirements, e.g., MPR/A-MPR, configured transmitted power etc., on the operation of combining inter-band, intra-band contiguous CA and UL-MIMO.

In current RAN4 specs, RF requirements are divided into sub-clauses with different suffix corresponding to the features, e.g., suffix A refers to CA operation, B refers to DC operation, C refers to SUL operation, and D refers to UL-MIMO operation. Considering Tx switching between 2 uplink bands in Rel-17 is the combination of inter-band, intra-band CA and UL-MIMO, if some RF requirements related to the operation need to be specified, then in which sub-clause in the specs these RF requirements should be captured should be discussed at first.

Proposal 2: If some RF requirements related to the operation need to be specified, then RAN4 should discuss at first in which sub-clause these RF requirements should be captured in the specs.

# Conclusion

In this paper, we have the following observations and proposal for Tx switching between two uplink bands in Rel-17:

Observation 1: Tx switching between two uplink bands in Rel-17 involves combining inter-band and intra-band contiguous CA with UL-MIMO.

Proposal 1: RAN4 to study and clarify some RF requirements, e.g., MPR/A-MPR, configured transmitted power etc., on the operation of combining inter-band, intra-band contiguous CA and UL-MIMO.

Proposal 2: If some RF requirements related to the operation need to be specified, then RAN4 should discuss at first in which sub-clause these RF requirements should be captured in the specs.

# Reference

R4-2105466, Email discussion summary for [98b-e][129] NR_RF_FR1_enh_Part_2, China Telecom