3GPP TSG-RAN WG4 Meeting#99-e	R4-2109464

Online, 19th May – 27th May, 2021

Source: 	Qualcomm Incorporated

Title: 	Simulation Results for Application Layer Throughput Tests

Agenda item:	10.7.3

Document for:	Discussion

- Introduction

In RAN4#99-e, RAN4 agreed on simulation assumptions in [1] to study the feasibility of defining requirements with link adaptation. In this paper, we provide simulation results based on [1].

- Simulation Results

Based on simulation assumptions in [1], below are the link adaptation results for each case:

Table 1: Link Adaptation Throughput results for FR1 FDD 10MHz, 15kHz

| SNR (dB) | 2x2 |  |  | 2x4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Median CQI | Median Rank | Throughput (Mbps) | Median CQI | Median Rank | Throughput (Mbps) |
| 0 | 3 | 1 | 3.14 | 3 | 1 | 10.93 |
| 2 | 3 | 1 | 4.55 | 4 | 1 | 14.54 |
| 4 | 4 | 1 | 6.15 | 4 | 1 | 20.43 |
| 6 | 6 | 1 | 8.06 | 6 | 1 | 25.37 |
| 8 | 6 | 1 | 9.98 | 6 | 1 | 33.16 |
| 10 | 8 | 1 | 12.16 | 7 | 1 | 39.81 |
| 12 | 5 | 2 | 14.63 | 5 | 2 | 47.31 |
| 14 | 6 | 2 | 17.29 | 6 | 2 | 53.96 |
| 16 | 6 | 2 | 20.05 | 6 | 2 | 62.51 |
| 18 | 8 | 2 | 22.99 | 7 | 2 | 71.73 |
| 20 | 8 | 2 | 25.75 | 8 | 2 | 82.65 |

Table 2: Link Adaptation Throughput results for FR1 TDD 40MHz, 30kHz

| SNR (dB) | 2x2 |  |  | 2x4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Median CQI | Median Rank | Throughput (Mbps) | Median CQI | Median Rank | Throughput (Mbps) |
| 2 | 3 | 1 | 7.56 | 3 | 2 | 22.90 |
| 4 | 4 | 2 | 9.98 | 3 | 2 | 29.74 |
| 6 | 5 | 2 | 13.11 | 5 | 2 | 41.33 |
| 8 | 6 | 2 | 16.53 | 5 | 2 | 52.25 |
| 10 | 6 | 2 | 20.14 | 6 | 2 | 64.32 |
| 12 | 8 | 2 | 23.75 | 7 | 2 | 74.48 |
| 14 | 8 | 2 | 28.03 | 8 | 2 | 85.31 |
| 16 | 9 | 2 | 32.21 | 8 | 2 | 97.85 |
| 18 | 10 | 2 | 35.91 | 10 | 2 | 112.10 |
| 20 | 12 | 2 | 40.47 | 11 | 2 | 123.50 |

Table 3: Link Adaptation Throughput results for FR2 TDD 100MHz, 120kHz, 2x2

| SNR (dB) | Median CQI | Median Rank | Throughput (Mbps) |
| --- | --- | --- | --- |
| 0 | 5 | 1 | 17.29 |
| 2 | 6 | 1 | 22.90 |
| 4 | 7 | 1 | 30.50 |
| 6 | 8 | 1 | 38.95 |
| 8 | 9 | 1 | 46.84 |
| 10 | 9 | 1 | 56.34 |
| 12 | 11 | 1 | 68.97 |
| 14 | 8 | 2 | 80.85 |
| 16 | 9 | 2 | 95.95 |

- Test Parameters

Based on the simulation results provided in Section 2 above, SNR of 20dB in FR1 and 16dB in FR2 are clearly in Rank 2 regime. Therefore, we propose the following.

Proposal 1: Use 20dB SNR for FR1 and 16dB SNR for FR2 as baseline for studying the feasibility of defining NR link adaptation throughput requirements.

- Conclusions

This paper provides simulation results for studying LA throughput requirements in RAN4. Following has been proposed.

Proposal 1: Use 20dB SNR for FR1 and 16dB SNR for FR2 as baseline for studying the feasibility of defining NR link adaptation throughput requirements.

# References

[1] R4-2106122, “WF on 5G NR UE Application Layer Data Throughput Performance”, 3GPP TSG RAN WG4 Meeting #98bis-e, Online, April 12 - April 20, 2021.