3GPP TSG RAN WG4 Meeting #99-e	R4-2109470

Electronic Meeting, 19 - 27 May., 2021

Agenda item:	8.11.2

Source:	SoftBank Corp.

Title:	TP for TR 38.717-03-01: CA_n3-n77-n79

Document for:	Approval

1	Introduction

3B NR CA of CA_n3-n77-n79 was approved in RAN#91[1]. This TP is to capture the basic aspects for NR CA.

2.  Reference

[1]  RP-210901 Revised WID: Rel-17 NR inter-band CA for 3 bands DL with 1 band UL, CATT

Annex.

[Unchanged Parts Skipped]

6.X	CA_n3-n77-n79

6.X.1	Operating band for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n77-n79 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | NR Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n77A-n79A4 | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n3A-n77(2A)-n79A4 | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| NOTE 3: The SCS of each channel bandwidth for NR band refer to Table 5.3.5-1. NOTE 4: The minimum requirements only apply for non simultaneous Tx/Rx between all carriers for TDD combinations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.X.3	Co-existence studies

Co-existence studies can be omitted because harmonic interference have been already studied for 2DL/1UL fallback combinations such as CA n3-n77, CA_n3-n79 and n77-n79.

6.X.4	∆TIB and ∆RIB values

For CA_n3-n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n77-n79 | n3 | 0.8 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 6.X.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n3-n77-n79 | n3 | 0.5 |
|  | n77 | 0.5 |
|  | n79 | 0 |

6.X.5	REFSENS requirements

Based on 6.X.3, there are no additional MSD requirements for this band combination since harmonic interference from n3 to n77 and n79, and from n77 to n3 and n79, and from n79 to n3 and n77 have been already studied.

[Unchanged Parts Skipped]