3GPP TSG-WG RAN4 Meeting #99e	R4-2109583

# Electronic Meeting, 19 - 27 May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0783 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UL MIMO coherence for Tx switching between two carriers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2021-05-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For some UE architectures, UE will lose cohrence between two antenna ports when it uses one or both TX chains to transmit on an other band; while for some other UE architectures, the coherence can be maintianed as the same regardless of whether Tx switching is triggered or not. Different capabibities are needed to differentiate the two types of UEs. For the former type of UE architectures, exception needs to be added to the list of coherent UL MIMO section. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the former type of UE architectures listed above, added exception for TX switching for coherent UL MIMO requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE is mandated to maintain the same UL MIMO coherence regardless of whether Tx switching is triggered or not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4D.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of first change >

6.4D.4	Requirements for coherent UL MIMO

For coherent UL MIMO, Table 6.4D.4-1 lists the maximum allowable difference between the measured relative power and phase errors between different antenna ports in any slot within the specified time window from the last transmitted SRS on the same antenna ports, for the purpose of uplink transmission (codebook or non-codebook usage) and those measured at that last SRS. The requirements in Table 6.4D.4-1 apply when the UL transmission power at each antenna port is larger than 0 dBm for SRS transmission and for the duration of time window.

Table 6.4D.4-1: Maximum allowable difference of relative phase and power errors in a given slot compared to those measured at last SRS transmitted

| Difference of relative phase error | Difference of relative power error | Time window |
| --- | --- | --- |
| 40 degrees | 4 dB | 20 msec |

The above requirements when all the following conditions are met within the specified time window:

-	UE is not signaled with a change in number of SRS ports in SRS-config, or a change in PUSCH-config

-	UE remains in DRX active time (UE does not enter DRX OFF time)

-	No measurement gap occurs

-	No instance of SRS transmission with the usage antenna switching occurs

-	Active BWP remains the same

-	EN-DC and CA configuration is not changed for the UE (UE is not configured or de-configured with PSCell or SCell(s))

-	When UE is not configured with uplink switching with parameter uplinkTxSwitching-r16; or when UE is configured with uplink switching with parameter uplinkTxSwitching-r16, and the capability [uplinkTxSwitching-PSUCH-TransCoherence] is absent or indicated as [‘coherent’]; or when UE is configured with uplink switching with parameter uplinkTxSwitching-r16, the capability [uplinkTxSwitching-PSUCH-TransCoherence] is indicated as [‘non-coherent’], and uplink switching is not triggered by the switching mechanisms specified in sub-clause 6.1.6 of TS 38.214 [10] between last transmitted SRS and scheduled transmission.

< End of first change >