3GPP TSG-RAN WG4 Meeting #99-e	R4-2109589

Electronic meeting, 19th – 27th May, 2021

Source:	Ericsson

Title:	Simulation results for NR-U PDSCH demodulation requirement

Agenda item:	6.1.7.2

Document for:	information

# Introduction

In RAN4#98-bis-e meeting, all simulation assumptions are agreed. In this contribution, we resubmit the simulation based on the WF [1].

# Simulation assumptions and results for PDSCH

Table 2-1	PDSCH demodulation parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active DL BWP index |  |  | 1 |
| Slot Pattern |  |  | 7D1S2U |
| LBT failure probability |  |  | 0.25 |
| Common serving cell parameters | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | The first SSB |
|  | SSB Q factor |  | 8 |
| PDCCH configuration | TCI state |  | TCI state #0 |
| CSI-RS for tracking | Periodicity | slot | 40 |
|  | Offset | slot | 20 Slots for CSI-Resource 1,2 21 Slots for CSI-Resource 3,4 |
|  | RB size |  | NBWP,isize |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | According to DL Transmission Model |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| RB Allocation |  |  | Full BW |
| MIMO Layers |  |  | 2 |
| Number of HARQ Processes |  |  | 8 |
| TX EVM |  |  | 6% |

Table 2-2	DL transmission model

| DL Transmission Model | Maximum DL COT Duration | ms | 4.5 |
| --- | --- | --- | --- |
|  | Idle Time after UL COT | ms | 0.5 |
|  | DL Transmission Model Period/ Fixed Frame Period (Note 1) | ms | 5 |
|  | Probability of LBT Failure pLBT |  | 0.25 |
|  | UL scheduling for HARQ feedback within FFP |  | Starts at 4ms (8th slot) and lasts for 0.5 ms (1 slot). |
|  | Possible values of the DL Transmission duration within the COT | Slots | {2,4,6,7} |
|  | Possible values of the overall duration of the last slot in the DL Transmission duration | Symbols | If DL Transmission duration is equal to 2 Slots = {14}; If DL If DL Transmission duration is larger than 2 Slots = {6,9,12,14}; |
| Notes: - The Fixed Frame Period denomination applies only for ChannelAccessType-r16 = ‘semistatic’. For ChannelAccessType-r16 = ‘dynamic’ this parameter is identified only as DL Transmission Model Period. |  |  |  |

Table 2-3 Simulation results for PDSCH

| Test num. | Bandwidth [MHz] | SCS [kHz] | Modulation and code rate | Propagation condition | Correlation matrix and antenna configuration | Fraction of maximum throughput (%) | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Ideal | Impairme nt |
| 1-1a | 20 | 30 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 10.6 | 12.6 |
| 1-1b | 40 | 30 |  |  |  |  | 10.9 | 12.9 |
| 1-1c | 60 | 30 |  |  |  |  | 11.2 | 13.2 |
| 1-1d | 80 | 30 |  |  |  |  | 11.4 | 13.4 |
| 2-1a | 20 | 30 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 5.6 | 7.6 |
| 2-1b | 40 | 30 |  |  |  |  | 5.7 | 7.7 |
| 2-1c | 60 | 30 |  |  |  |  | 6 | 8 |
| 2-1d | 80 | 30 |  |  |  |  | 6 | 8 |

# Conclusion

In this contribution, we deliver our simulation results based on the WF in the last meeting.

# References

[1]	R4-2106173, “Way forward on NR-U UE demodulation requirements”, RAN4#98-bis-e, Qualcomm