3GPP TSG-RAN WG4 Meeting #99-e	R4-2109594

Electronic meeting, 19th – 27th May, 2021

Source:	Ericsson

Title:	Simulation results for NR-U PUCCH demodulation requirements

Agenda item:	6.1.7.4.3

Document for:	information

# Introduction

In RAN4#98-bis-e meeting, all PUCCH simulation assumptions were agreed [1]. Information bits pattern are agreed for PF0/PF1/PF3. The simulation results for PF3 are also quite divergent between companies. This contribution delivers all simulation results based on agreed assumptions.

# Simulation assumptions and results

### 2.1	PUCCH Format 0

Table 2.1.1 Simulation assumptions for PF0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits Note 1 | 1 |
| Number of symbol | 1 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note1 |
| Propagation conditions | TDLA30-10 low |
| Test metric | SNR@Prob(ACK miss)≤10^(−2) Note 3 |
| Note 1: The UCI information only include HARQ-ACK and the bit pattern is [1]. Note 2: RBs 0, 10, 20,…,100 are allocated for 15kHz if agreed and RBs 0,5,10,…,50 are allocated for 30kHz. Note 3: The DTX to ACK probability shall not exceed 1% |  |

Table 2.1.2 Simulation results for PF0

| ePF0 | Ideal SNR [dB] | Impairment SNR [dB] |
| --- | --- | --- |
| SCS = 15 kHz | -5.1 | -3.1 |
| SCS = 30 kHz | -5.2 | -3.2 |

### 2.2	PUCCH Format 1

Table 2.2.1 Simulation assumptions for PF1

| Parameter | Test |
| --- | --- |
| Number of information bits Note 1 | 2 |
| Number of symbols | 14 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note2 |
| Propagation conditions | TDLA30-10 low |
| Test metric | SNR@Prob(ACK miss)≤10^(−2)) SNR@Prob(NACK→ACK)≤10^(−3)) Note3 |
| Note 1: The UCI information only contain HARQ-ACK and the bit pattern is [0 1]. Note 2: RBs 0, 10, 20,…,100 are allocated for 15kHz if agreed and RBs 0,5,10,…,50 are allocated for 30kHz. Note 3: The DTX to ACK probability shall not exceed 1% |  |

Table 2.2.2 Ideal simulation results for PF1

| ePF1 | ACK miss |  | NACK to ACK |  |
| --- | --- | --- | --- | --- |
|  | Ideal SNR [dB] | Impairment SNR [dB] | Ideal SNR [dB] | Impairment SNR [dB] |
| SCS = 15 kHz | -16.3 | -14.3 | -16.1 | -14.1 |
| SCS = 30 kHz | -16.4 | -14.4 | -16.1 | -14.1 |

### 2.3	PUCCH Format 2

Table 2.3.1 Simulation assumptions for PF2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Intra-slot frequency hopping | N/A |
| Number of symbols | 1 |
| The number of UCI information bits Note 1 | 22 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note 2 |
| Propagation conditions | TDLA30-10 Low |
| OCC-Length-r16 | Not configured |
| Test metric | SNR @ ProbUCI block BLER=10-2 |
| Note 1: The UCI information only contain HARQ-ACK and CSI part 1. Note 2: RBs 0, 10, 20,…,100 are allocated for 15kHz and RBs 0,5,10,…,50 are allocated for 30kHz. |  |

Table 2.3.2 Ideal simulation results for PF2

| ePF2 | Ideal SNR [dB] | Impairment SNR [dB] |
| --- | --- | --- |
| SCS = 15 kHz | 1.3 | 3.3 |
| SCS = 30 kHz | 1.2 | 3.2 |

### 2.4	PUCCH Format 3

Table 2.4.1 Simulation assumptions for PF3

| Parameter | Test 1 |
| --- | --- |
| Modulation order | QPSK |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Additional DM-RS configuration | No additional DM-RS |
| Number of symbols | 4 |
| The number of UCI information bits Note 1 | 4 bits |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Antenna configuration | 1T2R |
| Number of interlaces | 1 |
| Interlace index | 0Note 2 |
| Propagation conditions | TDLA30-10 Low |
| Index of OCC | Not configure |
| Length of OCC | Not configure |
| Cyclic shift index for DMRS | 0 |
| Test metric | Option 1: SNR @ ProbUCI block BLER=10-2 Option 2: SNR@Prob(ACK miss)≤10^(−2) SNR@Prob(NACK→ACK)≤10^(−3)) Note3 |
| Note 1: The UCI information only contain HARQ-ACK and the bit pattern is [1 1 1 1]. Note 2: RBs 0, 10, 20,…,90 are allocated for 15kHz and RBs 0,5,10,…,45 are allocated for 30kHz. Note 3: The DTX to ACK probability shall not exceed 1% |  |

The simulation for ePF3 is still ongoing by the time of this contribution delivery.

Table 2.4.2 Ideal simulation results for PF3

| ePF3 | Ideal SNR [dB] | Impairment SNR [dB] |
| --- | --- | --- |
| SCS = 15 kHz |  |  |
| SCS = 30 kHz |  |  |

# Conclusion

In this contribution, we deliver our simulation results based on the WF [1] in the last meeting.

# References

[1]	R4-2016010, “Way forward on NR-U BS demodulation requirements”, RAN4#98-bis-e, Huawei, HiSilicon