3GPP TSG-RAN WG4 Meeting #99-e	R4-2109689

# Electronic Meeting, 19th - 27th May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0542 | rev | <Rev#> | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-3 Switching period position for NR V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-5-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The output power dynamics for NR V2X is captured for TS 38.101-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Sub-clause 6.3E Output power dynamics for V2X operation to the spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Core requirements for NR V2X will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change >

6.3E	Output power dynamics for V2X operation

6.3E.1	General

The E-UTRA V2X and NR V2X switching time mask defines the observation period between E-UTRA V2X subframe and NR V2X slot/mini-slot boundary. Both E-UTRA V2X subframe and NR V2X slot/mini-slot have ON power transmissions. The ON power is defined as the mean power over the symbol duration excluding any transient period. For E-UTRA V2X subframe or NR V2X slot/mini-slot having OFF power transmission, the general time mask for E-UTRA V2X or NR V2X shall apply.

For intra-band V2X operation, the output power dynamics specified in clause 6.3.2G, 6.3.3G, 6.3.4G and 6.3.5G [4] shall be applied for E-UTRA V2X transmission or the output power dynamics specified in clause 6.3E [2] shall be applied for NR V2X transmission, respectively.

For the inter-band con-current NR V2X operation, output power dynamics requirement specified in subclause 6.3 of TS 36.101 [4] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.3E of TS 38.101-1 [2] shall apply for the NR sidelink in V2X operating bands.

6.3E.2	Output power dynamics for intra-band V2X operation

#### 6.3E.2.1	E-UTRA V2X SL and NR V2X SL switching time mask in TDM manner

For intra-band V2X operations specified in subclause 5.3E.1 and 5.3E.2, the SL switching time mask from LTE-V2X to NR-V2X in Figure 6.3E.2.1-1 shall apply.

Switching period

ON power Requirement

NR V2X slot/minislot

ON power

Requirement

E-UTRA V2X subframe

Figure 6.3E.2.1-1: E-UTRA V2X SL to NR V2X SL switching time mask in TDM manner

#### 6.3E.2.1	NR V2X SL and E-UTRA V2X SL switching time mask in TDM manner

For intra-band V2X operations specified in subclause 5.3E.1 and 5.3E.2, the SL switching time mask from NR V2X to LTE V2X in Figure 6.3E.2.1-2 shall apply.

Switching period

ON power Requirement

E-UTRA V2X subframe

ON power

Requirement

NR V2X slot/minislot

Figure 6.3E.2.1-2:NR V2X SL to E-UTRA V2X SL switching time mask in TDM manner

<End of change >