3GPP TSG-RAN WG4 Meeting # 99-e	R4-2109703

# Electronic Meeting, May. 19-27, 2021

Source:	LG Electronics

Title:	MPR of transmit diversity for power class2

Agenda item:	6.7.1.1

Document for:	Discussion

Introduction

In this paper, we provide our view on MPR of NR Tx diversity for power class 2 based on simulation results.

Discussion

In last RAN4 meeting, one WF[1] was agreed for MPR evaluation for NR TxD & UL-MIMO. Based on the WF, the followings are assumed for simulations.

Simulation Assumption

- Architecture - PC2 with 2x23dBm PAs => 2Tx NR MPR (High priority) - Waveform - CP-OFDM , DFT-s-OFDM - Maximum total output power - 26dBm : 23dBm 2 PAs - Modulation order - QPSK/16QAM/64QAM/256QAM - Channel Bandwidth & SCS - CBW_20MHz & SCS_15kHz - CBW_100MHz & SCS_30kHz - LCRB sizes - [1 3 7 10 20 30 45 60 75 90 106] PRBs for CBW_20MHz & SCS_15kHz - [1 5 10 20 35 55 75 100 130 160 190 220 250 273] PRBs for CBW_100MHz & SCS_30kHz

The following figures show simulation results of MPR for power class 2 with Tx diversity.

- Figure 2-1 : CBW_20MHz & SCS_15kHz for CP-OFDM

- Figure 2-2 : CBW_100MHz & SCS_30kHz for CP-OFDM

- Figure 2-3 : CBW_20MHz & SCS_15kHz for DFT-s-OFDM

Figure 2.1 MPR of PC2 with Tx Diversity (23dBm+23dBm) for CBW 20MHz & SCS 15kHz with CP-OFDM

Figure 2.2 MPR of PC2 with Tx Diversity (23dBm+23dBm) for CBW 100MHz & SCS 30kHz with CP-OFDM

Figure 2.3 MPR of PC2 with Tx Diversity (23dBm+23dBm) for CBW 20MHz & SCS 15kHz with DFT-s-OFDM

Based on the simulation results, MPR for power class 2 with Tx diversity is proposed as Table 2.1.

Table 2.1 Maximum power reduction (MPR) for power class 2 with Tx Diversity

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM |  |  |  |  |
|  | QPSK | ≤ 6.5 | ≤ 4.5 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 2.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 4.5 |  |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 5.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: The MPR is applied to the sum of the output power at each transmit antenna connector. |  |  |  |  |

Proposal 1: Define MPR for power class 2 with Tx diversity with Table 2.1.

Conclusion

In this contribution, we provided simulation results of MPR for NR Tx Diversity for power class 2. Based on the simulation results, proposal is as follows.

Proposal 1: Define MPR for power class 2 with Tx diversity with Table 2.1.

Table 2.1 Maximum power reduction (MPR) for power class 2 with Tx Diversity

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM |  |  |  |  |
|  | QPSK | ≤ 6.5 | ≤ 4.5 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 2.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 4.5 |  |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 5.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: The MPR is applied to the sum of the output power at each transmit antenna connector. |  |  |  |  |

- Reference

[1] R4-2105331, “Way forward on MPR evaluation for NR TxD & UL-MIMO”, vivo, Skyworks