3GPP TSG-RAN WG4 Meeting #99-e	R4-2109749

Electronic Meeting, 19thMay- 27thMay, 2021

Source:	ZTE Corporation

Title:          Discussion on Reference Signal for UL and DL

Agenda Item:	9.8.5.1

Document for:	Discussion

- Introduction

In the RAN4 98e-bis meeting, several issues of reference signal for UL and DL were discussed but there are some issues open in the agreed WF[1] as below.

In this contribution, some discussion on reference signal for UL and DL in FR2 HST are provided.

- Discussion

RS for PUSCH:

DMRS+PTRS configuration for PUSCH demodulation requirement with single-tap channel model : - Option 1: 1 DMRS +PTRS (L=1,K=2) - Option 2: 1+1 DMRS +PTRS (L=1,K=2) - Option 3: 1+1+1 DMRS+PTRS(L=1, K=2)

In last meeting, the agreement[1] that it is feasible to support maximum speed with 350km for uplink with PTRS or DMRS+PTRS configuration used for frequency offset tracking with 120KHz SCS was reached.

Reference signal configuration for PUSCH demodulation requirement was discussed but this issue is still open, there are three options for this open issue in WF[1].

in our understanding, the doppler frequency offset estimated by PTRS(L=1,K=2) can cover the max doppler offset in 30GHz carrier frequency with speed of 350km/h. 1 DMRS is enough to demodulation requirement and less overhead could get higher throughput.

Proposal 1: one DMRS is enough to demodulation requirement for PUSCH.

RS for PDSCH:

RS as baseline for frequency offset tracking to support 350km/h : - Option 1: SSB+TRS - Option 2: - SSB+TRS+PTRS for bi-directional deployment - SSB+TRS for unidirectional deployment - Option 3:Other options are not precluded DMRS configuration for PDSCH demodulation requirement : - Option 1: 1 DMRS - Option 2: 1+1+1 DMRS

Because SSB and TRS are transmitted periodically, we assume UE perform frequency offset compensation before demodulation, there would be residual frequency offset(RFO) after frequency offset compensation in each SSB/TRS period. RFO is relate to the channel model, switching point, period of SSB/TRS.

It have not agreed whether channel model is used, we list the optional channel model in discussion of FR2 HST channel model[3] and analysis the doppler tracking trajectory for different channel with different switching point and different period of SSB.

The period of SSB can be configured to 5/10/20/40/80/120ms, the period of TRS can be configured to 10/20/40/80ms.we think the final RFO is decided by the short period RS if the RFO compensated by long period RS is within the estimation range of short period RS.

Table 1: FR2 HST deployment scenario

| Parameters | Value |  |
| --- | --- | --- |
|  | Scenario-A | Scenario-B |
| Ds(m) | 700 | 700 |
| Dmin(m) | 10 | 150 |

Channel model for uni-directional RRH deployment:

- Single-tap propagation channel:

- DPS propagation channel

- UE move towards:

- UE move away:

Channel model for bi-directional RRH deployment:

- Modified single-tap propagation channel:

- DPS propagation channel

Table 2: value of maximum RFO

| Maximum RFO(Hz) under different period of RS | Dmin=10m |  |  |  | Dmin=150m |
| --- | --- | --- | --- | --- | --- |
|  | 10ms | 20ms | 40ms | 80ms | 120ms |
| Uni- Single-tap channel | 846 | 1761 | 3429 | 5874 | 748 |
| Uni- DPS channel-move toward | 846 | 1761 | 3429 | 5874 | 748 |
| Uni- DPS channel-move away | 847 | 1766 | 3447 | 5922 | 748 |
| Bi- modified-Single-tap channel | 847 | 1766 | 3447 | 5922 | 748 |
| Bi- DPS channel | 0.02 | 0.04 | 0.08 | 0.2 | 44 |

Table 3: value of maximum RFO in different Ds_offset

| Maximum RFO(Hz) | Dmin=10m，RS=120ms |  |
| --- | --- | --- |
|  | Ds_offset=Ds | Ds_offset=Ds±0.1Ds |
| Uni- DPS channel-move toward | 7287 | 25(+0.1Ds) |
| Uni- DPS channel-move away | 7355 | 25(-0.1Ds) |

The maximum doppler frequency offset is 9722Hz in 30GHz carrier frequency with 350km/h velocity, maximum doppler frequency offset estimated by SSB is 28000Hz, maximum doppler frequency offset estimated by TRS is 14000Hz, from table 3, the maximum RFO with 120ms RS period is within the estimation range of TRS. So it is feasible to use SSB+TRS for frequency offset tracking to support 350km/h.

Proposal 2 : It is feasible to use SSB+TRS for frequency offset tracking to support 350km/h

Tc is Coherence time ,fm is maximum frequencyoffset

Table 4 value of coherence time

| Coherence time(ms) | Coherence time |  |
| --- | --- | --- |
|  | Dmin = 10m, 40ms period of RS | Dmin = 150, 120ms period of RS |
| Uni- Single-tap channel | 0.123 | 0.565 |
| Uni- DPS channel-move toward | 0.123 | 0.565 |
| Uni- DPS channel-move away | 0.122 | 0.565 |
| Bi- modified-Single-tap channel | 0.122 | 0.565 |
| Bi- DPS channel | 5287 | 9.6 |

Duration of a slot with SCS 120KHz is 0.125ms, from the table 4, the coherence time in bi-directional DPS channel with 120ms SSB periodis much larger than a slot duration, the coherence time inothers channel in scenario-B with 120ms SSB period is also larger than a slot duration.

The coherence time in another channel in scenario-A with 40ms SSB period is slight larger than a slot duration, so the coherence time would less than a slot duration in this deployment if SSB period larger than 40ms.

It would be no time selective fading in a slot when the coherence time larger than a slot duration, it means 1 DMRS is enough to demodulation.

Proposal 3 :TRS or SSB period <40ms

Proposal 4 :one DMRS is enough to demodulation requirement for PDSCH based on Proposal 3.

- Conclusion

In this contribution, we provide the view and analysis on discussion on reference signal for PUSCH and PDSCH in FR2 HST, based on these analysis, the following proposals are given:

Proposal 1 : one DMRS is enough to demodulation requirement for PUSCH.

Proposal 2 : Do not define a new HST network assisted signaling or UE capability for HST CA scenario

Proposal 3 : TRS or SSB period < 40ms

Proposal 4 : one DMRS is enough to demodulation requirement for PDSCH based on Proposal 3.

- Reference

R4-2106102 WF on Demodulation requirement for FR2 HST

R4-2106100 WF on FR2 HST Deployment Scenario Analysis

R4-2106101 Way Forward on Channel Modeling for FR2 HST

- Appendix

The figure of Doppler trajectory and Doppler estimation trajectory with different period RS and Dmin, and the figure of RFO trajectory with different Dmin, in Uni-directional Single-tap channel are as follows.

The figure of Doppler trajectory and Doppler estimation trajectory with different period RS and Dmin and D_s_offset, and the figure of RFO trajectory with different Dmin and D_s_offset, in Uni-directional DPS move toward channel are as follows.

The figure of Doppler trajectory and Doppler estimation trajectory with different period RS and Dmin and D_s_offset, and the figure of RFO trajectory with different Dmin and D_s_offset, in Uni-directional DPS move away channel are as follows.

The figure of Doppler trajectory and Doppler estimation trajectory with different period RS and Dmin, and the figure of RFO trajectory with different Dmin, in Bi-directional modified Single-tap channel are as follows.

The figure of Doppler trajectory and Doppler estimation trajectory with different period RS and Dmin, and the figure of RFO trajectory with different Dmin, in Bi-directional DPS channel are as follows.