3GPP TSG-RAN WG4 Meeting #99-e	R4-2109763

Electronic Meeting, 19th - 27th May, 2021

Source:	ZTE Corporation

Title:	System performance evaluation on FDD HPUE

Agenda Item:	10.5.4

Document for:	Discussion

- Introduction

In RAN#90 e-meeting, a new SI on high power UE (power class 2) for NR FDD band was approved [1]. One objective related to system performance evaluation is:

Evaluate system performance gains on spectrum efficiency, and other metrics if needed could also be taken into account, to support NR FDD HPUE.

In 98bis e-meeting, one WF [2] was approved. In this WF, some updated evaluation assumptions were given, especially multiple {P0, alpha} values were provided. In this contribution, we provide some additional system evaluation results for NR FDD HPUE based on the updated evaluation assumptions.

- Discussion

2.1 Simulation assumptions

Except for the dynamic system level evaluation, potential co-existence issue was also discussed in RAN4#98 e-meeting, so the evaluation assumption for Monte Carlo simulation was also discussed. But considering the co-existence study had already been done for LTE TDD HPUE in Rel-16, from the simulation assumption and simulation procedure perspectives, there is no big difference between TDD bands and FDD bands, so the conclusion from co-existence simulation evaluation for TDD HPUE can also be used for FDD HPUE. Only dynamic system level evaluation for FDD HPUE is needed.

The detailed simulation assumptions are shown in Table 1.

Table 1: Dynamic system level simulation assumptions

| Configuration parameters | Values |
| --- | --- |
| Scenario | Urban macro |
| ISD | 500 m |
| Duplexing | FDD |
| Carrier frequency | 1.8 GHz |
| Modulation | Up to 256QAM |
| Numerology | 15 kHz |
| Simulation bandwidth | 40 MHz |
| Transmission scheme | SU-MIMO |
| Codebook | For 2Tx, codebook [1 1]T is used for transmit diversity |
| SU dimension | 1 layer |
| Antenna configuration at TRxP | 4Rx, (M,N,P,Mg, Ng) = (1,2,10,1,1; 1,2) |
| Antenna configuration at UE | 1Tx, (M,N,P,Mg, Ng) = (1,1,1,1,1; 1,1), 2Tx, (M,N,P,Mg, Ng) = (1,1,2,1,1; 1,1), |
| UE maximal transmit power | For 1Tx, 23 dBm for each TXRU For 1Tx, 26 dBm for each TXRU (High power UE) For 2Tx, 23 dBm for each TXRU (High power UE) |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Channel estimation | Ideal |
| Power control parameter | P0=-60, alpha = 0.6 |
| TRxP number per site | 3 |
| TRxP number | 21 |
| Channel model | UMa following TR 38.901 |
| Electronic tilt | 102° |
| Traffic model | FTP3, packet size: 100k / 10k Byte, arrival rate: 5 packet / s. |

2.2 Simulation results

We provide the simulation for band n3. The detailed simulation results are shown in Table 2.

Table 2: Dynamic system level simulation results for 10kB packet size

| Antenna Configuration | Maximum Tx Power(dBm) | Duty cycle (%) | Packet size (k Byte) | Arrival rate (file/s) | 5% UPT (Mbps) | Average UPT (Mbps) |
| --- | --- | --- | --- | --- | --- | --- |
| 1Tx 4Rx | 23 | 100 | 10 | 5 | 20.65 0% | 63.81 0% |
| 1Tx 4Rx | 26 | 100 | 10 | 5 | 31.66 +53% | 72.38 +13% |
| 1Tx 4Rx | 26 | 50 | 10 | 5 | 32.69 +58% | 72.43 +14% |
| 2Tx 4Rx | 26 | 100 | 10 | 5 | 31.87 +54% | 73.22 +15% |
| 2Tx 4Rx | 26 | 50 | 10 | 5 | 32.31 +56% | 73.22 +15% |

Observation : From the simulation results, obvious performance gains on cell average throughput and cell edge throughput can be observed for the case of 10 kB packet size.

- Conclusion

In this contribution, we have the following observation for system performance evaluation on FDD HPUE:

Observation : From the simulation results, obvious performance gains on cell average throughput and cell edge throughput can be observed for the case of 10 kB packet size.

- Reference

RP-202870, New SID: Study on high power UE (power class 2) for one NR FDD band, China Unicom

R4-2105425, “WF on initial agreements on simulation results” , China Unicom