3GPP TSG-RAN WG4 Meeting #99-e              	R4-2109795

Electrical Meeting, 19th May-27th May, 2021

Agenda item:	6.1.7.4.2

Source:	Samsung

Title:	Simulation results of PUSCH for Rel-16 NR-U

Document for:	Discussion

# 1	Introduction

In the previous RAN4 meeting, it was agreed to define PUSCH with interlaced allocation performance requirement for Rel-16 NR-U WI.  The test parameters for related requirement were captured in WF [1].

In this contribution, the simulation results of PUSCH are provided for performance requirement derived.

# 2	Simulation Results

In this section, based on the agreed test parameters, both ideal and impairment results are provided.

2.1 Simulation Assumption

In this subsection, the details of simulation assumption for PUSCH and CG-UCI multiplexing on PUSCH are provided as indicated in table 1-1/1-2, respectively.

Table 1-1: simulation assumption for PUSCH

| Parameter | Value |
| --- | --- |
| Transform precoding | Disabled |
| Antenna configuration | 1T2R low |
| SCS and BW | 15KHz, 20MHz, 30KHz, 20 MHz |
| TDD pattern | 7D2S1U S=6D:4G:4U for 30KHz SCS 3D1S1U=10D:2G:2U for 15KHz |
| Interlace structure | First single interlace per slot |
| DM-RS type | Type 1 with single -symbol |
| Number of DMRS symbols | 1+1 |
| Symbol length | 14 |
| Start symbol index | 0 |
| Time domain resource allocation type | Type A and Type B |
| Frequency domain resource | Full applicable test bandwidth |
| MCS index | 20 |
| Carrier frequency | 4 |
| Propagation condition | TDA30-10 |
| Guard band | Not configured |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group, Frequency hopping, limited buffer rate matching | Disabled |
| Number of HARQ transmission | 4 |
| RV sequence | {0,2,3,1} |
| Testing metric | SNR@70% throughput |
| Note 1: RBs 0, 10, 20, …, 100 are allocated for 15KHz and RBs, 0, 5, 10, …, 50 are allocated for 30KHz |  |

Table 1-2: simulation assumption for CG-UCI multiplexing on PUSCH

| Parameter | Value |  |
| --- | --- | --- |
| Transform precoding | Disabled |  |
| Antenna configuration | 1T2R low |  |
| SCS and BW | 15KHz, 20MHz, 30KHz, 20 MHz |  |
| TDD pattern | 7D2S1U S=6D:4G:4U for 30KHz SCS 3D1S1U=10D:2G:2U for 15KHz |  |
| Interlace structure | First single interlace per slot |  |
| Time domain resource allocation | Type 1 with single -symbol |  |
| DM-RS configuration | 1+1 |  |
| MCS | 14 |  |
| Propagation condition | 0 |  |
| RV sequence | Type A and Type B |  |
| Test metric | Full applicable test bandwidth |  |
| MCS index | 20 |  |
| CG | Payload | 18 bits |
|  | BetaOffsetCG-UCI-index (value) | 8 (10) |
|  | Scaling | 1 |
|  | UCI partition for frequency hopping | Disabled |
| Pattern of CG-UCI information | [c0, c1, …c17], HARQ process number: [c0,c1,c2,c3] = [0 0 0 1], RV [c4,c5]=[0 0 ], NDI: [c6]=1, COT sharing information: [c7,c8,....c17]=[0 0 ...0] |  |
| Note 1: RBs 0, 10, 20, …, 100 are allocated for 15KHz and RBs, 0, 5, 10, …, 50 are allocated for 30KHz |  |  |

2.2 Simulation Results

In this subsection, the simulation results for PUSCH and CG-UCI multiplexing on PUSCH are provided as indicated in table 2-1/2-2, respectively.

Table 2-1: ideal and impairment simulation results for PUSCH

| Tx/Rx | SCS& BW | Mapping type | Symbol length | DMRS configuration | Channel | Number of interlace | Interlace index | SNR@70% of TP(ideal) | SNR@70% of TP(impairment ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1T2R | 15 KHz, 20MHz | A | 14 | 1+1 (2,11) | TDLA30-10 | 1 | {0,10, 20,…,100} | 10.9 | 13.4 |
| 1T2R | 30 KHz, 20MHz | A | 14 | 1+1 (2,11) | TDLA30-10 | 1 | {0,5, 10,…,50} | 10.9 | 13.4 |
| 1T2R | 15 KHz, 20MHz | B | 14 | 1+1 (0,10) | TDLA30-10 | 1 | {0,10, 20,…,100} | 10.8 | 13.3 |
| 1T2R | 30 KHz, 20MHz | B | 14 | 1+1 (0,10) | TDLA30-10 | 1 | {0,5, 10,…,50} | 10.8 | 13.3 |

Table 2-2: ideal and impairment simulation results for CG-UCI on PUSCH

| Tx/Rx | SCS& BW | Mapping type | CG-UCI payload | Symbol length | DMRS configuration | Channel | Number of interlace | Interlace index | 1% BLER for UCI SNR (ideal results) | 1% BLER for UCI SNR (impairment results) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1T2R | 15 KHz, 20MHz | A | 18 | 14 | 1+1 (2,11) | TDLA30-10 | 1 | {0,10, 20,…,100} | 3.75 | 6.25 |
| 1T2R | 30 KHz, 20MHz | A | 18 | 14 | 1+1 (2,11) | TDLA30-10 | 1 | {0,10, 20,…,100} | 2.78 | 5.28 |
| 1T2R | 15KHz | B | 18 | 14 | 1+1 (0,10) | TDLA30-10 | 1 | {0,10, 20,…,100} | 3.85 | 6.35 |
| 1T2R | 30KHz | B | 18 | 14 | 1+1 (0,10) | TDLA30-10 | 1 | {0,10, 20,…,100} | 2.56 | 5.06 |

# 3	Conclusion

In this contribution, the ideal and impairment results for PUSCH and CG-UCI multiplexing on PUSCH are provided for performance requirement derived.

# References

R4-2106010, WF on the NR-U BS demodulation requirement, Huawei