3GPP TSG-RAN WG4 Meeting #99-e              	R4-2109796

Electrical Meeting, 19th May-27th May, 2021

Agenda item:	6.1.7.4.3

Source:	Samsung

Title:	Simulation results of PUCCH for Rel-16 NR-U

Document for:	Discussion

# 1	Introduction

In the previous RAN4 meeting, it was agreed to define the PUCCH with interlaced allocation performance requirement for Rel-16 NR-U WI.  The test parameters for related requirement were captured in WF [1].

In this contribution, the simulation results of PUCCH are provided for performance requirement derived.

# 2	Simulation Results

In this section, based on the agreed test parameters, both ideal and impairment results are provided.

2.1 Simulation Assumption

In this subsection, the details of simulation assumption for ePF0/1/2/3 are provided as indicated in table 1-1/1-2/1-3/1-4, respectively.

Table 1-1: simulation assumption for ePF0

| Parameter | Value |
| --- | --- |
| Number of information bits | 1 |
| Number of symbols | 1 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20 MHz |
| SCS | 15 KHz and 30 KHz |
| Number of interlace | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLA30-10 low |
| Test metric | SNR@Prob (ACK missed<=10^(-2)) SNR@Prob(DTX->ACK<=10^(-2)) |
| Pattern of information bits | [1] |
| Note 1: RBs 0, 10, 20, …, 100 are allocated for 15KHz and RBs, 0, 5, 10, …, 50 are allocated for 30KHz |  |

Table 1-2: simulation assumption for ePF1

| Parameter | Value |
| --- | --- |
| Number of information bits | 1 |
| Number of symbols | 14 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code | 0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15KHz and 30KHz |
| Number of interlace | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLA30-10 low |
| Test metric | SNR @ Prob (ACK miss) <=10^(-2) SNR @ Prob (PUCCH DTX->ACK)<=10^(-2) SNR@Prob (NACK->ACK)<=10^(-3) |
| Pattern of information bits | [c0 c1]= [0 1] |
| Note 1: RBs 0, 10, 20, …, 100 are allocated for 15KHz and RBs, 0, 5, 10, …, 50 are allocated for 30KHz |  |

Table 1-3: simulation assumption for ePF2

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| Intra-slot frequency hopping | N/A |
| Number of symbols | 1 |
| The number of UCI information bits | 22 |
| Frist symbol | 13 |
| DM-RS sequence generation | N_ID=0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15 KHz and 30 KHz |
| Number of interlace | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLA30-10 low |
| OCC-length-r16 | Not configured |
| Test metric | SNR @ Prob (UCI block BLER) =10^-2 |
| Pattern of information bits | Random information bits selection |
| Note 1: RBs 0, 10, 20, …, 100 are allocated for 15KHz and RBs, 0, 5, 10, …, 50 are allocated for 30KHz Note 2: The UCI information does not contain CSI part 2 |  |

Table 1-4: simulation assumption for ePF3

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Additional DM-RS configuration | No additional DM-RS |
| Number of symbols | 4 |
| The number of UCI information bits | 4 bits |
| Channel bandwidth | 20MHz |
| SCS | 15KHz and 30KHz |
| Antenna configuration | 1T2R |
| Number of interlaces | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLA30-10 low |
| Index of OCC | Not configure |
| Length of OCC | Not configure |
| Cyclic shift index for DMRS | 0 |
| Test metric | Prob (DTX>ACK)<=1% and Prob(ACK miss) <1% |
| Pattern of information bits | [c0 c1 c2 c3]=[1 1 1 1] |
| Note 1: RBs 0, 10, 20, …, 90 are allocated for 15KHz and RBs, 0, 5, 10, …, 45 are allocated for 30KHz Note 2: The UCI information does not contain CSI part 2 |  |

2.2 Simulation Results

In this subsection, the simulation results for ePF0/1/2/3 are provided as indicated in table 2-1/2-2/2-3/2-4, respectively.

Table 2-1: ideal and impairment simulation results for ePF0

| Tx/Rx | SCS& BW | UCI bits | Symbol length | Channel | Number of interlaces | Interlace index | SNR (impairment results) | SNR (impairment results) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1T2R | 15 KHz, 20MHz | 1 | 1 | TDA30-10 | 1 | {0,10,20,…100} | -4.5 | -2.5 |
| 1T2R | 30 KHz, 20MHz | 1 | 1 | TDA30-10 | 1 | {0, 5, 10, …,50} | -3.5 | -1.5 |

Table 2-2: ideal and impairment simulation results for ePF1

| Tx/Rx | SCS& BW | UCI bits | Symbol length | Channel | Number of interlaces | Interlace index | Metric | SNR (impairment results) | SNR (impairment results) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1T2R | 15 KHz, 20MHz | 2 | 14 | TDLA30-10 | 1 | 0,10,…100 | 1% ACK miss | -15.9 | -13.9 |
|  |  |  |  |  |  |  | 0.1% NACK to ACK | -15.1 | -13.1 |
| 1T2R | 30 KHz, 20MHz | 2 | 14 | TDLA30-10 | 1 | 0, 5,…,50 | 1% ACK miss | -15.2 | -13.2 |
|  |  |  |  |  |  |  | 0.1% NACK to ACK | -14.4 | -12.4 |

Table 2-3: ideal and impairment simulation results for ePF2

| Tx/Rx | SCS& BW | UCI bits | Symbol length | Channel | Number of interlaces | Interlace index | SNR (impairment results) | SNR (impairment results) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1T2R | 15 KHz, 20MHz | 22 | 1 | TDLA30-10 | 1 | 0,10,…100 | 2.3 | 4.3 |
| 1T2R | 30 KHz, 20MHz | 22 | 1 | TDLA30-10 | 1 | 0,5,…,50 | 2.3 | 4.3 |

Table 2-4: ideal and impairment simulation results for ePF3

| Tx/Rx | SCS& BW | UCI bits | Symbol length | Channel | Number of interlaces | DMRS configuration | Interlace index | SNR (impairment results) | SNR (impairment results) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1T2R | 15 KHz, 20MHz | 4 | 4 | TDLA30-10 | 1 | 1 | 0,10,…90 | -7.7 | -5.7 |
| 1T2R | 30 KHz, 20MHz | 4 | 4 | TDLA30-10 | 1 | 1 | 0.5,…,45 | -7.6 | -5.6 |

# 3	Conclusion

In this contribution, the ideal and impairment results for PUCCH are provided for performance requirement derived.

# References

R4-2103989, WF on the NR-U PUCCH demodulation requirement, Ericsson