3GPP TSG RAN WG4 Meeting #99-e	R4-2109935

Electronic Meeting, May 19 – May 27, 2021

Agenda item:			6.5.1.2

Source:	vivo

Title:	Remaining issues on PRS RSRP measurement requirements

Document for:	Discussion

- Introduction

In the RAN4#98bis-e meeting, the measurement period requirements for PRS-RSRP when configured with RSTD or UE Rx-Tx were discussed. No agreement was reached. The remaining open issue on PRS-RSRP measurement core requirements is captured in [1].

Our views on PRS-RSRP measurement core requirements were provided in [2] in the last meeting. In this contribution, we further provide our views on PRS-RSRP measurement requirements when configured for DL-TDOA or multi-RTT.

# Discussion

The open issue for PRS-RSRP measurement is captured in [1] as follows.

- PRS-RSRP requirements when configured for DL-TDOA or multi-RTT - Option 1 (QC) - According to a prior agreement, RAN4 may revise the number of samples (Nsample) in the requirement if more samples are needed to meet PRS-RSRP accuracy requirements. - Option 2a (vivo, HW, CATT, OPPO, Intel) - UE behaviour is not defined when PRS-RSRP is configured additionally to RSTD or UE Rx-Tx measurement. - Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA or Multi-RTT. - Option 2b (Nokia, Intel) - Regarding UE behavior, a UE continues to measure PRS-RSRP over the entire RSTD/UE Rx-Tx measurement period, when PRS-RSRP is configured together with RSTD/UE Rx-Tx. - RSRP requirements in clause 9.9.3 also apply for the case when configuring PRS-RSRP with RSTD or UE Rx-Tx. - Option 3 (Ericsson, Nokia) - UE behavior when PRS-RSRP is configured together with RSTD/UE Rx-Tx and the required PRS-RSRP measurement period is shorter than that for RSTD/UE Rx-Tx (configured without PRS-RSRP), then the PRS-RSRP measurement continues over the entire RSTD/UE Rx-Tx measurement period.

In the last meeting, there was agreement on RSTD measurement period requirements when PRS-RSRP is additionally configured.

- When PRS-RSRP is configured for DL-TDOA, RSTD and RSRP are performed over the same measurement period.

Therefore, from PRS-RSRP measurement perspective it would be performed over the RSTD measurement period. According to existing measurement period requirements in clauses 9.9.3 and 9.9.2, the measurement period is the same for PRS-RSRPmeasurement and RSTD measurement. Thus, PRS-RSRP measurement period requirements in clause 9.9.3 also apply for the case when PRS-RSRP is configured additionally with RSTD.

Proposal 1: When PRS-RSRP is configured for DL-TDOA, RSTD and PRS-RSRP are performed over the same measurement period. Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA.

When PRS-RSRP is configured for multi-RTT, the same principle should apply.

Proposal 2: When PRS-RSRP is configured for multi-RTT, UE Rx-Tx time difference and PRS-RSRP are performed over the same measurement period.

Proposal 3: Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for Multi-RTT.

# Summary

In this contribution we provided our views on PRS-RSRP measurement requirements when configured for DL-TDOA or multi-RTT. Based on analysis following proposals are present.

Proposal 1: When PRS-RSRP is configured for DL-TDOA, RSTD and PRS-RSRP are performed over the same measurement period. Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA.

Proposal 2: When PRS-RSRP is configured for multi-RTT, UE Rx-Tx time difference and PRS-RSRP are performed over the same measurement period.

Proposal 3: Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for Multi-RTT.

- References

- R4-2105851	WF on UE PRS measurement requirements, Huawei, HiSilicon

- R4-2106625	Discussion on PRS-RSRP measurement requirements, vivo