3GPP TSG RAN WG4 #99-e meeting 		R4-2109947

Electronic Meeting, 19th – 27th May, 2021

Agenda Item:	9.14.5

Source:	LG Electronics

Title:	RF requirements for intra-band con-current V2X operation with NR PC5 and NR Uu in a licensed band

Document for:	Approval

- Introduction

In the last RAN4 #98BIS-e meetings, RAN4 agreed the priority for V2X operating scenarios in a licensed band and impact of RF requirements.

In agreed WF, RAN4 consider as following scenarios

  - Focus on prioritization on operating scenarios including TDM and FDM.

    - 1st priority: TDM (Single RF chain for Tx as baseline)

    - 2nd priority: FDM with adjacent carrier (Separate RF chain as baseline)

    - 3rd priority: FDM with non-adjacent carrier (Separate RF chain as baseline)

  - RAN4 allow TDM operation between spectrally partially used PC5 SL and Uu UL/DL operation in a licensed TDD band regardless of adjacent/ non-adjacent carrier (Option 1 as the majority view in 1st round).

  - Frequency separation for non-adjacent carriers

    - Option 1: For the FDD/TDD intra-band con-current operation with non-adjacent carrier, RAN4 need further discussion on the detail coexistence scenarios based on operator deployment scenarios and request.

    - Option 2: No need to introduce the frequency separation for the case Uu and SL are in different channels for intra-band con-current operation.

    - Option 3: To study the frequency separation in non-adjacent channel, the frequency separation between existing UL and DL of FDD bands can be considered as a starting point.

In this paper, we provide our views on the remaining issues and expected RF requirements for intra-band on-current V2X operation in a licensed band.

- Discussion on RF requirements for TDM operation

In previous RAN4 meeting, RAN4 decided to support TDM operation for intra-band con-current operation as 1st priority in a licensed band such as n79. And determine the baseline RF architecture according to V2X operating scenarios in licensed band.

To support TDM operation, RAN4 need to consider the intra device coexistence evaluation in adjacent carrier. In rel-16, RAN4 only focused on the UE-to-UE (inter-device) coexistence for SL operation with adjacent carrier. But, the TDM operation do not have any impact to each other RAT operation. So the intra device coexistence evaluation is not necessary to allow the TDM operation.

RAN4 only consider whether or not specify the additional RF requirements for SL and Uu operation as TDM manner in same/adjacent carrier or non-adjacent carrier.

It can be consider two ON/OFF time mask in RF perspective. One is ON/OFF time mask for TDM operation in same carrier. And the other is ON/OFF time mask for TDM operation in different carrier.

Currently, RAN4 only allow TDM operation within different carrier operation in ITS spectrum.

So, the switching period and position should be considered with different carrier. However, if RAN4 consider allow TDM operation in same carrier in a licensed band, RAN4 do not consider the switching period.

So, RAN4 can define the ON/OFF time mask for TDM operation in same carrier as follow

Figure 2-1: ON/OFF Time mask for TDM operation in same carrier from NR Uu to NR SL in licensed band

Figure 2-2: ON/OFF Time mask for TDM operation in same carrier from NR SL to NR Uu in licensed band (15kHz/30kHz SCS)

Figure 2-3: ON/OFF Time mask for TDM operation in same carrier from NR SL to NR Uu in licensed band (60kHz SCS)

For 60kHz SCS, 10us +10us transient period can over the 1 symbol punctured time period, then 1 slot will be exhausted due to the V2X resource scheduling perspective. Therefore, RAN4 can adjust the transient period with 8us + 8us to keep the 1symbol punctured time period for 60kHz SCS.

For the other ON/OFF time mask for TDM operation in different carrier, RAN4 can follow the decision of TDM operation between LTE V2X and NR V2X in ITS spectrum.

To decide the ON/OFF time mask in different carrier, one different point is that the high priority is belong to NR Uu operation except V2X safety message in NR SL in licensed band.

Based on RAN1 agreements, the priority in ITS spectrum will be decided as RF implementation when the LTE V2X and NR V2X has same priority. So it is quite difficult to define ON/OFF time mask in RF perspectives.

Hence, we propose as follow

Proposal 1: RAN4 specify ON/OFF Time Mask for TDM operation in same carrier as shown in Figure 2-1 to Figure 2-3.

Proposal 2: For the ON/OFF time mask for TDM operation in different carrier, RAN4 can follow the decision of TDM operation in ITS spectrum with different carrier.

- Discussion on RF requirements for FDM operation

  - Transmitter Time difference between PC5 and Uu

In Rel-16 timeline, some operators requested that the SL operation is in sync with the non-V2X operation in the same licensed bands such as n79 or n38.

However, RRM session already agreed the and  for SL transmission in licensed band is 0us in TS38.133 as follow

- 12.2.3	NR Cell as synchronization reference source

The requirements in this subclause are applicable when the reference timing used for sidelink transmissions is a NR serving cell on a non-V2X sidelink carrier.

The sidelink transmissions takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell, where  = 0 and=0.

Firstly, RAN4 shall consider separate RF chain for FDM operation.

The real transmission timing between NR UL/DL and SL has some offset as shown in Figure 3-1.

Figure 3-1. Tx/Rx timing descriptions for con-current operation

Figure 3-2. Analysis of the transmission time difference for con-current operation

In Figure 3-2, we analyse the self-interference problem between PC5 and Uu operation. When we consider the separate RF architecture, the SL transmitted signal power can leak into the own NR received signal (black region) as Case1.

But, NR DL slot have some time gap from UL slot as in Case1 in Figure3. So we can expect that there is no interference problem in Case1.

In Case2, the 1st slot of TDD DL/UL configuration is assigned UL slot but there was no transmitted signal in Uu operation, so the 1st SL slot is operated to receive SL signal, then the second slot of UL signal will leak into the SL reception (blue region) due to the TA difference as shown in Case2. However, RAN4 decided to puncture the last symbol (71.4us for SCS 15kHz, 35.7us for SCS 30kHz, 17.9us for SCS 60kHz) in SL slot.

Also the TA (Timing advance) can be estimated as the equation of TA = NTA + NTA offset. In other word, the general TA is decided by propagation delay of NTA and NTA offset which were specified in section 7.1.2 in TS38.133 as follow

Table 7.1.2-2: The Value of in TS38.133

| Frequency range and band of cell used for uplink transmission | (Unit: TC) |
| --- | --- |
| FR1 FDD or TDD band with neither E-UTRA–NR nor NB-IoT–NR coexistence case | 25600 (Note 1) = 13us |
| FR1 FDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case | 0 (Note 1) |
| FR1 TDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case | 39936 (Note 1) = 20us |
| FR2 | 13792 |

The maximum NTA offset is 20 us in FR1 with 39936*Tc, the expected NTA is 1~2 us when RAN4 consider the ISD 500m cell for E-UTRA and/or NR system by the propagation delay. Then the TA is estimated about 22 us.

But, RAN4 can consider NR only system will be deployed in n79 licensed band. Then 13us for NTA offset  in Table 7.1.2-2 is considered to decide Time Advance (TA) = 13us+1~2us =15us in ISD 500m cell in n79.

Therefore, we can expect that in Case2 UL transmission does not make any interference to the own SL reception considering the above expected TA (15us) time for intra-band non-contiguous CA in n79 for all SCS waveforms.

Observation 1: RAN4 can consider 13us NTA offset for NR only 500m ISD cell. The all SCS waveform do not have any interference problem in its own device based on RAN1 & RRM agreement.

Also, we did not consider the transient period with 10us since there are separate RF architecture per each NR Uu and NR SL in licenced band for FDM operation.

So RAN4 can concluded that SL and UL/DL operation could not make any interference with each other with current RAN4 RRM agreement as specified in section 12.2.3 in TS38.133.

We summarize the pros and cons for timing alignment between NR Uu and NR SL according to align the DL slot timing or UL slot timing as shown in Table 3-1

Table 3-1: Comparison of timing alignment between NR SL and NR Uu

|  | DL slot timing (current RAN1/RRM agreement) | UL slot timing (change the SL slot timing with UL slot of NR Uu) |
| --- | --- | --- |
| Pros | - No interference for all 15/30/60kHz with 13us NTA offset - All UE can align the DL slot time | - No interference for all 15/30/60kHz regardless NTA offset |
| Cons | - Can be interfered into SL reception for 60kHz with 20us NTA offset | - Problem for IDLE NR UE due to lack of timing advance information - Backward compatibility is not guaranteed. Rel-16 NR SL UE and Rel-17 NR SL UE have inference problem due to different time alignment |

Based on above summary table, we propose as follow

Proposal 3: For the SL transmission time alignment, RAN4 can keep the current RRM agreements as specified in section 12.2.3 in TS38.133.

  - In Device Coexistence evaluation for FDM operation with adjacent carrier in a licensed band

For the FDM operation between Uu UL/DL and PC5 SL operation in adjacent carrier, the important point for simulation assumptions is to decide the PC5-Uu simultaneous operation scenarios as shown in Figure 4.

Figure 3-3. Use cases for intra-band con-current operation within adjacent carrier

There are 4 cases for PC5-Uu simultaneous operation in intra-device. The case1 in figure 3-3 is already analysed that the PC5 SL transmission timing is aligned the Uu DL reception slot in Figure 3-2. So it is not possible scenario when SL is transmitting signal with NR Uu receiving DL signal simultaneously.

For the Case2 and Case3, these are same scenarios as con-current operation between PC5 SL Tx and Uu UL Tx in a licensed band. Furthermore, RAN4 already evaluated the UE-to-UE coexistence results in Rel-16 considering with these two cases.

For the final Case 4, this scenario also can be considered as simultaneous Uu UL transmission and PC5 SL receptions as shown in Figure 3-2 with blue region. But we can solve this problem without any coexistence analysis since RAN4 already decided to puncture the last symbol as mentioned above.

So, RAN4 only need to study the coexistence scenarios of Case2 and Case3 in Figure 3-3. But these operation is quite similar to intra-band contiguous CA or DC operation in a licensed band. So these operation already allowed in licensed band. Hence we prefer not to study the in-device coexistence analysis in Rel-17.

But, for the FDD intra-band con-current operation in n1 as shown in figure 3-3, RAN4 need to further study the detail coexistence scenarios. Since the NR V2X reception is operated in UL frequency in FDD band with simultaneous NR Uu transmission. So it will be impacted to NR V2X reception in FDD band.

Proposal 4: RAN4 would allow the intra-band con-current SL operation with adjacent carrier for FDM operation in TDD band without in-device coexistence study.

Proposal 5: RAN4 need study whether to allow the intra-band con-current SL operation with adjacent carrier for FDM operation in FDD specific band based on operator request due to self-interference problem.

  - In Device Coexistence evaluation for FDM operation with non-adjacent carrier in a licensed band

The FDM operation with non-adjacent carrier operation in a licensed bands was decided to treat as 3rd priority in last RAN4 meeting.

For the FDM operation between Uu UL/DL and PC5 SL operation in non-adjacent carrier, the important point for simulation assumptions is to decide the PC5-Uu simultaneous operation scenarios as shown in Figure 3-4.

Figure 3-4. Use cases for intra-band con-current operation within non-adjacent carrier

In TDD band, when RAN4 allow the intra-band non-contiguous concurrent operation between NR V2X and NR Uu, one of benefit is to allow simultaneous Transmission and reception for NR V2X and NR Uu operation in UL slot with frequency gap to protect each other.

So, this is same issue in FDD band as shown in Figure 3-4.

Therefore, RAN4 need to study how much frequency gap is needed to protect each system for FDM operation in both FDD/TDD band as following proposal 6.

Proposal 6: For the FDD/TDD intra-band con-current operation with non-adjacent carrier, RAN4 need further discussion on the detail coexistence scenarios based on operator deployment scenarios and request. It will be treated as 3rd priority in Rel-17 .

- Specification aspect for intra-band con-current V2X operation in a licensed band

To support simultaneous NR uplink and SL transmission in a licensed band, RAN4 needs to specify the con-current NR V2X operating bands in TS38.101-1.

For TDM operation, RAN4 only to define operating band for V2X_n79A-n79A and ON/OFF time mask in same carrier firstly as shown in Section 2. And the ON/OFF time mask in different carrier will be decided based on Rel-16 TDM operation between LTE V2X and NR V2X in ITS spectrum.

For the FDM operation, RAN4 shall define the follow RF core requirements in TS38.101-1 based on separate RF architecture.

Table 4-1: NR V2X UE Tx/Rx RF requirements in TS 38.101-1

| Section / Clause | Description |
| --- | --- |
| 5.3E | V2X operating band Define the con-current operating bands for V2X_n79-n79 |
| 6.2E.1 | UE maximum output power for intra-band con-current V2X UE Define V2X_n79-n79 with PC3 |
| 6.2E.2 | UE maximum output power for intra-band con-current V2X UE Define MPR for intra-band contiguous/non-contiguous V2X_n79-n79 with PC3 |
| 6.2E.3 | UE maximum output power with additional requirements, Define A-MPR for intra-band contiguous/non-contiguous V2X_n79-n79 if the regional regulatory requirements for PC3 are identified |
| 6.2E.4 | Configured transmitted Power for intra-band con-current V2X UE Define additional requirements for configured Tx power to support intra-band contiguous or non-contiguous con-current operation |
| 6.3E.1 | Minimum output power for intra-band con-current V2X UE Define min. output power requirements to support intra-band contiguous or non-contiguous con-current operation |
| 6.3E.2 | Transmit OFF power for intra-band con-current V2X UE Define Transmit OFF power requirements to support intra-band contiguous or non-contiguous con-current operation |
| 6.3E.3 | Transmit ON/OFF time mask for intra-band con-current V2X UE Define Transmit ON/OFF time mask to support intra-band contiguous or non-contiguous con-current operation |
| 6.3E.4 | Power control for intra-band con-current V2X UE Define power control requirements to support intra-band contiguous or non-contiguous con-current operation |
| 6.4E.1 | Frequency error for intra-band con-current V2X UE Define Frequency error requirements to support intra-band contiguous or non-contiguous con-current operation |
| 6.4E.2 | Transmit modulation quality for intra-band con-current V2X UE Define EVM, carrier leakage, IBE, spectrum flatness to support intra-band contiguous or non-contiguous con-current operation |
| 6.5E.1 | Occupied bandwidth for intra-band con-current V2X UE Define Occupied bandwidth requirements to support intra-band contiguous or non-contiguous con-current operation |
| 6.5E.2 | Out of band emission for intra-band con-current V2X UE Define ACLR, SEM, A-SEM for ITS spectrum to support intra-band contiguous or non-contiguous con-current operation |
| 6.5E.3 | Spurious emissions for intra-band con-current V2X UE Define UE-to-UE coexistence requirements for intra-band contiguous or non-contiguous con-current operation |
| 6.5E.4 | Transmit intermodulation for intra-band con-current V2X UE Define Transmit intermodulation requirements for intra-band contiguous or non-contiguous con-current operation |
| 7.3E.3 | REFSENS requirements for intra-band con-current V2X UE Define REFSENS requirements for intra-band contiguous or non-contiguous con-current operation |
| 7.4E.2 | Max. Input level for intra-band con-current V2X UE Define max. input levels requirements for intra-band contiguous or non-contiguous con-current operation |
| 7.5E.2 | ACS for intra-band con-current V2X UE Define ACS requirements for intra-band contiguous or non-contiguous con-current operation |
| 7.6E.2 and 7.6E.3 | Blocking characteristics for intra-band con-current V2X UE Define in-band blocking/out-of-band blocking requirements for intra-band contiguous or non-contiguous con-current operation |
| 7.7E.2 | Spurious response for intra-band con-current V2X UE Define spurious response requirements for intra-band contiguous or non-contiguous con-current operation |
| 7.8E.3 | Wideband intermodulation for intra-band con-current V2X UE Define wideband intermodulation requirements for intra-band contiguous or non-contiguous con-current operation |

Based on the above RF architecture and specification works, the intra-band con-current operation in a licensed band will be supported as the simultaneous transmission in Rel-17.

Based on above analysis, we propose as follow

Proposal 7: Based on Table 4-1, RAN4 define the detailed RF requirements for intra-band con-current V2X operation in TDD band.

  - Full duplexer and half duplexer for SL enhancement in Rel-17

In last RAN4 meeting, RAN4 discuss which duplexer mode will be considered for partial usage operation between NR Uu and NR SL in a licensed band.

Basically, NR SL UE only allow half-duplexer mode only in ITS spectrum and licensed band in previous SL operation and NR SL enhancement in Rel-17 as shown in Figure 3-3 and Figure 3-4

- TDM operation: only allow HD mode for SL operation as same as NR V2X UE

- FDM operation with adjacent carrier: only allow HD mode for SL operation. The NR SL UE only allow transmission or reception in UL slot in TDD band or UL frequency band in FDD band.

- FDM operation with non-adjacent carrier: still only allow HD mode for SL operation even though allow simultaneous transmission/reception for NR Uu and NR SL operation in different carrier.

Observation 2: The partial usage V2X operation scenarios in a licensed band are considered the half duplexer mode in SL operation perspective.

Maybe, the company has concerned simultaneous transmission/reception between NR SL and NR Uu operation for UE perspective as shown in Figure 3-4. This operation is specific operation which was not studied in previous SL operation.

But, it will be studied based on operator request as 3rd priority in Rel-17 timeline.

  - REFSENS requirements for intra-band con-current V2X UE

In TR38.886, RAN4 decided the detail RF parameters for REFSENS requirements in licensed bands as follow

The V2X UE REFSENS is defined by the following equation:

REFSENSV2X=kTB + SNRV2X +10log10(LCRB*SCS*12/RX_BW) +( NFV2X+ IM) – Diversity gain

Where

-	kTB: Thermal noise level is [-174dBm(kT) + 10*log10(RX BW)]dBm.

-	NF: Noise figure. 13 dB is used for LAA and can be reused for NR V2X requirements. Assumed NF is 9dB < 3GHz, NF is 10dB>= 3GHz (e.g B42, n77, n78, n79…) at licensed bands at FR1.

-	IM: 2.5 dB is assumed. When the number of RB size equal to or less than 24RBs, 0.5dB additional relaxation is allowed.

-	Target SNR: -0.5 dB

-	Diversity gain: 3dB

So, the above equation will be applied to decide the REFSENS requirements in licensed band such as n79 NR band. Basically, the existing REFSENS requirements for NR Uu operation in n79 are reused for intra-band con-current V2X operation.

Also, the expected REFSENS requirements are proposed in n79 NR band for SL operation as shown in Table 4-2, 4-3 and 4-4.

Table 4-2: Reference sensitivity for intra-band con-current V2X operating Band

| Intra-band con-current V2X Band |  | NR Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band (PC5) | NR Band (Uu) | NR Band | SCS kHz | 10MHz (dBm) | 20MHz (dBm) | 30MHz (dBm) | 40MHz (dBm) | 50MHz (dBm) | 60MHz (dBm) | 80MHz (dBm) | 100MHz (dBm) | Duplex Mode |
| n79 | n79 | n79 | 15 | N/A | N/A | N/A | -89.6 | -88.6 | N/A | N/A | N/A | TDD (Uu) |
|  |  |  | 30 | N/A | N/A | N/A | -89.7 | -88.7 | -87.9 | -86.6 | -85.6 |  |
|  |  |  | 60 | N/A | N/A | N/A | -89.9 | -88.8 | -88.0 | -86.7 | -85.7 |  |
|  |  | n79 | 15 | -95.5 | -92.2 | -90.4 | -89.1 | N/A | N/A | N/A | N/A | HD (PC5) |
|  |  |  | 30 | -95.1 | -92.4 | -90.7 | -89.2 | N/A | N/A | N/A | N/A |  |
|  |  |  | 60 | -95.9 | -92.1 | -90.9 | -89.4 | N/A | N/A | N/A | N/A |  |

Table 4-3: UL configuration for REFSENS for intra-band con-current V2X operating Band (PC5)

| Intra-band con-current V2X Band |  |  | NR operating Band / SCS/ Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band (PC5) | NR band (Uu) | UL band (Uu) |  | SCS (kHz) | 10 MHz (dBm) | 20 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50MHz (dBm) | 60MHz (dBm) | 80MHz (dBm) | 100MHz (dBm) | Duplex Mode |
| n79 | n79 | n79 |  | 15 | N/A | N/A | N/A | 216 | 270 | N/A | N/A | N/A | TDD (Uu) |
|  |  |  |  | 30 | N/A | N/A | N/A | 100 | 128 | 162 | 216 | 270 |  |
|  |  |  |  | 60 | N/A | N/A | N/A | 50 | 64 | 75 | 100 | 135 |  |

Table 4-4: SL Tx configuration for REFSENS for intra-band con-current V2X operating Band (Uu)

| Intra-band con-current V2X Band |  | NR operating Band / SCS/ Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band (PC5) | NR band (Uu) | V2X band (PC5) | SCS (kHz) | 10 MHz (dBm) | 20 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50MHz (dBm) | 60MHz (dBm) | 80MHz (dBm) | 100MHz (dBm) | Duplex Mode |
| n79 | n79 | n79 | 15 | 50 | 105 | 160 | 216 | N/A | N/A | N/A | N/A | HD (PC5) |  |
|  |  |  | 30 | 24 | 50 | 75 | 105 | N/A | N/A | N/A | N/A |  |  |
|  |  |  | 60 | 10 | 24 | 36 | 50 | N/A | N/A | N/A | N/A |  |  |

Proposal 8: Based on Table 4-2, 4-3 and 4-4, RAN4 define the REFSENS requirements for intra-band con-current V2X operation in n79.

- Conclusions

In this contribution, we provide our views on candidate RF architecture and simultaneous Uu and SL operation in time domain to support the intra-band V2X con-current operation in a licensed band.

Above analysis in session 2 and 3, we propose as following

Observation 1: RAN4 can consider 13us NTA offset for NR only 500m ISD cell. The all SCS waveform do not have any interference problem in its own device based on RAN1 & RRM agreement.

Observation 2: The partial usage V2X operation scenarios in a licensed band are considered the half duplexer mode in SL operation perspective.

Proposal 1: RAN4 specify ON/OFF Time Mask for TDM operation in same carrier as shown in Figure 2-1 to Figure 2-3.

Proposal 2: For the ON/OFF time mask for TDM operation in different carrier, RAN4 can follow the decision of TDM operation in ITS spectrum with different carrier.

Proposal 3: For the SL transmission time alignment, RAN4 can keep the current RRM agreements as specified in section 12.2.3 in TS38.133.

Proposal 4: RAN4 would allow the intra-band con-current SL operation with adjacent carrier for FDM operation in TDD band without in-device coexistence study.

Proposal 5: RAN4 need study whether to allow the intra-band con-current SL operation with adjacent carrier for FDM operation in FDD specific band based on operator request due to self-interference problem.

Proposal 6: For the FDD/TDD intra-band con-current operation with non-adjacent carrier, RAN4 need further discussion on the detail coexistence scenarios based on operator deployment scenarios and request. It will be treated as 3rd priority in Rel-17 .

Proposal 7: Based on Table 4-1, RAN4 define the detailed RF requirements for intra-band con-current V2X operation in TDD band.

Proposal 8: Based on Table 4-2, 4-3 and 4-4, RAN4 define the REFSENS requirements for intra-band con-current V2X operation in n79.

Reference

[1] R4-2105472, “Email discussion summary for [98-bis-e][135] NRSL_enh_Part_2,” Moderator (CATT)

[2] R4-2105403, “Way forward on operating scenarios for SL and Uu operated in the same licensed band”, CATT

[3] TS38.101-1