3GPP TSG-RAN WG4 Meeting #99-e	R4-2109996

Electronic Meeting, 19 May – 27 May 2021

Agenda Item:	10.7.2

Source:	Ericsson

Title:	Remaining issues on Test methodology for application layer data throughput performance

Document for:	Discussion

# 1	Introduction

In last meeting, RAN4 had agreed most of issues on application layer data throughput evaluation, such as simulation results aligning criteria, simulation assumption and test parameters as follow[1].

Simulation Assumptions - RAN4 should only evaluate the physical layer throughput. - Application of reported CQI/PMI/RI - TE schedules the PDSCH transport block, rank, and precoding slot by slot, according to the reported CQI/PMI/RI. - Test Metric - Absolute Physical Layer Throughput. - Throughput calculation - SS collects ACK, NACK and statDTX from the UE and records the time, elapsed from the beginning of the test. The payload size, received by the UE and acknowledged towards the SS, is determined according to TS38.214 Clause 5.1.3.2, where the MCS index is derived from the coding rate indicated by the reported CQI according to TS38.214 Clause 5.2.2.1, and the number of layers is set according to the reported RI. Throughput can be calculated in the SS by multiplying the payload size with the number of ACKs and dividing the accumulated payload in kilobits by the time in seconds, elapsed from the beginning of the test, being associated to the following ratio: ACK / (ACK+NACK + DTX). - Receiver Type - MMSE-IRC receiver, i.e., same as existing CSI reporting tests. - Focus on single-carrier scenario in this SI. Test Parameters - SNR Points for FR1 - Option 1: 0-30 dB - Option 2: 0:2:20 dB (baseline) - SNR Points for FR2 - 0:2:16 dB. - CSI Reporting - Aperiodic for both FR1 and FR2. - PMI Codebook - CodebookSubsetRestriction is set so that all the possible Type-I single panel PMI matrices are allowed. - If there is large span in simulation results, CodebookSubsetRestriction of 010011 can be considered. - Channel Model for FR1 - TDLA30-5 - Higher Doppler scenarios can be further discussed in WI phase. - Channel Model for FR2 - TDLA30-35 - TDLD30 channel model can be further discussed in WI phase. - Other Parameters are same as below RI tests in 38.101-4: - FR1 2 RX: Table 6.4.2.1-1 (Test 1) and Table 6.4.2.2-1 (Test 1) - FR1 4 RX: Table 6.4.3.1-1 (Test 1) and Table 6.4.3.2-1 (Test 1) - FR2: Table 8.4.2.2-1 (Test 1) Simulation Results - Companies are encouraged to provide the following based on simulation assumptions (slides 3-5) and test parameters (slides 6-10): - For each SNR point: - Absolute Physical Layer Throughput - CQI and RI statistics (At least Median CQI and Median RI) - Interested companies can also submit decode success rate.

In this meeting, we will continue to discuss the remaining issues on test methodology of application layer data throughput performance.

# 2	Simulation Results Alignment Criteria

A remaining issue in last meeting is how to define the alignment results criteria. There are two possible options: one is based on throughput; another is based on SNR level. We think average throughput with an absolute span is a simple and direct method to evaluate the simulation results since this SI mainly focus on evaluating the absolute physical layer throughput. However, as mentioned by some companies in last meeting, whether average throughput criteria is suitable mainly depends on the summary of companies results. RAN4 will firstly try to align simulation results based on average throughput, but the SNR criteria may be further considered once the throughput curves among companies do not align regards to the X% of the absolute throughput.

- Alignment Results Criteria - Option 1: Absolute throughput span within X% of average throughput across companies at a given SNR. - Decide X based on simulation results. Possible values of X = [5]% or [10]%. - Option 2: SNR G±Gspan can be reached for the T% of maximum throughput - Maximum throughput is derived with TBS corresponding to CQI index 15 with rank 2 for 2Rx/4Rx UE. - Decide G based on simulation results. Candidate option is G = [2.5] dB. - Additional Criteria - CQI and RI reporting statistics span to aid with alignment. - Interested companies can also provide decode success rate.

Proposal 1: RAN4 shall further discuss which simulation results alignment criteria is more suitable based on companies’ simulation results.

# 3	Requirements Definition

Another important remaining issue is how to set the physical layer throughput requirements. There are also two options which are corresponding to the similar options in simulation results alignment criteria discussion.

- If it is found to be feasible to define requirements for link adaptation (LA) physical layer throughput: - Set the physical layer throughput requirements: - Option 1: By multiplying the averaged throughput by Y (%), e.g., Y=95% or 90%. - Option 2: Use methodology from PDSCH demodulation requirements with fixed RMC (i.e. average of impairments results + X dB margin). - Whether to update LA throughput requirements in each release can be discussed in Plenary during planning for each release.

From our understanding, it’s better to just use the averaged throughput to define the requirements since this requirement mainly focus on the physical layer throughput test. Moreover, the throughput is easier to understand the performance by operators. However, as we discussed before, RAN4 may consider using the SNR instead of averaged throughput once RAN4 found the throughput curves among companies do not align. Thus, the requirement definition for link adaptation (LA) physical layer throughput shall follow the same criteria in simulation results alignment.

Proposal 2: The requirement definition for link adaptation (LA) physical layer throughput shall follow the same criteria in simulation results alignment.

# 4	Summary

In this paper, we further discuss the remaining issues on test methodology for application layer data throughput performance.

Proposal 1: RAN4 shall further discuss which simulation results alignment criteria is more suitable based on companies’ simulation results.

Proposal 2: The requirement definition for link adaptation (LA) physical layer throughput shall follow the same criteria in simulation results alignment.

# References

- R4-2106122, “WF on 5G NR UE Application Layer Data Throughput Performance”, Qualcomm Inc.