3GPP TSG-RAN WG4 Meeting # 99-e                  	R4-2110010

# Electronic Meeting, May. 19-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1967 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to update UE Rx-Tx time difference measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are some remaining open issues in UE Rx-Tx time difference measurement period: - Observation windows of L_PRS (PRS resource) and N (UE capability) - Notation of L_PRS. We found that LPRS,i is rather defined in TS38.211 as the size of the downlink PRS resource in the time domain LPRS,i ∈ 2,4,6,12 given by the higher-layer parameter dl-PRS-NumSymbols. It is true that it is quite confusing about LPRS,i definition between TS 38.211 and TS 38.214. If the window T ms is not set same as the window LPRS,i , then the requirement becomes invalid, since it turns out to be scaled by a wrong factor of LPRS,iN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the UE Rx-Tx time difference measurement period requirements for above issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE Rx-Tx time difference measurement period requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 9.9.4.5 Measurement Period Requirements

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.4.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured PRS frequency layers within the measurement period TUERxTx,Total ms.

If measurement gaps and processing time T have overlap between different frequency PRS frequency layers,

TUERxTx, Total=i=1LTUERxTx,i+L-1*maxTeffect,i.

Editor’s Note: FFS the UE Rx-Tx time difference measurement period when measurement gaps and processing time T do not have overlap between different PRS frequency layers.

where i is the index of PRS frequency layer,

TUERxTx,i is the measurement period for UE Rx-Tx time difference measurements in PRS frequency layer i as further defined in this clause,

L is total number of PRS frequency layers, and

Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in PRS frequency layer i as defined further in this clause.

TUERxTx,i=CSSFi*NRxBeam,i*NPRS,islotN'LPRS,iKLPRS,iN''N*Nsample-1*Teffect,i+Tlast

Where

CSSFi is the carrier-specific scaling factor for NR PRS-based measurement in the PRS frequency layer i as defined in clause 9.1.5.2,

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if PRS layer i is in FR1 and NRxBeam,i=8 if PRS layer i is in FR2,

LPRS,iK  is defined in 5.1.6.5 of TS 38.214 [4], that is equal to the duration K msec of DL PRS symbols within P msec window corresponding to the maximum PRS periodicity in a positioning frequency layer.

LPRS,i

NPRS,islot is the maximum number of DL PRS resources of frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in clause 4.2.7.2 of TS 37.355 [34],

N'' is equal to N, if the observation window T ms of the UE processing capability is same as Pms of the observation window of LPRS,iK .   If they are not same (T ms ≠P ms),   N''=N×PT.

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to maxNumOfDL-PRS-ResProcessedPerSlot as specified in clause 6.4.3  of TS 37.355 [34],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= [4],

Tlast is the measurement duration for the last UE Rx-Tx time difference measurement sample, including the sampling time and processing time, Tlast = Ti + LPRS,i ,

Teffect,i is periodicity of UE Rx-Tx time difference measurement in frequency layer i:

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i

where

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi

TPRS,i is the PRS resource periodicity in frequency layer i,

MGRPi is the measurement gap repetition periodicity in frequency layer i.

If the frequency layer i has more than one DL PRS resource sets with different PRS periodicities, the least common multiple of PRS periodicities among DL PRS resource sets is used to derive the measurement period of that PRS frequency layer.

Editor’s note: FFS: counting only PRS frequency layers for which there is at least one PRS resource with PRS symbols available in at least some measurement gaps

The time TUERxTx,i starts from the first MG instance aligned with DL PRS resources of PRS frequency layer i closest in time after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

The UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.

### Editors Note: The applicability of requirements if the time duration of a DL PRS resource exceeds the UE capability N.9.9.5	NR E-CID measurements

<End of Change 1>