3GPP TSG RAN4 Meeting #99-e	R4-2110085

Electronic Meeting, 19th – 27th May, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | 0301 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 37.145-2 to modify AAS BS OTA Spurious emissions limits for co-existence with systems operating in other frequency bands in R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Any other business |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To modify AAS BS OTA Spurious emissions limits for co-existence with systems operating in other frequency bands in Table 6.7.6.4.5.3.1-1. The reference index in this table has changed due to the iteration of the version. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To modify AAS BS OTA Spurious emissions limits for co-existence with systems operating in other frequency bands in Table 6.7.6.4.5.3.1-1. It should be corrected to 6.7.6.3.5.3 not 6.7.6.3.5.3.3 or others. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is wrong reference index in this table which may cause trouble in reading and editing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.6.4.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change1>>

Table 6.7.6.4.5.3-1: AAS BS OTA Spurious emissions limits for co-existence with systems operating in other frequency bands

| System type to co-exist with | Frequency range for co-existence requirement |  | Maximum Level |  | Measurement Bandwidth |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSM900 | 921 - 960 MHz |  | -45.4 dBm |  | 100 kHz |  | This requirement does not apply to BS operating in band 8 |  |
|  | 876 - 915 MHz |  | -49.4 dBm |  | 100 kHz |  | For the frequency range 880-915 MHz, this requirement does not apply to BS operating in band 8, since it is already covered by the requirement in clause 6.7.6.5.3.3 |  |
| DCS1800 (NOTE 3) | 1805 - 1880 MHz |  | -35.4 dBm |  | 100 kHz |  | This requirement does not apply to BS operating in band 3. |  |
|  | 1710 - 1785 MHz |  | -49.4 dBm |  | 100 kHz |  | This requirement does not apply to BS operating in band 3, since it is already covered by the requirement in clause 6.7.6.5.3.3. |  |
| PCS1900 | 1930 - 1990 MHz |  | -35.4 dBm |  | 100 kHz |  | This requirement does not apply to BS operating in band 2, 25, band 36 or band 70. |  |
|  | 1850 - 1910 MHz |  | -49.4 dBm |  | 100 kHz |  | This requirement does not apply to BS operating in band 2 or 25, since it is already covered by the requirement in clause 6.7.6.5.3.3. This requirement does not apply to BS operating in band 35. |  |
| GSM850 or CDMA850 | 869 - 894 MHz |  | -45.4 dBm |  | 100 kHz |  | This requirement does not apply to BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |  |
|  | 824 - 849 MHz |  | -49.4 dBm |  | 100 kHz |  | This requirement does not apply to BS operating in band 5 or 26, since it is already covered by the requirement in clause 6.7.6.5.3.3. For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| UTRA FDD Band I or | 2110 - 2170 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 1 or 65, |  |
| E-UTRA Band 1 or NR band n1 | 1920 - 1980 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 1 or 65, since it is already covered by the requirement in clause 6.7.6.5.3.3. |  |
| UTRA FDD Band II or | 1930 - 1990 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 2, 25 or 70. |  |
| E-UTRA Band 2 or NR band n2 | 1850 - 1910 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 2 or 25, since it is already covered by the requirement in clause 6.7.6.5.3.3 |  |
| UTRA FDD Band III or | 1805 - 1880 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 3 or 9. |  |
| E-UTRA Band 3 or NR band n3 (NOTE 3) | 1710 - 1785 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 3, since it is already covered by the requirement in clause 6.7.6.5.3.3. For BS operating in band 9, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in clause 6.7.6.5.3.3. |  |
| UTRA FDD Band IV or | 2110 - 2155 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 4, 10 or 66 |  |
| E-UTRA Band 4 | 1710 - 1755 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 4, 10 or 66, since it is already covered by the requirement in clause 6.7.6.5.3.3. |  |
| UTRA FDD Band V or E-UTRA Band 5 or NR band n5 | 869 - 894 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |  |
|  | 824 - 849 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 5 or 26, since it is already covered by the requirement in clause 6.7.6.5.3.3. For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| UTRA FDD Band VI, XIX or | 860 - 890 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 6, 18, 19 |  |
| E-UTRA Band 6, 18, 19 or NR Band n18 | 815 - 830 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 18 since it is already covered by the requirement in clause 6.7.6.5.3.3. |  |
|  | 830 - 845 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 6, 19, since it is already covered by the requirement in clause 6.7.6.5.3.3. |  |
| UTRA FDD Band VII or | 2620 - 2690 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 7. |  |
| E-UTRA Band 7 or NR band n7 | 2500 - 2570 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 7, since it is already covered by the requirement in clause 6.7.6.5.3.3. |  |
| UTRA FDD Band VIII or | 925 - 960 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 8. |  |
| E-UTRA Band 8 or NR band n8 | 880 - 915 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 8, since it is already covered by the requirement in clause 6.7.6.5.3.3. |  |
| UTRA FDD Band IX or | 1844.9 - 1879.9 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 3 or 9. |  |
| E-UTRA Band 9 | 1749.9 - 1784.9 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 3 or 9, since it is already covered by the requirement in clause 6.7.6.5.3.3. |  |
| UTRA FDD Band X or | 2110 - 2170 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 4, 10 or 66 |  |
| E-UTRA Band 10 | 1710 - 1770 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 10 or 66, since it is already covered by the requirement in clause 6.7.6.5.3.3. For BS operating in Band 4, it applies for 1755 MHz to 1770 MHz, while the rest is covered in clause 6.7.6.5.3.3. |  |
| UTRA FDD Band XI or XXI or | 1475.9 - 1510.9 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 11, 21 or 32 |  |
| E-UTRA Band 11 or 21 | 1427.9 - 1447.9 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 11, since it is already covered by the requirement in clause 6.7.6.5.3.3. For BS operating in Band 32, this requirement applies for carriers allocated within 1475.9 MHz and 1495.9 MHz. |  |
|  | 1447.9 – 1462.9 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 21, since it is already covered by the requirement in clause 6.7.6.5.3.3. For BS operating in Band 32, this requirement applies for carriers allocated within 1475.9 MHz and 1495.9 MHz. |  |
| UTRA FDD Band XII or | 729 - 746 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 12 or 85. |  |
| E-UTRA Band 12 or NR band n12 | 699 - 716 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 12 or 85, since it is already covered by the requirement in clause 6.7.6.5.3.3. For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (NOTE 7) |  |
| UTRA FDD Band XIII or | 746 - 756 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 13. |  |
| E-UTRA Band 13 | 777 - 787 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 13, since it is already covered by the requirement in clause 6.7.6.5.3.3. |  |
| UTRA FDD Band XIV or E-UTRA Band 14 | 758 - 768 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 14. |  |
|  | 788 - 798 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 14, since it is already covered by the requirement in clause 6.7.6.5.3.3. |  |
| E-UTRA Band 17 | 734 - 746 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 17. |  |
|  | 704 - 716 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 17, since it is already covered by the requirement in clause 6.7.6.5.3.3. For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (NOTE 7) |  |
| UTRA FDD Band XX or E-UTRA Band 20 or NR band n20 | 791 - 821 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 20 or 28. |  |
|  | 832 - 862 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 20, since it is already covered by the requirement in clause 6.7.6.5.3.3. |  |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510 – 3590 MHz |  | -40.0 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 22 or 42, 48. |  |
|  | 3410 – 3490 MHz |  | -37.0 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 22, since it is already covered by the requirement in clause 9.7.3.3. This requirement does not apply to Band 42. |  |
| E-UTRA Band 24 | 1525 – 1559 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 24. |  |
|  | 1626.5 – 1660.5 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 24, since it is already covered by the requirement in clause 6.7.6.5.3.3. |  |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1930 - 1995 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 2, 25 or 70. |  |
|  | 1850 - 1915 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 25, since it is already covered by the requirement in clause 6.7.6.5.3.3. For BS operating in Band 2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in clause 6.7.6.5.3.3. |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26 | 859 - 894 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |  |
|  | 814 - 849 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 26, since it is already covered by the requirement in clause 6.7.6.5.3.3. For BS operating in Band 5, it applies for 814 MHz to 824 MHz, while the rest is covered in clause 6.7.6.5.3.3. For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| E-UTRA Band 27 | 852 – 869 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in bands 5, 26 or 27. |  |
|  | 807 – 824 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 27, since it is already covered by the requirement in clause 6.7.6.5.3.3. For BS operating in Band 26, it applies for 807 MHz to 814 MHz, while the rest is covered in clause 6.7.6.5.3.3. This requirement also applies to BS operating in Band 28, starting 4 MHz above the Band 28 downlink operating band (NOTE 6). |  |
| E-UTRA Band 28 or NR band n28 | 758 - 803 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 20, 28, 44, 67 or 68. |  |
|  | 703 - 748 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 28, since it is already covered by the requirement in clause 6.7.6.5.3.3. This requirement does not apply to BS operating in Band 44. For BS operating in Band 67, it applies for 703-736 MHz. For E-UTRA BS operating in Band 68, it applies for 728 MHz to 733 MHz. |  |
| E-UTRA Band 29 or NR Band n29 | 717 – 728 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 29 or 85 |  |
| E-UTRA Band 30 | 2350 - 2360 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 30 or 40. |  |
|  | 2305 - 2315 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 30, since it is already covered by the requirement in clause 6.7.6.5.3.3. This requirement does not apply to BS operating in Band 40. |  |
| E-UTRA Band 31 | 462.5 – 467.5 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 31, 72, 73. |  |
|  | 452.5 – 457.5 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 31, since it is already covered by the requirement in clause 6.7.6.5.3.3. This requirement does not apply to E-UTRA BS operating in band 72 or 73. |  |
| UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 - 1496 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 11, 21 or 32. |  |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 33 |  |
| UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 - 2025 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 34 |  |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 35 |  |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 2, 25 or 36 |  |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 MHz |  | -40.4 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |  |
| UTRA TDD Band d) or E-UTRA Band 38 or NR band n38 | 2570 – 2620 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 38 or 69. |  |
| UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 – 1920 MHz |  | -40.4 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 39 |  |
| UTRA TDD Band e) or E-UTRA Band 40 or NR band n40 | 2300 – 2400 MHz |  | -40.4 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 30 or 40 |  |
| E-UTRA Band 41 or NR band n41 | 2496 – 2690 MHz |  | -40.4 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 41 or 53 |  |
| E-UTRA Band 42 | 3400 – 3600 MHz |  | -40.0 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 22, 42, 43, 48, 52. |  |
| E-UTRA Band 43 | 3600 – 3800 MHz |  | -40.0 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 42, 43, 48 |  |
| E-UTRA Band 44 | 703 - 803 MHz |  | -40.4 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 28 or 44 |  |
| E-UTRA Band 45 | 1447 - 1467 MHz |  | -40.4 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 45 |  |
| E-UTRA Band 46 | 5150 - 5925 MHz |  | -39.5 dBm |  | 1 MHz |  |  |  |
| E-UTRA Band 47 | 5855 - 5925 MHz |  | -52 dBm |  | 1 MHz |  |  |  |
| E-UTRA Band 48 or NR Band n48 | 3550 – 3700 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 22, 42, 43, 48 |  |
| E-UTRA Band 49 | 3550 – 3700 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 22, 42, 43, 48 |  |
| E-UTRA Band 50 or NR Band n50 | 1432 - 1517 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in Band 11, 21, 32, 45, 50, 51, 74, 75 or 76 |  |
| E-UTRA Band 51 or NR Band n51 | 1427 - 1432 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in Band 50, 51, 75 or 76. |  |
| E-UTRA Band 52 | 3300 – 3400 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to E-UTRA BS operating in Band 42 or 52. |  |
| E-UTRA Band 53 or NR band n53 | 2483.5 – 2495 MHz |  | -40.4 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 41 or 53 |  |
| E-UTRA Band 65 or NR band n65 | 2110 - 2200 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 1 or 65, |  |
|  | 1920 - 2010 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 65, since it is already covered by the requirement in clause 6.7.6.5.3.3. For BS operating in Band 1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in clause 6.7.6.5.3.3. |  |
| E-UTRA Band 66 or NR Band n66 | 2110 - 2200 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 4, 10, 23 or 66. |  |
|  | 1710 - 1780 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 66, since it is already covered by the requirement in clause 6.7.6.5.3.3. For BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in clause 6.7.6.5.3.3. For BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in clause 6.7.6.5.3.3. |  |
| E-UTRA Band 67 | 738 – 758 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 28 or 67. |  |
| E-UTRA Band 68 |  | 753 - 783 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in band 28, or 68. |  |
|  |  | 698 - 728 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in band 68, since it is already covered by the requirement in clause 9.7.3.3. For E-UTRA BS operating in Band 28, it applies between 698 MHz and 703 MHz, while the rest is covered in clause 9.7.3.3. |  |
| E-UTRA Band 69 |  | 2570 - 2620 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in Band 38 or 69. |  |
| E-UTRA Band 70 or NR Band n70 |  | 1995 - 2020 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in band 2, 25 or 70 |  |
|  |  | 1695 – 1710 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in band 70, since it is already covered by the requirement in clause 6.7.6.5.3.3 |  |
| E-UTRA Band 71 or NR Band n71 |  | 617 - 652 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 71. |  |
|  |  | 663 – 698 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 71/n71, since it is already covered by the requirement in clause 6.7.6.3.5.3 |  |
| E-UTRA Band 72 |  | 461 - 466 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 31, 72 or 73. |  |
|  |  | 451 - 456 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 72, since it is already covered by the requirement in clause 6.7.6.3.5.3 This requirement does not apply to BS operating in band 73. |  |
| E-UTRA Band 73 |  | 460 - 465 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 31, 72 or 73. |  |
|  |  | 450 - 455 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 73, since it is already covered by the requirement in clause 6.7.6.3.5.3 |  |
| E-UTRA Band 74 or NR band n74 |  | 1475 – 1518 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 11, 21, 32, 50 74 or 75. |  |
|  |  | 1427 – 1470 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 74, since it is already covered by the requirement in clause 6.7.6.3.5.3 This requirement does not apply to BS operating in band 32, 45, 50, 51, 75 or 76. |  |
| E-UTRA Band 75 or NR Band n75 |  | 1432 - 1517 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 11, 21, 32, 45, 50, 51, 74, 75 or 76. |  |
| E-UTRA Band 76 or NR Band n76 |  | 1427 - 1432 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 50, 51, 75 or 76. |  |
| NR Band n77 |  | 3300 – 4200 MHz |  | -40.0 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 42, 43, 48 |  |
| NR Band n78 |  | 3300 – 3800 MHz |  | -40.0 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 42, 43, 48 |  |
| NR Band n79 |  | 4400 – 5000 MHz |  | -39.5 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band n79 |  |
| NR Band n80 |  | 1710 - 1785 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 3, since it is already covered by the requirement in clause 6.7.6.3.5.3 For BS operating in band 9, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in clause 6.7.6.3.5.3 |  |
| NR Band n81 |  | 880 - 915 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 8, since it is already covered by the requirement in clause 6.7.6.3.5.3 |  |
| NR Band n82 |  | 832 - 862 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 20, since it is already covered by the requirement in clause 6.7.6.3.5.3 |  |
| NR Band n83 |  | 703 - 748 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 28, since it is already covered by the requirement in clause 6.7.6.3.5.3 This requirement does not apply to BS operating in Band 44. For BS operating in Band 67, it applies for 703-736 MHz. For BS operating in Band 68, it applies for 728 MHz to 733 MHz. |  |
| NR Band n84 |  | 1920 - 1980 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 1 or 65, since it is already covered by the requirement in clause 6.7.6.3.5.3 |  |
| E-UTRA Band 85 |  | 728 - 746 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 12, 29 or 85. |  |
|  |  | 698 - 716 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 85, since it is already covered by the requirement in clause 6.7.6.3.5.3 For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 7). |  |
| NR Band n86 |  | 1710 - 1780 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 66/n66, since it is already covered by the requirement in clause 6.7.6.3.5.3. For BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in clause 6.7.6.3.5.3. For BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in clause 6.7.6.3.5.3 |  |
| E-UTRA Band 87 |  | 420 - 425 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 87 or 88. |  |
|  |  | 410 – 415 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 87, since it is already covered by the requirement in clause 6.7.6.3.5.3 |  |
| E-UTRA Band 88 |  | 422 - 427 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 87 or 88. |  |
|  |  | 412 - 417 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 88, since it is already covered by the requirement in clause 6.7.6.3.5.3. This requirement does not apply to BS operating in band 87. |  |
| NR Band n89 |  | 869 - 894 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |  |
| NR Band n91 |  | 1427 - 1432 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 50, 51, 75 or 76. |  |
|  |  | 832 - 862 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 20, since it is already covered by the requirement in clause 6.7.6.3.5.3 |  |
| NR Band n92 |  | 1432 - 1517 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 11, 21, 32, 45, 50, 51, 74, 75 or 76. |  |
|  |  | 832 - 862 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 20, since it is already covered by the requirement in clause 6.7.6.3.5.3 |  |
| NR Band n93 |  | 1427 - 1432 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 50, 51, 75 or 76. |  |
|  |  | 880 - 915 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 8, since it is already covered by the requirement in clause 6.7.6.3.5.3 |  |
| NR Band n94 |  | 1432 - 1517 MHz |  | -40.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 11, 21, 32, 45, 50, 51, 74, 75 or 76. |  |
|  |  | 880 - 915 MHz |  | -37.4 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 8, since it is already covered by the requirement in clause 6.7.6.3.5.3 |  |
| NR Band n95 |  | 2010 - 2025 MHz |  | -40.4 dBm |  | 1 MHz |  |  |  |

NOTE 1:	As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band 25, Band 27, Band 28 or Band 29, the co-existence requirements in table 6.7.6.4.5.3-1 do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see clause 6.7.1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.7.6.4.5.3-1 assumes that two operating bands, where the frequency ranges in clause 4.7 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	For the protection of DCS1800, UTRA Band III or E-UTRA Band 3 in China, the frequency ranges of the downlink and uplink protection requirements are 1805 – 1850 MHz and 1710 – 1755 MHz respectively.

NOTE 4:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations (except in Band 46), special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 6:	For Band 28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with Band 27 UL operating band.

NOTE 7:	For Band 29 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with UTRA Band XII or E-UTRA Band 12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL operating band.

.<<End of Change1>>