3GPP TSG-RAN4 Meeting #99-e	 R4-2110159

# Online, May 19th – 27th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0567 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-3: MSD test configurations modifications for US EN-DC combinations with Band n77 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R17_1BLTE_1BNR_2DL2UL-Core |  |  |  |  |  | Date: |  |  | 2021-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - There are a few US EN-DC combinations with Band n77 where the MSD test points for n77 either as UL aggressor carrier or DL victim carrier do not fall within US Band n77 frequency ranges. These MSD requirements would not be testable. - In Table 7.3B.2.3.5.2-1, there are a few duplicate EN-DC combinations specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify US EN-DC combinations with Band n77 where the MSD test points for n77 do not fall within US Band n77 frequency ranges based on the changes proposed in R4-2110158. - Remove duplicate EN-DC combinations in Table 7.3B.2.3.5.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The US EN-DC combinations with Band n77 where the MSD test points for n77 do not fall within US Band n77 frequency ranges are not testable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

##### 7.3B.2.3.5	MSD for intermodulation interference due to dual uplink operation for EN-DC in NR FR1

7.3B.2.3.5.0	General

For EN-DC configurations in NR FR1 the UE may indicate capability of not supporting simultaneous dual uplink operation due to possible intermodulation interference overlapping in frequency to its own primary downlink channel bandwidth if

-	the intermodulation order is 2;

-	the intermodulation order is 3 when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz

In the case for EN-DC configurations in NR FR1 for which the intermodulation products caused by dual uplink operation do not interfere with its own primary downlink channel bandwidth as defined in Annex I the UE is mandated to operate in dual and triple uplink mode.

For EN-DC configurations in NR FR1 with uplink and downlink assigned to E-UTRA and NR FR1 bands given in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1. For these test points the reference sensitivity levels specified in clause 7.3.1 in TS 36.101 [4] and 7.3.2 of TS 38.101-1 [2] for the corresponding channel bandwidths or in clause 7.3.1 of TS 36.101 [4] are relaxed by the amount of the parameter MSD given in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1.

The throughput on each of the CGs shall be ≥ 95% of the maximum throughput of the respective reference measurement channels as specified in Annex A of TS 38.101-1 [2] and Annex A of TS 36.101 [4], with parameters specified in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1 with dual UL transmissions overlapping in time unless otherwise stated.

###### 7.3B.2.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for PC3 EN-DC in NR FR1 involving two bands

Table 7.3B.2.3.5.1-1: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1_n3 | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
| DC_1A-n5A | 1 | 1965 | 5 | 25 | 2155 | 6 | IMD4 |
|  | n5 | 836.5 | 5 | 25 | 876.5 | N/A | N/A |
| DC_1A_n8A | 1 | 1965 | 5 | 25 | 2155 | 6.0 | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
| DC_1A_n71A DC_1A_n71B | 1 | 1958 | 5 | 25 | 2148 | N/A | N/A |
|  | n71 | 668 | 5 | 25 | 622 | 15.1 | IMD3 |
| DC_1A_n77A, DC_1A_SUL_n77A-n84A, DC_1A_n77(2A), | 1 | 1950 | 5 | 25 | 2140 | 29.8 | IMD23 |
|  |  |  |  |  |  | 32.54 |  |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | N/A |
| DC_1A_n77A, DC_1A_SUL_n77A-n84A, DC_1A_n77(2A), DC_1A_n78A, DC_1A_SUL_n78A-n84A, DC_1A_n78(2A) | 1 | 1950 | 5 | 25 | 2140 | 8.0 | IMD43 |
|  |  |  |  |  |  | 10.74 |  |
|  | n77, n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_2A_n46A | 2 | 1880 | 5 | 25 | 1960 | 12.0 | IMD3 |
|  | n46 | 5720 | 20 | 100 | 5720 | N/A | N/A |
| DC_2A_n48A | 2 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | N/A |
| DC_2A_n66A, DC_2A-2A_n66A DC_2A_n66(2A) | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_2A_n66A, DC_2A-2A_n66A DC_2A_n66(2A) | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
| DC_2A_n77A DC_2A-2A_n77A | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  |  |  |  |  |  | 28.74 |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 8.0 | IMD4 |
|  |  |  |  |  |  | 10.74 |  |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |
|  |  |  |  |  |  | 7.74 |  |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
| DC_2A_n78A DC_2A_n78(2A) | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD23 |
|  |  |  |  |  |  | 28.74 |  |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_2A_n78A DC_2A_n78(2A) | 2 | 1885 | 5 | 25 | 1965 | 8.0 | IMD43 |
|  |  |  |  |  |  | 10.74 |  |
|  | n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_3_n1 | 3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
| DC_3_n5 | 3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
|  | n5 | 838 | 5 | 25 | 883 | N/A | N/A |
|  | 3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | IMD23 |
| DC_3A_n7A DC_3C_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | IMD4 |
| DC_3_n8 | n8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
|  | 3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_3A-n20A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | n20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | n20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
| DC_3A_n38A | 3 | 1713 | 5 | 25 | 1808 | 8.2 | IMD4 |
|  | n38 | 2617 | 5 | 25 | 2617 | N/A | N/A |
| DC_3A_n41A DC_3C_n41A DC_3A_SUL_n41A-n80A, DC_3C_SUL_n41A-n80A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | 1740 | 5 | 25 | 1835 | 26 | IMD23 |
|  |  |  |  |  |  | 28.74 |  |
|  | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | 1765 | 5 | 25 | 1860 | 8.0 | IMD43 |
|  |  |  |  |  |  | 10.74 |  |
|  | n77, n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_4A_n2A | 2 | 1860 | 20 | 502 | 1940 | 5 | IMD3 |
|  | 4 | 1752.5 | 5 | 25 | 2152.5 | N/A | N/A |
|  | 2 | 1868.3 | 5 | 25 | 1948.3 | N/A | N/A |
|  | 4 | 1735 | 5 | 25 | 2135 | 5 | IMD5 |
| DC_4A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | 4 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_4A_n7A | 4 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |
| DC_5_n7 | n7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 12 | IMD33 |
| DC_5_n38 | 5 | 844 | 5 | 25 | 889 | 12 | IMD33 |
|  | n38 | 2577 | 10 | 50 | 2577 | N/A | N/A |
| DC_5A_n66A | 5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_5A_n78A DC_5A_n78(2A) DC_5A_n78C | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_7_n3 | 7 | 2535 | 10 | 50 | 2655 | 13 | IMD4 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_7_n5 | 7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | 12 | IMD33 |
| DC_7A_n20A | 7 | 2512 | 10 | 50 | 2632 | N/A | N/A |
|  | n20 | 851 | 5 | 25 | 810 | 12 | IMD33 |
| DC_7_n40 | 7 | 2510 | 5 | 25 | 2630 | 23 | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
| DC_7A_n66A DC_7A-7A_n66A DC_7C_n66A | 7 | 2535 | 10 | 50 | 2655 | 15 | 4th IMD |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
| DC_7A_n77A DC_7A-7A_n77(2A) DC_7A_n77(2A) DC_7C_n77A DC_7C_n77(2A) | 7 | 2540 | 5 | 25 | 2660 | 7.1 | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | N/A |
| DC_8A_n1A | 8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
|  | n1 | 1965 | 5 | 25 | 2155 | 6 | IMD4 |
| DC_8A_n3A | 8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
|  | n3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_8A_n20A | n20 | 849.5 | 5 | 25 | 808.5 | 25 | IMD33 |
|  | 8 | 890.5 | 5 | 25 | 935.5 | N/A | N/A |
|  | n20 | 847.5 | 5 | 25 | 806.5 | N/A | N/A |
|  | 8 | 892.5 | 5 | 25 | 937.5 | 25 | IMD33 |
| DC_8A_n41A DC_8A_SUL_n41A-n81A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | IMD33 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| DC_8A_n77A, DC_8A_n78A, DC_8A_n78(2A), DC_8A_SUL_n78A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | IMD4 |
|  | n77, n78 | 3635 | 10 | 50 | 3635 | N/A | N/A |
| DC_8A_n79A, DC_8A-n79C, DC_8A_SUL_n79A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | IMD5 |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | N/A |
| DC_11A_n28A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 10.4 | IMD4 |
| DC_12_n78 | 12 | 710 | 5 | 25 | 740 | 5.5 | IMD5 |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_13_n5 | 13 | 783 | 5 | 25 | 752 | N/A | N/A |
|  | n5 | 828 | 5 | 25 | 873 | 25 | IMD3 |
| DC_13A_n7A DC_13A_n7(2A) | 13 | 784.5 | 5 | 25 | 753.5 | N/A | N/A |
|  | n7 | 2520 | 40 | 216 | 2640 | 2.5 | IMD5 |
| DC_13A_n77A | 13 | 784.5 | 5 | 20 | 753.5 | 5.5 | IMD5 |
|  | n77 | 3891.5 | 10 | 50 | 3891.5 | N/A | N/A |
| DC_18A_n3A | 18 | 823 | 5 | 25 | 868 | N/A | N/A |
|  | n3 | 1721 | 5 | 25 | 1816 | 4 | IMD4 |
| DC_18A_n77A DC_18A_n78A | 18 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_19A_n78A | 19 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20A_n3A | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
|  | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
| DC_20A_n38A | 20 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n38 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20_n7 | 20 | 851 | 5 | 25 | 810 | 12 | IMD33 |
|  | n7 | 2512 | 10 | 50 | 2632 | N/A | N/A |
| DC_20A_n8A | 20 | 849.5 | 5 | 25 | 808.5 | 25 | IMD3 |
|  | n8 | 892.5 | 5 | 25 | 937.5 | 25 | IMD3 |
| DC_20_n41 | 20 | 851 | 5 | 25 | 810 | 12.1 | IMD3 |
|  | n41 | 2512 | 10 | 50 | 2512 | N/A | N/A |
| DC_20_n41 | 20 | 841 | 5 | 25 | 800 | 8.1 | IMD5 |
|  | n41 | 2564 | 10 | 50 | 2564 | N/A | N/A |
| DC_20A_n77A, DC_20A_n78A, DC_20A_n78(2A), DC_20A_SUL_n78A-n82A | 20 | 850 | 5 | 25 | 809 | 11 | IMD4 |
|  | n77, n78 | 3359 | 10 | 50 | 3359 | N/A | N/A |
| DC_20A_n77A | 20 | 840 | 5 | 25 | 799 | 6.5 | IMD5 |
|  | n77 | 4159 | 10 | 50 | 4159 | N/A | N/A |
| DC_21A_n28A7 | 21 | 1450.4 | 5 | 25 | 1498.4 | 2.5 | IMD5 |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | N/A |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 18.4 | IMD3 |
|  | n79 | 4420.5 | 40 | 216 | 4420.5 | N/A | N/A |
| DC_25A_n77A DC_25A-25A_n77A | 25 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 8 | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
| DC_25A_n78A DC_25A-25A_n78A | 25 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 8 | IMD4 |
|  | n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |
|  | 25 | 1875 | 5 | 25 | 1955 | 5 | IMD5 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_26A_n41A | 26 | 839 | 5 | 25 | 884 | 15.6 | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | N/A |
| DC_28_n50 | 28 | 730 | 10 | 50 | 775 | 15.3 | IMD 2 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
|  | 28 | 740 | 10 | 50 | 785 | 6 | IMD 4 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
|  | 28 | 740 | 10 | 50 | 785 | 0.5 | IMD 5 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
| DC_28A_n51A | 28 | 742.3 | 5 | 25 | 797.3 | 5 | IMD4 |
|  | n51 | 1429.5 | 5 | 25 | 1429.5 | N/A | N/A |
| DC_26A_n77A, DC_26A_n78A | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | IMD4 |
|  | n77, n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |
| DC_28A_n77A, DC_28A_n78A, DC_28A_n78(2A), DC_28A_SUL_n78A-n83A | 28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
|  | n77, n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_41A_n3A DC_41C_n3A | n3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | N/A |
| DC_42_n3 | 42 | 3575 | 10 | 50 | 3575 | N/A | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | 26 | 2nd3 |
|  |  |  |  |  |  | 28.74 |  |
|  | 42 | 3435 | 10 | 50 | 3435 | N/A | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | 4th3 |
|  |  |  |  |  |  | 10.74 |  |
| DC_42_n28 | 42 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
|  | n28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
| DC_48A_n12A | 48 | 3557.5 | 10 | 50 | 3557.5 | N/A | N/A |
|  | n12 | 705.5 | 5 | 25 | 735.5 | 5.5 | IMD5 |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | 48 | 3625 | 20 | 100 | 3625 | N/A | N/A |
|  | n25 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |
| DC_66A_n2A, DC_66A-66A_n2A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_66A_n7A DC_66A-66A_n7A DC_66A_n7(2A) DC_66A-66A_n7(2A) | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |
| DC_66A_n25A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n46A | 66 | 1735 | 5 | 25 | 2135 | 12.0 | IMD3 |
|  | n46 | 5605 | 20 | 100 | 5605 | N/A | N/A |
| DC_66A_n48A | 66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |
|  | n48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
| DC_66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |
| DC_66A_n77A DC_66-66_n77A DC_66-66-66_n77A | 66 | 1775 | 5 | 25 | 2175 | 31.0 | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 5.0 | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_66A_n78A | 66 | 1730 | 5 | 25 | 2150 | 5.0 | IMD5 |
|  | n78 | 3660 | 10 | 50 | 3660 | N/A | N/A |
| DC_71A_n38A | 71 | 665 | 5 | 25 | 619 | 11 | IMD4 |
|  | n38 | 2614 | 5 | 25 | 2614 | N/A | N/A |
| DC_71A_n41A | 71 | 666 | 5 | 25 | 620 | 11 | IMD4 |
|  | n41 | 2618 | 5 | 25 | 2618 | N/A | N/A |
| DC_71A_n66A | 71 | 675 | 5 | 25 | 629 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
| DC_71A_n78A | 71 | 681.5 | 5 | 25 | 635.5 | 5.5 | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3582.5 | N/A | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. NOTE 2: RBstart = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 5: Void NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. |  |  |  |  |  |  |  |

Table 7.3B.2.3.5.1-1a: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n41A | 3 | 1740 | 5 | 25 | 1835 | 18.4 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |
| DC_3A_n78A | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n78A | 3 | 1765 | 5 | 25 | 1860 | 18.5 | IMD4 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_1A_n78A | 1 | 1950 | 5 | 25 | 2140 | 17.8 | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_8A_n78A | 8 | 897.5 | 5 | 25 | 942.5 | 15.5 | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | N/A |
| DC_2A_n77A | 2 | 1855 | 5 | 25 | 1935 | 32.10 | IMD2 |
|  |  |  |  |  |  | 34.852 |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 19.10 | IMD41 |
|  |  |  |  |  |  | 21.852 |  |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_13A_n77A | 13 | 782 | 5 | 20 | 751 | 15.37 | IMD5 |
|  | n77 | 3879 | 10 | 50 | 3879 | N/A | N/A |
| DC_66A_n77A | 66 | 1730 | 5 | 25 | 2130 | 34.33 | IMD2 |
|  | n77 | 3860 | 10 | 50 | 3860 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 11.27 | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |

###### 7.3B.2.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.3.5.2-0: MSD test points for Pcell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A | N/A |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n28A DC_1A-3C_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | IMD5 |
| DC_1A_n3A-n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | IMD5 |
| DC_1A-3A_n28A DC_1A-3C_n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_1A_n3A-n41A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n41 | 2507.5 | 5 | 25 | 2507.5 | 5.0 | IMD5 |
| DC_1A-3A_n71A DC_1A-3A_n71B | 1 | 1960 | 5 | 25 | 2150 | 5 | IMD4 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |
| DC_1A-7A_n28A DC_1A-7C_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | 7 | 2533 | 10 | 50 | 2653 | 30.0 | IMD2 |
| DC_1A-7A_n40A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 23 | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_1A-8A_n78A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 8 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A-3A_n77A DC_1A-3C_n77A DC_1A-3C_n77(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | IMD2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 8.5 | IMD4 |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
| DC_1A_n3A-n77A DC_1A_n3A-n77(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n77 | 3360 | 10 | 50 | 3360 | 11.2 | IMD4 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | IMD2 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 8.5 | IMD4 |
| DC_1A-3A_n78A DC_1A-3C_n78A DC_1A-3A_n78(2A) DC_1A-3C_n78(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.2 | IMD2 |
|  | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 2.8 | IMD5 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |
| DC_1A_n3A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | 27.9 | IMD2 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
| DC_1A-5A_n78A DC_1A-5A_n78C | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |
| DC_1A-7A_n78A DC_1A-7C_n78A DC_1A-7A_n78(2A) DC_1A-7C_n78(2A) DC_1A-7A_n78C DC_1A-7A-7A_n78C | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_1A_n7A-n78A DC_1A_n7B-n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |
| DC_1A-3A_n79A | 1 | 1950 | 5 | 25 | 2140 | 3.6 | IMD5 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | N/A |
| DC_1A-5A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 5 | 837.5 | 5 | 25 | 882.5 | 18.3 | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 5 | 837.5 | 5 | 25 | 882.5 | 8.9 | IMD4 |
|  | n79 | 4907.5 | 40 | 216 | 4907.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | IMD4 |
|  | 5 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |
| DC_1A-8A_n28A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | 8 | 905 | 5 | 25 | 950 | 3.3 | IMD5 |
| DC_1A_n8A-n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n8 | 885 | 5 | 25 | 930 | 8.0 | IMD4 |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
| DC_1A-8A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 3.3 | IMD5 |
| DC_1A-8A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3960 | 10 | 50 | 3960 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 14.4 | IMD3 |
| DC_1A_n8A-n78A | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3745 | 10 | 52 | 3745 | 14.9 | IMD3 |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | n8 | 895 | 5 | 25 | 940 | 3.3 | IMD5 |
|  | n78 | 3380 | 10 | 52 | 3330 | N/A | N/A |
| DC_1A-8A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n79 | 4815 | 40 | 216 | 4815 | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 15.8 | IMD3 |
| DC_1A-8A_n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n79 | 4845 | 40 | 216 | 4845 | N/A | N/A |
|  | 1 | 1955 | 5 | 25 | 2145 | 8.2 | IMD4 |
| DC_1A-11A_n3A | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 11 | 1432 | 5 | 25 | 1480 | 15.2 | IMD3 |
| DC_1A-11A_n28A | 11 | 1440 | 5 | 25 | 1488 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | N/A | N/A |
|  | 1 | 1960 | 5 | 25 | 2150 | 28.3 | IMD21 |
| DC_1A-11A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  | n77 | 3441 | 10 | 50 | 3441 | N/A | N/A |
|  | 11 | 1438 | 5 | 25 | 1486 | 31.4 | IMD2 |
| DC_1A-11A_n77A | 11 | 1438 | 5 | 25 | 1486 | N/A | N/A |
|  | n77 | 3578 | 10 | 50 | 3578 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 30.8 | IMD2 |
| DC_1A-11A_n78A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  | n78 | 3441 | 10 | 50 | 3441 | N/A | N/A |
|  | 11 | 1438 | 5 | 25 | 1486 | 31.4 | IMD2 |
| DC_1A-11A_n78A | 11 | 1438 | 5 | 25 | 1486 | N/A | N/A |
|  | n78 | 3578 | 10 | 50 | 3578 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 30.8 | IMD2 |
| DC_1A-18A_n77A DC_1A-18A_n77(2A) | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 825 | 5 | 25 | 870 | N/A | N/A |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |
| DC_1A-18A_n78A DC_1A-18A_n78(2A) | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 819 | 5 | 25 | 864 | N/A | N/A |
|  | n78 | 3758 | 10 | 50 | 3758 | N/A | N/A |
| DC_1A-18A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 18 | 822.5 | 5 | 25 | 867.5 | 18.3 | IMD3 |
|  | n79 | 4737.5 | 40 | 216 | 4737.5 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 8.9 | IMD4 |
|  | n79 | 4925 | 40 | 216 | 4925 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 8.1 | IMD4 |
|  | 18 | 822.5 | 5 | 25 | 867.5 | N/A | N/A |
|  | n79 | 4592.5 | 40 | 216 | 4592.5 | N/A | N/A |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 1 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | 19 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | IMD5 |
| DC_1A_n28A-n41A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | n41 | 2653 | 10 | 50 | 2653 | 30.1 | IMD2 |
|  | 1 | 1923 | 5 | 25 | 2113 | N/A | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
|  | n28 | 707 | 5 | 25 | 762 | 29.3 | IMD2 |
|  | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n41 | 2510 | 10 | 50 | 2510 | N/A | N/A |
|  | n28 | 730 | 10 | 50 | 785 | 4.5 | IMD5 |
| DC_1A-20A_n8A | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 846 | 5 | 25 | 805 | 11.5 | IMD4 |
| DC_1A-20A_n38A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 20 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n38 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A-28A_n3A | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_1A-28A_n7A DC_1A-1A-28A_n7A DC_1A-28A_n7B DC_1A-1A-28A_n7B | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 28 | 730 | 10 | 50 | 785 | 4.5 | IMD5 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.5 | 18.3 | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | IMD4 |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |
| DC_1A-20A_n78A | 1 | 1930 | 5 | 25 | 2120 | 20.3 | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_1A-20A_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |
| DC_1A-21A_n28A10 | 1 | 1975.3 | 5 | 25 | 2165.3 | 16.1 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | N/A |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD2 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |
|  | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 2.9 | IMD5 |
|  | n77, n78 | 3675 | 10 | 50 | 3675 | N/A | N/A |
| DC_1A-21A_n79A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A_n28A-n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n40 | 2374 | 5 | 25 | 2374 | 10.1 | IMD4 |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 713 | 5 | 25 | 768 | 8.6 | IMD4 |
|  | n40 | 2314 | 5 | 25 | 2314 | N/A | N/A |
| DC_1A-28A_n40A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 8.9 | IMD4 |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |
| DC_1A-28A_n77A DC_1A-28A_n78A | 1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n77/n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |
| DC_1A-28A_n77A DC_1A-28A_n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 28 | 739 | 5 | 25 | 794 | 4.2 | IMD5 |
|  | n77/n78 | 3352 | 10 | 50 | 3352 | N/A | N/A |
| DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 4.2 | IMD5 |
| DC_1A-28A_n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 28 | 733 | 5 | 25 | 788 | 15.2 | IMD3 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |
|  | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | 28 | 740 | 5 | 25 | 795 | 10.0 | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | 1.2 | IMD4 |
|  | 28 | 745.5 | 5 | 25 | 800.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 4.5 | IMD5 |
|  | 28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | n79 | 4807 | 40 | 216 | 4807 | N/A | N/A |
| DC_1A_n28A-n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | 15.2 | IMD39 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n79 | 4630 | 40 | 216 | 4630 | 14.9 | IMD34 |
| DC_1A-32A_n3A | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1480 | 15.2 | IMD34 |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
| DC_1A-32A_n78A DC_1A-32A_n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 31.8 | IMD2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 0 | IMD5 |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |
| DC_1A-40A_n78A DC_1A-40C_n78A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 40 | 2340 | 5 | 25 | 2340 | 10.6 | IMD4 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.1 | IMD4 |
|  | 40 | 2360 | 5 | 25 | 2360 | N/A | N/A |
|  | n78 | 3430 | 10 | 50 | 3430 | N/A | N/A |
| DC_1A_n40A-n78A DC_1A_n40A-n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |
|  | n78 | 3450 | 10 | 50 | 3450 | 9.8 | IMD4 |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 10.6 | IMD4 |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
| DC_1A-41A_n3A DC_1A-41C_n3A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 41 | 2507.5 | 5 | 25 | 2507.5 | 5.0 | IMD5 |
| DC_1A-41A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | 41 | 2653 | 10 | 50 | 2653 | 30 | IMD2 |
| DC_1A-41A_n77A DC_1A-41C_n77A DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A |  |
|  | 41 | 2510 | 5 | 25 | 2510 | N/A | IMD4 |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 11.0 | N/A |
|  | n77 | 4150 | 10 | 50 | 4150 | N/A |  |
|  | 41 | 2510 | 5 | 25 | 2510 | N/A | IMD5 |
| DC_1A-41A_n78A DC_1A-41C_n78A DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 41 | 2515 | 5 | 25 | 2515 | 12 | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
| DC_1A_n41A-n77A DC_1A_n41A-n78A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n41 | 2515 | 10 | 50 | 2515 | 11.5 | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n41 | 2650 | 10 | 25 | 2650 | N/A | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | 19.6 | IMD3 |
| DC_1A-41A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n79 | 4500 | 40 | 216 | 4500 | N/A |  |
|  | 41 | 2530 | 5 | 25 | 2530 | 29.4 | IMD2 |
| DC_1A_n75A-n78A DC_1A_n75A-n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | n75 | - | - | - | 1470 | 30.4 | IMD2 |
| DC_1A-42A_n3A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | 42 | 3425 | 5 | 25 | 3425 | 13.0 | IMD4 |
| DC_1A-42A_n28A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | 42 | 3416 | 5 | 25 | 3416 | 15.7 | IMD3 |
| DC_1A-42A_n28A | 42 | 3580 | 5 | 25 | 3580 | N/A | N/A |
|  | n28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | 1 | 1944 | 5 | 25 | 2134 | 15.7 | IMD3 |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 42 | 3490 | 5 | 25 | 3490 | 4.8 | IMD5 |
|  | 42 | 3402.5 | 5 | 25 | 3402.5 | N/A | N/A |
|  | n79 | 4640 | 40 | 216 | 4640 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | 15.5 | IMD3 |
|  | 42 | 3450 | 5 | 25 | 3450 | N/A | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
| DC_1A_SUL_n77A-n80A | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |
| DC_1A_SUL_n77A-n80A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | IMD5 |
| DC_1A_SUL_n78A-n80A | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |
|  | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_2A_n2A-n66A | 2 | 1875 | 5 | 25 | 1955 | N/A | N/A |
|  | n2 | 1895 | 5 | 25 | 1975 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_2A_n2A-n78A | 2 | 1852.5 | 5 | 25 | 1932.5 | N/A | N/A |
|  | n2 | 1862.5 | 5 | 25 | 1942.5 | 26 | IMD24 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_2A-4A_n28A | 2 | 1880 | 5 | 25 | 1960 | 11.0 | IMD4 |
|  | 4 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | N/A |
| DC_2A-4A_n41A | 2 | 1860 | 5 | 25 | 1940 | 11.0 | IMD4 |
|  | 4 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| DC_2A-5A_n12A8 | 2 | 1900 | 5 | 25 | 1980 | 5.9 | IMD5 |
|  | 5 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | n12 | 705 | 5 | 25 | 735 | N/A | N/A |
| DC_2A-5A_n48A DC_2A-5A_n48B | 2 | 1882 | 5 | 25 | 1962 | 15.6 | IMD3 ¦ fn48-2*fB5¦ |
|  | 5 | 839 | 5 | 25 | 884 | N/A | N/A |
|  | n48 | 3640 | 5 | 25 | 3640 | N/A | N/A |
| DC_2A-5A_n71A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | n71 | 686.5 | 5 | 25 | 640.5 | N/A | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | 4.2 | IMD5 |
| DC_2A_n5A-n77A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 16.0 | IMD3 |
| DC_2A-7A_n5A DC_2A-7C_n5A DC_2A-7A-7A_n5A | 2 | 1855 | 10 | 50 | 1935 | N/A | N/A |
|  | 7 | 2575 | 10 | 50 | 2685 | 30.0 | IMD2 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
| DC_2A-7A_n28A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | 7 | 1720 | 5 | 25 | 2120 | 29.0 | IMD2 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | N/A |
| DC_2A-7A_n77A DC_2A-7C_n77A DC_2A-7A-7A_n77A DC_2A-7A_n77(2A) DC_2A-7C_n77(2A) DC_2A-7A-7A_n77(2A) | 2 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | n77 | 3525 | 10 | 50 | 3475 | N/A | N/A |
|  | 2 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | 3.4 | IMD5 |
|  | n77 | 4120 | 10 | 50 | 4120 | N/A | N/A |
| DC_2A-7A_n78A DC_2A-2A-7A_n78A DC_2A-7C_n78A DC_2A-7A-7A_n78A DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) DC_2A-7A-7A_n78(2A) | 2 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3525 | 10 | 50 | 3475 | N/A | N/A |
| DC_2A_n7A-n78A, DC_2A_n7(2A)-n78A DC_2A_n7A-n78(2A) DC_2A_n7(2A)-n78(2A) | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n78 | 3775 | 10 | 50 | 3775 | 4.2 | IMD5 |
| DC_2-8_n2 | 2 | 1860 | 5 | 25 | 1940 | 4 | IMD4 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
| DC_2A-12A_n5A | 2 | 1900 | 5 | 25 | 1980 | 5.9 | IMD5 |
|  | 12 | 705 | 5 | 25 | 735 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 885 | N/A | N/A |
| DC_2A-12A_n41A DC_2A-2A-12A_n41A | 2 | 1872 | 5 | 25 | 1952 | 26 | IMD2 |
|  | 12 | 708 | 5 | 50 | 738 | N/A | N/A |
|  | n41 | 2660 | 10 | 50 | 2660 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | 12 | 708 | 5 | 50 | 738 | 28.7 | IMD24 |
|  | n41 | 2638 | 10 | 50 | 2638 | N/A | N/A |
| DC_2A_12A-n66A | 2 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | 12 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_2A-12A_n78A DC_2A-2A-12A_n78A | 2 | 1874 | 5 | 25 | 1954 | 16.5 | IMD3 |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
| DC_2A-13A_n48A DC_2A-13A_n48B | 2 | 1903.5 | 5 | 25 | 1983.5 | 15.6 | IMD3 ¦ fn48-2*fB13¦ |
|  | 13 | 784.5 | 5 | 25 | 753.5 | N/A | N/A |
|  | n48 | 3552.5 | 5 | 25 | 3552.5 | N/A | N/A |
| DC_2A-13A_n66A DC_2A-2A-13A_n66A | 2 | 1860 | 5 | 25 | 1940 | 6.2 | IMD4 |
|  | 13 | 780 | 10 | 50 | 749 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
| DC_2A-13A_n77A | 2 | 1864 | 5 | 25 | 1944 | 16.0 | IMD3 |
|  | 13 | 783 | 5 | 25 | 752 | N/A | N/A |
|  | n77 | 3510 | 5 | 25 | 3510 | N/A | N/A |
| DC_2A_n38A-n71A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n38 | 2586 | 5 | 25 | 2586 | 29.2 | IMD2 |
|  | n71 | 686 | 5 | 25 | 640 | N/A | N/A |
| DC_2A_n38A-n78A | 2 | 1870 | 5 | 25 | 1950 | N/A | N/A |
|  | n38 | 2610 | 5 | 25 | 2610 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | 14.8 | IMD3 |
| DC_2A-14A_n66A | 2 | 1874 | 5 | 25 | 1954 | 7.2 | IMD4 |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_2A-28A_n66A | 2 | 1900 | 5 | 25 | 1980 | 11 | IMD4 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
| DC_2A_n41A-n71A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |
|  | n71 | 676 | 5 | 50 | 630 | 28.7 | IMD2 |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2586 | 10 | 50 | 2586 | 29.2 | IMD2 |
|  | n71 | 686 | 5 | 50 | 640 | N/A | N/A |
| DC_2A-46A_n66A5 DC_2A-46C_n66A5 DC_2A-46D_n66A5 | 2 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD3, IMD5 |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_2A-48A_n5A | 2 | 1870 | 5 | 25 | 1950 | 16.9 | IMD3 |
|  | 48 | 3610 | 10 | 50 | 3610 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 2 | 1890 | 5 | 25 | 1970 | N/A | N/A |
|  | 48 | 3570 | 5 | 25 | 3570 | 16.2 | IMD3 |
|  | n5 | 840 | 5 | 25 | 885 | N/A | N/A |
| DC_2A-48A_n66A DC_2A-48C_n66A DC_2A-48D_n66A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | 48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 28.3 | IMD2 |
|  | 48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
| DC_2A_n48A-n66A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 20 | IMD3 |
| DC_2A-66A_n2A | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
| DC_2A-66A_n5A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | 7.2 | IMD4 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
| DC_2A-66A_n25A | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
| DC_2A-66A_n28A | 2 | 1880 | 5 | 25 | 1960 | 11.0 | IMD4 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | N/A |
| DC_2A-66A_n41A DC_2A-66A_n41C DC_2A-66A_n41(2A) | 2 | 1860 | 5 | 25 | 1940 | 11.0 | IMD4 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | N/A |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1905 | 5 | 25 | 1985 | N/A | N/A |
|  | 66 | 1755 | 5 | 25 | 2155 | 12.1 | IMD4 |
|  | n48 | 3560 | 5 | 25 | 3560 | N/A | N/A |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1880 | 5 | 25 | 1960 | 28.3 | IMD5 |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
| DC_2A-66A_n77A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | 66 | 1715 | 5 | 25 | 2115 | 29.2 | IMD2 |
|  | n77 | 3970 | 5 | 25 | 3970 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | M/A | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | 10.4 | IMD4 |
|  | n77 | 3500 | 5 | 25 | 3500 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | M/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | 4.0 | IMD5 |
|  | n77 | 3915 | 5 | 25 | 3915 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3720 | 5 | 25 | 3720 | N/A | N/A |
|  | 2 | 1855 | 5 | 25 | 1935 | 4.2 | IMD5 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n77 | 3540 | 5 | 25 | 3540 | N/A | N/A |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | 29.2 | IMD2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) DC_2A_n66A-n78A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | 66/n66 | 1760 | 5 | 25 | 2160 | 10.3 | IMD4 |
|  | n78 | 3480 | 10 | 50 | 3480 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) DC_2A_n66A-n78(2A) DC_2A_n66(2A)-n78A DC_2A_n66(2A)-n78(2A | 2 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | 9.1 | IMD4 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | 2.1 | IMD5 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | N/A |
| DC_2A_n66A-n78A DC_2A_n66A-n78(2A) DC_2A_n66(2A)-n78A DC_2A_n66(2A)-n78(2A) | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | 8.9 | IMD4 |
| DC_2A-71A_n38A DC_2A-2A-71A_n38A | 2 | 1862 | 5 | 25 | 1942 | 26 | IMD2 |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
|  | n38 | 2610 | 10 | 50 | 2610 | N/A | N/A |
| DC_2A-71A_n41A DC_2A-2A-71A_n41A | 2 | 1862 | 5 | 25 | 1942 | 26 | IMD2 |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
|  | n41 | 2610 | 10 | 50 | 2610 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | 71 | 676 | 5 | 50 | 630 | 28.7 | IMD24 |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |
| DC_2A-71A_n78A DC_2A-2A-71A_n78A | 2 | 1874 | 5 | 25 | 1954 | 16.5 | IMD3 |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
| DC_2A_n71A-n78A | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | 8 | IMD3 |
| DC_3A_n1A-n28A DC_3C_n1A-n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_3A_n1A-n40A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 40 | 2380 | 5 | 25 | 2380 | 8.0 | IMD5 |
| DC_3A_n1A-n77A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
| DC_3A_n1A-n78A DC_3C_n1A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 3.5 | IMD5 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_3A_n3A-n41A | 3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | 8.2 | IMD4 |
|  | n41 | 2657.5 | 5 | 25 | 2657.5 | N/A | N/A |
| DC_3A-5A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 5 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-5A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 18.5 | IMD3 |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | IMD4 |
|  | 5 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_3A-7A_n5A | 3 | 1780 | 10 | 50 | 1875 | N/A | N/A |
|  | 7 | 2505 | 10 | 50 | 2625 | 30.0 | IMD21 |
|  | n5 | 845 | 5 | 25 | 890 | N/A | N/A |
| DC_3A-7A_n8A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 29.0 | IMD2 IMD33 |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 7 | 2562 | 10 | 50 | 2682 | 16.9 | IMD3 |
|  | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
| DC_3A-18A_n3A | 3 | 1719 | 5 | 25 | 1814 | 4 | IMD4 ¦2*fn3-2*fB18¦ |
|  | 18 | 823 | 5 | 25 | 868 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_3-18_n41 | 18 | 820 | 5 | 25 | 865 | 28.9 | IMD2 |
|  | 3 | 1765 | 5 | 25 | 1860 | N/A | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 19.0 | IMD3 |
|  | 3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | n41 | 2585 | 5 | 25 | 2585 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 28.8 | IMD2 |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | MSD | N/A |
| DC_3A-18A_n77A DC_3A-18A_n77(2A)DC_3A-18A_n78A DC_3A-18A_n78(2A) | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 18 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-19A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 19 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A_n7A-n28A | 3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
|  | n28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n7 | 2562 | 5 | 25 | 2682 | 17.0 | IMD3 |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
| DC_3A-7A_n40A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_3A-7A_n77A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_3A-7A_n77A | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n77 | 3475 | 10 | 50 | 3475 | N/A | N/A |
| DC_3A-7A_n77A | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | 7 | 2550 | 5 | 25 | 2670 | 5.2 | IMD5 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
| DC_3A-7A_n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | 3.4 | IMD5 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
| DC_3A-7A_n78A DC_3C-7A_n78A DC_3C-7C_n78A DC_3A-3A-7A_n78A DC_3A-3A-7A-7A_n78A DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A DC_3A-7A_n78(2A) DC_3C-7A_n78(2A) DC_3A-7C_n78(2A) DC_3C-7C_n78(2A) DC_3A-7A_n78C DC_3A-7A-7A_n78C | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | N/A |
| DC_3A-8A_n40A | 3 | 1779 | 5 | 25 | 1874 | 4 | IMD5 |
|  | 8 | 912 | 5 | 25 | 957 | N/A | N/A |
|  | n40 | 2305 | 5 | 25 | 2305 | N/A | N/A |
| DC_3A-8A_n77A DC_3C-8A_n77A DC_3C-8A_n77(2A) | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 9.7 | IMD4 |
| DC_3A-8A_n77A DC_3C-8A_n77A DC_3C-8A_n77(2A) | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_3A-8A_n78A DC_3A-3A-8A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_3A_n8A-n78A | 3 | 1740 | 5 | 25 | 1835 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | 16.3 | IMD3 |
| DC_3A-8A_n79A | 3 | 1755 | 5 | 25 | 1850 | N/A | N/A |
|  | n79 | 4465 | 40 | 216 | 4465 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 15.3 | IMD3 |
| DC_3A-8A_n79A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 8.8 | IMD4 |
| DC_3A_n7A-n78A DC_3A_n7B-n78A DC_3C_n7A-n78A DC_3C_n7B-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |
| DC_3A-11A_n77A DC_3A-11A_n77(2A) | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77 | 3675 | 10 | 50 | 3675 | N/A | N/A |
|  | 11 | 1443 | 5 | 25 | 1491 | 8.8 | IMD4 |
|  | 11 | 1435.4 | 5 | 25 | 1483.4 | N/A | N/A |
|  | n77 | 3905 | 10 | 50 | 3905 | N/A | N/A |
|  | 3 | 1753 | 5 | 25 | 1848 | 3.4 | IMD57 |
| DC_3A-19A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 19 | 840 | 5 | 25 | 885 | 18.5 | IMD3 |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | IMD4 |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_3A-20A_n7A DC_3C-20A_n7A | 3 | 1737 | 5 | 25 | 1832 | N/A | N/A |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | IMD2 |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
| DC_3A-20A_n8A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 27 | IMD2 |
| DC_3A-20A_n8A | 3 | 1765 | 5 | 25 | 1860 | 14.5 | IMD4 |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_3A-20A_n28A DC_3C-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 3 | 1723 | 5 | 25 | 1818 | 9.4 | IMD4 |
| DC_3A-20A_n38A | 3 | 1779 | 5 | 25 | 1874 | N/A | N/A |
|  | 20 | 852 | 10 | 20 | 811 | 26.0 | IMD21 |
|  | n38 | 2590 | 10 | 50 | 2590 | N/A | N/A |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1744 | 5 | 25 | 1839 | 26.0 | IMD2 |
|  | n41 | 2680 | 10 | 52 | 2680 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | N/A | N/A |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1779 | 5 | 25 | 1874 | N/A | N/A |
|  | n41 | 2590 | 10 | 52 | 2590 | N/A | N/A |
|  | 20 | 852 | 10 | 50 | 811 | 26.0 | IMD2 |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n41 | 2660 | 10 | 52 | 2660 | N/A | N/A |
|  | 20 | 841 | 5 | 25 | 800 | 12.5 | IMD3 |
| DC_3A_20A_SUL_n78A-n80A DC_3C_20A_SUL_n78A-n80A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |
| DC_3A_n20A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |
| DC_3A-20A_n78A DC_3C-20A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | 8.8 | IMD4 |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 3 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-21A_n77A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | N/A |
| DC_3A-21A_n79A | 3 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A-28A_n1A | 3 | 1725 | 5 | 25 | 1820 | 4 | IMD5 |
|  | 28 | 710 | 5 | 25 | 765 | N/A | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
| DC_3A-28A_n5A DC_3C-28A_n5A | 3 | 1735 | 5 | 25 | 1830 | 8.7 | IMD4 |
|  | 28 | 705 | 5 | 25 | 798 | N/A | N/A |
|  | n5 | 845 | 5 | 25 | 874 | N/A | N/A |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | 28 | 730 | 5 | 25 | 785 | 9.4 | IMD4 |
|  | n5 | 845 | 5 | 25 | 874 | N/A | N/A |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-3A-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B DC_3A-3A-28A_n7B | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
|  | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | 3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
| DC_3A-28A_n77A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 28 | 715 | 5 | 25 | 770 | 15.3 | IMD3 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 17.0 | IMD3 |
|  | 28 | 735 | 5 | 25 | 790 | N/A | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | N/A |
| DC_3A_n28A-n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n77 | 4173 | 10 | 50 | 4173 | 15.9 | IMD3 |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 28 | 715 | 5 | 25 | 770 | 15.3 | IMD3 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |
| DC_3A-28A_n41A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | N/A |
|  | 28 | 735 | 5 | 25 | 790 | 26.0 | IMD21 |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
|  | n41 | 2543 | 10 | 50 | 2543 | N/A | N/A |
|  | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
| DC_3A_n28A-n41A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 261 | IMD2 ¦fn41-fB3¦ |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | N/A |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | n41 | 2518 | 5 | 25 | 2518 | 27.4 | IMD2 ¦fB3+fn28¦ |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n41 | 2687 | 5 | 25 | 2687 | 15.9 | IMD3 ¦2*fB3-fn28¦ |
| DC_3A-28A_n78A DC_3C-28A_n78A DC_3A-3A-28A_n78A | 3 | 1775 | 5 | 25 | 1870 | 17.3 | IMD3 |
|  | 28 | 740 | 5 | 25 | 760 | N/A | N/A |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | N/A |
| DC_3A-28A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 10.3 | IMD4 |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 5.7 | IMD5 |
|  | 28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A_n28A-n78A DC_3C_n28A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | IMD5 |
| DC_3A_n28A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n28 | 725 | 5 | 25 | 780 | 10.3 | IMD4 |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n79 | 4585 | 40 | 216 | 4585 | 9.4 | IMD44 |
| DC_3A_SUL_n77A-n84A | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n84 | 1922.5 | 5 | 25 |  | N/A | N/A |
|  | n77 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_3A_n40A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 4.8 | IMD5 |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 4.4 | IMD5 |
|  | n78 | 3760 | 10 | 50 | 3760 | N/A | N/A |
| DC_3A_n40A-n79A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n40 | 2330 | 5 | 25 | 2330 | N/A | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | 4.7 | IMD5 |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n40 | 2330 | 5 | 25 | 2330 | 3.2 | IMD5 |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |
| DC_3A_n41A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | N/A |
|  | n79 | 4440 | 40 | 216 | 4440 | 30.8 | IMD24 |
| DC_3A-42A_n1A DC_3A-42C_n1A | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | 42 | 3425 | 5 | 25 | 3425 | 13.0 | IMD4 |
|  | n1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
| DC_3A_n75A-n78A DC_3A_n75A-n78(2A) | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | n75 | - | - | - | 1514.5 | 10.0 | IMD2 |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
|  | n79 | 4910 | 40 | 216 | 4910 | 16.3 | IMD3 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4510 | 40 | 216 | 4510 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | 4.2 | IMD5 |
| DC_3A_SUL_n78A-n82A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | n82 | 840 | 5 | 25 |  | N/A | N/A |
| DC_3A_SUL_n78A-n84A | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n84 | 1922.5 | 5 | 25 |  | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_3A-21A_n79A | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A-32A_n1A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1480 | 15.2 | IMD34 |
|  | n1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
| DC_3A-32A_n78A DC_3A-32A_n78(2A) | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 4.9 | IMD4 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1475 | 0 | IMD5 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
| DC_3A-40A_n1A DC_3A-40C_n1A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 40 | 2380 | 5 | 25 | 2380 | 8.0 | IMD5 |
| DC_3A-40A_n78A DC_3A-40C_n78A | 3 | 1775 | 5 | 25 | 1870 | 9.1 | IMD4 |
|  | 40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
|  | n78 | 3325 | 10 | 50 | 3325 | N/A | N/A |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 40 | 2360 | 5 | 25 | 2360 | 4.4 | IMD5 |
|  | n78 | 3760 | 10 | 50 | 3760 | N/A | N/A |
| DC_3A-41A_n3A DC_3A-41C_n3A | 3 | 1770 | 5 | 25 | 1865 | 8.2 | IMD4 ¦2*fB41-2*fn3¦ |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 | 2543 | 10 | 50 | 2543 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26 | IMD2 |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 41 | 2518 | 5 | 25 | 2518 | 27.4 | IMD2 |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 41 | 2687 | 5 | 25 | 2687 | 15.9 | IMD3 |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) DC_3A_n41A-n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
|  | 41/n41 | 2640 | 5 | 25 | 2640 | 5.3 | IMD5 |
|  | 41/n41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |
| DC_3A-41A_n78A DC_3A-41C_n78A DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n78 | 3400 | 10 | 52 | 3400 | N/A | N/A |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |
| DC_3A_n41A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n41 | 2560 | 10 | 50 | 2560 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.4 | IMD3 |
| DC_3A-41A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4440 | 40 | 216 | 4440 | N/A | N/A |
|  | 41 | 2670 | 5 | 25 | 2670 | 30.2 | IMD2 |
|  | 41 | 2570 | 5 | 25 | 2570 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 29.4 | IMD2 |
| DC_4A-7A_n28A | 4 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | 18.0 | IMD3 |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
| DC_5A-7A_n7A | 5 | 834 | 5 | 25 | 879 | 12 | IMD34 |
|  | 7 | 2527 | 10 | 50 | 2647 | N/A | N/A |
|  | n7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
| DC_5A-7A_n66A DC_5A-7C_n66A | 5 | 835 | 5 | 25 | 880 | 17.8 | IMD3 |
|  | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | 7 | 2504 | 5 | 25 | 2624 | 29.0 | IMD21 |
|  | 66 | 1777.5 | 5 | 25 | 2177.5 | N/A | N/A |
| DC_5A-7A_n71A | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | 6.5 | IMD5 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | N/A |
| DC_5A-7A_n78A DC_5A-7A_n78C DC_5A-7A-7A_n78C | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 30.2 | IMD2 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |
|  | 5 | 830 | 5 | 25 | 875 | 3.3 | IMD5 |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| DC_5A_n7A-n78A, DC_5A_n7(2A)-n78A DC_5A_n7A-n78(2A) DC_5A_n7(2A)-n78(2A) | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n78 | 3375 | 10 | 50 | 3375 | 29.7 | IMD2 |
| DC_5A-13A_n66A | 5 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | 13 | 781 | 5 | 25 | 750 | 9.4 | IMD4 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_5A_n38A-n66A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n38 | 2590 | 5 | 25 | 2590 | 28.9 | IMD2 |
| DC_5A_41A_n78A | 5 | 860 | 5 | 25 | 885 | 30.2 | IMD2 |
|  | 41 | 2615 | 5 | 25 | 2615 | N/A | N/A |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |
|  | 5 | 856.5 | 5 | 25 | 881.5 | 3.1 | IMD5 |
|  | 41 | 2620.5 | 5 | 25 | 2620.5 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | N/A | N/A |
| DC_5A-41A_n79A | 5 | 835 | 5 | 25 | 880 | 23.9 | IMD3 |
|  | 41 | 2665 | 5 | 25 | 2665 | N/A | N/A |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | 41 | 2517.5 | 5 | 25 | 2517.5 | 1.8 | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
| DC_5A-46A_n66A | 5 | 847 | 5 | 25 | 892 | N/A | N/A |
|  | 46 | 5163 | 10 | 50 | 5163 | 9.04 | IMD4 ¦2*fB5+2*fn66¦ |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_5A-48A_n12A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 48 | 3650 | 5 | 25 | 3650 | 4.4 | IMD5 |
|  | n12 | 705 | 5 | 25 | 735 | N/A | N/A |
|  | 5 | 830 | 5 | 25 | 875 | 5.9 | IMD5 |
|  | 48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
|  | n12 | 705 | 5 | 25 | 735 | N/A | N/A |
| DC_5A-48A_n71A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 48 | 3590 | 5 | 25 | 3590 | 4.4 | IMD5 |
|  | n71 | 690 | 5 | 25 | 644 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | 5.9 | IMD5 |
|  | 48 | 3600 | 5 | 25 | 3600 | N/A | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | N/A |
| DC_5A-66A_n2A DC_5BA-66A_n2A DC_5A-5A-66A_n2A DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A DC_5A-5A-66A-66A_n2A | 5 | 834 | 5 | 25 | 879 | N/A | N/A |
|  | 66 | 1712 | 5 | 25 | 2132 | 7.2 | IMD4 |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
| DC_5A-66A_n7A DC_5A-66A-66A_n7A | 5 | 835 | 5 | 25 | 880 | 18.0 | IMD3 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
| DC_5A-66A_n71A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 66 | 1761 | 5 | 25 | 2161 | 13 | IMD3 |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | 4.2 | IMD5 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
| DC_5A-66A_n77A | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | 66 | 1742 | 5 | 25 | 2142 | 13.2 | IMD3 ¦fn77-2*fB5¦ |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_5A-66A_n78A DC_5A-66A_n78(2A) | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | 66 | 1742 | 5 | 25 | 2142 | 13.2 | IMD3 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_5A_n66A-n78A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.6 | IMD3 |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | n66 | 1742 | 5 | 25 | 2142 | 13.2 | IMD3 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_7A_n1A-n40A | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n40 | 2335 | 5 | 25 | 2335 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 15.2 | IMD3 |
| DC_7A_n1A-n78A DC_7C_n1A-n78A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | 9.0 | IMD4 |
|  | n78 | 3610 | 10 | 50 | 3610 | N/A | N/A |
| DC_7A_n2A-n71A | 7 | 2530 | 5 | 25 | 2530 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n71 | 676 | 5 | 25 | 630 | 28.7 | IMD2 |
| DC_7A_n2A-n78A | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | n2 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | n78 | 3525 | 10 | 50 | 3525 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n78 | 3775 | 10 | 50 | 3775 | 4.2 | IMD5 |
| DC_7A_n3A-n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 15.6 | IMD3 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_7A_n8A-n40A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n8 | 905 | 5 | 25 | 950 | N/A | N/A |
|  | n40 | 2345 | 5 | 25 | 2345 | 3.0 | IMD5 |
| DC_7A-8A_n3A | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 7 | 2530 | 10 | 50 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 18.0 | IMD3 |
| DC_7A-8A_n3A | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 29.0 | IMD2+IMD33 |
| DC_7A-8A_n77A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 30.5 | IMD2 |
|  | n77 | 3470 | 10 | 50 | 3470 | N/A | N/A |
| DC_7A-8A_n77A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 3.1 | IMD5 |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_7A-8A_n77A | 7 | 2530 | 5 | 25 | 2650 | 28 | IMD2 |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | N/A |
| DC_7A-8A_n78A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 30.5 | IMD2 |
|  | n78 | 3470 | 10 | 50 | 3470 | N/A | N/A |
| DC_7A-8A_n78A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 3.1 | IMD5 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_7A-8A_n78A | 7 | 2530 | 5 | 25 | 2650 | 28 | IMD2 |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | n78 | 3545 | 10 | 50 | 3545 | N/A | N/A |
| DC_7A_n8A-n78A | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3455 | 10 | 50 | 3455 | 28.5 | IMD2 |
|  | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | 29.7 | IMD2 |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |
| DC_7A-12A_n66A | 7 | 2515 | 5 | 25 | 2635 | N/A | N/A |
|  | 12 | 712 | 5 | 25 | 742 | 31 | IMD2 |
|  | n66 | 1773 | 5 | 25 | 2173 | N/A | N/A |
| DC_7A-12A_n78A | 7 | 2542 | 5 | 25 | 2662 | 29.6 | IMD2 |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | 12 | 710 | 5 | 25 | 740 | 30.8 | IMD24 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
| DC_7A-13A_n66A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | 13 | 781 | 5 | 25 | 750 | 31 | IMD2 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_7A-13A_n66A | 7 | 2540 | 5 | 25 | 2660 | 18 | IMD3 |
|  | 13 | 780 | 5 | 25 | 749 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
| DC_7A-20A_n1A DC_7C-20A_n1A | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | 4.5 | IMD5 |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
| DC_7A-20A_n3A | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | IMD2 |
|  | n3 | 1737 | 5 | 25 | 1832 | N/A | N/A |
|  | 7 | 2510 | 10 | 50 | 2630 | 26.0 | IMD21 |
|  | 20 | 855 | 5 | 25 | 896 | N/A | N/A |
|  | n3 | 1775 | 10 | 50 | 1870 | N/A | N/A |
| DC_7A-20A_n8A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | 20 | 836 | 5 | 25 | 795 | 17.4 | IMD3 |
| DC_7A-20A_n8A | 7 | 2520 | 5 | 25 | 2640 | 21.1 | IMD3 |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_7A-20A_n8A | 7 | 2504 | 5 | 25 | 2624 | 18.8 | IMD3 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
| DC_7A-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 5.9 | IMD5 |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 30.5 | IMD2 |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_7A-20A_n78A | 7 | 2555 | 5 | 25 | 2675 | 30.8 | IMD2 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
| DC_7A-25A_n77A DC_7A-7A-25A_n77A DC_7C-25A_n77A DC_7C-25A-25A_n77A DC_7A-25A-25A_n77A DC_7A-7A-25A-25A_n77A | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | 25 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | 3.4 | IMD5 |
|  | 25 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | n77 | 4120 | 10 | 50 | 4120 | N/A | N/A |
| DC_7A-25A_n78A DC_7A-7A-25A_n78A DC_7C-25A_n78A DC_7A-25A-25A_n78A DC_7A-7A-25A-25A_n78A DC_7C-25A-25A_n78A | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | 25 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | n78 | 3525 | 10 | 50 | 3525 | N/A | N/A |
| DC_7A-28A_n1A | 7 | 2535 | 5 | 25 | 2655 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 4.3 | IMD5 |
|  | n1 | 1950 | 5 | 25 | 2165 | N/A | N/A |
|  | 7 | 2545 | 5 | 25 | 2665 | 29.0 | IMD2 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
| DC_7A-28A_n2A | 7 | 2510 | 10 | 50 | 2630 | 27.6 | IMD2 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
| DC_7A-28A_n3A DC_7C-28A_n3A | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2685 | 18 | IMD3 |
|  | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2540 | 5 | 25 | 2725 | N/A | N/A |
|  | 28 | 721 | 5 | 25 | 776 | 4.4 | IMD5 |
|  | n5 | 829 | 5 | 25 | 854 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 874 | N/A | N/A |
| DC_7A-28A_n40A | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |
|  | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_7A-28A_n66A DC_7C-28A_n66A | 7 | 2562 | 10 | 50 | 2682 | 16.9 | IMD3 |
|  | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796 | 20.0 | IMD2 |
|  | n66 | 1747 | 5 | 25 | 2147 | N/A | N/A |
| DC_7A-28A_n78A | 7 | 2567.5 | 5 | 25 | 2687.5 | N/A | N/A |
|  | 28 | 727.5 | 5 | 25 | 782.5 | 28.8 | IMD2 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 7 | 2567.5 | 5 | 25 | 2687.5 | N/A | N/A |
|  | 28 | 727.5 | 5 | 25 | 782.5 | 3.0 | IMD5 |
|  | n78 | 3460 | 10 | 50 | 3460 | N/A | N/A |
|  | 7 | 2530 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| DC_7A_n28A-n78A DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 28.8 | IMD2 |
| DC_7A-32A_n1A | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |
|  | 32 | N/A | 5 | N/A | 1454.5 | 15.2 | IMD3 |
| DC_7A-32A_n78A | n78 | 3560.5 | 10 | 50 | 3560.5 | N/A | N/A |
|  | 7 | 2517.5 | 5 | 25 | 2637.5 | N/A | N/A |
|  | 32 | N/A | 5 | N/A | 1474.5 | 17.6 | IMD3 |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | 32 | N/A | 5 | N/A | 1492 | 4.9 | IMD4 |
| DC_7A-40A_n1A DC_7A-40C_n1A | n1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 7 | 2530 | 5 | 25 | 2650 | 32.1 | IMD3 |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_7A-40A_n78A DC_7A-40C_n78A | 7 | 2510 | 5 | 25 | 2630 | 10.1 | IMD4 |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
|  | n78 | 3625 | 10 | 50 | 3625 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | N/A | N/A |
|  | 40 | 2310 | 5 | 25 | 2310 | 8.7 | IMD4 |
|  | n78 | 3785 | 10 | 50 | 3785 | N/A | N/A |
| DC_7A-46A_n78A6 | 7 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_7A-66A_n5A DC_7C-66A_n5A DC_7A-66A-66A_n5A DC_7C-66A-66A_n5A DC_7A-7A-66A_n5A DC_7A-7A-66A-66A_n5A | 7 | 2505 | 10 | 50 | 2625 | 30.0 | IMD26 |
|  | 66 | 1775 | 10 | 50 | 2175 | N/A | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
| DC_7A-66A_n7A DC_7A-66A-66A_n7A | 7 | 2555 | 10 | 50 | 2675 | 15 | IMD4 |
|  | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2515 | 10 | 50 | 2635 | N/A | N/A |
| DC_7A-66A_n28A | 7 | 2565 | 5 | 25 | 2685 | 18.0 | IMD3 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
| DC_7A-66A_n77A DC_7A-7A-66A_n77A DC_7A-7A-66A_n77(2A) DC_7A-66A_n77(2A) DC_7C-66A_n77A DC_7C-66A_n77(2A) | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 8.7 | IMD4 ¦2*fB7-2*fn77¦ |
|  | n77 | 3625 | 10 | 50 | 3475 | N/A | N/A |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | 7 | 2550 | 5 | 25 | 2670 | 5.2 | IMD5 |
|  | N77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | 3.4 | IMD5 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
| DC_7A-66A_n78A DC_7C-66A_n78A DC_7A-7A-66A_n78A DC_7A-66A-66A_n78A DC_7A-7A-66A-66A_n78A DC_7C-66A-66A_n78A DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) DC_7A-7A-66A_n78(2A) DC_7A-66A-66A_n78(2A) DC_7A-7A-66A-66A_n78(2A) DC_7C-66A-66A_n78(2A) | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | 66/n66 | 1750 | 5 | 25 | 2150 | 8.7 | IMD4 |
|  | n78 | 3625 | 10 | 50 | 3475 | N/A | N/A |
| DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A | 7 | 2542 | 5 | 25 | 2662 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3344 | 10 | 50 | 3344 | 16.0 | IMD3 |
| DC_7A-71A_n78A | 7 | 2550 | 5 | 25 | 2670 | 29.6 | IMD2 |
|  | 71 | 680 | 5 | 25 | 634 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | 71 | 686 | 5 | 25 | 640 | 3.0 | IMD5 |
|  | n78 | 3490 | 10 | 50 | 3490 | N/A | N/A |
| DC_7A_n71A-n78A | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n78 | 3714 | 10 | 50 | 3714 | 9.7 | IMD4 |
|  | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
|  | n71 | 671 | 5 | 25 | 625 | 3.9 | IMD5 |
| DC_7A_SUL_n78A-n80A | n80 | 1730 | 5 | 25 |  | N/A | N/A |
|  | 7 | 2535 | 10 | 50 | 2655 | 13 | IMD4 |
| DC_8A_n1A-n78A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n1 | 1945 | 5 | 25 | 2135 | N/A | N/A |
|  | n78 | 3745 | 10 | 50 | 3745 | 14.9 | IMD3 |
| DC_8A_n3A-n28A | 8 | 912.5 | 5 | 25 | 957.5 | N/A | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 30.4 | IMD2 |
| DC_8A-n3A_n77A DC_8A-n3A_n77(2A) | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 16.3 | IMD3 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_8A-11A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 11 | 1443 | 5 | 25 | 1491 | 18.8 | IMD3 |
| DC_8A-11A_n77A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | N/A |
|  | n77 | 3791 | 10 | 50 | 3791 | N/A | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 18.2 | IMD3 |
| DC_8A-11A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 11 | 1443 | 5 | 25 | 1491 | 18.8 | IMD3 |
| DC_8A-11A_n78A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | N/A |
|  | n78 | 3791 | 10 | 50 | 3791 | N/A | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 18.2 | IMD3 |
| DC_8A-20A_n78A | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | n78 | 3470 | 10 | 50 | 3470 | N/A | N/A |
|  | 20 | 841 | 5 | 25 | 800 | 12.1 | IMD4 |
|  | 8 | 895 | 5 | 25 | 940 | 12.1 | IMD4 |
|  | n78 | 3481 | 10 | 50 | 3481 | N/A | N/A |
|  | 20 | 847 | 5 | 25 | 806 | N/A | N/A |
| DC_8A_n28A-n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n77 | 3473 | 10 | 50 | 3473 | 10.3 | IMD4 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 11.6 | IMD4 |
|  | n77 | 3495 | 10 | 50 | 3495 | N/A | N/A |
| DC_8A_n28A-n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n78 | 3455 | 10 | 50 | 3455 | 10.3 | IMD4 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 11.6 | IMD4 |
|  | n78 | 3495 | 10 | 50 | 3495 | N/A | N/A |
| DC_8A-40A_n1A DC_8A-40C_n1A | 8 | 885 | 5 | 25 | 930 | 8.0 | IMD4 |
|  | 40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
| DC_8A-40A_n78A DC_8A-40C_n78A | 8 | 905 | 5 | 25 | 950 | 30.5 | IMD2 |
|  | 40 | 2380 | 5 | 25 | 2380 | N/A | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |
|  | 8 | 890 | 5 | 25 | 935 | 19.8 | IMD3 |
|  | 40 | 2320 | 5 | 25 | 2320 | N/A | N/A |
|  | n78 | 3705 | 10 | 50 | 3705 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 40 | 2395 | 5 | 25 | 2395 | 28 | IMD2 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
| DC_8A_n40A-n79A | 8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | n40 | 2305 | 5 | 25 | 2305 | N/A | N/A |
|  | n79 | 4960 | 40 | 216 | 4960 | 10.7 | IMD4 |
|  | 8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | n40 | 2305 | 5 | 25 | 2305 | 9.2 | IMD4 |
|  | n79 | 4960 | 40 | 216 | 4960 | N/A | N/A |
| DC_8A_n41A-n79A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | N/A |
|  | n79 | 4470 | 40 | 216 | 4470 | 16.3 | IMD3 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | 15.5 | IMD3 |
|  | n79 | 4470 | 40 | 216 | 4470 | N/A | N/A |
| DC_8A-42A_n3A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | N/A |
|  | 42 | 3540 | 5 | 25 | 3540 | 16.3 | IMD3 |
| DC_8A-42A_n28A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 42 | 3443 | 5 | 25 | 3443 | 8.7 | IMD4 |
| DC_8A_SUL_n78A-n80A | n80 | 1755 | 10 | 50 |  | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 8 | IMD4 |
|  | n80 | 1750 | 10 | 50 |  | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3550 | 10 | 50 | 3550 | 8 | IMD33 |
| DC_11A-n3A_n28A | 11 | 1435 | 5 | 25 | 1483 | N/A | N/A |
|  | n3 | 1753 | 5 | 25 | 1848 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 3.0 | IMD5 |
| DC_11A-n3A_n77A DC_11A-n3A_n77(2A) | 11 | 1440 | 5 | 25 | 1488 | N/A | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | 10.8 | IMD4 |
|  | 11 | 1440 | 5 | 25 | 1488 | N/A | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 29.0 | IMD2 |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_11A-18A_n77A | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |
|  | n77 | 3706 | 10 | 50 | 3706 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 18.7 | IMD3 |
| DC_11A-18A_n78A | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |
|  | n78 | 3706 | 10 | 50 | 3706 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 18.7 | IMD3 |
| DC_11A_n28A-n77A DC_11A_n28A-n77(2A) | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n77 | 3629 | 10 | 50 | 3629 | 17.5 | IMD3 |
|  | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |
|  | n77 | 3684 | 10 | 50 | 3684 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 15.8 | IMD3 |
| DC_12A_n2A-n38A | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n38 | 2608 | 5 | 25 | 2608 | 28.7 | IMD2 |
| DC_12A_n2A-n41A | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2608 | 5 | 25 | 2608 | 28.7 | IMD2 |
| DC_12A_n7A-n78A, DC_12A_n7(2A)-n78A DC_12A_n7A-n78(2A) DC_12A_n7(2A)-n78(2A) | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3624 | 10 | 50 | 3624 | 9 | IMD4 |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
|  | n7 | 2542 | 5 | 25 | 2662 | 29.6 | IMD2 |
| DC_12A-30A_n2A | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 30 | 2308 | 5 | 25 | 2353 | 12.0 | IMD4 |
|  | n2 | 1885 | 5 | 25 | 1965 | N/A | N/A |
| DC_12A-66A_n5A | 12 | 712 | 5 | 25 | 742 | 9.4 | IMD4 |
|  | 66 | 1745 | 5 | 25 | 2145 | N/A | N/A |
|  | n5 | 829 | 5 | 25 | 874 | N/A | N/A |
| DC_13A_n2A-n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | 1896 | 5 | 25 | 1976 | N/A | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | 17.3 | IMD3 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 16.0 | IMD3 |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | N/A |
| DC_13A_n48A-n66A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n48 | 3584 | 5 | 25 | 3584 | 2.8 | IMD5 |
|  | n66 | 1716 | 5 | 25 | 2116 | N/A | N/A |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
|  | n66 | 1731 | 5 | 25 | 2131 | 17.1 | IMD3 |
| DC_13A-66A_n2A DC_13A-66A-66A_n2A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | 66 | 1736 | 5 | 25 | 2156 | 7..2 | IMD4 |
|  | n2 | 1860 | 5 | 25 | 1940 | N/A | N/A |
| DC_13A-66A_n5A | 13 | 781 | 5 | 25 | 750 | 9.4 | IMD4 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 885 | N/A | N/A |
| DC_12A-66A_n25A | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
| DC_12A-66A_n41A | 12 | 712 | 5 | 25 | 742 | 31 | IMD2 |
|  | 66 | 1773 | 5 | 25 | 2173 | N/A | N/A |
|  | n41 | 2515 | 5 | 25 | 2515 | N/A | N/A |
| DC_12A-66A_n78A | 12 | 710 | 5 | 25 | 740 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 17.1 | IMD3 |
|  | n78 | 3580 | 5 | 25 | 3580 | N/A | N/A |
| DC_13A-66A_n48A DC_13A-66A_n48B DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | 66 | 1731 | 5 | 25 | 2131 | 17.1 | IMD3 |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
| DC_13A-66A_n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | 66 | 1756 | 5 | 25 | 2156 | 17.1 | IMD3 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_18A_n3A-n41A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n41 | 2540 | 10 | 50 | 2540 | 29.4 | IMD2 |
|  | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 28.2 | IMD2 |
| DC_18A_n3A-n77A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | 16.3 | IMD3 |
|  | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | 15.7 | IMD3 |
|  | n77 | 3505 | 10 | 50 | 3505 | N/A | N/A |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1762 | 5 | 25 | 2162 | 7.6 | IMD4 |
|  | n2 | 1874 | 5 | 25 | 1954 | N/A | N/A |
| DC_18A_n3A-n78A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 15.2 | IMD33 |
| DC_18A-28A_n77A DC_18A_n28A-n77A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | 28/n28 | 723 | 5 | 25 | 778 | 4.4 | IMD5 |
|  | n77 | 4058 | 10 | 50 | 4058 | N/A | N/A |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | 3.9 | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n77 | 3757 | 10 | 50 | 3757 | N/A | N/A |
| DC_18A-28A_n78A | 18 | 819 | 5 | 25 | 864 | 3.8 | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n78 | 3756 | 10 | 50 | 3756 | N/A | N/A |
| DC_18A_n28A-n77A DC_18A_n28A-n78A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | N/A | N/A |
|  | n77/n78 | 3770 | 10 | 50 | 3770 | 4.0 | IMD5 |
| DC_18A-41A_n3A DC_18A-41C_n3A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | 41 | 2630 | 5 | 25 | 2630 | 16.0 | IMD3 |
|  | 18 | 820 | 5 | 25 | 865 | 28.9 | IMD21 |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | N/A |
|  | 41 | 2630 | 5 | 25 | 2630 | N/A | N/A |
| DC_18A-41A_n77A DC_18A-41C_n77A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 |
|  | n77 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
| DC_18A_n41A-n77A DC_18A_n41A-n78A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n41 | 2570 | 5 | 25 | 2570 | N/A | N/A |
|  | n77/n78 | 3390 | 10 | 50 | 3390 | 30.1 | IMD2 |
|  | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n77/n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | n41 | 2630 | 5 | 25 | 2630 | 28.5 | IMD2 |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 |
|  | n78 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
| DC_19A_n1A-n77A DC_19A_n1A-n78A | 19 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n77/n78 | 3655 | 10 | 50 | 3655 | [21.4] | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | n77/n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | 837.5 | 5 | 25 | 882.5 | 18.7 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3783.3 | 10 | 50 | 3783.3 | N/A | N/A |
| DC_19A-21A_n77A | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | 21 | 1454.5 | 5 | 25 | 1502.5 | 9.0 | IMD4 |
|  | n77 | 4015 | 10 | 50 | 4015 | N/A | N/A |
| DC_19A-21A_n79A | 19 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.2 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 3.8 | IMD5 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | N/A |
| DC_20A_n1A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | 16.0 | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | 15.3 | IMD3 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | IMD3 |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | N/A |
| DC_20A_38A-n78A | 20 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | 38 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 20 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 38 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20A_n7A-n28A | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
|  | n7 | 2512 | 5 | 25 | 2632 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 13.9 | IMD3 |
|  | 20 | 852 | 5 | 25 | 811 | N/A | N/A |
|  | n7 | 2550 | 10 | 50 | 2670 | 5.9 | IMD5 |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
| DC_20A_SUL_n78A-n80A | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
|  | n80 | 1735 | 5 | 25 |  | N/A | N/A |
| DC_20A_n41A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n41 | 2675 | 10 | 50 | 2675 | 29.8 | IMD2 |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
|  | 20 | 850 | 5 | 25 | 809 | N/A | N/A |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | 28.8 | IMD2 |
| DC_21A_n1A-n77A DC_21A_n1A-n78A | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD24 |
|  | n77/n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |
| DC_21A-28A_n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | 28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD3 |
|  | n77 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |
|  | 21 | 1450.5 | 5 | 25 | 1498.5 | 9.9 | IMD4 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_21A-28A_n79A | 21 | 1450 | 5 | 25 | 1498 | 5.2 | IMD5 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_21A_n28A-n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
| DC_21A_n28A-n78A | n28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD39 |
|  | n77/n78 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | n28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n77/n78 | 3634.5 | 10 | 50 | 3634.5 | 17.3 | IMD39 |
| DC_21A_n28A-n79A | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n28 | 735.5 | 5 | 25 | 790.5 | 2.8 | IMD5 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
|  | 21 | 1460.4 | 5 | 25 | 1508.4 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | [6.3] | IMD44 |
| DC_21A-42A_n1A | 21 | 1452 | 5 | 25 | 1500 | 31.4 | IMD2 |
|  | 42 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
| DC_28A_n1A-n40A | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2374 | 5 | 25 | 2374 | 10.1 | IMD4 |
| DC_28A_n1A-n78A | 28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |
| DC_28A_n3A-n77A | 28 | 735 | 5 | 25 | 790 | N/A | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 17.0 | IMD3 |
|  | n77 | 3320 | 10 | 52 | 3320 | N/A | N/A |
|  | 28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77 | 4173 | 10 | 50 | 4173 | 15.9 | IMD3 |
| DC_28A_n7A-n78A DC_28A_n7B-n78A | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| DC_28A-41A_n77A | 28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | 29.5 | IMD2 |
| DC_28A-41A_n77A | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | N/A |
|  | 28 | 743 | 5 | 25 | 798 | 30.8 | IMD2 |
| DC_28A-41A_n77A | 41 | 2567.5 | 10 | 50 | 2567.5 | N/A | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | N/A |
|  | 28 | 727.5 | 5 | 25 | 782.5 | 3.0 | IMD5 |
| DC_28A-41A_n78A | 28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | 29.5 | IMD2 |
| DC_28A-41A_n78A | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | N/A |
|  | 28 | 743 | 5 | 25 | 798 | 30.8 | IMD2 |
| DC_28A-41A_n79A | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n79 | 4739 | 40 | 216 | 4739 | N/A | N/A |
|  | 41 | 2510 | 5 | 25 | 2510 | 8.6 | IMD4 |
| DC_28A-41A_n79A | 41 | 2650 | 5 | 25 | 2650 | N/A | N/A |
|  | n79 | 4502 | 40 | 216 | 4502 | N/A | N/A |
|  | 28 | 743 | 5 | 25 | 798 | 15.9 | IMD3 |
| DC_28A-42A_79A | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | 42 | 3420 | 5 | 25 | 3420 | 15.3 | IMD3 |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | N/A |
|  | 28 | 745 | 5 | 25 | 800 | 16.2 | IMD2 |
|  | 42 | 3597.5 | 5 | 25 | 3597.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_28A-66A_n7A | 28 | 735 | 5 | 25 | 790 | 27.6 | IMD2 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n7 | 2505 | 5 | 50 | 2625 | N/A | N/A |
| DC_28A-66A_n66A | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 66 | 1729 | 5 | 25 | 2129 | 11.0 | IMD4 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | N/A |
|  | n79 | 4515 | 40 | 216 | 4515 | 29.3 | IMD2 |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |
|  | n78 | 3715 | 10 | 50 | 3715 | 28.8 | IMD2 |
| DC_20A-28A_n3A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | 28 | 730 | 5 | 25 | 785 | 9.4 | IMD4 |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
| DC_20A_n28A-n78A, DC_20A_SUL_n78A-n83A | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
|  | n28, n83 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3314 | 10 | 50 | 3314 | 8.7 | IMD4 |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | n28 | 744 | 5 | 25 | 799 | 9.4 | IMD4 |
| DC_20A-32A_n1A | n1 | 1950.5 | 5 | 50 | 2140.5 | N/A | N/A |
|  | 20 | 852.5 | 5 | 25 | 811.5 | N/A | N/A |
|  | 32 | N/A | 5 | N/A | 1459.5 | 4.0 | IMD5 |
| DC_20A-40A_n78A | 20 | 856 | 5 | 25 | 815 | 19.8 | IMD3 |
|  | 40 | 2302.5 | 5 | 25 | 2302.5 | N/A | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | N/A | N/A |
|  | n79 | 4873 | 40 | 216 | 4873 | 30.1 | IMD2 |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n79 | 4940 | 40 | 216 | 4940 | N/A | N/A |
|  | n78 | 3487 | 10 | 50 | 3487 | 29.8 | IMD2 |
| DC_25A-66A_n77A DC_25A-25A-66A_n77A | 25 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | 66 | 1715 | 5 | 25 | 2115 | 29.2 | IMD2 |
|  | n77 | 3970 | 10 | 25 | 3970 | N/A | N/A |
|  | 25 | 1880 | 5 | 25 | 1960 | M/A | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | 10.4 | IMD4 |
|  | n77 | 3500 | 10 | 25 | 3500 | N/A | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | M/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | 4.0 | IMD5 |
|  | n77 | 3915 | 10 | 25 | 3915 | N/A | N/A |
|  | 25 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3720 | 10 | 25 | 3720 | N/A | N/A |
|  | 25 | 1855 | 5 | 25 | 1935 | 4.2 | IMD5 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n77 | 3540 | 10 | 25 | 3540 | N/A | N/A |
| DC_25A-66A_n78A DC_25A-25A-66A_n78A | 25 | 1880 | 5 | 25 | 1960 | M/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 10.4 | IMD4 |
|  | n78 | 3480 | 10 | 50 | 3480 | N/A | N/A |
|  | 25 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |
|  | 25 | 1880 | 5 | 25 | 1960 | 9.1 | IMD4 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 25 | 1900 | 5 | 25 | 1980 | 4.2 | IMD5 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n78 | 3645 | 10 | 25 | 3645 | N/A | N/A |
| DC_28A_n8A-n78A | 28 | 728 | 5 | 25 | 783 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3458 | 10 | 50 | 3458 | 9.1 | IMD4 |
|  | 28 | 713 | 5 | 25 | 768 | N/A | N/A |
|  | n8 | 890 | 5 | 25 | 935 | 4.3 | IMD5 |
|  | n78 | 3787 | 10 | 50 | 3787 | N/A | N/A |
| DC_29A-30A_n66A | 29 | N/A | 5 | 25 | 719.5 | 4.5 | IMD5 |
|  | 30 | 2307.5 | 5 | 25 | 2352.5 | N/A | N/A |
|  | n66 | 1777.5 | 5 | 25 | 2177.5 | N/A | N/A |
| DC_30A-66A_n5A, DC_30A-66A-66A_n5A, DC_30A-66A-66A-66A_n5A | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | 66 | 1730 | 5 | 25 | 2130 | 2.5 | IMD5 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
| DC_39A_n40A-n79A | 39 | 1917.5 | 5 | 25 | 1917.5 | N/A | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | 5.8 | IMD4 |
| DC_39A_n41A-n79A | 39 | 1900 | 5 | 25 | 1900 | N/A | N/A |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | 29.8 | IMD24 |
|  | 39 | 1900 | 5 | 25 | 1900 | N/A | N/A |
|  | n41 | 2620 | 10 | 50 | 2620 | 30.2 | IMD24 |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | N/A |
| DC_41A_n3A-n77A DC_41C_n3A-n77A DC_41A_n3A-n78A DC_41C_n3A-n78A | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |
|  | n77/n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 41 | 2580 | 5 | 25 | 2580 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77/n78 | 3440 | 10 | 50 | 3440 | 16.8 | IMD34 |
| DC_41A_n28A-n77A DC_41C_n28A-n77A DC_41A_n28A-n78A DC_41C_n28A-n78A | 41 | 2580 | 5 | 25 | 2580 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n77/n78 | 3323 | 10 | 50 | 3323 | 28.2 | IMD21 |
|  | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 30.8 | IMD21 |
|  | n77/n78 | 3440 | 10 | 50 | 3440 | N/A | N/A |
| DC_46A-66A_n5A | 46 | 5163 | 10 | 50 | 5163 | 9.0 | IMD4 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n5 | 847 | 5 | 25 | 892 | N/A | N/A |
| DC_46A-66A_n25A4 DC_46C-66A_n25A4 DC_46D-66A_n25A4 | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
| DC_48A-66A_n12A | 48 | 3580 | 5 | 25 | 3580 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 17.1 | IMD3 |
|  | n12 | 710 | 5 | 25 | 740 | N/A | N/A |
| DC_48A-66A_n25A DC_48C-66A_n25A DC_48D-66A_n25A | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
|  | 66 | 1730 | 5 | 25 | 2130 | 8.3 | IMD4 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | N/A |
| DC_48A-66A_n71A | 48 | 3560 | 5 | 25 | 3560 | N/A | N/A |
|  | 66 | 1774 | 5 | 25 | 2174 | 15.8 | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | 48 | 3697.5 | 5 | 25 | 3697.5 | 13.0 | IMD4 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
| DC_66A_n2A-n66A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n2 | 1870 | 5 | 25 | 1950 | N/A | N/A |
|  | n66 | 1770 | 5 | 25 | 2170 | 4.0 | IMD5 |
| DC_66A_n2A-n77A |  |  |  |  |  |  |  |
|  | n2 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_66A_n5A-n48A | 66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | N/A | N/A |
|  | n48 | 3582 | 5 | 25 | 3582 | 3.3 | IMD5 |
| DC_66A_n5A-n77A | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n5 | 845 | 5 | 25 | 890 | N/A | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | 16.6 | IMD3 |
| DC_66A_n7A-n78A, DC_66A-66A_n7A-n78 DC_66A_n7(2A)-n78A DC_66A-66A_n7(2A)-n78A DC_66A_n7A-n78(2A) DC_66A-66A_n7A-n78(2A) DC_66A-66A_n7(2A)-n78(2A) | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |
| DC_66A_n25A-n41A | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
|  | n25 | 1860 | 5 | 25 | 1940 | 5 | 11.0 |
| DC_66A_n25A-n48A | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 28.3 | IMD2 |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
| DC_66A_n38A-n78A | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n38 | 2610 | 5 | 25 | 2610 | N/A | N/A |
|  | n78 | 3460 | 10 | 50 | 3460 | 15.0 | IMD3 |
| DC_66A_n66A-n71A | 66 | 1752 | 5 | 25 | 2152 | N/A | N/A |
|  | n66 | 1718 | 5 | 25 | 2118 | 5.0 | IMD4 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | N/A |
| DC_66A_n66A-n77A | 66 | 1730 | 5 | 25 | 2140 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2170 | 31 | IMD2 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
| DC_66A_n66A-n78A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n66 | 1725 | 5 | 25 | 2125 | 2.8 | IMD5 |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |
| DC_66A_n71A-n78A | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
|  | n78 | 3709 | 5 | 25 | 3709 | 13.0 | IMD4 |
| DC_71A_n2A-n41A | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2586 | 5 | 25 | 2586 | 29.2 | IMD2 |
|  | 71 | 686 | 5 | 50 | 640 | N/A | N/A |
|  | n2 | 1862 | 5 | 25 | 1942 | 26 | IMD2 |
|  | n41 | 2610 | 5 | 25 | 2610 | N/A | N/A |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
| DC_71A_n2A-n78A | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | 71 | 695.5 | 5 | 25 | 649.5 | N/A | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | 8.0 | IMD3 |
|  | n2 | 1874 | 5 | 25 | 1954 | 16.5 | IMD3 |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
| DC_71A_n38A-n78A | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n38 | 2615 | 5 | 25 | 2615 | N/A | N/A |
|  | n78 | 3308 | 10 | 50 | 3308 | 29.1 | IMD2 |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n78 | 3308 | 10 | 50 | 3308 | N/A | N/A |
|  | n38 | 2615 | 5 | 25 | 2615 | 28.7 | IMD2 |
| DC_71A_n66A-n78A | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n78 | 3546 | 10 | 50 | 3546 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 15.5 | IMD3 |
|  | 71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
|  | n78 | 3697.5 | 10 | 50 | 3697.5 | 13.0 | IMD4 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 2: For DC_3A_n3A-n77A, DC_3A_n3A-n78A paired with UL_DC_3A_n3A, the 3rd DL bands n77/n78 are subject to IMD2 which MSD is not specified NOTE 3: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: This band is also subject to IMD2 which is not specified. The frequency range below 3400MHz in n77 is not used for this combination. NOTE 8: Band 5 is also affected by IMD5 from UL DC_2A_n12A, but MSD value is not specified as there is only partial overlap of IMD5 with DL carrier. NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 10: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2 fall in B1 also which MSD is not specified. |

<<< End of changed sections >>>