3GPP TGS-RAN WG4 Meeting #99-e	R4-2110171
Electronic Meeting, May 19 – 27, 2021

Agenda item:	9.15.3

Source:	Intel Corporation

Title:	On System Parameters in 60 GHz

Document for:	Discussion

# 1	Introduction

In the RAN4#98bis-e, there were quite a few progresses in system parameter discussions [1]. This contribution continues discussing open issues in 60 GHz NR system parameters.

# 2	Discussion

## 2.1	Channel bandwidths

## Maximum channel bandwidth for 960 kHz

Last meeting left the maximum channel bandwidth for 960 kHz open with the following options:

- 2000 MHz for both licensed and unlicensed operations

- 2160 MHz as the max. bandwidth, also 2000 MHz will be specified as a channel bandwidth, both licensed and unlicensed operations

- 2160 MHz for unlicensed operation and 2000 MHz for licensed operation

- Make a decision for unlicensed operation and FFS for licensed operation

In principle, different maximum channel bandwidth can apply for licensed and unlicensed bands, i.e., 2160 MHz for unlicensed band and 2000 MHz for licensed. However, it is also important to keep the design principle from FR2 to reuse the design where channel bandwidth is a multiple of 100 MHz. Therefore, 2000 MHz channel bandwidth as the maximum for 960 kHz is attractive for 60 GHz.

Proposal #1: 2000 MHz for both licensed and unlicensed operations for 960 kHz SCS.

## Channel bandwidths between minimum and maximum CBWs

Currently, carrier aggregation (CA) support is being discussed. Considering the 14 GHz as a whole spectrum range, too many supported channel bandwidths would create excessive channel bandwidths combinations in CA operation.

Observation #1: Introducing too many channel bandwidths creates excessive channel bandwidths combinations.

Proposal #2: Minimize number of supported channel bandwidths

The table 1 shows the summary of proposed channel bandwidths for each numerology.

| Subcarrier spacing [kHz] | Minimum bandwidths [MHz] | Other bandwidths between min. and max. CBW | Maximum bandwidths [MHz] |
| --- | --- | --- | --- |
| 120 | 100 | 200 | 400 |
| 480 | 400 | 800, 1200 | 1600 |
| 960 | 400 | 500, 800, 1000, 1600 | 2000 |

Table 1. Summary of proposed numerologies and channel bandwidths

## 2.2	Spectrum utilization

Based on ETSI EN 302 567 (V2.1.1) the occupied channel bandwidth shall be between 70 % and 100 % of the declared nominal channel bandwidth. While IEEE 802.11ad/ay has ~ 87 %, FR2 aimed to achieve max. 95 %. Considering the channel bandwidth could be very wide, emission related requirements could be more challenging with higher spectrum utilization. We think maximum spectrum utilization of 90 % could be a good compromise. Table 2 shows the proposed number of PRBs to meet 90 % spectrum utilization.

Proposal #3: Target spectrum utilization is 90 % across all supported channel bandwidths

|  |  | Channel Bandwidth (MHz)/PRBs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 100 | 200 | 400 | 500 | 800 | 1000 | 1200 | 1600 | 2000 |
| SCS (kHz) | 120 | 63 | 125 | 250 | - | - | - | - | - | - |
|  | 480 | - | - | 63 | - | 125 | - | 188 | 250 | - |
|  | 960 | - | - | 31 | 39 | 63 | 78 | - | 125 | 156 |

Table 2. Proposed number of PRBs which meet 90 % spectrum utilization

## 2.3	Operation Scenario

In the latest approved WID, operations of standalone, aggregated via CA, or DC with an anchor carrier is to be supported. Given the situation where unlicensed band was agreed, and CA is still being discussed, we propose to prioritize standalone mode.

Proposal #4: RAN4 prioritizes on standalone mode operation.

# 3	Conclusions

This paper made the following observation and proposals:

Channel bandwidths

Observation #1: Introducing too many channel bandwidths creates excessive channel bandwidths combinations.

Proposal #1: 2000 MHz for both licensed and unlicensed operations for 960 kHz SCS.

Proposal #2: Minimize number of supported channel bandwidths

| Subcarrier spacing [kHz] | Minimum bandwidths [MHz] | Other bandwidths between min. and max. CBW | Maximum bandwidths [MHz] |
| --- | --- | --- | --- |
| 120 | 100 | 200 | 400 |
| 480 | 400 | 800, 1200 | 1600 |
| 960 | 400 | 500, 800, 1000, 1600 | 2000 |

Table 1. Summary of proposed numerologies and channel bandwidths

Spectrum utilization

Proposal #3: Target spectrum utilization is 90 % across all supported channel bandwidths

|  |  | Channel Bandwidth (MHz)/PRBs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 100 | 200 | 400 | 500 | 800 | 1000 | 1200 | 1600 | 2000 |
| SCS (kHz) | 120 | 63 | 125 | 250 | - | - | - | - | - | - |
|  | 480 | - | - | 63 | - | 125 | - | 188 | 250 | - |
|  | 960 | - | - | 31 | 39 | 63 | 78 | - | 125 | 156 |

Table 2. Proposed number of PRBs which meet 90 % spectrum utilization

Operation scenario

Proposal #4: RAN4 prioritizes on standalone mode operation.

# 4	References

- R4-2105410, WF on [137] NR_ext_to_71GHz_Part1, Intel (Moderator), RAN4#98Bis-e, April 2021