3GPP TGS-RAN WG4 Meeting #99-e	R4-2110172
Electronic Meeting, May 19 – 27, 2021

Agenda item:	9.15.4.1

Source:	Intel Corporation

Title:	UE RF Tx requirements for NR in 60 GHz

Document for:	Discussion

# 1	Introduction

This paper continues discussing open issues in UE RF Tx requirements.

# 2	Discussion

## 2.1	Tx power class

UE RF discussions for the NR extension to 71 GHz WI started during the last RAN4 meeting and included potentially using FR2 requirements as baseline for 52.6 to 71 GHz, which companies agreed was reasonable to do [1]. For the Tx requirements, we also discussed power classes. In the following sections we will focus on how we can approach power class discussions, by considering its parameters and definitions.

## Power classes above 52.6 GHz

To help with power class definition, RAN4 discussed whether to define a target UE form factor [2]. Most companies supported the moderator’s suggestion to consider a target UE form factor aspect in the power class definition. This essentially implies we can follow the FR2 baseline which considers a UE type, and its form factor, when defining the requirements of a specific power class. Therefore, we think it is reasonable to continue using the FR2 power class framework, (including the four parameters, and per-band specification), up to 71 GHz. However, we do emphasize that this means the framework or approach can be reused, not necessarily the values or assumptions used in FR2; these need to be further discussed.

Observation 1-1: Majority view in the last RAN4 meeting was that the current FR2 UE RF requirements can be used as baseline for 52.6 to 71 GHz discussions. The FR2 power class approach, which captures relevant regulatory limits and aligns on a reference UE type to derive the requirements, can also be reused.

Proposal 1-1: Reuse FR2 power class framework for the 52.6 to 71 GHz frequency range. This means power classes will continue to be a package of four parameters: minimum peak EIRP, EIRP spherical coverage, maximum TRP and maximum EIRP (regulatory defined, captured in our specs for reference). Values and assumptions to be used for the four parameters are FFS.

### Power class parameters

Before we can address how to proceed with the power class performance requirements, we have to discuss whether the FR2 limits for maximum peak EIRP and TRP can be used for the 52.6 to 71 GHz frequency range. This implies reviewing relevant regulatory limits and capturing any differences in their definition. We will examine these in the upcoming section, starting with the maximum peak EIRP.

Maximum peak EIRP

In FR2, the maximum peak EIRP is either 43 dBm, or 55 dBm, depending on whether the device is a mobile station or a transportable one [3]. For the 52.6 to 71 GHz frequency range, applicable limits generally depend on device type, and antenna gain. For fixed devices, limits also depend on whether the transmitter is located indoors or outdoors.

Observation 1-2: For fixed devices, regulatory limits for 52.6 to 71 GHz consider the device antenna gain and transmitter location (indoors vs outdoors).

To determine what max EIRP value may be captured for the 52.6 to 71 GHz spectrum, we reviewed the content of TR 38.805 [4] and compared it to the available ETSI harmonized standards [5]. The table below summarizes the maximum EIRP limits considering the UE type, antenna gain and transmitter location. For FCC, mobile devices in this frequency range can continue using the 43 dBm max peak EIRP. For mobile devices, ETSI defines max average EIRP with the same 40 dBm value, but does not provide a max peak EIRP. They do include another limit for spectral power density, which we will address in another section.

Table 1. Summary of maximum EIRP regulatory limits [4-6]

| Location | UE type | Max average EIRP | Max peak EIRP |
| --- | --- | --- | --- |
| Anywhere | Mobile - FCC | 40 dBm | 43 dBm |
|  | Mobile - ETSI | 40 dBm |  |
|  | Fixed - FCC | 40 dBm | 43 dBm |
|  | Fixed - ETSI | 27 dBm + GAnt, for GAnt < 13 dBi 40 dBm, for 13 dBi ≤ GAnt < 30 dBi1 |  |
| Outdoor only | Fixed - ETSI | 55 dBm, for 30 dBi ≤ GAnt |  |
|  | Fixed P2P - FCC | 82 dBm for GAnt > 51 dBi 82 - 2*(51 – GAnt) for GAnt ≤ 51 dBi | 85 dBm for GAnt > 51 dBi 85 - 2*(51 - GAnt) for GAnt ≤ 51 dBi |
| 1 For UE arrays of 8, 16, and 32 elements, the antenna gain falls in 13 dBi ≤ GAnt < 30 dBi, so the max avg EIRP limit is 40 dBm. |  |  |  |

Observation 1-3: For FCC, the 43 dBm max peak EIRP value used in FR2 can continue to be used for mobile devices up to 71 GHz.

On the other hand, the limits for fixed devices depend on the antenna gain and transmitter location in FCC and ETSI. As noted in Table 1, fixed devices in ETSI can follow a max avg EIRP of 40 dBm if the antenna gain is 13 dBi ≤ GAnt < 30 dBi. Doing a simple calculation for antenna gain, we expect an array of 8, 16 and 32 elements to have a gain in this range and can then follow the 40 dBm max mean EIRP. Furthermore, given the tall order on the antenna gain for outdoor fixed devices, we think it is unlikely for us to define a power class with this limit. In our view, using 40 dBm max avg EIRP for all UEs is reasonable. Considering the impact of the transmitter location and antenna gain in the regulatory limits, we think it would be useful to capture these details in the reference UE type description of a power class.

Observation 1-4: Since regulatory limits for 52.6 to 71 GHz depend on the UE type considered, and antenna gain, it is useful to discuss these as part of the reference UE type description to help determine which regulatory limits to include.

Maximum TRP

The maximum TRP parameter in FR2 power classes considers both UL co-channel interference and regulatory limits. Depending on the power class, the FR2 value is either 23 or 35 dBm [7]. From co-existence perspective, there are on-going discussions that should determine if there are issues with these numbers up to 71 GHz. From regulatory perspective, we summarize the limits in Table 2.

Table 2. Summary of maximum TRP limits [4-6]

| Emission bandwidth 1 | Max TRP |
| --- | --- |
| BW ≥ 100 MHz | 500 mW = 27 dBm2 |
| BW < 100 MHz | 500 mW * (emission BW/100MHz) |
| 1 Emission BW is determined by measuring the width of the signal between two points, one below the carrier center freq. and one above the carrier center freq., that are 26 dB down relative to the max level of the modulated carrier [9] 2 Captured as max power at antenna ports in ETSI. In case of lack of suitable methods to measure maximum power level at antenna port or ports, the requirement at antenna port or ports is verified with the test metrics of Total Radiated Power (TRP), i.e. maximum TRP shall be less than or equal to 27dBm [5] |  |

For the maximum TRP power, the limits state it shall not exceed 500 mW (or 27 dBm). Additionally, in FCC this value is proportionally reduced depending on the emission bandwidth value (captured in Table 2). Because the absolute maximum TRP value is 27 dBm, we know the 35 dBm used in FR2 cannot be used in any power class definition, or revision, for this frequency range. As for the 23 dBm value, pending the outcome of co-existence discussions, the value may be used in a power class definition for this range.

Observation 1-5: The maximum TRP value allowed for 52.6 to 71 GHz is 27 dBm, which means 35 dBm cannot be used for power classes in this frequency range. A lower value for maximum TRP value (e.g. 23 dBm) may be captured pending the outcome of co-existence discussions.

Maximum power spectral density

Lastly, ETSI considers a limit for the maximum power spectral density. In this one, fixed devices outdoors with 30 dBi or larger antenna gain have a different limit. However, this one is unlikely to apply to any UE we define, so the applicable limit is 23 dBm/MHz.

Table 3. Power spectral density limit [5]

| Condition | Maximum EIRP0 |
| --- | --- |
| fixed outdoor installations with 30 dBi transmit antenna gain | 38 dBm/MHz |
| otherwise | 23 dBm/MHz |

Observation 1-6: Considering the significant antenna gain threshold, the limit for power spectral density we should consider is 23 dBm/MHz.

Given the sheer the amount of regulatory limits worldwide, and that these are regularly updated, we recommend doing a thorough review and discussing whether we are aligned in our interpretation of them.

Proposal 1-3: RAN4 should discuss relevant regulatory limits and align on which values to capture in power classes. Table 1, Table 2 and Table 3 can be used in our discussions during this meeting.

UE types

To assess the performance related power class requirements in FR2, RAN4 considered a reference UE type in the budget derivation and simulation discussions. The reference UE type provided guidance for form factor aspects, array size and overall loss parameters to be used in derivations. It is very useful to have this reference in our power class discussions, thus we recommend also establishing a reference UE type per power class for NR above 52.6 GHz.

Observation 1-7: For performance related requirements of 52.6 to 71 GHz, RAN4 should first agree on the reference UE type to consider in power class discussions.

As was done in FR2, we recommend starting power class discussions for this frequency range with handheld and FWA UEs. Because these two represent the lowest and highest specified requirements, their discussion will allow us to get a clearer picture of what a reasonable performance for this frequency range is. This should be prioritized in RAN4 as this discussion will help us determine what values are feasible and communicate them to RAN1 [9].

Proposal 1-4: Prioritize handheld and FWA UEs in power class discussions for the 52.6 to 71 GHz frequency range.

Budget derivations and array size

After aligning on applicable regulatory limits and reference UE types, the discussion can progress to budget derivations and detailed performance assessment. Given the losses and integration challenges for the 52.8 to 71 GHz frequency range, concern over the achievable peak EIRP level was raised [1]. In this context, there were suggestions to consider using larger antenna arrays than the ones assumed in FR2. Overall, this is a reasonable approach to take as the smaller wavelength and antenna dimensions of this frequency range allow for more elements to be integrated. However, as we increase the array size, there are other aspects we should be mindful of, including regulatory limits.

Observation 1-7: Larger-sized antenna arrays are possible given the smaller wavelength of this frequency range. However, this needs to be further discussed once we agree on a reference UE and are better aligned on applicable regulatory limits.

Finally, we provide some guidelines for power class discussions in RAN4.

Proposal 1-5: We recommend power class discussions in RAN4 follow the steps outlined below:

- Focus on requirements for the agreed unlicensed band: 57 to 71 GHz [2]

- Agree on reference UE - prioritize handheld and FWA UEs

  - Confirm applicable max EIRP limit

  - Confirm applicable max TRP limit

  - Confirm applicable power spectral density

- Capture relevant regulatory limits (max EIRP, max TRP, power spectral density) in power class definition

  - Updates to some FR2 values are expected

  - Remaining power class parameters are minimum peak EIRP and spherical coverage

- Start budget derivations

  - Discuss array sizes while considering design challenges of the frequency range, and regulatory requirements

  - Provide budgets for different array sizes and assess feasible values

## 2.2	Tx mask and ACLR

ETSI harmonized requirements, i.e., EN 302 567, EN 303 722, and EN 303 753, define the transmit mask in 60 GHz. In this mask, BW represents the declared nominal channel BW or an absolute level of – 30 dBm with 1 MHz measurement bandwidth, whichever is the greater. IEEE 802.11ad/ay also has similar transmit mask but with different in frequency offsets. Figure 1 shows transmit masks from ETSI and IEEE 802.11ad/ay, respectively.

| (a) ETSI transmit mask | (b) IEEE 802.11ad/ay transmit mask |
| --- | --- |
| (c) Transmit mask comparison between ETSI and IEEE 802.11ad/ay with 2.16 GHz channel bandwidth |  |

Figure 1. (a) Tx masks from ETSI (upper left), (b) IEEE 802.11ad/ay (upper right), and (c) comparison between two masks with normalized with 2.16 GHz channel bandwidth (bottom)

ACLR can be estimated from the integration over the ETSI mask and it comes to 13.4 dBc ACLR. With 90 % of measurement bandwidth, ACLR can be 17 dBc. This is well aligned with 60 GHz coexistence study in TR 38.803 and recent proposal based on the latest PA study [13]. Since the Tx mask is independent of channel bandwidth, the same ACLR requirement can be applicable to all supported channel bandwidths.

Proposal 2-1: Agree on 17 dBc UE ACLR for all supported channel bandwidths in 60 GHz

## 2.3	Transient Period and DL-UL switching time

## 2.3.1 	Relationship among transient period, DL-UL switching time, and maximum modulation scheme

In Rel-15 NR, there were three different transient periods were defined which are ON-to-OFF, OFF-to-ON, and ON-to-ON transient periods. While ON-to-OFF and ON-to-ON can be much faster than OFF-to-ON where OFF-to-ON takes much longer time due to power amplifier ramping up and PLL settling time, RAN4 specified a single transient period to all three cases.

Observation 3-1: In Rel-15, FR2 5 uS transient period was defined for all three transient periods, i.e., ON-to-OFF, OFF-to-ON, and ON-to-ON.

While these three transient periods have been discussed under same issues in FR1 and FR2, their implications in system design are different, especially in 60 GHz where larger SCS is playing a critical role in system performance. The ON/OFF transient periods, i.e., both ON-to-OFF and OFF-to-ON transient periods, determines cell coverage because this is the main contributor to DL/UL switching time, i.e., DL/UL switching time = ON/OFF transient period + extra time for the rest of configuration.

Observation 3-2: DL/UL switching time determines cell coverage.

Observation 3-3: ON/OFF transient period is the main factor of UL/DL switching time, i.e., UL/DL switching time = ON/OFF transient period + extra time for the rest of configuration.

Observation 3-4: ON/OFF transient period needs to be defined first, to determine UL/DL switching time.

ON/ON transient period plays a critical role in system throughput performance. In FR2, a blanked symbol was introduced for a consecutive power change. Since 5 uS ON/ON transient period is comparable with ~ 7 uS OFDM symbol duration with the highest SCS 120 kHz, the system throughput performance was limited. However, an OFDM symbol duration is quite short with 480/960 kHz SCS in 60 GHz, where 5 uS ON/ON transient period is corresponding to multiple OFDM symbol durations. Therefore, multiple blanked symbols have to be introduced if FR2 transient period is being reused in 60 GHz.

Observation 3-5: ON/ON transient period directly connected to system throughput performance.

Observation 3-6: In FR2, a blanked symbol was introduced for a consecutive power change which is comparable with one OFDM symbol duration with 120 kHz SCS and throughput degradation was limited as 5 uS transient period was comparable with ~7 uS OFDM symbol duration.

Observation 3-7: In 60 GHz, with the existing FR2 transient period, i.e., 5 uS, multiple blanked symbols have to be introduced with 480/960 kHz SCS in 60 GHz which severely degrades system throughput performance.

In the next two sections, we will provide detail analysis on ON/OFF and ON/ON transient periods, respectively.

## 2.3.3	ON/OFF Transient period impact on cell coverage

To support very broad channel bandwidth in 52.6 – 71 GHz spectrum, higher subcarrier spacings have been enabled, i.e., 480 and 960 kHz. Thus, the impact of transient time will be larger and even critical with these larger subcarrier spacings as the symbol durations are much smaller than NR FR2. Table 4 shows a summary of numerologies associated with different transient periods where how much portion the transient periods take compared to an OFDM symbol. As an example, 5 uS FR2 UE transient time is corresponding to 60 % of an OFDM symbol duration with 120 kHz SCS. In this example, considering ON-to-OFF and OFF-to-ON power transition scenario, the whole OFDM symbol will be lost and the 20 % of the second symbol will be also affected.

The situation becomes worse with larger SCS in the new spectrum range. If the same transient periods are kept, i.e., 5 uS for UE and 3 uS for BS, then these are corresponding to 480 % of an OFDM symbol duration, i.e., 2 x 240 % with 5 uS UE transient period and 480 kHz SCS, which means that almost 5 OFDM symbols will be affected and cannot be used for data transmission. Similarly, 3 uS transient period is corresponding to 288 % of an OFDM symbol duration, i.e., 2 x 144 % with 3 uS BS transient time and 480 kHz SCS, which means almost 3 OFDM symbol durations. Assuming 1 uS transient time, 1 OFDM symbol will be impacted with 480 kHz SCS and almost 2 OFDM symbol will be impacted with 960 kHz SCS.

During through NR release 15 and 16, the performance impact of transient period had been evaluated and discussed [10, 11]. As an outcome of the series of discussions, UE transient period report capability has been introduced in Rel-16 where 2 uS is the fastest transient period.

| Subcarrier spacing (kHz) | 120 | 480 | 960 |
| --- | --- | --- | --- |
| Slot duration (uS) | 125.00 | 31.25 | 15.63 |
| OFDM symbol duration (uS) | 8.33 | 2.08 | 1.04 |
| CP duration (uS) | 0.60 | 0.15 | 0.07 |
| OFDM symbol + CP (uS) | 8.93 | 2.23 | 1.12 |
| TP portion in OFDM symbol - 5 uS | 60.0% | 240.0% | 480.0% |
| TP portion in OFDM symbol - 3 uS | 36.0% | 144.0% | 288.0% |
| TP portion in OFDM symbol - 1 uS | 12.0% | 48.0% | 96.0% |

Table 4. Performance implication of transient period for different subcarrier spacings

In one our previous paper [12], we evaluated the implication of ON/OFF transient period where it is directly related to DL/UL switching time and main contributor of limiting cell coverage distance. Table 5 reproduced the summary of the analysis. In the analysis, 1 symbol guard period is assumed.

Table 5. The summary of DL-UL switching time with 100 m cell coverage distance target

From the analysis, 1 uS DL/UL switching time is needed in order to efficiently support 100 m cell coverage with the minimum system overhead. In case 1 symbol guard period cannot be kept due to longer switching time, that introduces multiple guard periods which creates additional overhead to system. Based on current technologies, 1 uS period is achievable without any major technical issue.

ON/OFF transient period is the major factor to decide DL/UL switching time. While ON/OFF transient period is sole RF parameter, DL/UL switching time also consider extra time for configuration changes in transceiver, i.e., loading firmware and parameters change, etc.

Observation 3-8: In FR2, 1 symbol guard period was assumed.

Observation 3-9: In 60 GHz, multiple symbols are required for guard period for higher SCS with the same FR2 ON/OFF transient period, which makes additional overhead to the system.

Observation 3-10: ON/OFF transient period has impact on the DL/UL switching time and determines the cell coverage distance as well as DL/UL switching overhead.

Observation 3-11: 1uS ON-OFF and OFF-ON switching time are feasible from implementation perspective.

## 2.3.4 ON/ON Transient period impact on UL performance

During the SI phase, it was concluded that the existing FR2 transient period, i.e., 5 uS for UE and 3 uS for BS, might not applicable with higher subcarrier spacings, i.e., 480 kHz and 960 kHz. As an example, 5 uS transient period is corresponding to 240 % of an OFDM symbol duration with 480 kHz SCS. Considering a pair of ON-to-OFF and OFF-to-ON transition, this 5 uS transient period means that the first 4 OFDM symbols and 80% of the 5th OFDM symbol are affected and cannot be used for data transmission. This causes significant system throughput degradation especially with higher SCS. To improve system performance, a shorter transient period is necessary in 53.6 GHz – 71 GHz.

The FR2 transient scenarios are defined in clause 6.3.3 of TS 38.101-2. The specification defines multiple different scenarios for transient period time masks. In our analysis we focus on the potential PUSCH throughput impact due to ON/ON transient periods on the slot boundaries or on the boundaries of PUSCH and SRS transmissions.  The typical examples are such scenarios are 1) PUSCH-PUCCH and PUSCH-SRS (Figure 2); 2) Consecutive slot or long subslot transmission and short subslot transmission time mask (Figure 3); 3) Consecutive long PUSCH slot transmission. Typically, in such scenarios the short signals (e.g. SRS or short subslot) are prioritized and the transient period is fully placed in the PUSCH slot, thus resulting in the negative impact on the demodulation performance. In particular, in case of relatively long transient period one or more PUSCH symbols need to be blanked to allow sufficient time for the transient processes and the impact on the PUSCH throughput/BLER is expected.

Figure 2. PUCCH/PUSCH/SRS time mask when there is a transmission before or after or both before and after SRS

Figure 3. Consecutive slot or long subslot transmission and short subslot transmission time mask

In our analysis, we have evaluated the impact of the ON/ON transition period on the slot boundaries on the PUSCH demodulation performance. The following key assumptions were used for the analysis.

- Scenario #1: Regular NR PUSCH transmissions (single slot)

  - Single slot PUSCH performance is modelled as shown in the figure below with PUSCH occupying 14 OFDM symbols. The transient period is located in one or more last symbols of the PUSCH slot and the affected symbols are blanked and not scheduled by the gNB. The actual number of blanked symbols depends on transient period (TP) duration and we assume that the symbol is blanked in case the TP is longer than the CP duration. In case of PUSCH transmission in such slot the effective code-rate will be higher comparing to the full slot PUSCH transmission which would lead to demodulation performance loss comparing to the ideal scenario.

Figure 4. Transient period model for regular NR PUSCH transmission (single slot)

- Scenario #2: Multiple PUSCH/PUCCH slot transmissions (under discussion in RAN1)

  - In addition to the evaluation assumptions above, where the transition scenarios are based on the existing FR2 specification, we also consider multiple PUSCH/PUCCH slot transmissions, which is currently being discussed in RAN1. The motivation of the multiple PUSCH/PUCCH slot transmission is that a short period of symbol duration might have a critical impact with higher SCS when ON/OFF or ON/ON transition case is happening and bundling multiple slots might reduce BS scheduling burden.

  - The modelled multi-slot transmission scenario is shown in the figure below. In this scenario we assume that the transient period is located on the boundaries of the bundles each of which includes multiple UL slots. The scenarios with 2, 4 and 8 UL slots per bundle are evaluated.

Figure 5. Transient period model for multiple PUSCH/PUCCH slot transmissions (under RAN1 discussion)

For the evaluations we assess the performance of 1, 2, 3, and 5 us TP against the ideal scenario with 0 uS TP (no blanked symbols). One of key assumptions we make during the evaluation is that UL slot symbol blanking is applied when a TP is longer than the CP duration. The following table 6 summarizes the assumptions on the number of affected blanked symbols for different SCS and TP values.

| SCS | 120 kHz | 480 kHz | 960 kHz |
| --- | --- | --- | --- |
| Symbol duration, us | 8.33 | 2.08 | 1.04 |
| CP duration, us | 0.59 | 0.15 | 0.07 |
| Number of blanked OFDM symbol with 5us TP | 1 | 3 | 5 |
| Number of blanked OFDM symbol with 3us TP | 1 | 2 | 3 |
| Number of blanked OFDM symbol with 1us TP | 1 | 1 | 1 |

Table 6. Number of blanked symbols depending on transient period

The detailed simulation assumptions are provided in Annex A. Tables 7 – 10 show the summary of link-level simulation results.

| Table 7. SNR loss with single slot scheduling |  |
| --- | --- |
| Table 8. SNR loss with multi-slot transmissions (bundling size is 2) |  |
| Table 9. SNR loss with multi-slot transmissions (bundling size is 4) |  |
| Table 10. SNR loss with multi-slot transmissions (bundling size is 8) |  |

The key observations based on the evaluations are as follows:

Observation 3-12:

- Single slot scheduling case:

  - The existing 5 uS transient period cannot provide reliable performance for 16QAM MCS 16 with 480 kHz and 960 kHz SCS (note that ∞ means that there is scenario with certain TP cannot reach 1 % BLER).

  - 3uS transient period shows 13.6 dB performance loss compared to the ideal transient period (0 uS) for the MCS 16 with 960 kHz SCS

  - 2us transient period provides up to 3.5 dB performance loss for MCS 16 with 960 kHz SCS

  - 1us transient period allows < 2 dB performance loss for all considered scenarios

- Multiple PUSCH/PUCCH slot transmissions

  - Comparing the same evaluation condition with the multiple PUSCH/PUCCH slot transmission, the performance get improved with larger number of bundling. For example, for 3 uS transient period for MCS 16 with 960 kHz SCS the performance is 13.6 dB (without bundling)  11.3 dB (with 2 slot bundling)  9.2 dB (with 4 slot bundling)  7.6 dB (with 8 slot bundling).

  - While there could be 6 dB throughput improvement with multiple-slot transmission, we would like to point out that the evaluation is based on optimistic assumption, i.e., there is no transient period between the multiple slots illustrated in figure 4, which may or may not hold true based on the final design. Furthermore, the 7.6 dB loss from the 3 uS transient period with MCS 16 with 960 kHz SCS is still quite large compared to the ideal transient period (0 uS TP).

Observation 3-13: 16 QAM is not possible with FR2 5 uS transient period in 60 GHz and at least 3 uS transient period is required to support 16 QAM in 60 GHz.

Table 11 shows SNR loss with single slot scheduling with 64 QAM with the phase noise models in TR 38.803 and ideal CPE compensation. A cell with red-color fill indicates even 0 uS transient period cannot reach 1 % BLER due to PN impact.

|  |  | 5us | 3us | 1us |
| --- | --- | --- | --- | --- |
| 120 kHz | MCS 17 | 1.6 | 1.6 | 1.6 |
|  | MCS 22 |  |  |  |
|  | MCS 26 |  |  |  |
| 480 kHz | MCS 17 | 5.3 | 2.8 | 1.3 |
|  | MCS 22 | ∞ | ∞ | 7.4 |
|  | MCS 26 |  |  |  |
| 960 kHz | MCS 17 | 18.1 | 4.5 | 1.1 |
|  | MCS 22 | ∞ | ∞ | 2.2 |
|  | MCS 26 | ∞ | ∞ | ∞ |

Table 11. SNR loss with single slot scheduling with 64 QAM

Observation 3-14: 64 QAM seems not to be feasible with a single slot scheduling even with 1 uS transient period.

Based on the evaluation, 1 uS transient period for both ON/OFF and ON/ON provides optimal performance. Considering extra time for configuration changes, i.e., loading firmware and change parameters, etc., 3 uS DL/UL switching time would be feasible. While OFF-to-ON time typically longer than ON-to-OFF time, we think specifying the same transient period for both OFF-to-ON and ON-to-OFF to keep the consistency from FR2 specification but we are open for further discussion and optimization.

Observation 3-15: 1 uS transient period (both ON/OFF and ON/ON) is feasible and 3 uS DL/UL switching time is feasible.

However, some implementations may not be fully optimized in the early stage production and 1 uS transient period or 3 uS DL/UL switching time might be a demanding goal. Considering the situation, we think 2 or 3 uS transient period would be a good compromise.

Proposal 3-1: RAN4 agrees on [1 – 3] uS transient period for both ON/OFF and ON/ON transient period for 52.6 – 71 GHz range.

Proposal 3-2: RAN4 agrees on either [3 – 5] uS for switching time for both DL-to-UL and UL-to-DL.

There was a RAN1 LS on beam switching [14] and the draft reply LS is in Annex B based on the discussion in this paper.

## 2.4 	UE Tx and Rx beam switching times

During the study item phase, it was concluded that ~59 nS beam switching time case can be feasible in 60 GHz. In our view, phase shifter can switch much faster in typical case and 50 nS is sufficient with enough margin.

Proposal 4-1: 50 nS is UE switching Tx beam and Rx beam times

## 2.5	Draft Reply LS on beam switching

The draft reply LS on beam switching is provided in Annex B

Proposal 5-1: Agree on the draft reply LS on beam switching in the Annex B as the official reply from RAN4

# 3	Conclusions

Power class framework

Observation 1: Majority view in the last RAN4 meeting was that the current FR2 UE RF requirements can be used as baseline for 52.6 to 71 GHz discussions. The FR2 power class approach, which captures relevant regulatory limits and aligns on a reference UE type to derive the requirements, can also be reused.

Proposal 1: Reuse FR2 power class framework for the 52.6 to 71 GHz frequency range. This means power classes will continue to be a package of four parameters: minimum peak EIRP, EIRP spherical coverage, maximum TRP and maximum EIRP (regulatory defined, captured in our specs for reference). Values and assumptions to be used for the four parameters are FFS.

Maximum peak EIRP

Observation 2: For fixed devices, regulatory limits for 52.6 to 71 GHz consider the device antenna gain and transmitter location (indoors vs outdoors).

Observation 3: For FCC, the 43 dBm max peak EIRP value used in FR2 can continue to be used for mobile devices up to 71 GHz.

Observation 4: Since regulatory limits for 52.6 to 71 GHz depend on the UE type considered, and antenna gain, it is useful to discuss these as part of the reference UE type description to help determine which regulatory limits to include.

Maximum TRP

Observation 5: The maximum TRP value allowed for 52.6 to 71 GHz is 27 dBm, which means 35 dBm cannot be used for power classes in this frequency range. A lower value for maximum TRP value (e.g. 23 dBm) may be captured pending the outcome of co-existence discussions.

Maximum power spectral density

Observation 6: Considering the significant antenna gain threshold, the limit for power spectral density we should consider is 23 dBm/MHz.

General for all regulatory limits

Proposal 3: RAN4 should discuss relevant regulatory limits and align on which values to capture in power classes. Table 1, Table 2 and Table 3 can be used in our discussions during this meeting.

UE types

Observation 7: For performance related requirements of 52.6 to 71 GHz, RAN4 should first agree on the reference UE type to consider in power class discussions.

Proposal 4: Prioritize handheld and FWA UEs in power class discussions for the 52.6 to 71 GHz frequency range.

Budget derivations and array size

Observation 7: Larger-sized antenna arrays are possible given the smaller wavelength of this frequency range. However, this needs to be further discussed once we agree on a reference UE and are better aligned on applicable regulatory limits.

General guidelines for power class discussions

Proposal 5: We recommend power class discussions in RAN4 follow the steps outlined below:

- Focus on requirements for the agreed unlicensed band: 57 to 71 GHz [2]

- Agree on reference UE - prioritize handheld and FWA UEs

  - Confirm applicable max peak EIRP limit (consider UE type and antenna gain)

  - Confirm applicable max TRP limit (consider co-existence discussion outcome)

- Capture max peak EIRP and TRP parameters in power class definition

  - Updates to some FR2 values are expected

  - Remaining power class parameters are minimum peak EIRP and spherical coverage

- Start budget derivations

  - Discuss array sizes while considering design challenges of the frequency range, and regulatory requirements

  - Provide budgets for different array sizes and assess feasible values

Tx mask and ACLR

Proposal 2-1: Agree on 17 dBc UE ACLR for all supported channel bandwidths in 60 GHz

Transient Period and DL-UL switching time

Observation 3-1: In Rel-15, FR2 5 uS transient period was defined for all three transient periods, i.e., ON-to-OFF, OFF-to-ON, and ON-to-ON.

Observation 3-2: DL/UL switching time determines cell coverage.

Observation 3-3: ON/OFF transient period is the main factor of UL/DL switching time, i.e., UL/DL switching time = ON/OFF transient period + extra time for the rest of configuration.

Observation 3-4: ON/OFF transient period needs to be defined first, to determine UL/DL switching time.

Observation 3-5: ON/ON transient period directly connected to system throughput performance.

Observation 3-6: In FR2, a blanked symbol was introduced for a consecutive power change which is comparable with one OFDM symbol duration with 120 kHz SCS and throughput degradation was limited as 5 uS transient period was comparable with ~7 uS OFDM symbol duration.

Observation 3-7: In 60 GHz, with the existing FR2 transient period, i.e., 5 uS, multiple blanked symbols have to be introduced with 480/960 kHz SCS in 60 GHz which severely degrades system throughput performance.

Observation 3-8: In FR2, 1 symbol guard period was assumed.

Observation 3-9: In 60 GHz, multiple symbols are required for guard period for higher SCS with the same FR2 ON/OFF transient period, which makes additional overhead to the system.

Observation 3-10: ON/OFF transient period has impact on the DL/UL switching time and determines the cell coverage distance as well as DL/UL switching overhead.

Observation 3-11: 1uS ON-OFF and OFF-ON switching time are feasible from implementation perspective.

Observation 3-12:

- Single slot scheduling case:

  - The existing 5 uS transient period cannot provide reliable performance for 16QAM MCS 16 with 480 kHz and 960 kHz SCS (note that ∞ means that there is scenario with certain TP cannot reach 1 % BLER).

  - 3uS transient period shows 13.6 dB performance loss compared to the ideal transient period (0 uS) for the MCS 16 with 960 kHz SCS

  - 2us transient period provides up to 3.5 dB performance loss for MCS 16 with 960 kHz SCS

  - 1us transient period allows < 2 dB performance loss for all considered scenarios

- Multiple PUSCH/PUCCH slot transmissions

  - Comparing the same evaluation condition with the multiple PUSCH/PUCCH slot transmission, the performance get improved with larger number of bundling. For example, for 3 uS transient period for MCS 16 with 960 kHz SCS the performance is 13.6 dB (without bundling)  11.3 dB (with 2 slot bundling)  9.2 dB (with 4 slot bundling)  7.6 dB (with 8 slot bundling).

  - While there could be 6 dB throughput improvement with multiple-slot transmission, we would like to point out that the evaluation is based on optimistic assumption, i.e., there is no transient period between the multiple slots illustrated in figure 4, which may or may not hold true based on the final design. Furthermore, the 7.6 dB loss from the 3 uS transient period with MCS 16 with 960 kHz SCS is still quite large compared to the ideal transient period (0 uS TP).

Observation 3-13: 16 QAM is not possible with FR2 5 uS transient period in 60 GHz and at least 3 uS transient period is required to support 16 QAM in 60 GHz.

Observation 3-14: 64 QAM seems not to be feasible with a single slot scheduling even with 1 uS transient period.

Observation 3-15: 1 uS transient period (both ON/OFF and ON/ON) is feasible and 3 uS DL/UL switching time is feasible.

Proposal 3-1: RAN4 agrees on [1 – 3] uS transient period for both ON/OFF and ON/ON transient period for 52.6 – 71 GHz range.

Proposal 3-2: RAN4 agrees on either [3 – 5] uS for switching time for both DL-to-UL and UL-to-DL.

UE Tx beam switching and Rx beam switching times

Proposal 4-1: 50 nS is UE switching Tx beam and Rx beam times

Draft Reply LS to RAN1 on beam switching

Proposal 5-1: Agree on the draft reply LS on beam switching as the official reply from RAN4

# 4	References

- R4-2105496, “Email discussion summary for [98-bis-e][138] NR_ext_to_71GHz_Part_2,” Qualcomm (moderator), RAN4 #98Bis-e, April 2021

- R4-2105474, “Email discussion summary for [98-bis-e][137] NR_ext_to_71GHz_Part_1_NWM,” Intel (moderator), RAN4 #98Bis-e, April 2021

- FCC 47 CFR § 30.202 - Power limits

- 3GPP TR 38.805, “Study on new radio access technology; 60 GHz unlicensed spectrum,” version 14.0.0

- ETSI EN 302 567, ETSI 303 722, Draft ETSI EN 303 753

- FCC 47 CFR § 15.255 - Operation within the band 57-71 GHz

- 3GPP TS 38.101-2, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone,” version 17.1.0

- FCC 47 CFR § 15.403 - Definitions

- R1-2104061, “LS to RAN4 on maximum UE conducted power and maximum UE EIRP for operation in the 52.6 – 71 GHz band,” RAN1, RAN1 #104Bis-e, April 2021

- R4-1706810, BS demodulation performance implication of UE transient time, RAN4 NR#2 (Ad-hoc), June 2017

- R4-1901398, UE reporting supported transient time parameters, Ericsson, RAN4#90, Feb. 2019

- R4-2015563, On numerology and channel bandwidth in 52.6 – 71 GHz, Intel, RAN4#97-e, Nov. 2020

- R4-2105090, On UE TX requirements for operations up to 71 GHz, Ericcson, RAN4#98bis-e, April 2021

- R1-2102202, LS on beam switching gap for 60 GHz band, Intel, RAN1#104-e

# Annex A: Simulation assumptions

| Transmitter |  |
| --- | --- |
| Carrier frequency | 60 GHz |
| Duplex mode | TDD |
| Antenna configuration | 2x2 |
| SCS + BW | 120 kHz; 480 kHz; 960 kHz with 400 MHz CBW |
| Number of RBs | 120 kHz: 256 480 kHz: 64 960 kHz: 32 |
| CP type | Normal |
| Mapping type | Type B |
| PUSCH allocation | S = 2; D = 12 |
| DMRS configuration | Type1, Single symbol, 1 Additional DMRS |
| MCS | MCS 10,16 (16QAM) |
| Rank | 1 |
| PTRS Configuration | K = 4; L = 1 |
| Propagation conditions |  |
| Channel model | TDLA30-10; |
| PN models | Tx: TR 38.803 UE model Rx: TR 38.803 BS model Correlated noise between Rx chains |
| Receiver |  |
| Channel estimation | Practical |
| PN Estimation/Compensation | Ideal CPE Estimation + Compensation |
| Demodulation | MMSE-IRC |
| Test metric | SNR @ 1% BLER |

Table 9. Evaluation assumptions for throughput impact due to transient periods

# Annex B: Draft reply LS to RAN1 on beam switching gap for 60 GHz band

3GPP TSG RAN WG4 Meeting #99-e						R4-21xxxx

e-Meeting, May 19 – 27, 2021

Title:	Reply LS on beam switching gap for 60 GHz band

Response to:

Release:	Rel-17

Work Item:	NR_ext_to_71GHz

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

#### Name:	Jiwoo Kim

E-mail Address:	jiwoo.kim (at) intel (dot) com,

Attachments:

1. Overall Description:

RAN4 discuss transient period for gNB and UEs operating in 52.6 GHz to 71 GHz. Please find the reply below:

- Switching Tx beams for gNB and UE

RAN4 answer: TR 38.808 subclause captures the outcome of RAN4 study where the order of 50 nS beam switching time is feasible for both Tx and Rx beam switching.

- Switching Rx beams for gNB and UE

RAN4 answer: TR 38.808 subclause captures the outcome of RAN4 study where the order of 50 nS beam switching time is feasible for both Tx and Rx beam switching.

- Switching from DL to UL for UE

RAN4 answer: [3 – 5] uS DL to UL switching time is feasible

- Switching from UL to DL for UE

RAN4 answer: [3 – 5] uS UL to DL switching time is feasible

- Switching from DL to UL for gNB

RAN4 answer: [3 – 4] uS DL to UL switching time is feasible

- Switching from UL to DL for gNB

RAN4 answer: [3 – 4] uS UL to DL switching time is feasible

2. Actions:

To RAN1: 	RAN4 would like to kindly ask RAN1 to provide information on the above questions.

3. Date of Next TSG-RAN WG4 Meetings: