3GPP TSG-RAN4 Meeting #99-e	R4-2110207

# Online, , 19th May 2021 – 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0331 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.141-2: BS PUCCH Format 1 demod test AWGN level error correction (8.3.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is critical error in TS38.141-2 document PUCCH format 1 demodulation test AWGN level table. - FR2 120 kHz SCS setting is missing. Rel -17 aslo has the same error. - For ACK missed detection test (8.3.2.2), for FR2, AWGN level descriptino has error on 200MHz CBW There are different set of error from Rel-15 document. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 8.3.2.1.4.2-1 AWGN level table (for NACK to ACK detection) - BS type 2-O, 120 kHz SCS setting is missing. In Table 8.3.2.2.4.2-2 AWGN level table (for ACK missed detection) - BS type 2-O, 120 kHz SCS setting is missing. - BS type 2-O, AWGN level description are incorrect |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this correction, BS type 2-O PUCCH format 1 demodulation text can not be performed because of incorrect AWGN level description as well as 120 kHz SCS setting is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table 8.3.2.1.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | -83.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | -80.3 – ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | -77.2 – ΔOTAREFSENS dBm / 19.08 MHz |
|  | 30 kHz | 10 | -80.6 – ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | -77.4 – ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | -74.2 – ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | -70.1 – ΔOTAREFSENS dBm / 98.28 MHz |
| BS type 2-O | 60 kHz | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  | 120 kHz | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and clause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

<skip text>

Table 8.3.2.2.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | -83.5 – ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | -80.3 – ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | -77.2 – ΔOTAREFSENS dBm / 19.08 MHz |
|  | 30 kHz | 10 | -80.6 – ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | -77.4 – ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | -74.2 – ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | -70.1 – ΔOTAREFSENS dBm / 98.28 MHz |
| BS type 2-O | 60 kHz | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  | 120 kHz | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz |
|  |  | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
|  |  | 200 | EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and clause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |