3GPP TSG-RAN WG4 Meeting # 99-e	R4-2110349

# Electronic Meeting, May. 19-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7092 | rev | - | Current version: | 13.21.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on RRC re-establishment for NB-IoT R13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core |  |  |  |  |  | Date: |  |  | 2021-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the RRC re-establishment requirements for NB-IoT in enhanced coverage, there is descriptions that “If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch_NB1-EC = 80 ms”. Howerver, according to the conditions of requirements of enhanced coverage shown below, the channel condition can not be better than that of normal coverage where 1400ms is needed for cell searching. Thus, it is impossible to define “sufficient signal quality for first attempt” in enhanced coverage, which is misleading. - The requirements for enhanced coverage shall apply for UE category NB1 provided that the radio condition of its serving cell are: -15 dB NSCH Ês/Iot < -6 dB and -15 dB NRS Ês/Iot < -6 dB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the content “If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch_NB1-EC = 80 ms” in the requirements of enhanced coverage. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are not correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage

The UE re-establishment delay (TUE_re-establish_delay_NB-IoT) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 36.331 [2] is detected by the UE to the time when the UE sends PRACH preamble to the target cell. The UE re-establishment delay (TUE_re-establish_delay_NB-IoT) requirement shall be less than:

TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB1-EC + TSI_NB1-EC + TPRACH_NB-IoT

Tsearch_NB1-EC: It is the time required by the UE to search the target cell:

If the target cell is known, then Tsearch_NB1-EC = 0 ms. Otherwise, Tsearch_NB1-EC = 14800 ms.

TSI_NB1-EC: It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [2] for the target cell for a UE in enhanced coverage.

TPRACH_NB-IoT: The additional delay caused by the random access procedure. The actual value of TPRACH_NB-IoT shall depend upon the NPRACH configuration used in the target cell and the number of repetition used by UE for sending random access to the target cell. There might be additional delay due to ramping procedure.

NNB-Iot-freq: It is the total number of NB-IoT frequencies to be monitored for RRC re-establishment; NNB-Iot-freq = 1 if the target cell is known.

There is no requirement if the target cell does not contain the UE context.

<End of Change 1>