3GPP TSG-RAN WG4 Meeting # 99-e	R4-2110354

# Electronic Meeting, May. 19-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7097 | rev | - | Current version: | 14.18.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on requirements of cell reselection for NB-IoT R14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MediaTek inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core |  |  |  |  |  | Date: |  |  | 2021-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the cell reselection requirements for inter-frequency measurement for NB-IoT, the definition of Pcarrier is not defined and there is redundant content need to be removed. There are also some misalignments between Rel-13 spec and subsequent release where some changes are not implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the defination of Pcarrier - Remove the redundant content - Correct the misalignments. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are not correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage

The UE shall be able to identify new inter-frequency cells and perform NRSRP measurements of identified inter-frequency cells if carrier frequency information is provided by the serving NB-IoT cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev ≤ SnonIntraSearchP then the UE shall search for and measure inter-frequency layers in preparation for possible reselection.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS36.304 within Pcarrier * Tdetect,NB_Inter_NC, if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving NB-IoT cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least Y dB, where Pcarrier is the number of inter-frequency measured and ‘Y’ is specified by Table 4.6.2.6-3. An inter-frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NB-IoT_SCH_RP and SCH Ês/Iot defined in Annex B.1.5 for a corresponding Band.

The UE shall filter NRSRP measurements of each measured inter-frequency cell using at least [2] measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure, Inter_NB-IoT_NC/2.

If an inter-frequency cell has been already detected but that has not been reselected to the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 36.304 within Pcarrier * Tevaluate,NB_Inter_NC. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and inter-frequency cells.

If Treselection timer has a non zero value and the inter-frequency cell is better ranked than the serving NB-IoT cell, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then the UE shall reselect that cell.

For UE not configured with eDRX_IDLE cycle, Tdetect,NB_Inter_NC, Tmeasure,NB_Inter_NC and Tevaluate, NB_inter_NC are specified in Table 4.6.2.5-1. For UE configured with eDRX_IDLE cycle, Tdetect,NB_Inter_NC, Tmeasure,NB_Inter_NC and Tevaluate, NB_inter_NC are specified in Table 4.6.2.5-2, where the requirements apply provided that the serving NB-IoT cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,NB_Inter_NC, Tmeasure,NB_Inter_NC and Tevaluate, NB_inter_NC when multiple PTWs are used.

<End of Change 1>