3GPP TSG-RAN WG4 Meeting #99-e		                    R4-2110420

Online, 19th May – 27th May, 2021

Source: 	Keysight Technologies

Title: 	IAB-MT conformance Test about EVM annex text
Agenda Item:	6.3.2.2.3

Document for:	Agreement

# Introduction

In RAN4#98bis-e IAB-MT conformance test discussion topic, agreement reached on EVM measurement to allow both BS method and UE method. This document provide our review result of draft CR annex text proposed in last meeting.

# Description

In discussion summary [1] following agreement reached as meeting conclusion; (excerpt from [1])

| Issue 1-1: Which EVM equalizer generation method should be used by IAB-MT, BS or UE? | Agreements: Both BS and UE method should be allowed in the specification. |
| --- | --- |
| Issue 1-2: How to adapt “Averaged EVM” clause to IAB-MT | Agreements: For IAB-MT EVM measurement procedure, use two different approaches for BS method and UE method as followings, BS approach: Copy the whole content from TS 38.141 and make the necessary modifications. The spec drafting is discussed in the TP. UE approach: Refer TS 38.521. |
| Issue 1-3: Proposal 2 in R4-2107046 | Agreements: Refer TS 38.521 for IAB-MT EVM measurement when UE procedure is used. The spec drafting is discussed in the TP. |
| Issue 1-4: Proposal 2 in R4-2107225 | Agreements: PTRS declaration for IAB-MT is needed, the exact number can be discussed in the TP for TS 38.176-2. |

As described in agreement, BS method refers to TS38.141 [5][6] and UE method refers to TS 38.521 [3][4]. Because signal processing has some difference between BS method and UE method for EVM calculation, each text in Annex should correctly be referred but not mixed together.

Main point for UE method is to use “E.3.3 Post FFT equalization” and related text which mostly in “E.3 Signal Processing” annex sub-clause of [3][4]. However, these sub-clauses also refer to other part of “Annex E Global In-Channel TX-Test” and are referred by other part of text too. Also, additionally, there are some text which irrelevant for IAB-MT EVM calculation such as calculation for flatness measurement (which not applicable for IAB-MT) exist.

Observation:

Observation of description in Annex E clause in TS38.521 [3][4]

- For allowing UE method, “Annex E in TS38.521” should be referred to.

- There are cross-references inside Annex text. So that it’s not easy to pick relevant portion of text only then entire Annex E should be referred to.

- Because of Annex E is about in-channel Tx test, there is irrelevant text not directly related with EVM calculation too.

- To make it clear what part is applicable for IAB-MT, description should be added about what relevant to calculation of EVM, which is, EVM calculation of CP-OFDM waveform of PUSCH.

# Conclusion

For TS 38.174 annex text regarding with IAB-MT EVM measurement,

Proposal;

- For allowing UE method, “Annex E in TS38.521” should be referred to. However, add description about calculation of EVM on CP-OFDM waveform of PUSCH, is only test and related description applicable for IAB-MT.

- We are OK with draft CR text in previous meeting (R4-2106040)

# References

[1] R4-2106137	Email discussion summary for [98e][303] NR_IAB_RF_Maintenance

[2] R4-2106040	Draft CR for TS 38.174: IAB-MT EVM measurement

[3] TS38.521-1 v16.7.0

[4] TS38.521-2 v16.7.0

[5] TS38.141-1 v16.7.0

[6] TS38.141-2 v16.7.0