3GPP TSG-RAN WG4 Meeting # 99-e                                                             R4-2110456

Electronic Meeting, May. 19-May. 27, 2021

Source:	ZTE Corporation

Title:	TP for 37.717-11-31_DC_8A_n39A-n40A-n79A

Agenda Item:	8.23.2

Document for:	Approval

- Introduction

This contribution provides a text proposal on DC band combination of DC_8A_n39A-n40A-n79A for TR37.717-11-31[1].

- Reference

- TR37.717-11-21, Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL, v0.2.0

- Text Proposal

----- Start of TP -----

- 6.x			DC_8A_n39-n40-n79

- 6.x.1	Operating bands for DC

Table 6.x.1-1: EN-DC band combination (four bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_8_n39-n40-n79 | 8 | CA_n39-n40-n79 |

### 6.x.2	Inter-band DC Configurations

Table 6.x.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A_n39A-n40A-n79A | DC_8A_n39A DC_8A_n40A DC_8A_n79A |

- 6.x.3	Co-existence studies

For DC_8A_n39A-n40A-n79A co-existence studies, the lower order DC_8A_n40A-n79A, DC_8A_n39A-n40A and DC_8A_n39A-n79A can be applied.

- 6.x.4	∆TIB and ∆RIB values

For DC_8A_n40A-n79A-n79A , the TIB,c and RIB,c values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n39-n40-n79 | 8 | 0.3 |
|  | n39 | 0.3 |
|  | n40 | 0.3 |
|  | n79 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n39-n40-n79 | 8 | 0 |
|  | n39 | 0.3 |
|  | n40 | 0.3 |
|  | n79 | 0.5 |

- 6.x.5	MSD

No additional MSD requirement is needed.

----- End of TP -----