3GPP TSG-RAN WG4 Meeting #99-e	R4-2110542

# Electronic Meeting, 19th - 27th May, 2021

Title: 	Updated simulation assumptions for NR IAB-MT demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	6.3.6.3

Document for:	Approval

Introduction

This contribution provides the simulation assumption for demodulation performance requirements for NR IAB-MT PDSCH and PDCCH.

PDSCH

For PDSCH, total 13 cases need to be introduced and 6 cases need to be re-simulated as per current TS 38.101-4 and RAN4 Email discussion. The overview of the workload and the simulation assumption on NR IAB PDSCH is shown in Clause 2-1 and 2-2 respectively as following.

Overview of the workload

In following Table 2.1-1 we list all case need to be introduced for IAB-MT PDSCH and identify whether to be re-simulated or not.

Table 2.1-1 Overview of the workload on NR IAB PDSCH

| Clause in TS 38.101-4 | Test number in TS 38.101-4 to introduce | Whether to re-simulate | Mapping to the test number in this paper |
| --- | --- | --- | --- |
| 5.2.2.2.1 | 1-3 | N |  |
|  | 1-4 | Y | 1 |
|  | 2-1 | N |  |
| 5.2.3.2.1 | 1-3 | N |  |
|  | 1-4 | Y | 2 |
|  | 2-1 | N |  |
|  | 3-1 | Y | 3 |
|  | 4-1 | N |  |
| 7.2.2.2.1 | 1-2 | Y | 4 |
|  | 1-3 | Y | 5 |
|  | 2-2 | Y | 6 |
|  | 2-5 | N |  |
|  | 2-6 | N |  |

Simulation assumption

- HARQ RV sequence: {0, 2, 3, 1} for all the modulation orders

- Max number of HARQ transmission: 4

- PDSCH scheduling

  - Skip scheduling PDSCH

    - slot#0 per 20ms

    - slot#20, 21 for FR1 case 1/2/3

    - slot#80, 81 for FR2 case 4/5/6

  - PDSCH mapping type: Type A

  - PRB bundling: 2

  - Full PRB allocation

  - Time domain mapping

    - Start symbol (S): 2 for FR1 case 1/2/3

    - Start symbol (S): 1 for FR2 case 4/5/6

  - Precoding model: Random Precoding, per slot , PRB bundling granularity(codebook configuration Single panel Type I)

- DMRS

  - DMRS Type: Type 1 single symbol front loaded

  - Number of additional DMRS: 1

  - FDM is applied between DMRS and data for test cases with 1 Layer and 2 Layers

- PTRS

  - Not configured for FR1 case 1/2/3

  - Kptrs = 2, Lptrs = 1 for FR2 case 4/5/6

- Overhead for TBS determination

  - 0 for FR1 case 1/2/3

  - 6 for FR2 case 4/5/6

- Impairments modeling for alignment purpose: TX EVM = 6%

- Receiver assumption:

  - MMSE-IRC

Table 2.2-1 Simulation assumption on NR IAB PDSCH

| Test | CBW(MHz)/SCS(KHz) | Modulation and code rate | Rank | TDD UL-DL pattern | Propagation condition | Antenna configuration | Fraction of maximum throughput (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40/30 | 16QAM, 0.48 | 1 | FR1.30-1 (7DS2U,S:6+4+4) | TDLA30-10 | 2x2 ULA Low | 30 |
| 2 | 40/30 | 16QAM, 0.48 | 1 | FR1.30-1 (7DS2U,S:6+4+4) | TDLA30-10 | 2x4 ULA Low | 30 |
| 3 | 40/30 | 16QAM, 0.48 | 3 | FR1.30-1 (7DS2U,S:6+4+4) | TDLA30-10 | 4x4, ULA Low | 70 |
| 4 | 100/120 | 16QAM, 0.48 | 1 | FR2.120-1 (DDDSU,S:10+2+2) | TDLA30-75 | 2x2 ULA Low | 30 |
| 5 | 100/120 | 64QAM, 0.46 | 1 | FR2.120-1 (DDDSU,S:10+2+2) | TDLA30-75 | 2x2 ULA Low | 70 |
| 6 | 100/120 | 16QAM, 0.48 | 2 | FR2.120-1 (DDDSU,S:10+2+2) | TDLA30-75 | 2x2 ULA Low | 70 |

The FRC definition by following the BS approach defined in TS 38.104/141-1/2 that only consider the scheduled ‘D’ slots.

Table 2.2-2 FRC on NR IAB PDSCH

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 | Test 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 40 | 40 | 40 | 100 | 100 | 100 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 120 | 120 | 120 |
| Allocated resource blocks |  | 106 | 106 | 106 | 66 | 66 | 66 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 13 | 13 | 13 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 13 | 13 | 18 | 13 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 64QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 | 0.46 | 0.48 |
| Number of MIMO layers |  | 1 | 1 | 3 | 1 | 1 | 2 |
| Number of DMRS REs |  | 12 | 12 | 24 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 6 | 6 | 6 |
| Information Bit Payload per Slot | Bits | 26632 | 26632 | 73776 | 17424 | 25104 | 34816 |
| Transport block CRC per Slot | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  | 4 | 4 | 9 | 3 | 3 | 5 |
| Binary Channel Bits Per Slot | Bits | 55968 | 55968 | 152640 | 36564 | 54846 | 73128 |

PDCCH

For PDCCH, total 9 cases need to be introduced and total 5 cases need to be re-simulated as per current TS 38.101-4 and RAN4 Email discussion. The overview of the workload and the simulation assumption on NR IAB PDCCH is shown in Clause 3-1 and 3-2 respectively as following.

Overview of the workload

In following Table 3.1-1 we list all case need to be introduced for IAB-MT PDCCH and identify whether to be re-simulated or not.

Table 3.1-1 Overview of the workload on NR IAB PDCCH

| Clause in TS 38.101-4 | Test number in TS 38.101-4 to introduce | Whether to re-simulate | Mapping to the test number in this paper |
| --- | --- | --- | --- |
| 5.3.2.2.1 | 1 | N |  |
|  | 2 | Y | 1 |
| 5.3.2.2.2 | 1 | Y | 2 |
| 5.3.3.2.1 | 1 | N |  |
|  | 2 | Y | 3 |
| 5.3.3.2.2 | 1 | Y | 4 |
| 7.3.2.2.1 | 1-1 | N |  |
|  | 1-2 | Y | 5 |
| 7.3.2.2.2 | 2-1 | N |  |

Simulation assumption

Table 3.2-1 Simulation assumption on NR IAB PDCCH

| Test | CBW(MHz)/SCS(KHz) | CORESET RB | CORESET duration | Aggregation level | DCI format | Payload(without CRC) | TDD UL-DL pattern | CCE-to-REG Mapping | Interleaver size | REG bundle size | Propagation condition | Antenna configuration | Pm-dsg(%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40/30 | 102 | 1 | 4 | 1_1 | 53 | FR1.30-1 (7DS2U,S:6+4+4) | Interleaved | 3 | 2 | TDLA30-10 | 1x2 Low | 1 |
| 2 | 40/30 | 90 | 1 | 8 | 1_1 | 53 | FR1.30-1 (7DS2U,S:6+4+4) | Interleaved | 3 | 6 | TDLA30-10 | 2x2 Low | 1 |
| 3 | 40/30 | 102 | 1 | 4 | 1_1 | 53 | FR1.30-1 (7DS2U,S:6+4+4) | Interleaved | 3 | 2 | TDLA30-10 | 1x4 Low | 1 |
| 4 | 40/30 | 90 | 1 | 8 | 1_1 | 53 | FR1.30-1 (7DS2U,S:6+4+4) | Interleaved | 3 | 6 | TDLA30-10 | 2x4 Low | 1 |
| 5 | 100/120 | 60 | 1 | 4 | 1_1 | 56 | FR2.120-1 (DDDSU,S:10+2+2) | Interleaved | 2 | 6 | TDLA30-75 | 1x2 Low | 1 |

Table 3.2-2 FRC on NR IAB PDCCH

| Parameter | Unit | Test 1 | Test 2 | Test 3 | Test 4 | Test 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 120 |
| CORESET frequency domain allocation |  | 102 | 90 | 102 | 90 | 60 |
| CORESET time domain allocation |  | 1 | 1 | 1 | 1 | 1 |
| Aggregation level |  | 4 | 8 | 4 | 8 | 4 |
| DCI Format |  | 1_1 | 1_1 | 1_1 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 53 | 53 | 53 | 53 | 56 |

Conclusion

For PDSCH and PDCCH, total 22 cases need to be introduced and total 11 cases need to be re-simulated as per current TS 38.101-4 and RAN4 Email discussion. Therefore, the workload can be acceptable.

Reference

R4-2103994, WF on Rel-16 NR IAB demodulation requirements, RAN4#98-e, Nokia

R4-2103979, Simulation assumptions for demodulation performance requirements for IAB-MT, RAN4#98-e, Huawei, HiSilicon

R4-2106172, WF on Rel-16 NR IAB demodulation requirements, RAN4#98-bis-e, Nokia

R4-2106815, Updated simulation assumptions for NR IAB-MT demodulation requirements, RAN4#98-bis-e, Huawei, HiSilicon