3GPP TSG-RAN4 Meeting #99-e	R4-2110577

# Online, , 19th May - 27th May 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0580 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for missing delta T & delta R of EN-DC with intra-band non-contiguous LTE CA combos in Rel.16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CHTTL, SGS Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R16_1BLTE_1BNR_2DL2UL-Core |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some missing delta T & delta R values for EN-DC with intra-band non-contiguous LTE CA combos on LTE part are missing in Rel.16 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the following EN-DC configuration to the delta T table: DC_1-1_n7, DC_2-2_n5, DC_2-2_n38, DC_2-2_n41, DC_2-2_n66, DC_2-2_n71, DC_2-2_n78, DC_3-3_n7, DC_7-7_n5, DC_66-66_n2, DC_66-66_n7, DC_66-66_n38, DC_66-66_n71, DC_66-66_n78 Adding the following EN-DC configuration to the delta R table: DC_2-2_n66, DC_2-2_n78, DC_66-66_n2, DC_66-66_n7, DC_66-66_n38, DC_66-66_n78 Correction: DC_28A_n51 DC_28_n51 DC_39-n41 DC_39_n41 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Above delta T & delta R requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.4.2.3, 7.3B.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Related Clauses for reference>>

5.5B.4	Inter-band EN-DC within FR1

5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration |  | Uplink EN-DC configuration (NOTE 1) |  | Single UL allowed |  | DL interruption allowed (Note 14) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n3A DC_1C_n3A |  | DC_1A_n3A DC_1C_n3A |  | DC_1_n3 |  |  |  |
| DC_1A_n5A |  | DC_1A_n5A |  | No |  |  |  |
| DC_1A_n7A DC_1A_n7B |  | DC_1A_n7A |  | No |  |  |  |
| DC_1A-1A_n7A DC_1A-1A_n7B |  | DC_1A_n7A |  | No |  |  |  |
| DC_1A_n8A |  | DC_1A_n8A |  | No |  |  |  |
| DC_1A_n20A |  | DC_1A_n20A |  | No |  |  |  |
| DC_1A_n28A |  | DC_1A_n28A |  | No |  |  |  |
| DC_1A-1A_n28A |  | DC_1A_n28A |  | No |  |  |  |
| DC_1A_n38A DC_1C_n38A |  | DC_1A_n38A |  | No |  |  |  |
| DC_1A_n40A DC_1A_n40B |  | DC_1A_n40A |  | No |  |  |  |
| DC_1A_n41A |  | DC_1A_n41A |  | No |  |  |  |
| DC_1A_n50A |  | DC_1A_n50A |  | No |  |  |  |
| DC_1A_n51A |  | DC_1A_n51A |  | No |  |  |  |
| DC_1A_n71A DC_1A_n71B |  | DC_1A_n71A |  | No |  |  |  |
| DC_1A_n77A7 DC_1A_n77C7 |  | DC_1A_n77A |  | DC_1_n77 |  | No |  |
| DC_1A_n77(2A) |  | DC_1A_n77A |  | DC_1_n77 |  | No |  |
| DC_1A_n78A7 DC_1A_n78C7 |  | DC_1A_n78A |  | No |  | No |  |
| DC_1A_n78(2A)7 DC_1A-1A_n78A |  | DC_1A_n78A |  | No |  | No |  |
| DC_1A_n79A7 DC_1A_n79C7 |  | DC_1A_n79A |  | No |  | No |  |
| DC_2A_n5A |  | DC_2A_n5A |  | No |  |  |  |
| DC_2A-2A_n5A |  | DC_2A_n5A |  | No |  |  |  |
| DC_2A_n7A |  | DC_2A_n7A |  | No |  |  |  |
| DC_2A_n7(2A) |  | DC_2A_n7A |  | No |  |  |  |
| DC_2A_n12A |  | DC_2A_n12A |  | No |  |  |  |
| DC_2A_n28A |  | DC_2A_n28A |  | No |  |  |  |
| DC_2A_n38A |  | DC_2A_n38A |  | No |  |  |  |
| DC_2A-2A_n38A |  | DC_2A_n38A |  | No |  |  |  |
| DC_2A_n41A DC_2A_n41C DC_2C_n41A |  | DC_2A_n41A DC_2C_n41A |  | No |  |  |  |
| DC_2A-2A_n41A DC_2A_n41(2A) |  | DC_2A_n41A |  | No |  |  |  |
| DC_2A_n46A |  | DC_2A_n46A |  | No |  |  |  |
| DC_2A_n48A DC_2A_n48B |  | DC_2A_n48A |  | No |  |  |  |
| DC_2A_n66A DC_2A_n66(2A) |  | DC_2A_n66A |  | DC_2_n66 |  |  |  |
| DC_2A-2A_n66A |  | DC_2A_n66A |  | DC_2_n66 |  |  |  |
| DC_2A_n71A DC_2A_n71B DC_2C_n71A |  | DC_2A_n71A DC_2C_n71A |  | No |  |  |  |
| DC_2A-2A_n71A |  | DC_2A_n71A |  | No |  |  |  |
| DC_2A_n77A |  | DC_2A_n77A |  | DC_2_n77 |  |  |  |
| DC_2A-2A_n77A |  | DC_2A_n77A |  | DC_2_n77 |  |  |  |
| DC_2A_n78A |  | DC_2A_n78A |  | DC_2_n78 |  |  |  |
| DC_2A_n78(2A) |  | DC_2A_n78A |  | DC_2_n78 |  |  |  |
| DC_2A-2A_n78A |  | DC_2A_n78A |  | DC_2_n78 |  |  |  |
| DC_3A_n1A DC_3C_n1A |  | DC_3A_n1A DC_3C_n1A |  | DC_3_n1 |  |  |  |
| DC_3A-3A_n1A |  | DC_3A_n1A |  | DC_3_n1 |  |  |  |
| DC_3A_n5A DC_3C_n5A |  | DC_3A_n5A DC_3C_n5A |  | DC_3_n5 |  |  |  |
| DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B |  | DC_3A_n7A DC_3A_n7B DC_3C_n7A |  | No |  |  |  |
| DC_3A-3A_n7A DC_3A-3A_n7B |  | DC_3A_n7A |  | No |  |  |  |
| DC_3A_n8A |  | DC_3A_n8A |  | No |  |  |  |
| DC_3A_n20A |  | DC_3A_n20A |  | No |  |  |  |
| DC_3A_n28A DC_3C_n28A |  | DC_3A_n28A DC_3C_n28A |  | No |  |  |  |
| DC_3A_n34A |  | DC_3A_n34A |  | No |  |  |  |
| DC_3A_n38A DC_3C_n38A |  | DC_3A_n38A |  | No |  |  |  |
| DC_3A_n40A DC_3A_n40B |  | DC_3A_n40A |  | No |  |  |  |
| DC_3A_n41A DC_3C_n41A |  | DC_3A_n41A DC_3C_n41A |  | DC_3_n41 |  | No |  |
| DC_3A_n50A |  | DC_3A_n50A |  | No |  |  |  |
| DC_3A_n51A |  | DC_3A_n51A |  | No |  |  |  |
| DC_3A_n77A7 DC_3A_n77C7 DC_3C_n77A7 |  | DC_3A_n77A DC_3C_n77A |  | No |  |  |  |
| DC_3A_n77(2A)7 DC_3C_n77(2A)7 |  | DC_3A_n77A DC_3C_n77A |  | DC_3_n77 |  | No |  |
| DC_3A-3A_n77A |  | DC_3A_n77A |  | DC_3_n77 |  | No |  |
| DC_3A_n78A7 DC_3A_n78C7 DC_3C_n78A7 |  | DC_3A_n78A DC_3C_n78A |  | DC_3_n77 |  | No |  |
| DC_3A_n77A7 DC_3A_n77C7 DC_3C_n77A7 |  | DC_3A_n77A DC_3C_n77A |  | DC_3_n78 |  | No |  |
| DC_3A_n78(2A)7 DC_3C_n78(2A)7 |  | DC_3A_n78A |  | DC_3_n78 |  | No |  |
| DC_3A-3A_n78A |  | DC_3A_n78A |  | DC_3_n78 |  | No |  |
| DC_3A_n79A7 DC_3A_n79C7 DC_3C_n79A7 |  | DC_3A_n79A DC_3C_n79A |  | No |  | No |  |
| DC_4A_n2A |  | DC_4A_n2A |  | No |  |  |  |
| DC_4A_n5A |  | DC_4A_n5A |  | DC_4_n5 |  |  |  |
| DC_4A_n7A |  | DC_4A_n7A |  | No |  |  |  |
| DC_4A_n28A |  | DC_4A_n28A |  | No |  |  |  |
| DC_4A_n38A |  | DC_4A_n38A |  | No |  |  |  |
| DC_4A_n41A |  | DC_4A_n41A |  | No |  |  |  |
| DC_4A_n78A |  | DC_4A_n78A |  | No |  |  |  |
| DC_4A_n78(2A) |  | DC_4A_n78A |  | No |  |  |  |
| DC_5A_n2A DC_5B_n2A |  | DC_5A_n2A |  | No |  |  |  |
| DC_5A-5A_n2A |  | DC_5A_n2A |  | No |  |  |  |
| DC_5A_n7A |  | DC_5A_n7A |  | DC_5_n7 |  |  |  |
| DC_5A_n7(2A) |  | DC_5A_n7A |  | DC_5_n7 |  |  |  |
| DC_5A_n12A |  | DC_5A_n12A |  | No |  |  |  |
| DC_5A_n38A |  | DC_5A_n38A |  | DC_5_n38 |  |  |  |
| DC_5A_n40A |  | DC_5A_n40A |  | No |  |  |  |
| DC_5A_n48A DC_5A_n48B |  | DC_5A_n48A |  | No |  |  |  |
| DC_5A_n66A DC_5B_n66A |  | DC_5A_n66A |  | DC_5_n66 |  |  |  |
| DC_5A-5A_n66A |  | DC_5A_n66A |  | DC_5_n66 |  |  |  |
| DC_5A_n77A |  | DC_5A_n77A |  | No |  |  |  |
| DC_5A_n71A |  | DC_5A_n71A |  | No |  |  |  |
| DC_5A_n78A7 DC_5A_n78C7 |  | DC_5A_n78A |  | No |  | No |  |
| DC_5A_n78(2A)7 |  | DC_5A_n78A |  | No |  | No |  |
| DC_5A_n79A |  | DC_5A_n79A |  | No |  | No |  |
| DC_7A_n1A DC_7C_n1A |  | DC_7A_n1A DC_7C_n1A |  | No |  |  |  |
| DC_7A-7A_n1A |  | DC_7A_n1A |  | No |  |  |  |
| DC_7A_n2A DC_7C_n2A |  | DC_7A_n2A |  | No |  |  |  |
| DC_7A_n3A DC_7C_n3A |  | DC_7A_n3A DC_7C_n3A |  | No |  |  |  |
| DC_7A_n5A DC_7C_n5A |  | DC_7A_n5A DC_7C_n5A |  | DC_7_n5 |  |  |  |
| DC_7A-7A_n5A |  | DC_7A_n5A |  | DC_7_n5 |  |  |  |
| DC_7A_n8A |  | DC_7A_n8A |  | No |  |  |  |
| DC_7A-7A_n78A7 DC_7A-7A_n78C7 |  | DC_7A_n78A |  | No |  |  |  |
| DC_7A-7A_n78(2A)7 |  | DC_7A_n78A |  | No |  |  |  |
| DC_7A_n20A |  | DC_7A_n20A |  | No |  |  |  |
| DC_7A_n28A DC_7C_n28A |  | DC_7A_n28A DC_7C_n28A |  | No |  |  |  |
| DC_7A_n40A |  | DC_7A_n40A |  | Yes |  |  |  |
| DC_7A_n51A |  | DC_7A_n51A |  | No |  |  |  |
| DC_7A_n66A DC_7C_n66A |  | DC_7A_n66A |  | No |  |  |  |
| DC_7A-7A_n66A |  | DC_7A_n66A |  | No |  |  |  |
| DC_7A_n71A |  | DC_7A_n71A |  | No |  |  |  |
| DC_7A_n77A DC_7A_n77(2A) DC_7C_n77A DC_7C_n77(2A) |  | DC_7A_n77A |  | No |  |  |  |
| DC_7A-7A_n77A DC_7A-7A_n77(2A) |  | DC_7A_n77A |  | No |  |  |  |
| DC_7A_n78A7 DC_7C_n78A7 DC_7A_n78C7 |  | DC_7A_n78A DC_7C_n78A |  | No |  |  |  |
| DC_7A_n78(2A)7 DC_7C_n78(2A)7 |  | DC_7A_n78A DC_7C_n78A |  | No |  |  |  |
| DC_8A_n1A |  | DC_8A_n1A |  | No |  |  |  |
| DC_8A_n2A |  | DC_8A_n2A |  | DC_8_n2 |  |  |  |
| DC_8A_n3A |  | DC_8A_n3A |  | No |  |  |  |
| DC_8A_n7A |  | DC_8A_n7A |  | No |  |  |  |
| DC_8A_n20A |  | DC_8A_n20A |  | Yes |  |  |  |
| DC_8A_n28A |  | DC_8A_n28A |  | No |  |  |  |
| DC_8A_n34A |  | DC_8A_n34A |  | No |  |  |  |
| DC_8A_n39A |  | DC_8A_n39A |  | No |  |  |  |
| DC_8A_n40A7 |  | DC_8A_n40A |  | No |  |  |  |
| DC_8A_n41A DC_8A_n41C |  | DC_8A_n41A |  | No |  | No |  |
| DC_8A_n41(2A) |  | DC_8A_n41A |  | No |  | No |  |
| DC_8A_n77A7 |  | DC_8A_n77A |  | No |  | No |  |
| DC_8A_n77(2A)7 |  | DC_8A_n77A |  | No |  | No |  |
| DC_8A_n78A7 |  | DC_8A_n78A |  | No |  | No |  |
| DC_8A_n78(2A)7 |  | DC_8A_n78A |  | No |  | No |  |
| DC_8A_n79A7 DC_8A_n79C |  | DC_8A_n79A DC_8A_n79C |  | No |  | No |  |
| DC_8A_n93A |  | DC_8A_n93A_ULSUP-TDM |  | N/A |  |  |  |
| DC_8A_n94A |  | DC_8A_n94A_ULSUP-TDM |  | N/A |  |  |  |
| DC_11A_n3A |  | DC_11A_n3A |  | No |  |  |  |
| DC_11A_n28A |  | DC_11A_n28A |  | No |  |  |  |
| DC_11A_n77A7 |  | DC_11A_n77A |  | No |  | No |  |
| DC_11A_n77(2A)7 |  | DC_11A_n77A |  | No |  | No |  |
| DC_11A_n78A7 |  | DC_11A_n78A |  | No |  | No |  |
| DC_11A_n79A7 |  | DC_11A_n79A |  | No |  |  |  |
| DC_12A_n2A |  | DC_12A_n2A |  | No |  |  |  |
| DC_12A_n5A |  | DC_12A_n5A |  | No |  |  |  |
| DC_12A_n7A DC_12A_n7(2A) |  | DC_12A_n7A |  | No |  |  |  |
| DC_12A_n25A |  | DC_12A_n25A |  | No |  |  |  |
| DC_12A_n38A |  | DC_12A_n38A |  | No |  |  |  |
| DC_12A_n41A |  | DC_12A_n41A |  | No |  |  |  |
| DC_12A_n66A DC_12A_n66(2A) |  | DC_12A_n66A |  | No |  |  |  |
| DC_12A_n71A |  | DC_12A_n71A18,19 |  | DC_12_n71 |  |  |  |
| DC_12A_n78A DC_12A_n78(2A) |  | DC_12A_n78A |  | DC_12_n78 |  |  |  |
| DC_13A_n2A |  | DC_13A_n2A |  | No |  |  |  |
| DC_13A_n5A |  | DC_13A_n5A |  | DC_13_n5 |  |  |  |
| DC_13A_n7A DC_13A_n7(2A) |  | DC_13A_n7A |  | No |  |  |  |
| DC_13A_n48A DC_13A_n48B |  | DC_13A_n48A |  | No |  |  |  |
| DC_13A_n66A |  | DC_13A_n66A |  | No |  |  |  |
| DC_13A_n71A |  | DC_13A_n71A |  | No |  |  |  |
| DC_13A_n77A |  | DC_13A_n77A |  | No |  |  |  |
| DC_13A_n78A DC_13A_n78(2A) |  | DC_13A_n78A |  | No |  |  |  |
| DC_14A_n2A |  | DC_14A_n2A |  | No |  |  |  |
| DC_14A_n66A |  | DC_14A_n66A |  | No |  |  |  |
| DC_18A_n3A |  | DC_18A_n3A |  | No |  |  |  |
| DC_18A_n28A8 |  | DC_18A_n28A |  | No |  |  |  |
| DC_18A_n41A16 |  | DC_18A_n41A |  | No |  |  |  |
| DC_18A_n77A7 DC_18A_n77(2A)7 |  | DC_18A_n77A |  | No |  | No |  |
| DC_18A_n78A7 DC_18A_n78(2A)7 |  | DC_18A_n78A |  | No |  | No |  |
| DC_20A_n91A |  | DC_20A_n91A_ULSUP-TDM |  | N/A |  |  |  |
| DC_20A_n92A |  | DC_20A_n92A_ULSUP-TDM |  | N/A |  |  |  |
| DC_18A_n79A7 |  | DC_18A_n79A |  | No |  |  |  |
| DC_19A_n1A |  | DC_19A_n1A |  | No |  |  |  |
| DC_19A_n77A7 DC_19A_n77C7 |  | DC_19A_n77A |  | No |  |  |  |
| DC_19A_n78A7 DC_19A_n78C7 |  | DC_19A_n78A |  | No |  | No |  |
| DC_19A_n79A7 DC_19A_n79C7 |  | DC_19A_n79A |  | No |  | No |  |
| DC_20A_n1A |  | DC_20A_n1A |  | No |  |  |  |
| DC_20A_n3A |  | DC_20A_n3A |  | No |  |  |  |
| DC_20A_n7A |  | DC_20A_n7A |  | DC_20_n7 |  |  |  |
| DC_20A_n8A |  | DC_20A_n8A |  | DC_20_n8 |  |  |  |
| DC_20A_n28A8, 11,13 |  | DC_20A_n28A |  | No |  |  |  |
| DC_20A_n38A |  | DC_20A_n38A |  | No |  |  |  |
| DC_20A_n41A |  | DC_20A_n41A |  | DC_20_n41 |  |  |  |
| DC_20A_n50A |  | DC_20A_n50A |  | No |  |  |  |
| DC_20A_n51A |  | DC_20A_n51A |  | No |  |  |  |
| DC_20A_n77A7 |  | DC_20A_n77A |  | No |  |  |  |
| DC_20A_n78A7 |  | DC_20A_n78A |  | No |  |  |  |
| DC_20A_n78(2A)7 |  | DC_20A_n78A |  | No |  |  |  |
| DC_21A_n1A |  | DC_21A_n1A |  | No |  |  |  |
| DC_21A_n28A17 |  | DC_21A_n28A |  | DC_21_n28 |  |  |  |
| DC_21A_n77A7 DC_21A_n77C7 |  | DC_21A_n77A |  | No |  |  |  |
| DC_21A_n78A7 DC_21A_n78C7 |  | DC_21A_n78A |  | No |  | No |  |
| DC_21A_n79A7 DC_21A_n79C7 |  | DC_21A_n79A |  | No |  | No |  |
| DC_25A_n41A |  | DC_25A_n41A |  | No |  |  |  |
| DC_25A-25A_n41A |  | DC_25A_n41A |  | No |  |  |  |
| DC_25A_n77A |  | DC_25A_n77A |  | DC_25_n77 |  |  |  |
| DC_25A-25A_n77A |  | DC_25A_n77A |  | DC_25_n77 |  |  |  |
| DC_25A_n78A |  | DC_25A_n78A |  | DC_25_n78 |  |  |  |
| DC_25A-25A_n78A |  | DC_25A_n78A |  | DC_25_n78 |  |  |  |
| DC_26A_n25A |  | DC_26A_n25A |  | No |  |  |  |
| DC_26A_n41A |  | DC_26A_n41A |  | No |  |  |  |
| DC_26A_n77A7 |  | DC_26A_n77A |  | No |  |  |  |
| DC_26A_n78A7 |  | DC_26A_n78A |  | No |  |  |  |
| DC_26A_n79A7 |  | DC_26A_n79A |  | No |  |  |  |
| DC_28A_n1A |  | DC_28A_n1A |  | No |  |  |  |
| DC_28A_n2A |  | DC_28A_n2A |  | No |  |  |  |
| DC_28A_n3A |  | DC_28A_n3A |  | No |  |  |  |
| DC_28A_n5A8 |  | DC_28A_n5A |  | No |  |  |  |
| DC_28A_n7A DC_28A_n7B |  | DC_28A_n7A DC_28A_n7B |  | No |  |  |  |
| DC_28A_n51A |  | DC_28A_n51A |  | No |  |  |  |
| DC_28A_n8A |  | DC_28A_n8A |  | No |  |  |  |
| DC_28A_n40A |  | DC_28A_n40A |  | No |  |  |  |
| DC_28A_n41A |  | DC_28A_n41A |  | No |  |  |  |
| DC_28A_n50A |  | DC_28A_n50A |  | No |  |  |  |
| DC_28A_n66A |  | DC_28A_n66A |  | No |  |  |  |
| DC_28A_n77A7 DC_28A_n77C7 |  | DC_28A_n77A |  | No |  | No |  |
| DC_28A_n77(2A)7 |  | DC_28A_n77A |  | No |  | No |  |
| DC_28A_n78A7 DC_28A_n78C7 |  | DC_28A_n78A |  | No |  | No |  |
| DC_28A_n78(2A) |  | DC_28A_n78A |  | No |  | No |  |
| DC_28A_n79A7 DC_28A_n79C7 |  | DC_28A_n79A |  | No |  |  |  |
| DC_30A_n2A |  | DC_30A_n2A |  | No |  |  |  |
| DC_30A_n5A |  | DC_30A_n5A |  | No |  |  |  |
| DC_30A_n66A |  | DC_30A_n66A |  | No |  |  |  |
| DC_38A_n78A7 |  | DC_38A_n78A |  | No |  |  |  |
| DC_39A_n40A3 |  | DC_39A_n40A |  | No |  |  |  |
| DC_39A_n41A3 DC_39C_n41A3 |  | DC_39A_n41A DC_39C_n41A |  | No |  | No |  |
| DC_39A_n78A5,7 |  | DC_39A_n78A |  | No |  |  |  |
| DC_39A_n79A7 DC_39A_n79C7 |  | DC_39A_n79A |  | No |  | No |  |
| DC_40A_n1A DC_40C_n1A |  | DC_40A_n1A |  | No |  |  |  |
| DC_40A_n41A3 DC_40A_n41C3 DC_40C_n41A3 |  | DC_40A_n41A |  | No |  |  |  |
| DC_40A_n41(2A)3 |  | DC_40A_n41A |  | No |  |  |  |
| DC_40A_n77A |  | DC_40A_n77A |  | No |  |  |  |
| DC_40A_n78A DC_40C_n78A |  | DC_40A_n78A DC_40C_n78A |  | No |  |  |  |
| DC_40A_n78(2A) DC_40C_n78(2A) |  | DC_40A_n78A DC_40C_n78A |  | No |  |  |  |
| DC_40A_n79A7,12 DC_40A_n79C7,12 DC_40C_n79A7,12 |  | DC_40A_n79A |  | No |  | No |  |
| DC_41A_n3A DC_41C_n3A |  | DC_41A_n3A DC_41C_n3A |  | No |  |  |  |
| DC_41A_n28A DC_41C_n28A |  | DC_41A_n28A DC_41C_n28A |  | No |  |  |  |
| DC_41A_n77A DC_41C_n77A |  | DC_41A_n77A DC_41C_n77A |  | No |  |  |  |
| DC_41A_n77(2A) DC_41C_n77(2A) |  | DC_41A_n77A DC_41C_n77A |  | No |  |  |  |
| DC_41A_n78A DC_41C_n78A DC_41D_n78A |  | DC_41A_n78A DC_41C_n78A |  | No |  |  |  |
| DC_41A_n78(2A) DC_41C_n78(2A) |  | DC_41A_n78A DC_41C_n78A |  | No |  |  |  |
| DC_41A_n79A6,7 DC_41A_n79C6,7 DC_41C_n79A6,7 |  | DC_41A_n79A DC_41C_n79A |  | No |  | No |  |
| DC_42A_n1A DC_42C_n1A |  | DC_42A_n1A |  | No |  |  |  |
| DC_42A_n3A DC_42C_n3A |  | DC_42A_n3A DC_42C_n3A |  | DC_42_n3 |  |  |  |
| DC_42A_n28A DC_42C_n28A |  | DC_42A_n28A DC_42C_n28A |  | No |  |  |  |
| DC_42A_n51A |  | DC_42A_n51A |  | No |  |  |  |
| DC_42A_n77A3,4,9,11 DC_42A_n77C3,4,9,11 DC_42C_n77A3,4,9,11 DC_42C_n77C3,4,9,11 DC_42D_n77A3,4,9,11 DC_42D_n77C DC_42E_n77A3,4,9,11 DC_42E_n77C |  | N/A |  | N/A |  |  |  |
| DC_42A_n77(2A)3,4,9,11 DC_42C_n77(2A)3,4,9,11 |  | N/A |  | N/A |  |  |  |
| DC_42A_n78A3,4,9,11 DC_42A_n78C3,4,9,11 DC_42C_n78A3,4,9,11 DC_42C_n78C3,4,9,11 DC_42D_n78A3,4,9,11 DC_42D_n78C DC_42E_n78A3,4,9,11 DC_42E_n78C |  | N/A |  | N/A |  |  |  |
| DC_42A_n79A9,15 DC_42A_n79C9,15 DC_42C_n79A9,15 DC_42C_n79C9,15 DC_42D_n79A9,15 DC_42D_n79C9,15 DC_42E_n79A9,15 DC_42E_n79C9,15 |  | N/A |  | N/A |  |  |  |
| DC_46A_n77A2 |  | N/A |  | N/A |  |  |  |
| DC_46A_n78A2 DC_46C_n78A2 DC_46D_n78A2 DC_46E_n78A2 |  | N/A |  | N/A |  |  |  |
| DC_48A_n5A |  | DC_48A_n5A |  | No |  |  |  |
| DC_48A_n12A |  | DC_48A_n12A |  | No |  |  |  |
| DC_48A_n46A DC_48B_n46A DC_48C_n46A DC_48D_n46A DC_48E_n46A DC_48A_n46B DC_48B_n46B DC_48C_n46B DC_48D_n46B DC_48E_n46B DC_48A_n46C DC_48B_n46C DC_48C_n46C DC_48D_n46C DC_48E_n46C DC_48A_n46D DC_48B_n46D DC_48C_n46D DC_48D_n46D DC_48E_n46D DC_48A_n46E DC_48B_n46E DC_48C_n46E DC_48D_n46E DC_48E_n46E |  | DC_48A_n46A DC_48B_n46A |  | No |  |  |  |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A |  | DC_48A_n25A |  | No |  |  |  |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A |  | DC_48A_n66A |  | No |  |  |  |
| DC_48A_n71A DC_48B_n71A DC_48C_n71A DC_48D_n71A |  | DC_48A_n71A |  | No |  |  |  |
| DC_48A-48A_n71A DC_48A-48A-48A_n71A |  | DC_48A_n71A |  | No |  |  |  |
| DC_66A_n2A DC_66B_n2A DC_66C_n2A |  | DC_66A_n2A |  | DC_66_n2 |  |  |  |
| DC_66A-66A_n2A DC_66A-66A-66A_n2A |  | DC_66A_n2A |  | DC_66_n2 |  |  |  |
| DC_66A_n5A DC_66B_n5A DC_66C_n5A |  | DC_66A_n5A |  | DC_66_n5 |  |  |  |
| DC_66A-66A_n5A DC_66A-66A-66A_n5A |  | DC_66A_n5A |  | DC_66_n5 |  |  |  |
| DC_66A_n7A DC_66A-66A_n7A DC_66A_n7(2A) DC_66A-66A_n7(2A) |  | DC_66A_n7A |  | No |  |  |  |
| DC_66A_n12A |  | DC_66A_n12A |  | No |  |  |  |
| DC_66A_n25A |  | DC_66A_n25A |  | DC_66_n25 |  |  |  |
| DC_66A_n28A |  | DC_66A_n28A |  | No |  |  |  |
| DC_66A_n38A |  | DC_66A_n38A |  | No |  |  |  |
| DC_66A-66A_n38A |  | DC_66A_n38A |  | No |  |  |  |
| DC_66A_n41A DC_66A_n41C |  | DC_66A_n41A |  | No |  |  |  |
| DC_66A_n41(2A) |  | DC_66A_n41A |  | No |  |  |  |
| DC_66A_n46A |  | DC_66A_n46A |  | No |  |  |  |
| DC_66A_n48A DC_66A_n48B |  | DC_66A_n48A |  | No |  |  |  |
| DC_66A-66A_n48A DC_66A-66A_n48B |  | DC_66A_n48A |  | No |  |  |  |
| DC_66A_n71A DC_66C_n71A DC_66A_n71B |  | DC_66A_n71A |  | No |  |  |  |
| DC_66A-66A_n71A |  | DC_66A_n71A |  | No |  |  |  |
| DC_66A_n77A |  | DC_66A_n77A |  | DC_66_n77 |  |  |  |
| DC_66A-66A_n77A DC_66A-66A-66A_n77A |  | DC_66A_n77A |  | DC_66_n77 |  |  |  |
| DC_66A_n78A |  | DC_66A_n78A |  | No |  |  |  |
| DC_66A_n78(2A) |  | DC_66A_n78A |  | No |  |  |  |
| DC_66A-66A_n78A |  | DC_66A_n78A |  | No |  |  |  |
| DC_66A-66A_n78(2A) |  | DC_66A_n78A |  | No |  |  |  |
| DC_71A_n2A |  | DC_71A_n2A |  | No |  |  |  |
| DC_71A_n5A |  | DC_71A_n5A |  | No |  |  |  |
| DC_71A_n38A |  | DC_71A_n38A |  | No |  |  |  |
| DC_71A_n41A |  | DC_71A_n41A |  | No |  |  |  |
| DC_71A_n48A |  | DC_71A_n48A |  | No |  |  |  |
| DC_71A_n66A |  | DC_71A_n66A |  | No |  |  |  |
| DC_71A_n78A |  | DC_71A_n78A |  | No |  |  |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: The minimum requirements for intra-band contiguous or non-contiguous EN-DC apply. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 5: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 8: The frequency range in band n28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. NOTE 9: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. NOTE 10: Void. NOTE 11: The minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 12: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 13: The minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirements for NR DL carrier(s) and E-UTRA DL carrier(s) are specified in clause 8.2.1.2.14 of 38.133 [15] and clause 7.32.2.12 of 36.133 [16] respectively. NOTE 15: Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. Same restrictions are applied to related higher order configurations. NOTE 16: The frequency range in band n41 is restricted for this band combination to 2595 – 2645 MHz. NOTE 17: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 18: Only single switched UL is supported. NOTE 19: The implementation with 4 antennas is targeted for FWA form factor for this band combination. |  |  |  |  |  |  |  |

<< Start of changes >>

6.2B.4.2.3	Inter-band EN-DC within FR1

6.2B.4.2.3.1	ΔTIB,c for EN-DC two bands

Table 6.2B.4.2.3.1-1: ΔTIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration |  |  |  |  | E-UTRA or NR Band |  |  |  |  | ΔTIB,c (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1_n3 |  |  |  |  | 1 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n3 |  |  |  |  | 0.3 |  |  |  |  |
| DC_1_n5 |  |  |  |  | 1 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n5 |  |  |  |  | 0.3 |  |  |  |  |
| DC_1_n7, DC_1-1_n7 |  |  |  |  | 1 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n7 |  |  |  |  | 0.6 |  |  |  |  |
| DC_1_n8 |  |  |  |  | 1 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n8 |  |  |  |  | 0.3 |  |  |  |  |
| DC_1_n20 |  |  |  |  | 1 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n20 |  |  |  |  | 0.3 |  |  |  |  |
| DC_1_n28 |  |  |  |  | 1 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n28 |  |  |  |  | 0.6 |  |  |  |  |
| DC_1_n38 |  |  |  |  | 1 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n38 |  |  |  |  | 0.5 |  |  |  |  |
| DC_1_n40 |  |  |  |  | 1 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n40 |  |  |  |  | 0.5 |  |  |  |  |
| DC_1_n50 |  |  |  |  | 1 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n50 |  |  |  |  | 0.5 |  |  |  |  |
| DC_1_n41 |  |  |  |  | 1 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.5 |  |  |  |  |
| DC_1_n51 |  |  |  |  | 1 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n51 |  |  |  |  | 0.6 |  |  |  |  |
| DC_1_n71 |  |  |  |  | 1 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n71 |  |  |  |  | 0.3 |  |  |  |  |
| DC_1_n77 |  |  |  |  | 1 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_1_n78 |  |  |  |  | 1 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_2_n5 |  |  |  |  | 2 |  |  |  |  | 0.3 |  |  |  |  |
| DC_2-2_n5 |  |  |  |  | n5 |  |  |  |  | 0.3 |  |  |  |  |
| DC_2_n7 |  |  |  |  | 2 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n7 |  |  |  |  | 0.5 |  |  |  |  |
| DC_2_n12 |  |  |  |  | 2 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n12 |  |  |  |  | 0.3 |  |  |  |  |
| DC_2_n28 |  |  |  |  | 2 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n28 |  |  |  |  | 0.3 |  |  |  |  |
| DC_2_n38, DC_2-2_n38 |  |  |  |  | 2 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n38 |  |  |  |  | 0.9 |  |  |  |  |
| DC_2_n41, DC_2-2_n41 |  |  |  |  | 2 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.41 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 0.92 |  |  |  |  |
| DC_2_n48 |  |  |  |  | 2 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n48 |  |  |  |  | 0.8 |  |  |  |  |
| DC_2_n66 |  |  |  |  | 2 |  |  |  |  | 0.5 |  |  |  |  |
| DC_2-2_n66 |  |  |  |  | n66 |  |  |  |  | 0.5 |  |  |  |  |
| DC_2_n71, |  |  |  |  | 2 |  |  |  |  | 0.3 |  |  |  |  |
| DC_2-2_n71 |  |  |  |  | n71 |  |  |  |  | 0.3 |  |  |  |  |
| DC_2_n77 DC_2-2_n77 |  |  |  |  | 2 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_2_n78, DC_2-2_n78 |  |  |  |  | 2 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_3_n1 |  |  |  |  | 3 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n1 |  |  |  |  | 0.3 |  |  |  |  |
| DC_3_n5 |  |  |  |  | 3 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n5 |  |  |  |  | 0.3 |  |  |  |  |
| DC_3_n8 |  |  |  |  | 3 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n8 |  |  |  |  | 0.3 |  |  |  |  |
| DC_3_n7, DC_3-3_n7 |  |  |  |  | 3 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n7 |  |  |  |  | 0.5 |  |  |  |  |
| DC_3_n20 |  |  |  |  | 3 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n20 |  |  |  |  | 0.3 |  |  |  |  |
| DC_3_n28 |  |  |  |  | 3 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n28 |  |  |  |  | 0.3 |  |  |  |  |
| DC_3_n34 |  |  |  |  | 3 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n34 |  |  |  |  | 0.5 |  |  |  |  |
| DC_3_n38 |  |  |  |  | 3 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n38 |  |  |  |  | 0.5 |  |  |  |  |
| DC_3_n40 |  |  |  |  | 3 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n40 |  |  |  |  | 0.5 |  |  |  |  |
| DC_3-n41 |  |  |  |  | 3 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.33 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 0.84 |  |  |  |  |
| DC_3_n50 |  |  |  |  | 3 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n50 |  |  |  |  | 0.5 |  |  |  |  |
| DC_3_n51 |  |  |  |  | 3 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n51 |  |  |  |  | 0.3 |  |  |  |  |
| DC_3_n71 |  |  |  |  | 3 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n71 |  |  |  |  | 0.3 |  |  |  |  |
| DC_7_n66, DC_7-7_n66 |  |  |  |  | 7 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n66 |  |  |  |  | 0.5 |  |  |  |  |
| DC_3_n77, DC_3-3_n77 |  |  |  |  | 3 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_3_n78, DC_3-3_n78 |  |  |  |  | 3 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_4_n2 |  |  |  |  | 4 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n2 |  |  |  |  | 0.5 |  |  |  |  |
| DC_4_n5 |  |  |  |  | 4 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n5 |  |  |  |  | 0.3 |  |  |  |  |
| DC_4_n7 |  |  |  |  | 4 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n7 |  |  |  |  | 0.5 |  |  |  |  |
| DC_4_n28 |  |  |  |  | 4 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n28 |  |  |  |  | 0.6 |  |  |  |  |
| DC_4_n38 |  |  |  |  | 4 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n38 |  |  |  |  | 0.8 |  |  |  |  |
| DC_4_n41 |  |  |  |  | 4 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.81 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 1.32 |  |  |  |  |
| DC_4_n78 |  |  |  |  | 4 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_5_n2, DC_5-5_n2 |  |  |  |  | 5 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n2 |  |  |  |  | 0.3 |  |  |  |  |
| DC_5_n7 |  |  |  |  | 5 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n7 |  |  |  |  | 0.3 |  |  |  |  |
| DC_5_n12 |  |  |  |  | 5 |  |  |  |  | 0.8 |  |  |  |  |
|  |  |  |  |  | n12 |  |  |  |  | 0.4 |  |  |  |  |
| DC_5_n38 |  |  |  |  | 5 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n38 |  |  |  |  | 0.3 |  |  |  |  |
| DC_5_n40 |  |  |  |  | 5 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n40 |  |  |  |  | 0.3 |  |  |  |  |
| DC_5_n48 |  |  |  |  | 5 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n48 |  |  |  |  | 0.3 |  |  |  |  |
| DC_5_n66, DC_5-5_n66 |  |  |  |  | 5 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n66 |  |  |  |  | 0.3 |  |  |  |  |
| DC_5_n71 |  |  |  |  | 5 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n71 |  |  |  |  | 0.5 |  |  |  |  |
| DC_5_n77 |  |  |  |  | 5 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_5_n78 |  |  |  |  | 5 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_7_n1, DC_7-7_n1 |  |  |  |  | 7 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n1 |  |  |  |  | 0.5 |  |  |  |  |
| DC_7_n2 |  |  |  |  | 7 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n2 |  |  |  |  | 0.5 |  |  |  |  |
| DC_7_n3 |  |  |  |  | 7 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n3 |  |  |  |  | 0.5 |  |  |  |  |
| DC_7_n5, DC_7-7_n5 |  |  |  |  | 7 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n5 |  |  |  |  | 0.3 |  |  |  |  |
| DC_7_n8 |  |  |  |  | 7 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n8 |  |  |  |  | 0.6 |  |  |  |  |
| DC_7_n20 |  |  |  |  | 7 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n20 |  |  |  |  | 0.3 |  |  |  |  |
| DC_7_n28 |  |  |  |  | 7 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n28 |  |  |  |  | 0.3 |  |  |  |  |
| DC_7_n40 |  |  |  |  | 7 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n40 |  |  |  |  | 0.6 |  |  |  |  |
| DC_7_n51 |  |  |  |  | 7 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n51 |  |  |  |  | 0.3 |  |  |  |  |
| DC_7_n71 |  |  |  |  | 7 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n71 |  |  |  |  | 0.6 |  |  |  |  |
| DC_7_n77, DC_7-7_n77 |  |  |  |  | 7 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_7_n78, DC_7-7_n78 |  |  |  |  | 7 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_8_n1 |  |  |  |  | 8 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n1 |  |  |  |  | 0.3 |  |  |  |  |
| DC_8_n2 |  |  |  |  | 8 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n2 |  |  |  |  | 0.3 |  |  |  |  |
| DC_8_n3 |  |  |  |  | 8 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n3 |  |  |  |  | 0.3 |  |  |  |  |
| DC_8_n7 |  |  |  |  | 8 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n7 |  |  |  |  | 0.3 |  |  |  |  |
| DC_8_n20 |  |  |  |  | 8 |  |  |  |  | 0.4 |  |  |  |  |
|  |  |  |  |  | n20 |  |  |  |  | 0.4 |  |  |  |  |
| DC_8_n28 |  |  |  |  | 8 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n28 |  |  |  |  | 0.5 |  |  |  |  |
| DC_8_n34 |  |  |  |  | 8 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n34 |  |  |  |  | 0.3 |  |  |  |  |
| DC_8_n39 |  |  |  |  | 8 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n39 |  |  |  |  | 0.3 |  |  |  |  |
| DC_8_n40 |  |  |  |  | 8 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n40 |  |  |  |  | 0.3 |  |  |  |  |
| DC_8_n41 |  |  |  |  | 8 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.3 |  |  |  |  |
| DC_8_n77 |  |  |  |  | 8 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_8_n78 |  |  |  |  | 8 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_11_n3 |  |  |  |  | 11 |  |  |  |  | 0.8 |  |  |  |  |
|  |  |  |  |  | n3 |  |  |  |  | 0.9 |  |  |  |  |
| DC_11_n28 |  |  |  |  | 11 |  |  |  |  | 0.4 |  |  |  |  |
|  |  |  |  |  | n28 |  |  |  |  | 0.6 |  |  |  |  |
| DC_11_n77 |  |  |  |  | 11 |  |  |  |  | 0.4 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_11_n78 |  |  |  |  | 11 |  |  |  |  | 0.4 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_12_n2 |  |  |  |  | 12 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n2 |  |  |  |  | 0.3 |  |  |  |  |
| DC_12_n5 |  |  |  |  | 12 |  |  |  |  | 0.4 |  |  |  |  |
|  |  |  |  |  | n5 |  |  |  |  | 0.8 |  |  |  |  |
| DC_12_n7 |  |  |  |  | 12 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n7 |  |  |  |  | 0.3 |  |  |  |  |
| DC_12_n25 |  |  |  |  | 12 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n25 |  |  |  |  | 0.3 |  |  |  |  |
| DC_12_n38 |  |  |  |  | 12 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n38 |  |  |  |  | 0.3 |  |  |  |  |
| DC_12_n41 |  |  |  |  | 12 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.3 |  |  |  |  |
| DC_12_n66 |  |  |  |  | 12 |  |  |  |  | 0.8 |  |  |  |  |
|  |  |  |  |  | n66 |  |  |  |  | 0.3 |  |  |  |  |
| DC_12_n71 |  |  |  |  | 12 |  |  |  |  | 1 |  |  |  |  |
|  |  |  |  |  | n71 |  |  |  |  | 1 |  |  |  |  |
| DC_12_n78 |  |  |  |  | 12 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_13_n2 |  |  |  |  | 13 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n2 |  |  |  |  | 0.3 |  |  |  |  |
| DC_13_n5 |  |  |  |  | 13 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n5 |  |  |  |  | 0.5 |  |  |  |  |
| DC_13_n7 |  |  |  |  | 13 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n7 |  |  |  |  | 0.5 |  |  |  |  |
| DC_13_n48 |  |  |  |  | 13 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n48 |  |  |  |  | 0.3 |  |  |  |  |
| DC_13_n66 |  |  |  |  | 13 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n66 |  |  |  |  | 0.3 |  |  |  |  |
| DC_13_n71 |  |  |  |  | 13 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n71 |  |  |  |  | 0.5 |  |  |  |  |
| DC_13_n77 |  |  |  |  | 13 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_13_n78 |  |  |  |  | 13 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_14_n2 |  |  |  |  | 14 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n2 |  |  |  |  | 0.3 |  |  |  |  |
| DC_14_n66 |  |  |  |  | 14 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n66 |  |  |  |  | 0.3 |  |  |  |  |
| DC_18_n3 |  |  |  |  | 18 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n3 |  |  |  |  | 0.3 |  |  |  |  |
| DC_18_n28 |  |  |  |  | 18 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n28 |  |  |  |  | 0.5 |  |  |  |  |
| DC_18_n41 |  |  |  |  | 18 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.33 |  |  |  |  |
| DC_18_n77 |  |  |  |  | 18 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_18_n78 |  |  |  |  | 18 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_19_n1 |  |  |  |  | 19 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n1 |  |  |  |  | 0.3 |  |  |  |  |
| DC_19_n77 |  |  |  |  | 19 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_19_n78 |  |  |  |  | 19 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_20_n1 |  |  |  |  | 20 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n1 |  |  |  |  | 0.3 |  |  |  |  |
| DC_20_n3 |  |  |  |  | 20 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n3 |  |  |  |  | 0.3 |  |  |  |  |
| DC_20_n7 |  |  |  |  | 20 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n7 |  |  |  |  | 0.3 |  |  |  |  |
| DC_20_n8 |  |  |  |  | 20 |  |  |  |  | 0.4 |  |  |  |  |
|  |  |  |  |  | n8 |  |  |  |  | 0.4 |  |  |  |  |
| DC_20_n28 |  |  |  |  | 20 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n28 |  |  |  |  | 0.5 |  |  |  |  |
| DC_20_n38 |  |  |  |  | 20 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n38 |  |  |  |  | 0.3 |  |  |  |  |
| DC_20_n41 |  |  |  |  | 20 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.3 |  |  |  |  |
| DC_20_n50 |  |  |  |  | 20 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n50 |  |  |  |  | 0.4 |  |  |  |  |
| DC_20_n51 |  |  |  |  | 20 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n51 |  |  |  |  | 0.5 |  |  |  |  |
| DC_20_n77 |  |  |  |  | 20 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_20_n78 |  |  |  |  | 20 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_21_n1 |  |  |  |  | 21 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n1 |  |  |  |  | 0.3 |  |  |  |  |
| DC_21_n28 |  |  |  |  | 21 |  |  |  |  | 0.4 |  |  |  |  |
|  |  |  |  |  | n28 |  |  |  |  | 0.3 |  |  |  |  |
| DC_21_n77 |  |  |  |  | 21 |  |  |  |  | 0.4 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_21_n78 |  |  |  |  | 21 |  |  |  |  | 0.4 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_25_n41, DC_25-25_n41 |  |  |  |  | 25 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.41 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 0.92 |  |  |  |  |
| DC_25_n77 DC_25-25_n77 |  |  |  |  | 25 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_25_n78 DC_25-25_n78 |  |  |  |  | 25 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_26_n25 |  |  |  |  | 26 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n25 |  |  |  |  | 0.3 |  |  |  |  |
| DC_26_n41 |  |  |  |  | 26 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.3 |  |  |  |  |
| DC_26_n77 |  |  |  |  | 26 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_26_n78 |  |  |  |  | 26 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_28_n1 |  |  |  |  | n1 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | 28 |  |  |  |  | 0.6 |  |  |  |  |
| DC_28_n2 |  |  |  |  | 28 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n2 |  |  |  |  | 0.3 |  |  |  |  |
| DC_28_n3 |  |  |  |  | 28 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n3 |  |  |  |  | 0.3 |  |  |  |  |
| DC_28_n5 |  |  |  |  | 28 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n5 |  |  |  |  | 0.5 |  |  |  |  |
| DC_28_n7 |  |  |  |  | 28 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n7 |  |  |  |  | 0.3 |  |  |  |  |
| DC_28_n8 |  |  |  |  | 28 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n8 |  |  |  |  | 0.6 |  |  |  |  |
| DC_28_n40 |  |  |  |  | 28 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n40 |  |  |  |  | 0.3 |  |  |  |  |
| DC_28_n41 |  |  |  |  | 28 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.3 |  |  |  |  |
| DC_28_n50 |  |  |  |  | 28 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n50 |  |  |  |  | 0.4 |  |  |  |  |
| DC_28_n51 |  |  |  |  | 28 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n51 |  |  |  |  | 0.5 |  |  |  |  |
| DC_28_n66 |  |  |  |  | 28 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n66 |  |  |  |  | 0.3 |  |  |  |  |
| DC_28_n77 |  |  |  |  | 28 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_28_n78 |  |  |  |  | 28 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_30_n2 |  |  |  |  | 30 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n2 |  |  |  |  | 0.5 |  |  |  |  |
| DC_30_n5 |  |  |  |  | 30 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n5 |  |  |  |  | 0.3 |  |  |  |  |
| DC_30_n66 |  |  |  |  | 30 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n66 |  |  |  |  | 0.8 |  |  |  |  |
| DC_38_n78 |  |  |  |  | n78 |  |  |  |  | 0.5 |  |  |  |  |
| DC_39-n41 |  |  |  |  | 39 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.5 |  |  |  |  |
| DC_39_n78 |  |  |  |  | 39 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_39_n79 |  |  |  |  | 39 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n79 |  |  |  |  | 0.8 |  |  |  |  |
| DC_40_n1 |  |  |  |  | n1 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | 40 |  |  |  |  | 0.5 |  |  |  |  |
| DC_40_n415 |  |  |  |  | 40 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.5 |  |  |  |  |
| DC_40_n77 |  |  |  |  | n77 |  |  |  |  | 0.5 |  |  |  |  |
| DC_40_n78 |  |  |  |  | n78 |  |  |  |  | 0.56 |  |  |  |  |
| DC_40_n79 |  |  |  |  | 40 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n79 |  |  |  |  | 0.8 |  |  |  |  |
| DC_41_n3 |  |  |  |  | 41 |  |  |  |  | 0.33 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 0.84 |  |  |  |  |
|  |  |  |  |  | n3 |  |  |  |  | 0.5 |  |  |  |  |
| DC_41_n28 |  |  |  |  | 41 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n28 |  |  |  |  | 0.3 |  |  |  |  |
| DC_41_n77 |  |  |  |  | 41 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_41_n78 |  |  |  |  | 41 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_41_n79 |  |  |  |  | 41 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n79 |  |  |  |  | 0.8 |  |  |  |  |
| DC_42_n1 |  |  |  |  | 42 |  |  |  |  | 0.8 |  |  |  |  |
|  |  |  |  |  | n1 |  |  |  |  | 0.3 |  |  |  |  |
| DC_42_n3 |  |  |  |  | 42 |  |  |  |  | 0.8 |  |  |  |  |
|  |  |  |  |  | n3 |  |  |  |  | 0.6 |  |  |  |  |
| DC_42_n28 |  |  |  |  | 42 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n28 |  |  |  |  | 0.8 |  |  |  |  |
| DC_42_n51 |  |  |  |  | 42 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n51 |  |  |  |  | 0.8 |  |  |  |  |
| DC_48_n5 |  |  |  |  | 48 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n5 |  |  |  |  | 0.3 |  |  |  |  |
| DC_48_n12 |  |  |  |  | 48 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n12 |  |  |  |  | 0.3 |  |  |  |  |
| DC_48_n25 |  |  |  |  | 48 |  |  |  |  | 0.8 |  |  |  |  |
|  |  |  |  |  | n25 |  |  |  |  | 0.6 |  |  |  |  |
| DC_48_n46 |  |  |  |  | 48 |  |  |  |  | 0.8 |  |  |  |  |
| DC_48_n66 |  |  |  |  | 48 |  |  |  |  | 0.8 |  |  |  |  |
|  |  |  |  |  | n66 |  |  |  |  | 0.6 |  |  |  |  |
| DC_48_n71 DC_48-48_n71 DC_48-48-48_n71 |  |  |  |  | 48 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n71 |  |  |  |  | 0.3 |  |  |  |  |
| DC_66_n2 |  |  |  |  | 66 |  |  |  |  | 0.5 |  |  |  |  |
| DC_66-66_n2 DC_66-66-66_n2 |  |  |  |  | n2 |  |  |  |  | 0.5 |  |  |  |  |
| DC_66_n5, DC_66-66_n5, DC_66-66-66_n5 |  |  |  |  | 66 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n5 |  |  |  |  | 0.3 |  |  |  |  |
| DC_66_n7, |  |  |  |  | 66 |  |  |  |  | 0.5 |  |  |  |  |
| DC_66-66_n7 |  |  |  |  | n7 |  |  |  |  | 0.5 |  |  |  |  |
| DC_66_n12 |  |  |  |  | 66 |  |  |  |  | 0.8 |  |  |  |  |
|  |  |  |  |  | n12 |  |  |  |  | 0.3 |  |  |  |  |
| DC_66_n25 |  |  |  |  | 66 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n25 |  |  |  |  | 0.5 |  |  |  |  |
| DC_66_n28 |  |  |  |  | n28 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | 66 |  |  |  |  | 0.3 |  |  |  |  |
| DC_66_n38, DC_66-66_n38 |  |  |  |  | 66 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n38 |  |  |  |  | 0.5 |  |  |  |  |
| DC_66_n41 |  |  |  |  | 66 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.81 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 1.32 |  |  |  |  |
| DC_66_n48, DC_66-66_n48 |  |  |  |  | 66 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n48 |  |  |  |  | 0.8 |  |  |  |  |
| DC_66_n71, DC_66-66_n71 |  |  |  |  | 66 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n71 |  |  |  |  | 0.3 |  |  |  |  |
| DC_66_n77 DC_66-66_n77 DC_66-66-66_n77 |  |  |  |  | 66 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n77 |  |  |  |  | 0.8 |  |  |  |  |
| DC_66_n78, DC_66-66_n78 |  |  |  |  | 66 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| DC_71_n2 |  |  |  |  | 71 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n2 |  |  |  |  | 0.3 |  |  |  |  |
| DC_71_n5 |  |  |  |  | 71 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n5 |  |  |  |  | 0.5 |  |  |  |  |
| DC_71_n38 |  |  |  |  | 71 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n38 |  |  |  |  | 0.3 |  |  |  |  |
| DC_71_n41 |  |  |  |  | 71 |  |  |  |  | 0.6 |  |  |  |  |
|  |  |  |  |  | n41 |  |  |  |  | 0.3 |  |  |  |  |
| DC_71_n48 |  |  |  |  | 71 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n48 |  |  |  |  | 0.3 |  |  |  |  |
| DC_71_n66 |  |  |  |  | 71 |  |  |  |  | 0.3 |  |  |  |  |
|  |  |  |  |  | n66 |  |  |  |  | 0.3 |  |  |  |  |
| DC_71_n78 |  |  |  |  | 71 |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  | n78 |  |  |  |  | 0.8 |  |  |  |  |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 3: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. NOTE 5: Applicable for UE supporting inter-band EN-DC without simultaneous Rx/Tx. NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< Second of changes >>

7.3B.3.3	Inter-band EN-DC within FR1

7.3B.3.3.1	ΔRIB,c for EN-DC in two bands

Table 7.3B.3.3.1-1: ΔRIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1_n28 | n28 | 0.2 |
| DC_1_n51 | n51 | 0.1 |
| DC_1_n77 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1_n78 | n78 | 0.5 |
| DC_2_n48 | 2 | 0.2 |
|  | n48 | 0.5 |
| DC_2_n66 | 2 | 0.3 |
| DC_2-2_n66 | n66 | 0.3 |
| DC_2_n77 DC_2-2_n77 | 2 | 0.2 |
|  | n77 | 0.5 |
| DC_2_n78, DC_2-2_n78 | 2 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n41 | n41 | 03 |
|  |  | 0.54 |
| DC_3_n51 | 3 | 0.2 |
|  | n51 | 0.2 |
| DC_3_n77, DC_3-3_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n78, DC_3-3_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_5_n77 | 5 | 0.2 |
|  | n77 | 0.5 |
| DC_5_n78 | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_4_n2 | 4 | 0.3 |
|  | n2 | 0.3 |
| DC_4_n28 | n28 | 0.2 |
| DC_4_n38 | 4 | 0.5 |
|  | n38 | 0.5 |
| DC_4_n41 | 4 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| DC_4_n78 | 4 | 0.2 |
|  | n78 | 0.5 |
| DC_5_n12 | 5 | 0.5 |
|  | n12 | 0.3 |
| DC_7_n8 | n8 | 0.2 |
| DC_7_n40 | n40 | 0.5 |
| DC_7_n51 | n51 | 0.2 |
| DC_7_n66, DC_7-7_n66 | 7 | 0.5 |
|  | n66 | 0.5 |
| DC_7_n71 | n71 | 0.2 |
| DC_7_n77, DC_7-7_n77 | n77 | 0.5 |
| DC_7_n78, DC_7-7_n78 | n78 | 0.5 |
| DC_8_n7 | 8 | 0.2 |
| DC_8_n28 | 8 | 0.2 |
|  | n28 | 0.1 |
| DC_8_n77 | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_8_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_11_n3 | 11 | 0.3 |
|  | n3 | 0.5 |
| DC_11_n28 | n28 | 0.2 |
| DC_11_n77 | n77 | 0.5 |
| DC_11_n78 | n78 | 0.5 |
| DC_12_n5 | 12 | 0.3 |
|  | n5 | 0.5 |
| DC_12_n66 | 12 | 0.5 |
| DC_12_n71 | 12 | 0.8 |
|  | n71 | 0.8 |
| DC_12_n78 | 12 | 0.2 |
|  | n78 | 0.5 |
| DC_13_n7 | 13 | 0.5 |
|  | n7 | 0.5 |
| DC_13_n77 | 13 | 0.2 |
|  | n77 | 0.5 |
| DC_13_n78 | 13 | 0.2 |
|  | n78 | 0.5 |
| DC_18_n41 | n41 | 03 |
| DC_18_n77 | n77 | 0.5 |
| DC_19_n77 | n77 | 0.5 |
| DC_19_n78 | n78 | 0.5 |
| DC_20_n38 | 20 | 0.2 |
| DC_20_n51 | n51 | 0.2 |
| DC_20_n77 | n77 | 0.5 |
| DC_20_n78 | n78 | 0.5 |
| DC_21_n77 | n77 | 0.5 |
| DC_21_n78 | n78 | 0.5 |
| DC_25_n41, DC_25-25_n41 | n41 | 01 |
|  |  | 0.52 |
| DC_25_n77 DC_25-25_n77 | 25 | 0.2 |
|  | n77 | 0.5 |
| DC_25_n78 DC_25-25_n78 | 25 | 0.2 |
|  | n78 | 0.5 |
| DC_26_n77 | n77 | 0.5 |
| DC_26_n78 | n78 | 0.5 |
| DC_28_n1 | 28 | 0.2 |
| DC_28_n8 | 28 | 0.1 |
|  | n8 | 0.2 |
| DC_28_n51 | n51 | 0.2 |
| DC_28_n66 | 28 | 0.2 |
| DC_28_n77 | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_28_n78 | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_30_n66 | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_38_n78 | 38 | 0.4 |
|  | n78 | 0.5 |
| DC_39_n40 | 39 | 0.3 |
|  | n40 | 0.3 |
| DC_39_n41 | 39 | 0.2 |
|  | n41 | 0.2 |
| DC_39_n78 | n78 | 0.5 |
| DC_39_n79 | n79 | 0.5 |
| DC_40_n77 | 40 | 0.4 |
|  | n77 | 0.5 |
| DC_40_n78 | 40 | 0.45 |
|  | n78 | 0.55 |
| DC_40_n79 | n79 | 0.5 |
| DC_41_n3 | 41 | 03 |
|  |  | 0.54 |
| DC_42_n1 | 42 | 0.5 |
| DC_42_n3 | 42 | 0.5 |
|  | n3 | 0.2 |
| DC_42_n28 | 42 | 0.2 |
|  | n28 | 0.5 |
| DC_41_n77 | n77 | 0.5 |
| DC_41_n78 | n78 | 0.5 |
| DC_41_n79 | n79 | 0.5 |
| DC_42_n51 | n51 | 0.2 |
| DC_48_n25 | 48 | 0.5 |
|  | n25 | 0.2 |
| DC_48_n46 | 48 | 0.5 |
| DC_48_n66 | 48 | 0.5 |
|  | n66 | 0.2 |
| DC_66_n2 | 66 | 0.3 |
| DC_66-66_n2 DC_66-66-66_n2 | n2 | 0.3 |
| DC_66_n7 DC_66-66_n7 | 66 | 0.5 |
|  | n7 | 0.5 |
| DC_66_n12 | 66 | 0.5 |
| DC_66_n25 | 66 | 0.3 |
|  | n25 | 0.3 |
| DC_66_n28 | n28 | 0.2 |
| DC_66_n38, DC_66-66_n38 | 66 | 0.5 |
|  | n38 | 0.5 |
| DC_66_n41 | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| DC_66_n48, DC_66-66_n48 | 66 | 0.2 |
|  | n48 | 0.5 |
| DC_66_n77 DC_66-66_n77 DC_66-66-66_n77 | 66 | 0.2 |
|  | n77 | 0.5 |
| DC_66_n78, DC_66-66_n78 | 66 | 0.2 |
|  | n78 | 0.5 |
| DC_71_n38 | 71 | 0.2 |
| DC_71_n41 | 71 | 0.2 |
| DC_71_n78 | 71 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. NOTE 3: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

<< End of changes >>