3GPP TSG-RAN WG4 Meeting#99-e                                R4-2110607

E-meeting, 19th May– 27th May, 2021

Agenda item:	9.6.2.2

Source: 	ZTE Corporation

Title: 	Discussion on BS requirements for NR 1024QAM

Document for:	Approval

Introduction

In the RAN#89e meeting, introduction of DL 1024QAM for NR FR1 [1] was approved to further enhance the network throughput similar as LTE 1024QAM. Currently, corresponding PHY related work has been completed in RAN1. In this contribution, we want to share some initial considerations on potential constraints from BS RF perspective.

- Specify corresponding UE and BS RF core requirements [RAN4]

  - UE and BS RF core requirements are specified for stationary wireless scenarios with up to 2 layer DL MIMO

  - The cell size(s) and type of stationary wireless scenarios for which UE and BS RF core requirements are defined will be studied and decided by RAN4.

Discussion

The analysis for NR 1024QAM should focus on the achieved EVM performance, identify performance gain in certain scenarios and its applicable deployment scenarios similar as what has been did for LTE 10241QAM.

Regarding the achieved EVM performance, it might be necessary to identify the essential difference between LTE and NR system which might contribute to potential EVM difference. Compared with LTE system, some major difference for FR1 NR should be larger channel bandwidth supported in lots of bands (e.g.up to 100MHz specified), massive MIMO architecture (e.g. 8x8 antenna array with digital beamforming) and higher frequency range supported (e.g. n77/n78/n79).

For first two factors mentioned above, this would mainly have the impacts on the I/Q compression/decompression due to the limited bandwidth of fiber link and increasing channel bandwidth and MIMO layers of NR. Indeed, this kind of I/Q compression and decompression was also considered and implemented in LTE system, the key question would be whether the same compression rate for LTE and NR are used or not. If lower compression rate like 8/6 bit compression algorithm are used, then EVM contribution would be worse than that of 11 bit compression algorithm, however of course this really depend whether the capacity of fiber link is sufficient or not and this need some further discussions among vendors before reaching the consensus.

Figure 1. illustration of I/Q compression and decompression

For rest of factors contributing to EVM performance as following, these traditional factors might be not quite related with larger channel bandwidth and massive MIMO architecture of NR system, however we still need to consider all factors one by one.

- CFR distortion;

- I/Q imbalance which could be alleviated by internal self-calibration of BS;

- Transmit chain non-linearity, mainly referring to PA non-linearity;

- Phase noise originated from LO;

For EVM contribution of CFR distortion and I/Q imbalance, it should be similar between LTE and NR, therefore there should be no much difference from EVM contributor perspective.

For EVM contribution of PA non-linearity of NR, its performance should be worse than that of LTE due to the larger channel bandwidth supported for NR system since PA non-linearity would be more complicated within larger channel bandwidth and lower PSD of NR compared with LTE if the same output power is assumed for LTE and NR would result in relative worse DPD performance.

For EVM contribution of phase noise of NR, it should make no difference between LTE and NR system, however the supported frequency range for licensed band of FR1 NR is up to 7125MHz (licensed 6GHz would be coming soon) which is also much higher than upper frequency range of licensed band of LTE (e.g. B42, B43), therefore at least for band n77/n78/n79 and licensed 6GHz band of NR, phase noise difference should also be taken into account.

Based on above considerations, from the EVM contributor perspective, at least following factor should be taken into account for FR1 NR 1024QAM.

- I/Q compression and decompression due to larger channel bandwidth and higher MIMO layers;

- Transmit chain non-linearity, mainly referring to PA non-linearity due to larger channel channel bandwidth;

- Phase noise due to higher frequency range supported for NR compared with LTE.

Based on the experience for the commercial FR1 NR BS, the achievable EVM to support 256QAM without any back off is close to 3%, in other words, if to achieve the same EVM requirement as LTE 1024QAM (e.g. 2.5%) for NR, then power back off for NR BS is definitely needed, we have some concerns on the shrink of coverage to support 1024QAM in sacrifice of other legacy UE’s performance, therefore we propose to start with the scenarios where there are no coverage issues or at least power backoff is not needed for NR 1024QAM.

Proposal 1: to start with scenarios (LA, MR) where no coverage issue or power back off is not needed to support 1024QAM firstly.

Proposal 2: propose the EVM requirement for NR 1024QAM as 2.5%.

Conclusion

- In this contribution, we want to share some initial considerations on potential constraints from BS RF perspective.

Proposal 1: to start with scenarios (LA, MR) where no coverage issue or power back off is not needed to support 1024QAM firstly.

Proposal 2: propose the EVM requirement for NR 1024QAM as 2.5%.

References

- RP-202044, New WI: Introduction of DL 1024QAM for NR FR1, Ericsson, approved.

- R4-1803257,CR on BS RF requirments for DL 1024QAM in TS 36.104, approved.

- R4-1706112, Reply LS on regarding 1024 QAM, Huawei, HiSilicon, approved.

- TR 38.921.

- R4-1811723, Simulation assumptions for NR UE PDSCH, approved