3GPP TSG-RAN4 Meeting #99-e	R4-2110639

# Online, , 19th May 2021 - 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0244 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of [] from Noc power level for n257/n258 PC5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_FWA_Bn257_Bn258-Perf |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Noc values for n257/n258 PC5 is in [ ]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of Noc values for n257/n258 PC5 is in [ ]. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot set Noc values for n257/258 for FWA PC5 UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-4 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

#### 4.5.3.2	Noc for NR operating bands in FR2

Values for Noc according to operating band and power class for single carrier requirements are specified in Table 4.5.3.2-1 for ∆BB =1dB.

Table 4.5.3.2-1: Noc power level for different UE power classes and frequency bands

| Operating band | UE Power class |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| n257 | -166.8 | -161.3 | -157.6 | -166.3 | -161.9 |
| n258 | -166.8 | -161.3 | -157.6 | -166.3 | -162.1 |
| n259 |  |  | -154.0 |  |  |
| n260 | -163.8 |  | -155.0 | -164.3 |  |
| n261 | -166.8 | -161.3 | -157.6 | -166.3 |  |
| Note 1: Noc levels are specified in dBm/Hz |  |  |  |  |  |

For PC3 multi-band devices, the Noc power level (NocMB) shall increase by multi-band relaxation defined in Table 6.2.1.3-4 of TS 38.101-2 [7]:

NocMB = NocSB + ∆MBP,n

-	NocSB is the Noc defined in Table 4.5.3.2-1

-	∆MBP,n values are specified in TS 38.101-2 [7].

For CA case, the Noc power level (NocCA) shall increase by a relaxation factor defined in TS 38.101-2 [7] Table 7.3A.2.1-1:

NocCA = NocSC + ΔRIB

-	NocSC is derived by assuming UE supports single carrier.

-	ΔRIB values are specified in TS 38.101-2 [7].

------------------------------------------------------------- End of change ------------------------------------------------------------