3GPP TSG-RAN WG4 Meeting #99-e	R4-2110640

Electronic Meeting, 19 May – 27 May 2021

Agenda Item:	9.7.3.2

Source:	Ericsson

Title:	Update of simulation result for CA PDSCH with HST

Document for:	Information

# 1	Introduction

This contribution updates our simulation from [1] and add impairment results.

# 2	Simulation results

## 2.1	HST-SFN joint transmission

| Test number | Duplex mode / SCS | Bandwidth (MHz) | MCS / Rank | Propagation condition | Antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD SCS=15kHz | 5, 10, 15, 20, 25, 30, 40, 50 | MCS13 / Rank 2 | HST-SFN | 2x2 |
| 2 | FDD SCS=15kHz | 5, 10, 15, 20, 25, 30, 40, 50 | MCS13 / Rank 2 | HST-SFN | 2x4 |
| 3 | TDD SCS=30kHz | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | MCS13 / Rank 2 | HST-SFN | 2x2 |
| 4 | TDD SCS=30kHz | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | MCS13 / Rank 2 | HST-SFN | 2x4 |

Table 1 summarizes our simulation results for HST-SFN joint transmission with FDD SCS=15kHz and TDD SCS=30kHz.

Table 1	Simulation results for HST-SFN JT.

| CBW (MHz) | FDD 15kHz |  |  |  | TDD 30kHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx |  | 4Rx |  | 2Rx |  | 4Rx |  |
|  | Alignment | Impairments | Alignment | Impairments | Alignment | Impairments | Alignment | Impairments |
| 5 | 10.4 | 12.4 | 8.0 | 10.0 | 11.2 | 13.2 | 9.2 | 11.2 |
| 10 | 10.5 | 12.5 | 8.0 | 10.0 | 11.7 | 13.7 | 9.7 | 11.7 |
| 15 | 10.5 | 12.5 | 8.0 | 10.0 | 11.9 | 13.9 | 9.9 | 11.9 |
| 20 | 10.9 | 12.9 | 8.4 | 10.4 | 11.7 | 13.7 | 9.8 | 11.8 |
| 25 | 10.9 | 12.9 | 8.3 | 10.3 | 11.8 | 13.8 | 9.9 | 11.9 |
| 30 | 11.1 | 13.1 | 8.4 | 10.4 | 12.1 | 14.1 | 10.2 | 12.2 |
| 40 | 11.0 | 13.0 | 8.4 | 10.4 | 12.1 | 14.1 | 10.2 | 12.2 |
| 50 | 11.0 | 13.0 | 8.4 | 10.4 | 12.1 | 14.1 | 10.2 | 12.2 |
| 60 |  |  |  |  | 12.1 | 14.1 | 10.1 | 12.1 |
| 80 |  |  |  |  | 12.1 | 14.1 | 10.1 | 12.1 |
| 90 |  |  |  |  | 12.3 | 14.3 | 10.3 | 12.3 |
| 100 |  |  |  |  | 12.0 | 14.0 | 10.0 | 12.0 |

## 2.2	HST-DPS

| Test number | Duplex mode / SCS | Bandwidth (MHz) | MCS | Propagation condition | Antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD SCS=15kHz | 5, 10, 15, 20, 25, 30, 40, 50 | MCS17 / Rank 2 | HST-DPS | 2x2 |
| 2 | FDD SCS=15kHz | 5, 10, 15, 20, 25, 30, 40, 50 | MCS17 / Rank 2 | HST-DPS | 2x4 |
| 3 | TDD SCS=30kHz | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | MCS17 / Rank 2 | HST-DPS | 2x2 |
| 4 | TDD SCS=30kHz | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | MCS17 / Rank 2 | HST-DPS | 2x4 |

Table 2 summarizes our simulation results for HST-DPS transmission with FDD SCS=15kHz and TDD SCS=30kHz, where the alignment results are same as our previous contribution [2].

Table 2	Simulation results for HST-DPS.

| CBW (MHz) | FDD 15kHz |  |  |  | TDD 30kHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx |  | 4Rx |  | 2Rx |  | 4Rx |  |
|  | Alignment | Impairments | Alignment | Impairments | Alignment | Impairments | Alignment | Impairments |
| 5 | 9.7 | 11.7 | 7.2 | 9.2 | 9.5 | 11.5 | 6.8 | 8.8 |
| 10 | 10.1 | 12.1 | 7.3 | 9.7 | 9.8 | 11.8 | 7.2 | 9.2 |
| 15 | 10.1 | 12.1 | 7.3 | 9.3 | 9.7 | 11.7 | 7.2 | 9.2 |
| 20 | 9.7 | 11.7 | 7.2 | 9.2 | 9.6 | 11.6 | 7.1 | 9.1 |
| 25 | 10.1 | 12.1 | 7.3 | 9.3 | 9.7 | 11.7 | 7.2 | 9.2 |
| 30 | 10.2 | 12.2 | 7.4 | 9.4 | 9.6 | 11.6 | 7.1 | 9.1 |
| 40 | 10.1 | 12.1 | 7.3 | 9.3 | 9.6 | 11.6 | 7.0 | 9.0 |
| 50 | 10.2 | 12.2 | 7.4 | 9.7 | 10.0 | 12.0 | 7.3 | 9.3 |
| 60 |  |  |  |  | 9.8 | 11.8 | 7.3 | 9.3 |
| 80 |  |  |  |  | 9.7 | 11.7 | 7.2 | 9.2 |
| 90 |  |  |  |  | 10.2 | 12.2 | 7.3 | 9.3 |
| 100 |  |  |  |  | 9.9 | 11.9 | 7.3 | 9.3 |

# 3	References

- R4-2106862, “Initial simulation result of PDSCH for CA in HST”, Ericsson.

- R4-2106099, “Summary for FR1 HST demodulation results”, Ericsson.

# Appendix:	Simulation assumption

| Parameters | HST-SFN JT | HST-DPS |
| --- | --- | --- |
| DMRS type | Type 1 | Type 1 |
| Number of DMRS symbols | 1+1+1 | 1+1+1 |
| TDD pattern (for TDD) | 7DS2U for SCS=30kHz | 7DS2U for SCS=30kHz |
| MCS | MCS13 (64QAM table) | MCS17 (64QAM table) |
| TRS periodicity | 10ms, 2 slots pattern | 10ms, 2 slots pattern |
| PDSCH mapping | Type A, Start symbol: 2, Duration: 12 | Type A, Start symbol: 2, Duration: 12 |
| Ds | 700m | 700m |
| Dmin | 150m | 150m |
| Max Doppler shift | 870Hz for SCS=15kHz 1667Hz for SCS=30kHz | 870Hz for SCS=15kHz 1667Hz for SCS=30kHz |
| Rank | 2 | 2 |