3GPP TSG-RAN WG4 Meeting #99-e	R4-2110644

Electronic Meeting, 19 May – 27 May 2021

Agenda Item:	8.28.5

Source:	Ericsson

Title:	PDSCH simulation results for CBW 35MHz/45MHz

Document for:	Discussion

# 1	Introduction

RAN4#98-bis-e agreed with the way forward on UE demodulation and CQI reporting for channel bandwidths 35/45MHz for NR FR1 [1], where RAN4 agreed to define PDSCH CA requirements for 35MHz and 45MHz bandwidth in FDD. This contribution provides our PDSCH simulation results for 35/45MHz.

# 2	Simulation results

Table 1 shows our simulation results. We propose to consider our simulation results to define PDSCH demodulation requirements for CA.

Table 1	Test case list for added PDSCH requirements.

| Test number | Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) for alignment | SNR (dB) with impairments |
| 1 | 35 | R.PDSCH.1-10.X FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 10.9 | 12.9 |
| 2 | 45 | R.PDSCH.1-10.Y FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 11.6 | 13.6 |
| 3 | 35 | R.PDSCH.1-10.X FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 5.6 | 7.6 |
| 4 | 45 | R.PDSCH.1-10.Y FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 6.1 | 8.1 |

# References

- R4-2106090, “Way forward on UE demodulation and CQI reporting for channel bandwidths 35MHz and 45MHz for NR FR1”, Ericsson.

# Appendix

## A.1	FRC tables

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-10.1 FDD | R.PDSCH.1-10.2 FDD | R.PDSCH.1-10.X FDD | R.PDSCH.1-10.Y FDD |  |
| Channel bandwidth | MHz | 40 | 50 | 35 | 45 |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |  |
| Number of allocated resource blocks | PRBs | 216 | 270 | 188 | 242 |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 |  |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 13 | 13 | 13 | 13 |  |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 |  |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |  |
| Number of DMRS REs |  | 12 | 12 | 12 | 12 |  |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | Bits | 108552 | 135296 | 94248 | 122976 |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | CBs | 13 | 17 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 10, 11 | Bits | 217728 | 272160 | 189504 | 243936 |  |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 228096 | 285120 | 198528 | 255552 |  |
| Max. Throughput averaged over 2 frames | Mbps | 103.124 | 128.531 | 89.536 | 116.827 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |