3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2110649

Electronic Meeting, 19– 27 May, 2021

Title: 	Adding 90 and 100MHz bandwidth for band n40

Source: 	Huawei, HiSilicon, CMCC

Agenda item:	8.27.2

Document for:	Approval

Introduction

In the last plenary meeting, proposals for adding new existing BWs are approved in the revised WID RP-202062 [1]. It is copied below that for every proposal, RAN4 considers to specify the corresponding RF requirements:

- Analyze and specify requirements: - Reference sensitivity and associated RB allocation. - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR) - NS signalling. - Any other RF requirement which might be relevant.

In last meeting, way forward on adding 90 and 100MHz channel BW for UE in band n40 was approved in [8].

  - Introduce 90 and 100 MHz channel BW, considering specifying Δ -MPR, new co-existence requirements with n41.

    - Option 1: Restrict UL RBs to max 80MHz aggregated BW. RBs might be placed at lower frequency edge of channel (Δ-MPR is not required)

    - Option 2: No restriction to UL RBs

  - Introduce Δ-MPR for 90 MHz and 100MHz.

    - Option 1: For 90MHz channel BW: 0.5dB (assuming coexistence limit option1)

For 100MHz chanel BW: 2dB (assuming coexistence limit option1)

    - Option 2: 1 dB

  - Specify coexistence limit with n41: -40dBm/MHz which will be applicable:

    - Option 1: on the full band

    - Option 2: only in 2496MHz-2505MHz, and -50dBm/MHz would be applicable in the rest of the band, i.e. 2505MHz-2690MHz

In this paper, we provide further discussion on other remaining issues.

MPR for 100MHz UE channel bandwidth

According to the agreement in [2], the limitation for maximum UE channel bandwidth is the relative bandwidth, which is the key to ensure the MPR performance (Memory effects, bandwidth limitation, and output matching Q). The agreement for maximum channel bandwidth criteria in [2] is as follows:

In order to introduce new channel bandwidths in the future we propose to introduce a criteria to decide if the general MPR applies or additional MPR should be evaluated:

- If max UL CH BW rel ≤3% for FDD bands and ≤4% for TDD bands general MPR requirement applies

- If max UL CH BW rel >3% for FDD bands and >4% for TDD bands additional band and channel bandwidth specific MPR should be evaluated

According to equation of maximum relative channel bandwidth as following,

max UL CH BW rel = (2*max CH BW) / (FUL low + FUL high)

We can calculate the maximum channel bandwidth for NR band n40, which is 94MHz.

According to the results of series of measurements carried out upon PA, and RFIC, 0.5dB~1dB delta is found for ACLR evaluation and no difference for EVM evaluation.

Based on the discussion above, we propose,

Proposal 1: Introduce 1 dB Δ-MPR for 100MHz.

Co-existence with WIFI or Bluetooth

From the discussion in last meeting, one of the concern to introduction of 90 and 100MHz UL is co-existence with Wi-Fi/BT. In this clause we provide discussion on this aspect.

The interference avoidance for in-device coexistence between LTE band 40 and 2.4 GHz ISM band had been studied in Rel-11 SI [6]. The co-existence are guaranteed by implementation measures, such FDM solutions, TDM solutions and power control solutions. We believe the situation on the co-existence keep unchanged for NR.

Table 4-1 shows a typical attenuation in the band 40 bandpass filter and Table 4-2 shows a typical attenuation in the ISM bandpass filter.

Table 4-1: Typical attenuation in the Band 40 bandpass filter

| Freq. (MHz) | 2402-2418 | 2418-2436 | 2436-2480 |
| --- | --- | --- | --- |
| Typical attenuation | 10 | 39 | 45 |

Table 4-2: Typical attenuation in the ISM bandpass filter

| Freq. (MHz) | 2300-2370 | 2370-2380 | 2380-2400 |
| --- | --- | --- | --- |
| Typical attenuation | 50 | 40 | 4 |

As shown in Table 4-1 and 4-2, the attenuation is small to the adjacent interference. Due to band 40 and 2.4GHz ISM band are adjacent, the whole n40 band will suffer large blocking from WIFI or BT TX in the frequency range 2400~2418, and correspondingly, the WIFI or BT will suffer blocking from n40 TX in upper edge frequency 2380~2400. In existing specification, there is no restriction for NR operation in the range of 2380~2400 MHz for all channel bandwidths, i.e. it is left to the UE implementation. The co-existence are guaranteed by implementation measures in frequency domain, time domain, power domain or their combinations. Hence we should not limit operator to flexibly use the 90/100 MHz spectrum, e.g. in some scenarios WIFI/BT does not appear.

Proposal 2: No restriction to UL RBs for introduction of 90 and 100 MHz channel BW.

n41 co-existence

The analysis and measurements results can be found in [9], in general the spurious emission is defined as -30 dBm/MHz without filter rejection assumed, which applies for the frequency ranges that are more than FOOB (MHz) from the edge of the channel bandwidth, e.g. offset 105 MHz for 100 MHz CBW. With larger than 20 dB filter rejection, -50 dBm/MHz co-existence can be met without A-MPR. We have checked several commercial band 40 filters, 30~35 dB rejection is achievable. Hence option 2 is proposed for coexistence limit with n41, i.e. -40dBm/MHz applies in 2496MHz-2505MHz and -50dBm/MHz is applicable in 2505MHz-2690MHz.

Proposal 3: For co-existence with n41 for 90/100 MHz CBW, -40dBm/MHz is applicable in 2496MHz-2505MHz and -50dBm/MHz is applicable in 2505MHz-2690MHz, and the requirements can be met without A-MPR.

Conclusions

In this paper, we provide further discussion on corresponding RF requirements for adding 90 and 100MHz for band n40.

Proposal 1: Introduce 1 dB Δ-MPR for 100MHz.

Proposal 2: No restriction to UL RBs for introduction of 90 and 100 MHz channel BW.

Proposal 3: For co-existence with n41 for 90/100 MHz CBW, -40dBm/MHz is applicable in 2496MHz-2505MHz and -50dBm/MHz is applicable in 2505MHz-2690MHz, and the requirements can be met without A-MPR.

As a package proposal, we propose,

  - Introduce 90 and 100 MHz channel BW, considering specifying Δ –MPR and new co-existence requirements with n41.

    - Introduce 1 dB Δ-MPR for 100MHz

    - For co-existence with n41, -40dBm/MHz is applicable in 2496MHz-2505MHz and -50dBm/MHz is applicable in 2505MHz-2690MHz

References

[1] RP-202062, Revised Basket WID on Rel-17 adding channel bandwidth support to existing NR bands, Ericsson

[2] Skyworks, Huawei, Qorvo, Broadcom, R4-1801008, “WF on maximum channel bandwidth criteria”, 3GPP TSG RAN4 Meeting #94e.

[3] R4-2015297, draftCR to 38101-1 to add 90 and 100MHz BW for band n40, Huawei, HiSilicon

[4] R4-2015298, draftCR to 38104 to add 90MHz BW for band n40, Huawei, HiSilicon

[5] R4-2016861, draftCR to 38101-1 to add 90 and 100MHz BW for band n40, Huawei, HiSilicon

[6] TR 36.816, Study on signalling and procedure for interference avoidance for in-device coexistence

[7] R4-2014593, n40 MPR and Interference for Additional Channel Bandwidths, Skyworks Solutions, Inc.

[8] R4-2105360,	Way forward on adding 90 and 100MHz channel BW for UE in band n40, Huawei, HiSilicon, CMCC

[9] R4-2110397, Discussion on spurious emission about UE co-existence between band n40 and n41, Huawei, HiSilicon, CMCC