3GPP TSG-RAN WG4 meeting #99-e	R4-2110777

Electronic Meeting, 19 May – 27 May 2021

Agenda Item:	5.1.5.1.1

Source:	Ericsson

Title:	Channel matrix of LOS path in TDLD30

Document for:	Discussion

# 1	Introduction

RAN4#98-e has completed the UE demodulation performance part on DL 256QAM FR2, but RAN4 agreed to continues to discuss the channel matrix used for LOS path in TDLD30 channel model [1]. This contribution provides our simulation results with several channel matrix options.

# 2	Simulation results

Table 1 shows the simulation parameters according to [2].

Table 1	Simulation parameters

| Parameters | Values |
| --- | --- |
| Tx EVM | 3% |
| SCS and CBW | 120kHz, 50MHz (32PRB) |
| Antenna configuration | 2x2 |
| Rank | 1 |
| MCS | 20 |
| TDD configuration | DDDSU, S=10D:2G:2U |
| PDCCH configuration | Symbol #0 (AL2) |
| PDSCH configuration | Mapping type A DL slots: start symbol 1, duration 13 Special slots: start symbol 1, duration 9 |
| PDSCH PRB bundling size and precoding | PRB bundling size: 2 Random precoding, per slot, WB granularity |
| PDSCH DMRS configuration | Type 1, 1+1 |
| TRS configuration | Periodicity: 20ms offset: 10ms |
| Overhead for TBS determination | 6 |
| HARQ RV sequence | {0, 2, 3, 1} |
| Propagation channel | TDLD30-75 |

Figure 1 compares the ideal simulation results different channel matrix for LOS path in TDLD30-75. The channel matrix of LOS path is given by H=ejϕ1ejϕ2ejϕ3ejϕ4, where ϕ1, ϕ2, ϕ3, and ϕ4 are phases for each Tx-Rx path. For example the static channel matrix, defined in TS38.101-4 B.1.1, uses H=1j1-j where ϕ1=ϕ3=0, ϕ2=π/2, and ϕ4=-π/2. Figure 1 compares PDSCH demodulation performance with the several channel matrixes where ϕi is selected randomly. It is observed from the simulation results that the simulation results are significantly affected by the selected channel matrix. If we focus on the SNR with 70% of the maximum throughput, the span of SNR values become more than 1.5dB.

Observation: It is observed the significant performance difference depending on the selected channel matrix for LOS path in TDLD30-75.

Figure 1	Comparison of DL 256QAM in FR2 with different channel matrix for LOS path in TDLD30-75.

From the observation, we propose RAN4 specify the channel matrix for the path delay profiles with LOS path in TS38.101-4 to avoid the different test results. One option is to set the same channel matrix as TS38.101-4 B.1.1, i.e., H = [1 j; 1 -j]. However we are open to discuss to set the channel matrix.

Proposal: TS38.101-4 should specify the channel matrix for LOS path in TDLD30, e.g., H = [1 j; 1 -j].

# 3	Summary

Observation: It is observed the significant performance difference depending on the selected channel matrix for LOS path in TDLD30-75.

Proposal: TS38.101-4 should specify the channel matrix for LOS path in TDLD30, e.g., H = [1 j; 1 -j].

# References

- R4-2103940, “Email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod”, China Telecom​.

- R4-2103834, “CR on demodulation performance requirements for DL 256QAM for FR2”, ZTE corporation.