3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2110798

# Electronic Meeting, May 19-27, 2021

Agenda Item:		10.5.4

Source: 		Qualcomm Incorporated

Title: 	TP to TR38.861: Simulaiton results for FDD HPUE

Document for:	Approval

# 1. Introduction

As discussed in [1], we propose the following TP to TR 38.861 for study on high power UE (power class 2) for one NR FDD band.

# 2. Text Proposal

--------------Start of text proposal -------------

## 8.2	Monte Carlo simulation

### 8.2.1	Simulation methodology

The Monte Carlo based static system level simulation is typical simulation method used in the coexistence study and can also be used for the simplified system performance evaluation.  The sub-snapshot method is agreed as one of possible methods to simulate the HPUE feature.

The difference for UL duty cycle is emulated by assuming there are N sub-snapshots within one snapshot. For example, we can set N=2 for the case of 50% duty cycle for FDD HPUE. Similarly, we can set N=4 for the case of 25% duty cycle. In each snapshot, UEs are randomly placed in a predefined deployment scenario. But within one snapshot, the UEs’ locations for N sub-snapshots are fixed. Table 8.2.1-1 illustrates the sub-snapshot concept and the simulation method. When the simulation performance is assumed to be measured within one frame (e.g., 10ms) and only 50% time can be used for UL transmission within one frame assuming 50% duty cycle. Then each sub-snapshot can be considered as 5ms in the Monte-Carlo simulation.

Table 8.2.1-1. An example to compare FDD PC2 UE (50% DC) and FDD PC3 UE (100% DC)

| UE type | The first sub-snapshot | The second sub-snapshot | UL duty cycle |
| --- | --- | --- | --- |
| FDD PC3 UEs | Max. Tx power is 23dBm | Max. Tx power is 23dBm | 100% |
| FDD PC2 UEs with transmission power >23dBm | Max. Tx power is 26dBm | No transmission | 50% |
| Note 1: For PC2 UEs with transmission power ≤ 23dBm, there is no need to reduce the UL duty cycle since there is no SAR issue. Note 2: For simplicity, 50% UL duty cycle will be applied once PC2 UEs transmit power is larger than 23dBm. Note 3: PC2 UEs with 50% UL duty can randomly transmit power at the first or the second sub-snapshot. |  |  |  |

Non-full buffer and full buffer scenarios are both considered in the simulation. For the non-full buffer scenario, the resource will not be used in the sub-snapshot which there is no transmission for the UE with >23dBm Tx power. For the full buffer scenario, a new user will be scheduled in one of the two sub-snapshots when the UE with >23dBm Tx power will not transmit in that correspondent sub-snapshot due to 50% duty cycle.

There are also other evaluation methods can be implemented to stimulate the duty cycle in the Monte Carlo simulation, such as drawing a random number in each snapshot. If the random number is larger than the duty cycle, then the HPUE will be replaced with another UE; If the random number is less than the duty cycle, then the HPUE will transmit normally with Tx power up to 26 dBm.

### 8.2.2	Simulation assumptions

The simulation assumptions using Monte-Carlo simulation are as Table 8.2.2-1.

Table 8.2.2-1.  System Assumptions

| Parameter | Base Station | UE |  |
| --- | --- | --- | --- |
| Carrier frequency | 2GHz |  |  |
| Channel bandwidth | 40MHz |  |  |
| Active UE number in UL | 3 |  |  |
| PC2 UE ratio | 25%, 50%, 100% |  |  |
| Inter-site distance | 750m |  |  |
| Cell layout | Wrap-around 19 tri-sector cells |  |  |
| Lognormal fading | 10 dB |  |  |
| Shadowing correlation | Between cells: 0.5, between sites: 1.0 |  |  |
| MCL (including antenna gain) | 70 dB (urban) |  |  |
| Antenna gain and horizontal antenna pattern | Aθ=-min⁡[ 12 θθ3dB 2,Am], 17 dBi, θ3dB = 65 degrees, Am = 20 dB |  | Omni-directional antenna with -3.5 dBi. |
| Noise figure | 5 dB |  | 9 dB |
| Transmit power | 46 dBm |  | 23 dBm for UE, 26dBm for HPUE |
| Antenna height | 45 m |  | 1.5 m |

The power control (PC) set 4A and 4B are selected for performance evaluation which refers to TR36.886 chapter 5.10.1.2. The parameters are presented in Table 8.2.2-2.

Table 8.2.2-2. PC set parameters for 23dBm UE and 26 dBm UE with 0.75 km ISD

| Power control set parameter | Gamma | CLx-ile for BW 40MHz |  |
| --- | --- | --- | --- |
|  |  | 23dBm | 26dBm |
| Set 4A | 1 | 104 | 107 |
| Set 4B | 1 | 100 | 103 |
| Note: the original table in 36.886 is for 20MHz bandwidth, thus the value needed to be adjusted for 40MHz bandwidth correspondently. |  |  |  |

Figure 8.2.2-1 and 8.2.2-2 are the CDF curve of Tx power for power control set 4A and 4B, respectively, to show the difference between FDD PC2 and PC3.

Figure 8.2.2-1.  UL Tx power CDF for power control set 4A

Figure 8.8.2-2.  UL Tx power CDF for power control set 4B

### 8.2.3	Simulation results

#### 8.2.3.1	Simulation results for power control set 4A

The ratio of UL Tx power > 23dBm is about 4.9% for power control set 4A.

In a more realistic situation, for example, considering a relatively long-time duration, the time restriction for UE duty cycle can be relaxed assuming that UE can keep transmit with >23dBm power in its scheduled duration. After the UE finishes its transmission, another UE will be scheduled. In this scenario, the UE can naturally realize the 50% duty cycle by the scheduling time since the UE will not be always scheduled. The duty cycle then does not need to be considered in the result processing. Therefore, the system performances are shown in Table 8.2.3.1-1 and Table 8.2.3.1-2.

Table 8.2.3.1-1. PC2 system performance for non-full buffer case with PC set 4A

when 50% duty cycle is considered in a longer duration

| Power control set | 4A | 4A | 4A |
| --- | --- | --- | --- |
| PC2 UE ratio | 25% | 50% | 100% |
| 5% throughput gain | 1.22% | 2.44% | 4.27% |
| Cell throughput gain | 0.40% | 1.04% | 2.39% |

Table 8.2.3.1-2.  PC2 system performance for full buffer with PC set 4A

when 50% duty cycle is considered in a longer duration

| Power control set | 4A | 4A | 4A |
| --- | --- | --- | --- |
| PC2 UE ratio | 25% | 50% | 100% |
| 5% throughput gain | 0.61% | 1.83% | 3.05% |
| Cell throughput gain | 0.48% | 1.12% | 2.55% |

As shown in Table 8.2.3.1-1 and Table 8.2.3.1-2:

- When 50% duty cycle can be realized in a longer duration, performance gain can be observed for the 5% throughput. The gain’s value depends on the PC2 UE ratio and the traffic loading.

- When 50% duty cycle can be realized in a longer duration, performance gain can be observed for the average cell throughput. The gain’s value depends on the PC2 UE ratio and the traffic loading.

- When 50% duty cycle can be realized in a longer duration, higher PC2 UE ratio will bring higher 5%-throughput gain as well as the average cell throughput gain. The system performance gain is highly related with PC2 UE ratio.

#### 8.2.3.2	Simulation results for power control set 4B

The Tx power ratio UL Tx power > 23dBm is about 17.8% for power control set 4B.

Similar consideration is applied, when 50% duty cycle can be realized in a longer duration, the 50% duty cycle need not be considered in the performance processing. Therefore, the system performance is shown in Table 8.2.3.2-1 and Table 8.2.3.2-2.

Table 8.2.3.2-1.  PC2 system performance for non-full buffer with PC set 4B

when 50% duty cycle is considered in a longer duration

| Power control set | 4B | 4B | 4B |
| --- | --- | --- | --- |
| PC2 UE ratio | 25% | 50% | 100% |
| 5% throughput gain | 5.13% | 10.26% | 17.31% |
| Cell throughput gain | 0.81% | 1.98% | 5.50% |

Table 8.2.3.2-2.  PC2 system performance for full buffer with PC set 4B

when 50% duty cycle is considered in a longer duration

| Power control set | 4B | 4B | 4B |
| --- | --- | --- | --- |
| PC2 UE ratio | 25% | 50% | 100% |
| 5% throughput gain | 3.85% | 8.33% | 15.38% |
| Cell throughput gain | 1.03% | 2.20% | 4.84% |

From the simulation results shown in chapter 8.2.3.2, the similar performance variance trend can be observed for the power control set 4B, compared with power control set 4A.

#### 8.2.3.3	Simulation observations

The system performance is sensitive to the power control set parameters. In realistic situation, where several UEs are multiplexed in the same cell, non-negligible gain in UL performance can be observed in both mean and 5%-tile throughput.

--------------End of text proposal -------------

# Reference

[1] R4-2106912 TR skeleton for Study on high power UE (power class 2) for one NR FDD band