3GPP TSG-RAN4 Meeting #99-e	R4-2110871

# Electronic Meeting, May 19–27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2087 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to update RSTD measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are some remaining open issues in RSTD measurement period: - PRS resource muting is not accounted - Parameter Lprs is not specified - Impact of MG reconfiguration is not accounted - Measurement period for PRS-RSRP configured for DL-TDOA is undefined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the RSTD measurement period requirements for above issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RSTD measurement period requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.9.2.5	Measurements Period Requirements

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure and report multiple (up to the UE capability specified in Clause 9.9.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

When measurement gaps and processing time T have overlap between different PRS frequency layers, TRSTD,Total is defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i

Editor’s note:	FFS the RSTD measurement period when measurement gaps and processing time T do not have overlap between different PRS  frequency layers.

Where ,

i is the index of PRS frequency layer,

L is total number of PRS frequency layers, and

Teffect,i is the periodicity of PRS-RSTD measurement in PRS frequency layer i as defined further in this clause.

TPRS-RSTD,i is the measurement period for PRS RSTD measurement in PRS frequency layer i as specified below:

TPRS-RSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iKiN*Nsample-1*Teffect,i+Tlast ,

where:

NRxBeam,i is the UE Rx beam sweeping factor. In FR1, NRxBeam,i = 1; and in FR2,  NRxBeam,i = [8].

CSSFPRS,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in frequency layer i as defined in clause 9.1.5.2.

Nsample is the number of PRS RSTD samples and Nsample= 4.

Tlast is the measurement duration for the last PRS RSTD sample, including the sampling time and processing time, Tlast = Ti + LPRS,i ,

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i , is the periodicity of PRS RSTD measurement in frequency layer i.

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi.

MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the PRS frequency layer i.

TPRS,i=LCMj(Tresource,j*Nmuting,j), where

Tresource,j is the configured periodicity of PRS resource j on the positioning frequency layer i which is fully or partially overlapped with measurement gap, and

Nmuting,j=X*Y is the scaling factor if muting option 1 is applied on PRS resource j, where X is the size of NR-MutingPattern-r16 for mutingOption1-r16, and Y is the value of dl-prs-MutingBitRepetitionFactor.

LPRS,i is FFS.Ki is parameter K for positioning frequency layer i as defined in clause 5.1.6.5 of TS 38.214 [26], and only accounts for duration of PRS resource instances within measurement gap and are not muted.

NPRS,islot is the maximum number of DL PRS resources configured in a slot in PRS frequency layer i.

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34].

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot specified in TS 37.355 [34].

If handover occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements. The UE shall also meet the RSTD measurement requirements in this clause and measurement accuracy requirements in clause 10.1.23. However, in this case the RSTD measurement period TRSTD,total.HO shall be as follows:

TRSTD, total,HO=TRSTD, Total+KNHO*Teffect+THO

Where,

-	NHOK is the number of times handover occurs during TPRS-RSRP,total.HO;

-	Teffect is the largest Teffect,i among all PRS layers;

-	THO is the time during which the RSTD measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

-	THO is the time during which the RSTD measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

If during the measurement period of one or more positioning frequency layers, the measurement gap is reconfigured, the measurement period can be longer.

When PRS-RSRP is configured for DL-TDOA, RSTD and PRS-RSRP measurements are performed over the same measurement period defined in this clause.

<End of Change 1>