3GPP TSG-RAN WG4 Meeting #99-e	R4-2110911

Electronic Meeting, 19 – 27 May, 2021

Title: 	Discussion on pre-configured MG

Source: 	Huawei, HiSilicon

Agenda item:	9.10.2.1

Document for:	Discussion

Introduction

RRM requirements for pre-configured MG were discussed in RAN4#98-bis-e, and the outcomes are captured in the WF [1]. Based on [1] the following issues are to be further discussed:

- User scenario

- Configuration of pre-configured MG

- Activation and deactivation of pre-configured MG

- RRM requirements related to pre-configured MG, including

- MGPs for pre-configured MG

In this paper we will provide our views on the above open issues for pre-configured MG.

Discussion

User scenario

- Pre-configured MG for SSB measurements shall be configured. - FFS whether the pre-configured MG shall be configured for PRS measurement: - Option 1. Yes - Option 2. No - FFS whether the pre-configured MG shall be configured for CSI-RS L3 measurement: - Option 1. Yes - Option 2. No

It was agreed that pre-configured MG can be used for SSB measurement, and it is FFS if it can be also used for PRS and CSI-RS measurement. In our view, there is no need to restrict use of pre-configured MG from standard point of view.

For PRS, we agree that PRS is always measured in MGs based on Rel-16 requirements, but it does not mean we need to exclude PRS measurement for pre-configured MG in standard. There can be two options when UE is to perform PRS measurement:

- Option 1: NW disables the pre-configured MG.

In this way, NW indicates the pre-configured MG to become legacy MG (which is always active), and there is no activation or deactivation until NW indicates to enable the pre-configured MG again (the legacy MG becomes pre-configured MG) when UE no longer performs PRS measurement.

- Option 2: the pre-configured MG is activated

In this way, depending on the activation and deactivation mechanism, NW and/or UE will activate the pre-configured MG until UE completes the PRS measurement. After that, NW and/or UE can determine the status of the pre-configured MG based on the need for MG of RRM measurements.

It is noted that neither option requires standardization, but can be left to NW and/or UE implementation. In fact, even RAN4 agrees on option 2, it still needs to be implemented by either option 1 or option 2, so we do not see clear point to further discuss this issue.

For CSI-RS, similar analysis applies. Intra-frequency measurement is always performed without MG based on the condition that CSI-RS is confined in serving cell BWP, so it can co-exist with pre-configured MG and it does not even impact the activation and deactivation. Inter-frequency measurement is always performed without MG, so it can be handled in the same way as PRS by NW and/or UE implementation.

Proposal 1: No need to restrict the use of pre-configured MG with PRS or CSI-RS measurements. How to handle PRS and CSI-RS measurements with pre-configured MG can be left to NW and/or UE implementation.

Configuration of pre-configured MG

Per BWP configuration

- Pre-configured MG(s) are configured per UE or per FR - FFS if pre-configured MGs can be additionally configured per BWP

In our view, configuring pre-configured MG per BWP does not work when UE is configured with multiple measurements and/or multiple serving cells.

- If UE is only configured with PCell and intra-frequency measurement on PCC, NW may associate a MG configuration to BWP1 (which does not contain the SSB for PCC) and no-gap configuration to BWP2 (which contains the SSB for PCC).

- If UE is configured with an additional inter-frequency measurement which requires MG, NW needs to associate MG configuration to both BWP1 and BWP2. It means when an MO is added or released, NW may have to change the associated MG configuration for a BWP. This will cause unnecessary complexity for NW, since BWP is a per serving cell configuration and is related to data transmission, while measurement is a per UE configuration and is related to measurement.

- If UE is configured two serving cells (PCell and SCell) and intra-frequency measurement on both PCC and SCC, NW may not even be able to determine the associated MG for a BWP. For example,

  - PCell

    - BWP P-1 does not contain the PCC SSB and

    - BWP P-2 contains the PCC SSB;

  - SCell

    - BWP S-1 does not contain the SCC SSB and

    - BWP S-2 contains the SCC SSB

In this case, should NW associate MG configuration or no-gap configuration to BWP P-2? This should depend on which BWP UE stays on the SCell, which can change dynamically, so it is not possible for NW to associate any MG configuration to the BWP P-2.

Based on above analysis, we suggest that

Proposal 2: Pre-configured MGs is not additionally configured per BWP.

New parameters

- How pre-configured MGs can be configured: - The common configuration parameters of pre-configured MG (e.g. MGRP, MGL, etc) which are same as these of Rel16 legacy MG can be configured by the similar way as the Rel16 legacy MGs - FFS on the specific configuration parameters for the new aspects of pre-configured MG to be introduced, - One single bit (or flag) per BWP to indicate whether the pre-configured MG is ON or OFF when that BWP becomes active - Others are not precluded.

NW and UE should have common understanding about whether a MG is used as pre-configured MG, i.e. whether it will be activated and deactivated following BWP switch, or it will always stay activated as in Rel-16. Therefore, enable/disable of the pre-configured MG should be explicitly configured by the NW. This can be done e.g. by introducing one single bit (or flag) associated to existing MG configuration to indicate whether the MG is used as pre-configured MG or not.

Besides the flag, another possible new configuration is the status (activated or deactivated) of the pre-configured MG. For example, when NW enables pre-configured MG, it may also configure whether the MG should be activated or deactivated after the configuration, and NW should configure this status based on the need for MG from all measurements. The status (activated or deactivated) configuration may also be used when NW reconfigures MO or serving cells, both may impact the need for MG for measurements.

Proposal 3: On top of common configuration parameters as Rel-16 MG configuration, NW also configures the following new parameters for pre-configured MG

- A flag to indicate whether the MG is used as pre-configured MG or not

- Status (activated or deactivated) of the pre-configured MG

Default status (activated or deactivated)

- FFS whether the pre-configured MG is activated or not after configuration completed - Option 1: Activated by default - Option 2: Not activated by default until being activated. - Option 3 Status of pre-configured MG is not fixed (no default status): - Option 3a: Whether pre-configured MG activated or not depends on whether reference signal to measure is within the active BWP or not. - Option 3b: Network signals the status (activated or deactivated) when pre-configured MG is configured

When the pre-configured MG is configured, the status (activated or deactivated) should depend on whether MG is needed by UE to perform the configured measurements, so it should not be fixed (either activated or deactivated). Both option 3a and 3b are viable:

- With option 3a, configuration of pre-configured MG is regarded as a condition to trigger activation or deactivation, similar as BWP switching.

- With option 3b, NW configures the status (with the second new parameter in Proposal 3), and there is no need for NW and/or UE to activate or deactivate the pre-configured MG upon configuration.

Our preference is option 3b as it is more reliable and efficient, i.e. UE can simply follow NW configuration, and there is no need for NW and/or UE to perform additional activation or deactivation following the RRC message.

Proposal 4: Status (activated or deactivated) of pre-configured MG is not fixed after configuration. NW configures the status when pre-configured MG is configured.

Change of MG configuration

- FFS on whether MG configuration can be changed after BWP switching - Option 1: Yes - Option 2: No

The motivation to support pre-configured MG is that following a BWP switch, a measurement may change from MG based to MG-less or vice versa, so the need for MG may change. As a results, the pre-configured MG can be deactivated or activated in a fast manner, avoiding the latency and signalling overhead caused by RRC (de)configuration of MG. In this case, the pre-configured MG is effectively turned ON/OFF following a BWP switch.

In RAN4#98-bis-e discussion, another use case has been discussed, which is to change the MG configuration following a BWP switch. In this case, MG would be needed both before and after the BWP switching, but a different MG configuration (e.g. MG pattern) can be used following a BWP switch.

We acknowledge that this could be a valid use case, e.g. it could happen that a measurement requiring large MGL when being a MG based becomes a MG-less one following a BWP switch, and thus a shorter MGL could be used for other measurements after the BWP switch. However, the gain in supporting this use case is not as large as the ON/OFF of the pre-configured MG, and it will require additional specification efforts and implementation complexity for both NW and UE. Therefore, we suggest that changing of MG configuration following a BWP switching is not considered in the WI.

Proposal 5: RAN4 to support ON/OFF of the pre-configured MG. Changing of MG configuration following a BWP switching is not considered.

Pre-configured MG and legacy MG

- FFS on relation of pre-configured MG pattern and with the current RRC configured MG - Opiton 1. - The already configured P-MGP is transformed into legacy MGP (with same MGL/MGRP) if the UE is configured to measure on any carrier (e.g. inter-RAT) which always need gaps for performing the measurement. - Network can transform an already configured P-MGP into legacy MGP with same MGL/MGRP or vice versa without deconfiguring the P-MGP - Others

As discussed in section 2.2.2, pre-configured MG and legacy MG share the same configurations and NW flags whether the MG is used as pre-configured MG or not. This is aligned with the 2nd bullet of option 1.

The 1st bullet of option 1, if we understand correctly, means NW and UE can autonomously transform the pre-configured MG into legacy MG or vice versa. We do not prefer this approach as it is mixing the configuration/de-configuration with activation/deactivation of the pre-configured MG.

Proposal 6: Network can transform a pre-configured MG into legacy MG or vice versa with same MG configuration.

Activation and deactivation of pre-configured MG

Conditions

- The conditions to activate/de-activate the Pre-configured MG are: - BWP switching - For activation decision, MO configured can perform gapless measurement before BWP switching but not after BWP switching. For deactivation decision, MO configured can perform gapwith measurement before BWP switching but can not after BWP switching. - Other conditions are not precluded

Besides BWP switching, there could be also other RRC and MAC procedures that could trigger a change in need for MG and thus activation and deactivation of pre-configured MG, e.g.

- RRC (re)configuration of MO

  - When an MO is added or released, the need for MG may change. For example, all existing measurements do not require MG, but a newly added MO requires MG. In this case, the pre-configured MG should be activated.

- RRC (re)configuration of serving cells

  - When a serving cell is added or released, the need for MG may change. For example, an inter-frequency measurement may become intra-frequency measurement when a serving cell is added. In this case, the pre-configured MG should be deactivated.

- SCell activation and deactivation

  - When an SCell is activated or deactivated, the need for MG may change. For example, intra-frequency measurement on deactivated SCC does not require MG, but if the SCell is activated and the active BWP does not contain the SSB, MG would be required. In this case, the pre-configured MG should be activated.

For RRC reconfigurations, as discussed in section 2.2.3 for pre-configured MG configuration, our preference is to have NW to configure the status (activated or deactivated) instead of have NW and/or UE to perform an additional activation or deactivation.

For SCell activation and deactivation, there can be two options:

- Option A: SCell activation and deactivation is considered as a triggering event for activation and deactivation of pre-configured MG. Similar as option 3a in section 2.2.3, whether pre-configured MG activated or not depends on the need for MG from the configured measurements after the SCell activation or deactivation.

- Option B: NW indicates the status (activated or deactivated) of pre-configured MG. Similar as option 3b in section 2.2.3, NW could determine whether pre-configured MG should be activated or deactivated after the SCell activation or deactivation, and use a MAC CE to explicitly indicate UE the status of pre-configured MG.

Proposal 7a: RRC reconfigurations of MO or serving cell are not considered as triggering event for activation or deactivation of pre-configured MG, but the status of pre-configured MG should be configured by the NW.

Proposal 7b: For SCell activation and deactivation, FFS

- Option A: SCell activation and deactivation is considered as triggering event for activation or deactivation of pre-configured MG

- Option B: NW indicates the status of pre-configured MG after SCell activation and deactivation

Mechanism

- FFS on whether NW can fully control the pre-configured MG being activated/deactivated - FFS on how pre-configured MGs can be activated/deactivated: - Option 1 Autonomously/implicitly triggered by condition change - Option1a Autonomously triggered by BWP switching DCI/timer but explicitly directed according to the pre-configured ON/OFF state/bit for the target BWP - Option 2: If the use case of MG pattern change following BWP switch is considered, to activate/deactivate the pre-configured MG by the network indication - FFS on evaluation on MG activation/deactivation mechanism

There are basically two mechanisms for activation and deactivation of pre-configured MG:

- Implicit

In this way, when the triggering events, e.g. BWP switching, occurs, NW and UE will separately determine the status of the pre-configured MG based on the need for MG from configured measurements. There is no explicit signaling for activation and deactivation of pre-configured MG.

The benefit of implicit activation and deactivation is that it can be done fast without additional signaling delay, and it can also avoid the signaling overhead of explicit activation and deactivation indication.

The drawback is that NW and UE may have misalignment regarding the triggering event, e.g. whether BWP switching happened or not, and robustness may be an issue.

- Explicit

In this way, NW will indicate UE the status of the pre-configured MG, e.g. NW could determine whether pre-configured MG should be activated or deactivated when the triggering event happens, and explicitly indicate UE to activate or deactivate.

The benefit of explicit activation and deactivation is that NW will fully control the status so there would be no misalignment. The process can be similar as SCell activation and deactivation.

The drawback is that the explicit signaling will incur additional delay and overhead.

Considering the main triggering event is BWP switching, which can be rather dynamic, we slightly prefer the implicit approach.

Proposal 8: Pre-configured MG is implicitly activated and deactivated based on the triggering events.

Criteria

Activation and deactivation of the pre-configured MG should be based on the need for MG. As the need for MG actually comes from the measurements, to determine whether the pre-configured MG should be activated and deactivated, NW and UE should go through all the configured measurements and check if MG is required by any of them. If the MG is not required by any of the configured measurements, it means the MG is not needed so it can be deactivated, otherwise the MG is needed and it should be activated.

Based on Rel-15/16 requirements, the need for MG for a particular measurement is summarized in Table 1.

Table 1: Need for MG for a particular measurement

| Type of measurement | Cases where measurement does not require MG | Cases where measurement requires MG |
| --- | --- | --- |
| SSB based RRM measurement | Case 1: Intra-frequency measurement with SSB contained in the active BWP of the serving cell Case 2: Intra-frequency measurement with SSB not contained in the active BWP of the serving cell, and UE supports no-gap for the corresponding serving cell in NeedForGapsInfoNR signaling Case 3: Intra-frequency measurement on deactivated SCC Case 4: Inter-frequency measurement with SSB contained in the active BWP of the serving cell, UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the NW Case 5: Inter-frequency measurement, and UE supports no-gap for the corresponding target band in NeedForGapsInfoNR signaling | All other cases than Case 1~5 |
| CSI-RS based RRM measurement | Intra-frequency | Inter-frequency |
| Inter-RAT RRM measurement |  | All |
| PRS based positioning measurement |  | All |

Proposal 9: Adopt the following criteria for activation and deactivation of pre-configured MG:

- If MG is not required by any of the configured measurements, the MG is deactivated.

- If MG is required by one or more of the configured measurements, the MG is activated.

RRM requirements related to pre-configured MG

Activation and deactivation delay

- FFS on activation/deactivation delay - Option 1: No separated activation delay for the pre-configured MG activation/deactivation - Option 2: some transition time (T) shall be included in the pre-configured MG activation/deactivation time. - Option 2a - MG deactivation delay of [Y]ms is defined from the DCI of BWP switch is received to the earliest time instant when MG can be aborted. - MG activation delay of [X]ms is defined from the completion of target BWP activation to the earliest time instant when a MG can be entered. - No requirements shall be defined for the measurements within the gaps during [Y] and/or [X]ms - Option 2b The pre-configured delay includes BWP switch delay with additional validation time. The validation time is 20ms. - Others

Ideally, NW and UE would complete activation and deactivation of pre-configured MG at the same time when UE completes BWP switching. In the real world, however, both NW and UE may need some extra time to adapt parameters for data and measurement scheduling due to the change of the status of pre-configured MG, so it is reasonable to allow an extra time Delta on top of BWP switching delay, as shown in Figure 1. The value of Delta can be FFS.

Proposal 10: The delay of activation and deactivation of pre-configured MG is defined as BWP switching delay plus Delta. The value of Delta can be FFS.

Figure 1: Illustration of delay of activation and deactivation of pre-configured MG

In Figure 1, the MG occasion starts right after the activation and deactivation delay, so it is clear that the MG is deactivated from this MG occasion. However, there could be cases where an MG occasion overlaps with the activation and deactivation delay, as in Figure 2.

Figure 2: MG occasion overlapping with delay of activation and deactivation

In our view, it is difficult for NW and UE to change the status of the pre-configured MG from somewhere in the middle of an MG occasion because NW and UE may have already schedule data or measurement before based on the old status. Therefore, we propose that the activation and deactivation of pre-configured MG takes effect from the first MG occasion after the activation and deactivation delay.

Proposal 11: Activation and deactivation of pre-configured MG takes effect from the first MG occasion after the activation and deactivation delay.

Measurement period and number of transitions

- FFS on measurement period for the measurements with the pre-configured MGs - FFS on transitions between gapless and gap-based measurement procedures during ongoing measurements - Option 1: - RAN4 is to investigate the maximum number of transitions (N1,max) allowed for switching from gapless measurement procedure to gap-based measurement procedure during the ongoing measurement. - RAN4 is to investigate the maximum number of transitions (N2,max) allowed for switching from gap-based measurement procedure to gapless measurement procedure during the ongoing measurement - Option2 : No need

Following a BWP switch, a measurement may transition from MG based to MG-less or vice versa, so its measurement period may be impacted. The measurement period for other measurements may also be impacted, e.g. due to change in CSSF. It is noted that the scenario is not new and already exists in Rel-15, e.g. an intra-frequency SSB based measurement can change between MG based and MG less measurement. In Rel-15 a transition requirement is defined in 9.1.6 of 38.133.

### 9.1.6 Minimum requirement at transitions When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within gaps or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply. The carrier-specific scaling factor specified in clause 9.1.5 that applies to the other impacted measurement objects will also apply based on the longer measurement or cell identification delay before or after the transition. When the UE transitions between DRX and non-DRX or when DRX cycle periodicity changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition. Subsequent to this measurement period, the cell identification and measurement period requirements on each measurement object are corresponding to the second mode after transition.

The new aspect is that in Rel-15 the MG is always ON once configured, while the pre-configured MG can be activated and deactivated. This mainly impacts the availability of resources for data scheduling, but from measurement perspective there is no difference compared to Rel-15, so we see no reason why the transition requirements defined in Rel-15 could not apply, or why different requirements are needed.

Proposal 12: The transition requirements defined in clause 9.1.6 apply also with pre-configured MG.

In RAN4#98-bis-e, some companies proposed to define a limit on the maximum number of transitions between MG based and MG-less measurement during the measurement period. In our view, this is not well justified:

- Following the transition requirements in 9.1.6, the measurement period will be based on the longer one between the MG based and MG-less measurement, but it is not increasing with number of transitions.

- The transition between the MG based and MG-less measurement due to BWP switch already exists in Rel-15, and no problem has been identified without such a limit.

- It will create unnecessary restriction and complexity for the NW, e.g. the NW will need to maintain the exact measurement period for each measurement and make sure the number of BWP switches causing the transition does not exceed the limit.

Proposal 13: RAN4 not to define limit on the maximum number of transitions (between MG based and MG-less measurement) during the measurement period.

Scheduling restriction

- FFS on scheduling restriction during pre-configured MGs when not used - Option 1: If the UE is measuring without pre-configured gaps and no other frequency layer which needs gaps is configured then the UE can be scheduled during the pre-configured gaps while meeting existing scheduling restriction requirements defined in TS 38.133 - Option 1a: Existing scheduling restriction for RRM measurement without MG applies when pre-configured MG is deactivated

When pre-configured MG is deactivated, UE should be able to Tx/Rx data during the occasions of the pre-configured MG, and this is the exact motivation to support pre-configured MG. On the other hand, all the measurements should be performed without MG (otherwise the pre-configured MG should be activated), and the existing scheduling restriction defined for RRM measurement without MG should apply, as if MG is not configured.

In our view, option 1 and option 1a are rather similar, so we can support both them. We slightly prefer to use the wording in option 1a for more precise and accurate wording.

Proposal 14: Existing scheduling restriction for RRM measurement without MG applies when pre-configured MG is deactivated.

MGPs for pre-configured MG

Number of configurable MGs

- Number of pre-configured MG patterns: - A single pre-configured MG is considered for the case of non-concurrent MG scenarios. - FFS if more pre-configured MGs shall be considered for the multiple concurrent MG scenarios

In our view, without concurrent MGs, only one MG can be configured to the UE, no matter if it is used as legacy MG or pre-configured MG, or it is activated or deactivated. When more than one MGs are configured to the UE, it already requires NW and UE to work in the framework of concurrent MGs, e.g. to determine which MGs are used for which measurements.

Based on the WID, the joint working of pre-configured MG and concurrent MG can be discussed when requirements for each of them are stabilized. Considering the current status of each objective, we suggest to focus the discussion on single pre-configured MG for now.

Proposal 15: Defer the discussion on multiple pre-configured MGs with concurrent MGs when requirements for pre-configured MG and concurrent MGs are stabilized.

Candidate MGPs

- FFS on MG patterns used for the pre-configured MG mechanism - Option 1: The existing gap patterns (0~23) in Rel16 can be reused for the pre-configured MG. - Option 2: The existing gap patterns (0~25) in Rel16 can be reused for the pre-configured MG.

MG pattern #24 and #25 can only be used when UE needs them to perform PRS measurement, which based on Rel-16 requirements always requires MG. Based on discussions in section 2.1, NW can disable pre-configured MG when it configures MG pattern #24 or #25. This is effectively same as excluding MG pattern #24 and #25 from being used for pre-configured MG, but it is more future proof e.g. considering that MG-less PRS measurement might be introduced.

Proposal 16: The existing gap patterns (0~25) in Rel-16 can be reused for the pre-configured MG.

Conclusions

In this paper we provided our views on RRM requirements for pre-configured MG.

Proposal 1: No need to restrict the use of pre-configured MG with PRS or CSI-RS measurements. How to handle PRS and CSI-RS measurements with pre-configured MG can be left to NW and/or UE implementation.

Proposal 2: Pre-configured MGs is not additionally configured per BWP.

Proposal 3: On top of common configuration parameters as Rel-16 MG configuration, NW also configures the following new parameters for pre-configured MG

- A flag to indicate whether the MG is used as pre-configured MG or not

- Status (activated or deactivated) of the pre-configured MG

Proposal 4: Status (activated or deactivated) of pre-configured MG is not fixed after configuration. NW configures the status when pre-configured MG is configured.

Proposal 5: RAN4 to support ON/OFF of the pre-configured MG. Changing of MG configuration following a BWP switching is not considered.

Proposal 6: Network can transform a pre-configured MG into legacy MG or vice versa with same MG configuration.

Proposal 7a: RRC reconfigurations of MO or serving cell are not considered as triggering event for activation or deactivation of pre-configured MG, but the status of pre-configured MG should be configured by the NW.

Proposal 7b: For SCell activation and deactivation, FFS

- Option A: SCell activation and deactivation is considered as triggering event for activation or deactivation of pre-configured MG

- Option B: NW indicates the status of pre-configured MG after SCell activation and deactivation

Proposal 8: Pre-configured MG is implicitly activated and deactivated based on the triggering events.

Proposal 9: Adopt the following criteria for activation and deactivation of pre-configured MG:

- If MG is not required by any of the configured measurements, the MG is deactivated.

- If MG is required by one or more of the configured measurements, the MG is activated.

Proposal 10: The delay of activation and deactivation of pre-configured MG is defined as BWP switching delay plus Delta. The value of Delta can be FFS.

Proposal 11: Activation and deactivation of pre-configured MG takes effect from the first MG occasion after the activation and deactivation delay.

Proposal 12: The transition requirements defined in clause 9.1.6 apply also with pre-configured MG.

Proposal 13: RAN4 not to define limit on the maximum number of transitions (between MG based and MG-less measurement) during the measurement period.

Proposal 14: Existing scheduling restriction for RRM measurement without MG applies when pre-configured MG is deactivated.

Proposal 15: Defer the discussion on multiple pre-configured MGs with concurrent MGs when requirements for pre-configured MG and concurrent MGs are stabilized.

Proposal 16: The existing gap patterns (0~25) in Rel-16 can be reused for the pre-configured MG.

Reference

- R4-2105791, WF on R17 NR MG enhancements – Pre-configured MG, Intel