3GPP TSG-RAN WG4 Meeting #99-e		R4-2110955

E-meeting, 19 May – 27 May, 2021

Agenda item:	9.8.4.3

Source:	Intel Corporation

Title:	Discussion on the RRM requirements impact of FR2 HST

Document for:	Discussion

# Introduction

During RAN4 #98-bis-e meeting the following topics were kept FFS [1]

- Connection Mobility Control - RRC Release with Redirection - Connection Mobility control - RRC re-establishment - Restriction on SMTC periodicity - L1 measurements - Handover - RRC CONNECTED mode requirements for DRX - Scaling factor N - Cell identification - Intra-frequency measurement

In this paper we provide our views on the above-mentioned topics and provide some timing analysis for mentioned RRM requirements considering different assumptions on TSMTC, Rx scaling factor, TDRX.

# Discussion

RRC Release with Redirection

Redirection to another frequency is out of FR2 HST scope based on WID. In addition to that, the following agreement was made during the previous meeting regarding inter-frequency operation:

- Requirements on inter-frequency measurements:

  - Do not define inter-frequency measurements requirements for FR2 HST

Proposal 1: Do not define requirements for RRC Release with Redirection procedure

## Criterion for the RRM requirements definition

In order to make a decision on the changes which should be done for RRM requirements we need to identify the corresponding criterion which we should look at while defining whether the specific requirements are applicable for HST FR2 scenario or not.

Let’s consider an example of uni-directional Scenario-A shown in Figure 1. Due to a big delay of the measurement report, UE switches to another RRH much later than it should. Considering the dash line is the SNR threshold for operating with maximum throughput, there will be an area where the system will work under its capabilities providing performance degradation. Whether the delay is acceptable or not from the system performance point of view is not clear without exact criterion.

Figure 1. Performance degradation due to long L1 measurements.

During the deployment discussion the focus was made on defining the beam dwelling time – this is the parameter which is going to be forwarded to RRM group to be used for requirement definition. However, for example for L1-RSRP measurements, beam dwelling time metric doesn’t work – report should be sent long before we pass the beam coverage.

Observation 1: Usage of Beam dwelling time as a metric for RRM requirements definition may not be applicable

Proposal 2: RAN4 to define metric of applicability of the RRM requirements in FR2 HST. This metric can be defined as:

Option 1: Maximum performance degradation allowed (Max serving DL SNR degradation below threshold)

- Can be used at least for L1 measurements, intra-frequency cell identification, handover.

Option 2: other options are not precluded

## RRM requirements estimation

Below we present the timing analysis of the following requirements:

- L1 measurements

- Intra-frequency cell identification

- Handover

- RRC re-establishment

- L1 measurements

For L1-RSRP measurement period TS 38.133 defines the following:

For estimation we will consider the following:

- Treport is small enough to neglect

- M = 3, P = 1

- TSSB = 20ms

N=8:

| TDRX (ms) | No DRX | 20 | 40 | 60 | 80 | 160 | 256 | 320 | 512 | 640 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TL1-RSRP_Measurement_Period (ms) | 480 | 720 | 1440 | 2160 | 2880 | 5760 | 9216 | 11520 | 18432 | 23040 |
| Distance travelled (m) | 47 | 70 | 140 | 210 | 280 | 560 | 896 | 1120 | 1792 | 2240 |

Following proposal made in [2] we may change multiplier N from 8 to 1 considering only 1 Rx beam is used for L1 measurements

N=1:

| TDRX (ms) | No DRX | 20 | 40 | 60 | 80 | 160 | 256 | 320 | 512 | 640 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TL1-RSRP_Measurement_Period (ms) | 60 | 90 | 180 | 270 | 360 | 720 | 1152 | 1440 | 2304 | 2880 |
| Distance travelled (m) | 6 | 9 | 18 | 26 | 35 | 70 | 112 | 140 | 224 | 280 |

The obtained table may be used for defining the limits for DRX cycle duration. However, the criterion on which delay (column of the table) is acceptable in FR2 HST need to be introduced first.

Observation 2: Limits for DRX cycle duration need to be defined based on the maximum L1 measurements delay allowed for HST FR2 by the corresponding metric.

- Intra-frequency measurements

TS 38.133 defines the requirements for intra-frequency cell identification as

Where TPSS/SSS_sync_intra and TSSB_measurement_period_intra are defined as

For estimation we will consider the following:

- Treport is small enough to neglect

- Kp = Klayer1_measurement = 1

- CSSFintra = 1, TSSB = 20ms

- DRX cycle > SMTC period

MPSS/SSS_sync_w/o_gaps = MSSB_measurement_period_w/o_gaps = 24:

| TDRX (ms) | No DRX | 20 | 40 | 60 | 80 | 160 | 256 | 320 | 512 | 640 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tidentify_intra_without_index (ms) | 960 | 1440 | 2880 | 4320 | 5760 | 11520 | 18432 | 23040 | 36864 | 46080 |
| Distance travelled (m) | 93 | 140 | 280 | 420 | 560 | 1120 | 1792 | 2240 | 3584 | 4480 |

Following proposal made in [2] we may change multipliers MPSS/SSS_sync_w/o_gaps and MSSB_measurement_period_w/o_gaps from 24 to 3 considering only 1 Rx beam is used at the UE for intra-frequency measurements

MPSS/SSS_sync_w/o_gaps = MSSB_measurement_period_w/o_gaps = 3

| TDRX (ms) | No DRX | 20 | 40 | 60 | 80 | 160 | 256 | 320 | 512 | 640 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tidentify_intra_without_index (ms) | 120 | 180 | 360 | 540 | 720 | 1440 | 2304 | 2880 | 4608 | 5760 |
| Distance travelled (m) | 12 | 17.5 | 35 | 52.5 | 70 | 140 | 224 | 280 | 448 | 560 |

Observation 3: Limits for DRX cycle duration and SMTC periodicity need to be defined based on the max intra-frequency cell identification time period allowed by the corresponding metric

- Handover:

TS 38.133 defines Handover delay as Dhandover = TRRC_delay + Tinterrupt, where

For estimation we will consider the following:

- T∆ = Trs, Trs is TSMTC

- TIU,max = (160 + 10) ms

- Tprocessing = 20ms,	Tmargin = 2ms

- TRRC_delay =16ms

Tsearch = 8* Trs

| TSMTC, ms | 5 | 10 | 20 | 40 | 80 | 160 |
| --- | --- | --- | --- | --- | --- | --- |
| Dhandover, ms | 333 | 442 | 692 | 1192 | 2192 | 4192 |
| Distance traveled (m) | 32 | 43 | 67 | 116 | 213 | 408 |

Following proposal made in [2] we may change multiplier 8 to 1 considering only 1 Rx beam is used at the UE for cell search

Tsearch = 1* Trs

| TSMTC, ms | 5 | 10 | 20 | 40 | 80 | 160 |
| --- | --- | --- | --- | --- | --- | --- |
| Dhandover, ms | 228 | 232 | 272 | 352 | 512 | 832 |
| Distance traveled (m) | 22 | 23 | 26 | 34 | 50 | 81 |

Observation 4: Limits for SMTC periodicity need to be defined based on maximum handover delay allowed by the corresponding metric.

- Connection mobility control - RRC re-establishment:

TS 38.133 defines UE re-establishment delay as

For estimation we will consider the following:

- For FR2 HST Tidentify_inter_NR,i = 0

- TSI-NR < MIB periodicity + SIB1 periodicity + RRC procedure delay = 160ms + 80ms + 10ms =250ms

- TPRACH, max = 160ms

Tidentify_intra_NR = MAX(1000ms, 80*TSMTC):

| TSMTC, ms | 5 | 10 | 20 | 40 | 80 | 160 |
| --- | --- | --- | --- | --- | --- | --- |
| TUE_re-establish_delay, ms | 1460 | 1460 | 2060 | 3660 | 6860 | 13260 |
| Distance travelled (m) | 142 | 142 | 200 | 356 | 667 | 1289 |

Following proposal made in [2] we may change multiplier 80 to 20 considering only 2 Rx beams are used at the UE for RRC re-establishment

Tidentify_intra_NR = MAX(1000ms, 20*TSMTC):

| TSMTC, ms | 5 | 10 | 20 | 40 | 80 | 160 |
| --- | --- | --- | --- | --- | --- | --- |
| TUE_re-establish_delay, ms | 1460 | 1460 | 1460 | 1460 | 1460 | 2060 |
| Distance travelled (m) | 142 | 142 | 142 | 142 | 200 | 356 |

Observation 5: Restrictions on SMTC periodicity need to be defined based on maximum UE Re-establishment delay allowed by corresponding criterion

# Conclusion

In this paper we made the following proposals:

Proposal 1: Do not define requirements for RRC Release with Redirection procedure

Observation 1: Usage of Beam dwelling time as a metric for RRM requirements definition is not applicable

Proposal 2: RAN4 to define metric of applicability of the RRM requirements in FR2 HST. This metric can be defined as:

Option 1: Maximum performance degradation allowed (Max serving DL SNR degradation below threshold)

- Can be used at least for L1 measurements, intra-frequency cell identification, handover.

Option 2: other options are not precluded

Observation 2: Limits for DRX cycle duration need to be defined based on the maximum L1 measurements delay allowed for HST FR2 by the corresponding metric.

Observation 3: Limits for DRX cycle duration and SMTC periodicity need to be defined based on the max intra-frequency cell identification time period allowed by the corresponding metric

Observation 4: Limits for SMTC periodicity need to be defined based on maximum handover delay allowed by the corresponding metric.

Observation 5: Restrictions on SMTC periodicity need to be defined based on maximum UE Re-establishment delay allowed by corresponding criterion

# References

- R4-2105794 “WF on FR2 HST RRM requirements”, Nokia, Nokia Shanghai Bell, RAN4 #98-bis-e, April, 2021

- R4-2110954 “Discussion on the number of RX beams for FR2 HST”, Intel Corporation, RAN4 #99-e, May, 2021