3GPP TSG-RAN WG4 # 99-e	R4-2111022

# Electronic Meeting, May 19-27, 2021

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5788 | rev |  | Current version: | 14.18.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS36.101: NB-IoT FCC emission requirements (Rel-14) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14 |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To meet the additional requirements, NB-IoT devices shall exclude 100KHz at the band edges related with FCC emission requirements. The additional spurious emission at FCC license block edges shall be reflected in 3GPP spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introuduce the additional requiremtens in section 6.6.2F.2.3 - Modify definition of NS_04 in section 6.2.4F |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot pass FCC regulation conformance test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5F, 6.2.4F, 6.6.2F.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<< Start of changed sections>>>

<<< Unchanged Parts Skipped>>>

## 5.5F	Operating bands for category NB1 and NB2

Category NB1 and NB2 are designed to operate in the E-UTRA operating bands 1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 21, 25, 26, 28, 31, 66 and 70 which are defined in Table 5.5-1. Category NB1 and NB2 systems operate in HD-FDD duplex mode.

In case UE receives network signaling value NS_04 on operating band 26 then the lower limit of band 26 uplink is 814.1 MHz and lower limit of downlink is 859.1 MHz to account for the PLMR emission requirement.

In case UE receives network signaling value NS_04 on operating band 2, 3, 4, 5, 17, 25, 66, 71, 85, then the lower and upper 100kHz of uplink and downlink shall be excluded to account for FCC emission requirement.

In case UE receives network signaling value NS_04 on operating band 12 and 13, then the upper 100kHz of uplink and downlink shall be excluded to account for FCC emission requirement.

<<< Unchanged Parts Skipped>>>

### 6.2.4F	UE maximum output power with additional requirements for category NB1 and NB2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2.4F-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4F-1 are in addition to the allowed MPR requirements specified in subclause 6.2.3F-1.

Table 6.2.4F-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.6.2F.1 | Operating bands defined in 5.5F | N/A |
| NS_02 | 6.6.2F.2.1 | [1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 21, 25, 26, 28, 66 and 70] | [N/A] |
| NS_03 | 6.6.2F.2.2 | [1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 21, 25, 26, 28, 66 and 70] | [N/A] |
| NS_04 | 5.5F, 6.6.2F.2.3 | 2, 3, 4, 5, 12, 13, 17, 25, 26, 66, 71, 85 | N/A |

<<< Unchanged Parts Skipped>>>

6.6.2F.2.3            Minimum requirement (network signalled value "NS_04")

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2F.1-1

Table 6.6.2F.2-1: Additional requirements

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | NBIOT Mode |  |
| 775.9 ≤ f ≤ 776 | -13 dBm | 30 kHz |
| 787 ≤ f ≤ 787.1 | -13 dBm | 30 kHz |
| 662.9≤ f ≤ 663 | -13 dBm | 30 kHz |
| 697.9 ≤ f ≤ 698 | -13 dBm | 30 kHz |
| 703.9≤ f ≤ 704 | -13 dBm | 30 kHz |
| 716 ≤ f ≤ 716.1 | -13 dBm | 30 kHz |
| 1709.9 ≤ f ≤ 1710 | -13 dBm | 2 kHz |
| 1755 ≤ f ≤ 1755.1 | -13 dBm | 2 kHz |
| 1780 ≤ f ≤ 1780.1 | -13 dBm | 2 kHz |
| 1785 ≤ f ≤ 1785.1 | -13 dBm | 2 kHz |
| 1849.1≤ f ≤ 1850 | -13 dBm | 2 kHz |
| 1910 ≤ f ≤ 1910.1 | -13 dBm | 2 kHz |
| 1915 ≤ f ≤ 1915.1 | -13 dBm | 2 kHz |
| 823.9 ≤ f ≤ 824 | -13 dBm | 2 kHz |
| 849 ≤ f ≤ 849.1 | -13 dBm | 2 kHz |

<<< End of changed sections>>>